3GPP TSG-RAN WG3 Meeting #129-bis	R3-257072

Prague, Czech Republic, October 13th–17th, 2025

Title:	(TP to TP to TR 38.745) Support for multi-hop UE trajectory

Source:	Ericsson

Agenda item:	12.2.1

Document Type:	Discussion and agreement

# Introduction

The Study Item Description (SID) for “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN Phase 3” in Rel-20, found in RP-251868, states the following objectives:

The aim of this study item is to further investigate new AI/ML-based use cases and identify enhancements to support AI/ML functionality. The detailed objectives of the SI are as follows: - Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios: - multi-hop UE trajectory across gNBs; - intra-CU LTM; - handover enhancements, e.g. inter-CU LTM. NOTE: No UE impacts.

In this contribution, we discuss issues and solutions regarding support for multi-hop UE trajectory (see highlighted text above), focusing on both predictions and measurement collection.

In particular, we first provide a brief overview of the evolution of this topic, which originated in the Rel-17 SI “Study on enhancement for data collection for NR and ENDC”. We then examine the key challenges that prevented its specification in both Rel-18 and Rel-19, and we finally propose a new direction that we believe could enable RAN3 to successfully address this topic in Rel-20.

# Discussion on support for multi-hop UE trajectory

## History and new way forward

During the Rel-18 normative phase, RAN3 introduced two new features aimed at supporting AI/ML-based mobility optimization: predicted UE trajectory and measured UE trajectory. Both features are implemented as a list of cells that a UE is either predicted to traverse or has already traversed, similar in concept to the UE History Information. After extensive discussion, RAN3 decided to limit the scope of these features to a “single hop”. Specifically, a source gNB can provide, for a UE being handed over, a prediction of the UE’s trajectory in cells of the target gNB. Conversely, a target gNB of a handed-over UE can provide, to the former source gNB, the measured UE trajectory in its own cells. This limitation was introduced to ensure a procedure that was both simple and robust, thereby enabling timely completion of the WI.

Some of the reasons why multi-hop UE trajectory did not advance in Rel-18 stem from two fundamental challenges, as depicted in Figure 1. First, the source gNB for a handed-over UE may not have an Xn connection to all relevant future target gNBs, preventing it from directly (i.e., without intermediaries) signaling the trajectory prediction or collecting the measured trajectory across multiple nodes. Second, while the UE is identified between the source gNB and the immediate target gNB, its identity is unknown to the source gNB in subsequent handovers; hence, it is not clear how to identify a UE in a trajectory prediction or in the measured trajectory, in case of a direct signaling between the first gNB and any future hop.

Observation 1:  A gNB cannot collect measured multi-hop UE trajectory beyond the first hop directly (i.e., without intermediaries) using the Data Collection framework developed in Rel-18.

Figure 1: Challenges with multi-hop UE trajectory.

Due to support from multiple companies, RAN3 agreed to revisit the topic in Rel-19 with the goal of enabling both predicted and measured multi-hop UE trajectory. To address the described challenges in Rel-18, several companies proposed a “hop-by-hop” approach during Rel-19. In this approach, the trajectory prediction is piggybacked forward as the UE moves, while the measured trajectory is propagated backward along the UE’s path as depicted in Figure 2. However, this approach introduces significant complexity. For measured UE trajectory reporting, it requires that all gNBs along the path have compatible Data Collection contexts configured (e.g., step 4 in Figure 2 needs to be in place just like with step 1). More problematic is the fact that intermediate nodes must store locally measured UE trajectory until exit conditions are triggered in the final node, at which point the information cascades backward; in Figure 2, gNB2 needs to store the UE trajectory before step 6, when the UE leaves the node, until it receives gNB3’s trajectory in step 8. Furthermore, these issues become more severe as more nodes are involved. The storing of information includes also UE context storing, which to a certain extent needs to be in place to be able to associate the information reported from a target gNB with the UE for which the measured trajectory monitoring was triggered.

Figure 2: Rel-19 "hop-by-hop" approach for measured multi-hop UE trajectory. Highlighted IEs with matching colors indicate identical information.

One of the main drawbacks of the “hop-by-hop” approach, though, is that intermediate gNBs must set up Data Collection contexts with multiple neighboring nodes, even if they do not intend to collect data themselves. This is because the Data Collection context must exist before handover, as the HANDOVER REQUEST message includes the Data Collection ID. A gNB cannot wait until handover is imminent to initiate the Data Collection Reporting Initiation procedure with the relevant target gNB. Instead, it must preemptively start the procedure with several neighbors. This leads to a cascading effect: if one gNB requests measured multi-hop UE trajectory, its neighbors must forward the request to their own neighbors, and so on. As a result, even though the “hop-by-hop” approach can technically be implemented using the Rel-18 Data Collection framework, it is not a technically sound solution due to its inherent complexity and inefficiency.

Observation 2:  The “hop-by-hop” approach proposed in Rel-19 for multi-hop UE trajectory is incompatible with the Rel-18 Data Collection framework because it requires intermediate gNBs to preemptively set up contexts with multiple neighbors, leading to unnecessary complexity and cascading configuration overhead.

Proposal 1:  RAN3 to agree that the “hop-by-hop” approach proposed in Rel-19 for multi-hop UE trajectory is incompatible with the Rel-18 Data Collection framework.

Given the difficulties encountered with the “hop-by-hop” approach, it is clear that repeating the same discussions in Rel-20 would likely lead to the same lack of consensus. A new approach (which is compatible with the Rel-18 Data Collection framework) is thus needed, one that avoids the original problems (i.e., lack of Xn connectivity and UE identification beyond first hop) and instead leverages a scenario where these issues are inherently mitigated.

Observation 3:  In order to avoid a deadlock in the discussion of multi-hop UE trajectory as in Rel-19, a new approach is needed.

Fortunately, the two previously described issues (i.e., lack of Xn connectivity and UE identification beyond first hop) are not present if multi-hop UE trajectory is considered in the context of inter-gNB (subsequent) LTM rather than legacy L3 mobility procedures, as depicted in Figure 3. In inter-gNB LTM, the source gNB has an Xn connection to all candidate gNBs (i.e., gNBs that serve the LTM candidate cells), and the identity of the UE is unique and known to all of them. Moreover, it is the source gNB that configures all subsequent gNBs in an LTM mobility trajectory, making it logical and feasible to collect measured multi-hop UE trajectory and even generate multi-hop UE trajectory predictions. This is in contrast to legacy L3 handover, where the new serving gNB (i.e. a node that serves the UE “in the middle” of the UE trajectory) decides on future mobility configurations, making it unclear why the original source gNB would need to collect information about future hops.

Observation 4:  In an inter-gNB (subsequent) LTM scenario, the issues affecting collection of measured multi-hop UE trajectory beyond the first hop directly with the Data Collection framework are not present.

Figure 3: The procedures developed for inter-gNB LTM solve the previous problems with multi-hop UE trajectory. The yellow highlighted IE, which is shared among the nodes, allows for the unique identification of the UE.

We thus propose that multi-hop UE trajectory (both prediction and measurement) should be only considered in the context of inter-gNB (subsequent) LTM scenarios, explicitly excluding other mobility types such as legacy L3 handover. This approach avoids the complexities associated with the solutions proposed in Rel-19 for legacy mobility procedures and offers a way forward for RAN3 to define a simpler, more robust, and (most importantly) more useful framework for multi-hop UE trajectory.

Proposal 2:  RAN3 to consider multi-hop UE trajectory only in inter-gNB (subsequent) LTM scenarios.

## Usefulness of multi-hop UE trajectory for inter-gNB LTM

The usefulness of multi-hop UE trajectory is evident when considered in the context of inter-gNB (subsequent) LTM. In these mobility scenarios, the source gNB is responsible for configuring multiple candidate cells, potentially across multiple nodes, by means of the Handover Preparation procedure, which creates a clear need for multi-hop trajectory information at the source gNB. By collecting measured multi-hop UE trajectory, the source gNB can train (or re-train) AI/ML models that predict UE mobility patterns across multiple nodes. These models can then be used to generate predictions and optimize LTM configurations, directly improving mobility performance and service continuity. Since the LTM configuration is determined by the first gNB across multiple hops, having access to multi-hop UE trajectory (both measured and predicted) at that node is clearly beneficial.

Observation 5:  In an inter-gNB (subsequent) LTM scenario, the LTM configuration is determined by the first gNB across multiple hops.

In contrast, multi-hop UE trajectory does not provide the same benefits in legacy L3 handover scenarios. This is because each handover is configured independently by the serving gNB at each hop, meaning that once the UE leaves the first gNB, subsequent decisions may be made without considering any predictions from that node. As a result, the first gNB has no practical reason to collect multi-hop UE trajectory and train AI/ML models that provide predictions across multiple gNBs, since it cannot influence the UE’s mobility beyond the initial handover. The “single hop” nature of legacy L3 handover preparation thus limits the usefulness of multi-hop UE trajectory collection at the source gNB.

Observation 6:  In a legacy L3 handover scenario, each handover is configured independently by the serving gNB at each hop. This implies that collection of a multi-hop UE trajectory at the first source gNB is not useful as the first source gNB has no influence on mobility after the UE leaves the first source gNB.

During Rel-19, some companies argued that multi-hop UE trajectory prediction could still be beneficial in legacy L3 handover, particularly for fast-moving UEs. The idea is that the first gNB, potentially equipped with more advanced AI/ML capabilities, could assist subsequent gNBs by forwarding trajectory predictions. However, this argument has a critical flaw: if the first gNB is capable of accurately predicting the UE’s trajectory, it should configure the UE for inter-gNB (subsequent) LTM instead. LTM is the preferred mobility option for fast-moving UEs due to its lower service interruption and better continuity. Relying on legacy L3 handover and attempting to forward predictions introduces unnecessary complexity and inefficiency, especially when a more suitable and more useful mobility framework is available.

Proposal 3:  RAN3 to exclude legacy L3 handover scenarios for multi-hop UE trajectory.

Proposal 4:  RAN3 to agree to the TP to TR 38.745 in Appendix A.

## Measured multi-hop UE trajectory

A simplified flow diagram with the most relevant messages or events for collection of measured multi-hop UE trajectory in an inter-gNB LTM scenario is shown in Figure 4 below.

Figure 4: Flow diagram for collection of measured multi-hop UE trajectory.

The main steps of the procedure are as follows:

- gNB1 sends a DATA COLLECTION REQUEST message to gNB2 with a request for measured UE trajectory, and gNB2 responds with a DATA COLLECTION RESPONSE message accepting the request. The values for NG-RAN node1 Measurement ID IE and NG-RAN node2 Measurement ID IE obtained in this step are called “Data Collection ID 1” for simplicity.

- gNB1 sends a DATA COLLECTION REQUEST message to gNB3 with a request for measured UE trajectory, and gNB3 responds with a DATA COLLECTION RESPONSE message accepting the request. The values for NG-RAN node1 Measurement ID IE and NG-RAN node2 Measurement ID IE obtained in this step are called “Data Collection ID 2” for simplicity.

- gNB1 decides to configure a UE with LTM towards cells served by gNB2 and by gNB3.

- gNB1 requests LTM for one or more candidate cells belonging to gNB2; for each candidate cell, it initiates a HANDOVER REQUEST message where it includes the “Data Collection ID 1” to indicate the request of measured UE trajectory for this UE. Although not shown in the flow diagram, gNB2 responds with a HANDOVER REQUEST ACKNOWLEDGE message for each accepted candidate cell. The source NG-RAN node UE XnAP ID and target NG-RAN node UE XnAP ID between gNB1 and gNB2 are the same in all exchanged messages.

- gNB1 requests LTM for one or more candidate cells belonging to gNB3; for each candidate cell, it initiates a HANDOVER REQUEST message where it includes the “Data Collection ID 2” to indicate the request of measured UE trajectory for this UE. Although not shown in the flow diagram, gNB3 responds with a HANDOVER REQUEST ACKNOWLEDGE message for each accepted candidate cell. The source NG-RAN node UE XnAP ID and target NG-RAN node UE XnAP ID between gNB1 and gNB3 are the same in all exchanged messages.

- gNB1 initiates LTM towards gNB2, sends the cell switch command to the UE, and sends the CELL SWITCH NOTIFICATION message to gNB2. After the UE successfully accesses the cell in gNB2, gNB2 initiates the LTM completion phase where, among other steps, it sends an LTM CONFIGURATION UPDATE message to other candidate gNBs (i.e., gNB3).

- With the UE being served by one of its cells, gNB2 starts collecting the measured UE trajectory.

- gNB2 determines to initiate LTM towards gNB3, sends the cell switch command to the UE, and sends the CELL SWITCH NOTIFICATION message to gNB3. After the UE successfully accesses the cell in gNB3, gNB3 initiates the LTM completion phase where, among other steps, it sends an LTM CONFIGURATION UPDATE message to other candidate gNBs (i.e., gNB2).

- Since the UE is no longer being served by gNB2, gNB2 sends a DATA COLLECTION UPDATE message to gNB1 with the measured UE trajectory in cells of gNB2. The UE Assistant Identifier IE linked to the measured UE trajectory is the source NG-RAN node UE XnAP ID signaled in step 4.

- With the UE being served by one of its cells, gNB3 starts collecting the measured UE trajectory.

- An exit condition for collection of measured UE trajectory is reached, e.g., the UE goes to idle or moves to a cell that is not part of the LTM configuration determined by gNB1. gNB3 thus terminates the measured multi-hop UE trajectory collection.

- gNB3 sends a DATA COLLECTION UPDATE message to gNB1 with the measured UE trajectory for cells of gNB3. The UE Assistant Identifier IE linked to the measured UE trajectory is the source NG-RAN node UE XnAP ID signaled in step 5.

As it is seen in the described steps, the legacy Data Collection framework allows the collection of measured multi-hop UE trajectory almost without any change. Only two main enhancements are needed:

- Modification to the start and exit conditions for collection of measured UE trajectory to incorporate LTM.

- Addition of signaling between gNB2 and gNB3, e.g., as part of the CELL SWITCH NOTIFICATION message, where gNB2 indicates the amount of time and the number of cells the UE was in gNB2. This is to limit the collection of the whole multi-hop UE trajectory and not only per node.

We believe that these enhancements have a very limited impact while enabling the collection of measured multi-hop UE trajectory. Therefore, RAN3 should consider them as a preferred solution.

Observation 7:  The enhancements needed to enable the collection of measured multi-hop UE trajectory in inter-gNB LTM scenarios have very limited impact in the specifications.

Proposal 5:  RAN3 to agree to the collection of measured multi-hop UE trajectory in inter-gNB LTM scenarios.

## Predicted multi-hop UE trajectory

As previously described, the main benefit of predicted multi-hop UE trajectory lies in enabling a more accurate selection of candidate cells for LTM configuration. This information is used internally by the gNB responsible for configuring LTM for the UE, so there is no need to signal predicted multi-hop UE trajectory to other gNBs.

However, in the inter-gNB LTM scenarios we propose to focus on, the signaling of predicted multi-hop UE trajectory to multiple gNBs comes at no additional cost. As shown in Figure 4, gNB1 sends HANDOVER REQUEST messages to both gNB2 and gNB3 (steps 4 and 5, respectively), which may already include the Cell Based UE Trajectory Prediction IE. This IE can be trivially reused to signal predicted multi-hop UE trajectory.

Observation 8:  The current specifications already support the signaling of predicted multi-hop UE trajectory in inter-gNB LTM scenarios.

Proposal 6:  RAN3 to agree to the signaling of predicted multi-hop UE trajectory in inter-gNB LTM scenarios given there is no specification impact.

# Conclusions

Based on the discussion in this paper, we derived the following observations and proposals:

Observation 1:  A gNB cannot collect measured multi-hop UE trajectory beyond the first hop directly (i.e., without intermediaries) using the Data Collection framework developed in Rel-18.

Observation 2:  The “hop-by-hop” approach proposed in Rel-19 for multi-hop UE trajectory is incompatible with the Rel-18 Data Collection framework because it requires intermediate gNBs to preemptively set up contexts with multiple neighbors, leading to unnecessary complexity and cascading configuration overhead.

Proposal 1:  RAN3 to agree that the “hop-by-hop” approach proposed in Rel-19 for multi-hop UE trajectory is incompatible with the Rel-18 Data Collection framework.

Observation 3:  In order to avoid a deadlock in the discussion of multi-hop UE trajectory as in Rel-19, a new approach is needed.

Observation 4:  In an inter-gNB (subsequent) LTM scenario, the issues affecting collection of measured multi-hop UE trajectory beyond the first hop directly with the Data Collection framework are not present.

Proposal 2:  RAN3 to consider multi-hop UE trajectory only in inter-gNB (subsequent) LTM scenarios.

Observation 5:  In an inter-gNB (subsequent) LTM scenario, the LTM configuration is determined by the first gNB across multiple hops.

Observation 6:  In a legacy L3 handover scenario, each handover is configured independently by the serving gNB at each hop. This implies that collection of a multi-hop UE trajectory at the first source gNB is not useful as the first source gNB has no influence on mobility after the UE leaves the first source gNB.

Proposal 3:  RAN3 to exclude legacy L3 handover scenarios for multi-hop UE trajectory.

Proposal 4:  RAN3 to agree to the TP to TR 38.745 in Appendix A.

Observation 7:  The enhancements needed to enable the collection of measured multi-hop UE trajectory in inter-gNB LTM scenarios have very limited impact in the specifications.

Proposal 5:  RAN3 to agree to the collection of measured multi-hop UE trajectory in inter-gNB LTM scenarios.

Observation 8:  The current specifications already support the signaling of predicted multi-hop UE trajectory in inter-gNB LTM scenarios.

Proposal 6:  RAN3 to agree to the signaling of predicted multi-hop UE trajectory in inter-gNB LTM scenarios given there is no specification impact.

# Appendix A – TP to TR 38.745

# 4	Use cases and Solutions

## 4.1	Multiple-hop UE trajectory

### 4.1.1	Use case description

Editor’s Note: Capture the description of use case

Efficient mobility optimization is essential in mobile networks to ensure seamless service continuity and optimal resource allocation as UEs move across cells and gNBs. AI/ML-based methods are powerful tools that can improve mobility decisions, particularly using trajectory predictions and UE performance forecasting. These methods rely on accurate and comprehensive measurement data to train and validate AI/ML models.

Multi-hop UE trajectory refers to the ability to predict and/or measure the path of a UE across multiple gNBs during a series of connected-mode mobility events. This capability is particularly relevant in inter-gNB L1/L2 Triggered Mobility (LTM) scenarios, where a source gNB is responsible for configuring multiple candidate cells across potentially different gNBs. In such cases, access to measured multi-hop UE trajectory data enables the source gNB to train and update AI/ML models, that reflect UE movement patterns across several hops, and to use those models to optimize LTM configurations. This results in improved mobility performance and service continuity, especially for fast-moving UEs.

In contrast, legacy L3 handover scenarios do not benefit from multi-hop UE trajectory in the same way. Each handover is configured independently by the serving gNB at each hop, and once the UE leaves the first gNB, subsequent decisions are made without reference to any predictions or measurements from that node.

Therefore, multi-hop UE trajectory is best suited for inter-gNB LTM scenarios, due to the intrinsic multi-hop nature of inter-gNB LTM.

### 4.1.2	Solutions and standard impacts

Editor’s Note: Capture the solutions for the use case, including potential standard impacts on existing Nodes, functions, and interfaces

#### 4.1.2.1	Locations for AI/ML Model Training and AI/ML Model Inference

The following solutions can be considered for supporting multi-hop UE trajectory for AI/ML-based mobility optimization:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB.

-	AI/ML Model Training and AI/ML Model Inference are both located in the gNB.

In case of CU-DU split architecture, the following solutions are possible:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.