3GPP TSG-RAN WG3 #129-bis																	R3-257073

Prague, Czech Republic, 13-17 October 2025

Title: 	(TP to TR 38.745) AI/ML based mobility – Intra-CU LTM

Source: 	Ericsson

Agenda item:	12.2.2

Document Type:	Discussion and Decision

# 1. Introduction

The detailed SID objectives of Study on AIML for NG-RAN Phase 3 (RP-252867) for further investigation of new AI/ML-based use cases and identification of enhancements to support AI/ML functionality are listed as follows:

- Study the AI/ML-based mobility use case based on the principles of AI/ML for NG-RAN as captured in TS 38.300 and TS 38.401 with existing NG-RAN interfaces and architecture, including the following scenarios:

  - multi-hop UE trajectory across gNBs;

  - intra-CU LTM;

  - handover enhancements, e.g. inter-CU LTM.

NOTE: No UE impacts.

In this contribution we initiate a discussion related to AI/ML based mobility enhancements for intra-CU LTM.

# 2. Discussion

## 2.1 Overview

As clearly described in the SI objectives on AI/ML for NG-RAN Phase 3, the principles agreed in previous releases for AI/ML for NG-RAN should be maintained.  A direct consequence of this is, in our view, that, when studying the proposed AI/ML-based mobility use case, the agreed principle related to AI/ML model deployment scenarios should be respected. In particular, for AI/ML model deployed in the RAN: “AI/ML Model Training and Model Inference are both located in the gNB-CU.” (TS 38.401).

Proposal 1: For AI/ML based intra-CU LTM, reuse the defined AI/ML framework for 5G. For a split gNB in particular, if the AI/ML model is deployed in the RAN, the Model inference is located in the gNB-CU.

Another aspect that we think is important, to reduce the effort in future normative work, is to reuse as much as possible the already defined procedures for collecting data, namely the Data Collection Reporting Initiation and Data Collection Reporting procedures, and extend them if needed.

Proposal 2: For AI/ML based intra-CU LTM, reuse the existing E1 Data Collection Reporting Initiation and Data Collection Reporting procedures.

Support for Intra-CU LTM mobility has been specified in Rel-18. With this type of mobility, a UE receives from a source gNB the RRC configuration of one or more LTM candidate cells, and it can move across multiple LTM candidate cells (subsequent LTM) without being reconfigured over the air. The intra-CU LTM includes the scenarios of intra-gNB-DU LTM, inter-gNB-DU LTM, and LTM change of CU-UP.

At the start of LTM preparation, there is a decision taken by the gNB-CU, which, based on UE reported L3 measurements, decides the potential candidate cells for LTM. If the gNB-DU(s) serving the candidate cells accept(s) to prepare a cell as LTM candidate, the configuration is prepared and sent to the UE. During the preparation, early synchronization in UL can be executed, and the Timing Advance for the UE is retrieved for a candidate cell, which enables RACH-less mobility. If the candidate cell is in a gNB-DU different from the source gNB-DU, the TA value is transferred from candidate gNB-DU to source gNB-DU via the gNB-CU.

Due to the complexity of LTM, a big effort was made in RAN3 to support it, extending existing F1AP procedures (UE Context Setup / Modification), to prepare the LTM configuration for the UE, and introducing new procedures for various purposes, such as transferring TA values acquired from the candidate cell (DU-CU / CU-DU TA Information Transfer), letting the gNB-CU trigger early synchronization or LTM cell switch execution (CU-DU Mobility Initiation), or to inform a candidate gNB-DU that LTM Cell switch execution is triggered (DU-CU / CU-DU Cell Switch Notification).

It is also worth mentioning that in Rel-19, the intra-gNB LTM mobility has been extended with support of Intra-CU Conditional LTM (CLTM). In case of CLTM, the UE is configured with execution conditions. Upon fulfillment of such conditions, the UE triggers CLTM execution. We note that, according to RAN2 signaling design, when a UE is configured for CLTM, i.e., when a certain cell can be used as target for CLTM execution, the gNB can also trigger LTM towards the same cell, i.e., a CLTM candidate cell is also an LTM candidate cell. Due to the coexistence of LTM and CLTM in case of intra-gNB mobility, potential optimizations introduced with AI/ML for intra-CU LTM could try to take also CLTM into account. However, we think that “pure” intra-CU LTM should be prioritized, in line with the agreed SI scope.

Proposal 3: For AI/ML based intra-CU LTM, the focus is on non-conditional LTM. CLTM aspects may be considered, if time allows.

## 2.1 AI/ML-based intra-CU LTM

In this section, we discuss some aspects for which we think it can be beneficial to support intra-CU LTM with AI/ML.

As described in the previous section, in the LTM preparation phase, the gNB-CU decides to use certain cells as potential candidate cells. This decision is currently based purely on radio measurements. On the other hand, if a UE is served by cells of the gNB-CU, (e.g., the UE moves among LTM candidate cells of different gNB-DUs, within the same gNB-CU), the gNB-CU can collect some UE performance by itself (UE throughput) and use existing procedures to request and to receive some other UE performance metrics (e.g., Packet loss, Packet Delay).

A possibility is then that, when an LTM Cell switch execution is triggered, the gNB-CU indicates to the new serving gNB-DU that it should collect and report UE performance in the new serving cell. As an example of how this could be achieved, we could think of using a similar approach as in previous normative work for AI/ML mobility use case, where, in case the target gNB receives a Data Collection ID in the HANDOVER REQUEST message, the target gNB understands that it should collect and then report to the source gNB the requested UE performance. The LTM Cell switch execution can be initiated by the gNB-CU (CU-DU LTM MOBILITY INITATION REQUEST), or by the gNB-DU (in which case the CU receives a DU-CU CELL SWITCH NOTIFICATION). Either way, the gNB-CU can request and obtain UE performance in the selected candidate cell. This information can be used by the gNB-CU to predict an optimal set of LTM candidate cells.

Proposal 4: For intra-CU LTM preparation, the gNB-CU can predict the cells to be used as LTM candidate.

If we now consider the LTM execution phase, this includes two logical steps: 1) the LTM Cell Switch Command is sent to the UE, and 2) the candidate gNB-DU is informed that the UE is coming (to a certain cell). When the execution is done, the target gNB-DU needs to accommodate some resources (e.g., Configured Grant) to serve the UE. One possibility to optimize these resources at the target can be that the gNB-CU sends a predicted timing for upcoming Cell Switch execution, and the target gNB-DU can take it into account to reduce the amount of allocated resources.

Proposal 5: For intra-CU LTM execution, the gNB-CU can send predictions to a target gNB-DU on when a cell switch will occur.

Also, the execution of LTM can be RACH-less or RACH based, depending on whether a valid TA is available or not. Therefore, if the gNB-CU can predict (e.g., based on radio measurements) whether the TA for a certain candidate cell will be outdated soon, it can send this information to the source gNB-DU, or trigger the gNB-DU to initiate an early UL synchronization, so that the chances of using RACH-less LTM are improved.

Proposal 6: To support RACH-less intra-CU LTM execution, the gNB-CU can send predictions to the source gNB-DU on when early UL synchronization will be needed.

In previous releases, one aspect discussed in the context of AI/ML support for mobility concerned the definition of exit conditions, stipulating when a gNB requested to provide certain UE performance, should terminate the collection, and report the results. A similar discussion is applicable to intra-CU LTM as well, and additional exit conditions should be discussed. For example, if there is a failure in LTM Cell Switch execution, this can be considered as a new exit condition applicable to the intra-CU LTM case.

Proposal 7: If UE performance is collected for intra-CU LTM, RAN3 should discuss whether and which exit conditions should be specified.

Finally, we propose to consider, as part of the AI/ML based intra-CU LTM discussion, the aspect of cancelling a certain prediction. The need for this type of support became relevant in recent discussions on CCO, where the gNB-CU sends predictions to the gNB-DU. Assuming that the existing AI/ML framework is reused, which, in the context of intra-CU LTM means that the gNB-CU can send predictions to a gNB-DU (source gNB-DU or target gNB-DU), we think RAN3 should discuss the aspect of cancelling predictions for this AI/ML-based use case as well.

Proposal 8: For AI/ML based intra-CU LTM, in case of gNB-CU sending predictions to a gNB-DU, RAN3 to discuss whether and how to cancel the predictions.

# 3. Conclusion

# In this contribution, the following observations and proposals are made:

Proposal 1: For AI/ML based intra-CU LTM, reuse the defined AI/ML framework of data. For a split gNB in particular, if the AI/ML model is deployed in the RAN, the Model inference is located in the gNB-CU.

Proposal 2: For AI/ML based intra-CU LTM, reuse the existing E1 Data Collection Reporting Initiation and Data Collection Reporting procedures.

Proposal 3: For AI/ML based intra-CU LTM, the focus is on non-conditional LTM. CLTM aspects may be considered, if time allows.

Proposal 4: For intra-CU LTM preparation, the gNB-CU can predict the cells to be used as LTM candidate.

Proposal 5: For intra-CU LTM execution, the gNB-CU can send predictions to a target gNB-DU on when a cell switch will occur.

Proposal 6: To support RACH-less intra-CU LTM execution, the gNB-CU can send predictions to the source gNB-DU on when early UL synchronization will be needed.

Proposal 7: If UE performance is collected for intra-CU LTM, RAN3 should discuss whether and which exit conditions should be specified.

Proposal 8: For AI/ML based intra-CU LTM, in case of gNB-CU sending predictions to a gNB-DU, RAN3 to discuss whether and how to cancel the predictions.

Proposal 9: Agree to the pCR to TR 38.745 in Appendix A.

# Appendix A – pCR to TR 38.745

# 4	Use cases and Solutions

## 4.2	AI/ML assisted Intra-CU LTM

### 4.2.1	Use case description

Editor’s Note: Capture the description of use case

In Rel-18, support for intra-CU LTM is introduced. LTM preparation and LTM execution can be optimized by means of AI/ML.

AI/ML assisted intra-CU LTM is studied by assuming inference at the gNB-CU only.

AI/ML assisted intra-CU LTM aims at the following objectives:

- optimal set of LTM candidate cells

- reduction in LTM execution failures,

- increased use of RACH-less LTM

- improved resource utilization at the source and LTM candidate cell