3GPP TSG-RAN WG3 Meeting #129bis	R3-257080

Prague, Czech Republic, 13 – 17 October, 2025

Agenda item:	9.2.8

Source:	Nokia

Title:	Discussion on ANR for OD-SIB1

Document for:	Discussion and Decision

# 1	Introduction

In light of introduction of OD-SIB1, we here provide discussion on use of the ANR (Automatic Neighbour Relation) feature in networks in networks that enable this new Rel-19 feature.

# 2	Discussion

RAN3 had earlier agreed that gNB-NES will send the OD-SIB1 related information to gNB-Cell A for it to broadcast this information in SIB26. However, it is not clear currently, how the gNB-NES candidate identifies gNB-Cell A’s and/or cells that can provide the anchor cell functionality. There are 2 options for the gNB-NES to know this information.

- OAM based solution – here, it may be assumed that the gNB-NES is configured by the OAM the list of cell As in all its neighboring gNBs.

- XnAP based solution – here, the gNB-Cell A indicates support of the anchor cell functionality, e.g. at a cell granularity.

An important principle for the NG-RAN is that the operator shall be able to set up a network based on auto-discovery of neighbours (Automatic Neighbour Relation function), including automated Xn setup. This should be done preferably without OAM configured information related to neighbour nodes. The OAM-based solution will therefore in our view break this principle.

Furthermore, the Cell A support in a cell may depend on different factors, e.g. related to energy considerations, so should in our view be considered as a kind of information that is suitable for transmission by signalling and not hard-coded in an OAM configuration. We therefore prefer the XnAP based solution. Also, since this support is at the cell level, we propose to add a new flag in the Served Cell Information NR to indicate if a given cell can provide anchor cell functionality.

Proposal: Add a new flag in the IE Served Cell Information NR to indicate whether a given cell is available to provide anchor cell functionality.

# 3	Conclusion

We have made the following observations and proposals:

Proposal: Add a new flag in the IE Served Cell Information NR to indicate whether a given cell is available to provide anchor cell functionality.

We have submitted a corresponding CR to this meeting in R3-257081.