3GPP TSG-RAN WG3 Meeting #129bis	R3-257083

Prague, Czech Republic, 13 – 17 October 2025

Agenda item:	8.1

Source:	Nokia

Title:	Handling of average window for alternative QoS

WID/SID:	TEI18

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#129bis RAN3 has received the following LS from SA2 in [1] to decide how to select and use the averaging window when AQP are present in the NG-RAN:

SA2 thanks for the LS from the RAN3, SA2 has the following answer for the LS:

- Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

- Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

RAN3 could not reach consensus on introducing averaging window in the alternative QoS Profile, and respectfully requests SA2 to provide clarification on introducing averaging window in the alternative QoS Profile.

[SA2 Answer:] SA2 has discussed the LS and there are other understandings beyond Understanding 1 and 2. There is no consensus on how to select and the impacts of using the averaging window of current activated QoS Profile by NG-RAN. SA2 would like to ask RAN3 to decide how to select and use the averaging window when AQP are present in the NG-RAN.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to decide how to select and use the averaging window when AQP are present in the NG-RAN.

Basically, the question is whether the average window used for an alternative QoS profile should be the same of the average window used for the nominal QoS profile or whether RAN3 wants to enable a different one.

This paper provides the background behind this question and a way forward.

# 2	Discussion

Background

The topic started with evidence that SA2 and RAN3 specifications diverge on which average window should be used by gNB when evaluating an alternative QoS profile, namely:

- In SA2 specifications (stage 2 TS 23.501), one dedicated average window can be specified per QoS profile,

- In RAN3 specification (stage 3 specification TS 38.413), one average window is common for all QoS profiles.

RAN3 already discussed this issue at RAN3#127bis and did not agree to align with SA2 specification after evaluating gain versus pain. Moreover, RAN3 concluded that in general stage 2 aligns with stage 3 for frozen releases and not the opposite. RAN3 sent an LS to SA2 in this direction at RAN3#127bis in tdoc R3-252397 [2]:

Companies in RAN3 discussed whether to include averaging window information in the Alternative QoS Parameter set and there are two different understandings:

- Understanding 1: NG-RAN node will re-use the same value as the averaging window configured in the QoS profile to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled.

- Understanding 2: The Alternative QoS Parameter sets may contain different averaging window configurations. It means that the NG-RAN will use different durations to determine whether the QoS profile can be fulfilled and which alternative QoS profile can be used as reference if the QoS profile is not fulfilled. This represents a new requirement to NG-RAN.

RAN3 could not reach consensus on introducing averaging window in the alternative QoS Profile, and respectfully requests SA2 to provide clarification on introducing averaging window in the alternative QoS Profile.

From the incoming LS it seems that even SA2 themselves now cannot converge on the need of an average window per QoS profile when they write in S2-2508118 [xx]:

“There is no consensus on how to select and the impacts of using the averaging window of current activated QoS Profile by NG-RAN”

In the lack of consensus to change the status quo usual rules prevail and single average window is used as per current stage 3. RAN3 can reply to the question of SA2 as follows:

Answer to SA2

When AQP are present in the NG-RAN node, each qos profile selects and uses the average window indicated in the qos parameter of the qos flow.

A reply LS to SA2 is also provided in [3].

As a way forward we propose to possibly even make it clearer in TS 38.413 through the CRs in [4] and [5].

Proposal 1: agree the response LS in [3].

Proposal 2: agree the CRs in [4] and [5].

# 3	Conclusion

This paper has recalled the past discussions on which average window is assumed to be used for alternative QoS profile according to current stage 3. It makes the following proposals:

Proposal 1: agree the response LS in [3].

Proposal 2: agree the CRs in [4] and [5].

# 4	References

- R3-256515, LS on Average window for alternative QoS, S2-2508118, RAN3#129bis

- R3-252397, LS on average window for alternative QoS, RAN3#127bis

- R3-257086, Reply LS on Average window for alternative QoS

- R3-257084, Correction on average QoS window for AQP

- R3-257085, Correction on average QoS window for AQP