3GPP TSG-RAN WG3 Meeting #129-bis	R3-257115

# Prague, Czech Republic, 13 – 17 October 2025

Agenda Item:	11.2.1

Source:	Ericsson

Title:	MRO for inter-gNB LTM

Document for:	Discussion and decision

# 1	Introduction

One of the main objectives of the SON/MDT Rel-20 WID [RP-252560] is to enhance the MRO features to optimize the LTM cell switch procedure.

- MRO enhancement for R19 mobility mechanisms, including inter-CU LTM and intra-CU conditional LTM [RAN3, RAN2]: - Specification of the inter-node information exchange, including possible enhancements on the existing interfaces [RAN3] - Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2] If needed, co-operate with RAN1, SA5.

This contribution discusses potential MRO enhancements for inter-CU LTM cell switch.

# 2	Discussion

## 2.1 Use-cases to be supported in Rel-20

During Rel-19 MRO for intra-CU LTM, a variety of different use-cases were discussed, including Too Late LTM cell switch, Too Early LTM cell switch, LTM cell switch to Wrong Cell, near failure LTM, BFD/BFR shortly after successful LTM cell switch, LTM failure due to wrong beam selection, LTM failure due to outdated TA, and LTM ping-pong issue.

The failure and near failure scenarios listed above for intra-CU LTM also apply for inter-CU LTM. Therefore, we propose that,

At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM.

For each use case in Rel-19 MRO concerning intra-CU LTM, extensive discussions were held, particularly regarding whether a network-based or UE-based approach should be adopted. From both specification and UE/NW implementation perspectives, a unified MRO solution covering both intra-CU and inter-CU LTM is crucial, which ensures alignment, facilitates interoperability, and leverages the optimizations derived from the Rel-19 work. Therefore, the solutions defined in Rel-19 should be used as the baseline for developing Rel-20 MRO for inter-CU LTM, if applicable.

Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

### 2.1.1 BFD/BFR shortly after successful LTM cell switch

The case of BFD/BFR shortly after successful LTM cell switch is defined as that the target DU identifies that a BFD/BFR has happened in the UE shortly after a successful LTM cell switch by detecting that a time gap between the successful LTM cell switch and the BFR in the same cell is smaller than the configured threshold (e.g. Tstore_UE_cntxt).

To perform MRO for intra-CU LTM, the target DU performs initial analysis and sends the recovery beam information to the source DU via the CU over F1AP. Similarly, for inter-CU LTM scenario, to send the recovery beam information to the source DU, it is straightforward that XnAP message enhancement is needed for signaling the recovery beam information transfer from the target CU to source CU. Here an existing message can be reused, such as the ACCESS AND MOBILITY INDICATION message.

Target CU sends the recovery beam information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

For intra-CU LTM, the CU always has the UE context when BFR shortly after successfully LTM happens. However, after a successful inter-CU LTM execution, the source CU has the possibility to release the UE context. The configured time threshold for detecting BFR shortly after successful LTM cell switch, e.g Tstore_UE_cntxt can be quite long. Below is the allowed range defined in TS 28.541 for Tstore_UE_cntxt for a reference.

Hence, the source CU may be required to retain the UE context for an extended period for MRO purposes solely in case an BFD/BFR issue arises afterwards, which could lead to additional costs.

RAN3 to discuss BFR shortly after successful LTM cell switch without retaining the UE context in the source CU after successful inter-CU LTM execution.

### 2.1.2 Failure due to wrong beam selection

Regarding the inter-CU LTM cell switch failure due to wrong beam selection, the target DU can identify that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell. To send the re-established or recovery beam information from target DU to source DU, an Xn message enhancement is needed in order to reuse the Rel-19 solution for intra-CU LTM case.

Target CU sends the re-established or recovery beam information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

### 2.1.3 Failure due to outdated TA

Similar to 2.1.2, the target DU can identify that the UE successfully performs a RACH-based access while re-establishing or recovering to the same target cell to identify a possible outdated TA problem. To send the TA value used in RACH procedure during the successful re-establishing or recovering, an Xn message enhancement is needed in order to reuse the Rel-19 solution for intra-CU LTM case.

Target CU sends the TA information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

### 2.1.4 LTM cell switch ping-pong

In Rel-19 MRO for LTM cell switch ping-pong, the CU can detect the potential LTM ping-pong issue based on local information and informs relevant information to the DU whenever CU decides this is necessary. Currently, UHI information is included in HANDOVER REQUEST and RETRIEVE UE CONTEXT RESPONSE XnAP messages. During inter-CU LTM initial preparation, the HANDOVER REQUEST with the UHI information is transmitted to all the gNBs with inter-CU LTM candidate cells. For subsequent inter-CU LTM cell switch, the gNBs of the subsequent LTM candidate cell may not have the up-to-date UHI information about the cells visited by the UE after LTM initial preparation phase. Without this up-to-date UHI information, the involving CU may not be able to detect the potential LTM ping-pong issue, and not be able to send the last consecutive LTM cell switch entries from the UHI to the related DU.

- The candidate gNB for Inter-CU LTM may have not the up-to-date UHI information after LTM initial preparation phase.

In order to leverage the Rel-19 MRO solution for LTM cell switch ping-pong, RAN3 should discuss and agree on whether and in what way, inter-CU LTM candidate gNBs can be provided with the up-to-date UHI information.

RAN3 to discuss whether and how inter-CU LTM candidate gNBs can be provided with the up-to-date UHI information.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	The candidate gNB for Inter-CU LTM may have not the up-to-date UHI information after LTM initial preparation phase.

Based on the discussion in the previous sections we propose the following:

Proposal 1	At least Rel-19 MRO use-cases for intra-CU LTM need to be supported for MRO for inter-CU LTM.

Proposal 2	Rel-19 MRO solutions for intra-CU LTM are taken as baseline for Rel-20 MRO, if applicable.

Proposal 3	Target CU sends the recovery beam information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

Proposal 4	RAN3 to discuss BFR shortly after successful LTM cell switch without retaining the UE context in the source CU after successful inter-CU LTM execution.

Proposal 5	Target CU sends the re-established or recovery beam information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

Proposal 6	Target CU sends the TA information to the source CU via an existing XnAP message i.e. ACCESS AND MOBILITY INDICATION.

Proposal 7	RAN3 to discuss whether and how inter-CU LTM candidate gNBs can be provided with the up-to-date UHI information.