3GPP TSG-RAN WG3 #129bis	R3-257159

Prague - Czech, 13th – 17th Oct. 2025

Agenda Item:	10.2.1 General principles and requirements

Source:	NTT DOCOMO INC.

Title:	Requirements and deployment scenarios toward 6G RAN

Document for:	Discussion

Introduction

Revised SID Study on 6G Radio has been approved in RAN#109 [1]. This contribution provides general aspects regarding RAN3 6G study.

Discussion

6G RAN architecture and migration requirements

According to latest approved pCR for 38.914 on Requirements for architecture and migration for 6G [2] in RAN plenary#109, the following requirements should be taken into consideration into RAN3 6G study.

- The 6G RAN architecture shall support standalone RAN architecture.

- The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

- The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

- The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

- The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

- 3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

- The 6G RAN architecture shall allow for control plane and user plane separation.

- The 6G RAN architecture shall support sharing of the RAN between multiple operators.

- The 6G RAN architecture shall allow for the operation of network slicing.

- The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

- The 6G RAN architecture shall support enhanced service awareness in RAN.

- The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

- The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

- The 6G RAN architecture shall allow non-public networks.

Proposal 1:	RAN3 6G study should be based on the above 6G RAN architecture and migration requirements captured in TR 38.914.

Regarding additional migration options (other than standalone, MRSS, and inter-RAT mobility between NR-6G), RAN will start the study in March 2026 and will make a decision by September 2026 whether expand WG SI scope to cover additional migration option(s) as stated in [1]. RAN will also task relevant RAN WGs for any specific technical analysis. Therefore, RAN3 should wait for liaison from RAN regarding additional migration options.

Observation 1:	RAN will start the study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). RAN will also task relevant RAN WGs for any specific technical analysis.

Proposal 2:	RAN3 should wait for liaison from RAN regarding additional migration options study.

Centralized/non-centralized deployment

Non-centralized deployment is one of the deployments widely used in existing 4G/5G deployments. It is a simple configuration that can guarantee high RAN performance even under limited transport network performance. On the other hand, it is necessary to place RAN-node components at the edge due to the limitation of the distance of the fronthaul. This monolithic RAN node configuration can be a baseline even in the 6G era.

Non-centralized deployment, which is also used in the existing generation, achieves optimized centralization and distribution of RAN functions according to the required delay and throughput. By centralizing the computing resources of the higher layer while considering the limitation of the extension distance of the fronthaul, performance can be maintained while reducing costs. This option allows operator flexible deployment, therefore we believe that this deployment can potentially become widespread in 5G, and this should also be needed in 6G, considering more RAN nodes would be virtualized, and HW/computing resources would be shared for multiple purposes.

Regarding the splitting layer of 6G RAN node, lower layer split (i.e. RU) was already discussed [3] in RAN#109 and they endorsed that 6G RAN includes a distinct logical Radio Unit (RU) and the interface for it will not be specified. For higher layer split (i.e. CU/DU), it can be further discussed, including different layer split between C-plane and U-plane. Similar options can be considered in 6G as in 5G TR [4], considering the protocol stacks discussed in RAN2.

Fig. 1. Non-centralized and centralized deployment

Proposal 3:	Support both monolithic RAN node and split RAN node architecture in 6G. In which layer do we split the RAN node can be further discussed.

Virtualized deployment

RAN virtualization and HW generalization have various advantages for operators, such as cost efficiency, flexibility of installation, scalability, redundancy, and so on. Virtualization has already started to be introduced in 5G RAN, and the ability to share HW between 5G and 6G would be a big boost to the rapid introduction of 6G. In 5G, implementation-based RAN virtualization has already been developed/developed, however in some cases, current protocol design of 3GPP assuming P2P connection between dedicated RAN equipment causes some difficulties. For example, from the point of view of virtualization, it is desirable for RAN nodes to be managed as distributed pods for scale/recovery, etc. On the other hand, since the existing 3GPP protocol does not assume dynamic interface termination points switching, it is required to rely on implementation-based solutions or to be integrated pods design that is inappropriate for micro services. In 6G, the transition to cloud-friendly transport protocols and protocol/procedure design assuming virtualization will promote faster/easier virtualized RAN deployment.

- Fig. 2. Virtualized RAN deployment

Proposal 4:	6G RAN should support virtualization and be cloud-native design.

Co-sited deployment with LTE/NR

Given the current 5G deployment and the fact that there will not be many new frequencies in 6G, it's possible that 4G/5G/6G could all be used to form coverage. Some country including Japan, 4G is used to form coverage, and NSA is used for higher capacity. In some areas with high traffic 5G SA is deployed, such as urban areas, for higher performance and/or advanced features. For early step of 6G deployment, it is expected that 6G will be deployed for further performance, i.e. in areas covered with 4G NSA or 5G NR. This means that in some area, 4G would be potentially migrated to/co-located with 6G skipping a migration to 5G SA. Considering this situation, from operator perspective, both 6G/5G and 6G/4G co-location should be considered. In actual deployment, 3 RAT co-location would happen in case that 4G would be still needed for legacy devices even if 6G/5G are overlaying or in case that neither VoNR nor 6G voice is supported. However, we are not so motivated to specify any enhancement to support that scenario, and in most cases, it can be considered as the combination of Site A and Site B in the Fig.2. It is also noted that MRSS for 6G and 4G and for 3 RAT is not approved in RAN plenary, while one for 6G and NR was approved. From RAN3 perspective, for these deployment scenarios, we should discuss whether and how to introduce an interface and procedures for coordination between RAN nodes across RATs. This coordination would include, e.g. for inter-RAT handover and MRSS. There are following approaches, therefore we suggest discussing which approach we should take.

- Single RAN node serves both 5G and 6G, or different RAN nodes serve each RAT, and the coordination would be done based on the implementation.

- Different RAN nodes serve each RAT, and related procedures would be done without the coordination between them.

- Different RAN nodes serve each RAT, and the interface between RAN nodes (similar as Xn) would be specified and used for the coordination between them.

- Different RAN nodes serve each RAT, and new interface between lower layer part of RAN nodes (similar as D2 specified in O-RAN) would be used for the coordination between them and specified, if necessary.

Fig. 3. Co-sited deployment with LTE/NR

Observation 2:	At least, co-site deployment for 6G/5G and 6G/4G would be needed in initial phase of 6G deployment.

Observation 3:	Co-site deployment for 4G/5G/6G would happen in initial phase of 6G deployment, however it can be considered as the combination of 6G/5G co-location and 6G/4G co-location in most cases.

Proposal 5:	Co-sited deployment with 6G-NR and with 6G-E-UTRAN should be supported. FFS on interworking between 6G and E-UTRAN pending on RAN plenary discussions.

Proposal 6:	RAN3 to discuss whether and how to specify interfaces and procedures for MRSS between RAN nodes.

Energy efficiency

In the 6G era, reducing power consumption on the NW side is important from the viewpoints of sustainability and OPEX reduction. In particular, at high frequencies, the increase in power consumption is remarkable, and this trend will still exist with the introduction of FR3, etc. Therefore, it is desirable from the operator's viewpoint to introduce related functions from day 1. In addition, reducing power consumption on the UE side is directly linked to the user experience. We believe that it is also important to support functions to reduce power consumption in UE, which requires higher capability from the performance viewpoint. Many functions for energy efficiency were introduced in 5G, however most of functions were introduced after 5G day 1, therefore it was difficult to introduce these functions for operators. For 6G, we think these functions should be supported from day 1.

Proposal 7:	Support functions for energy efficiency for both NW and UE.

Support of TN cells and NTN cells

For wider coverage and higher reliability, not only TN but also NTN should be utilized in the 6G era. We think that considering both TN and NTN and supporting smooth mobility and various topologies from day 1 will contribute to the success of deployment and commercialization. Mobility procedures would be discussed mainly in RAN2, and from RAN3 perspective, switch overs of RAN interfaces should be considered, because in NTN deployment, connection between RAN-CN and RAN nodes frequently switched, which characteristic is different from TN deployment. In 5G, suspend/resume/removal of NG interfaces are supported in release 19 as a fundamental function for gNB on board NTN, and at least it should be supported from 6G day 1.

Proposal 8:	Support both TN and NTN deployments.

Automated and advanced RAN optimization

In current 4G/5G, many RAN optimization functions specified as SON/MDT and implementation-based functions are used. In 5GA, AI/ML assisted optimization functions were introduced to optimize NW decisions, improve radio resource efficiency, reduce UE power consumption, and so on. From the operator's point of view, SON and AI/ML assisted optimization functions are required from day 1 in 6G to achieve automation and advanced optimization for increasing cells and optimization parameters due to the introduction of high frequency and wider coverage. Also, these enhancements are one of the important functions to maximize the motivation of introducing a new generation, 6G.

Proposal 9:	Support automated RAN optimization functionalities, including SON/MDT and AI/ML assisted RAN optimizations.

Resiliency

As described in the latest TR 38.914, higher resiliency is very important in both aspects of speedup of recovery from damage of RAN equipment due to disasters, etc., and improvement of operability by service continuity during operation/maintenance. Especially in disaster-prone countries such as Japan, the recovery speed of damaged RAN equipment directly affects human lives as in ETWS. In addition, supporting resiliency function in specifications will reduce cost due to redundancy and contribute to realization of more reliable NW.

RAN virtualization will also contribute to improving resiliency. So far, operators have basically ensured resiliency according to the characteristics of each region by redundancy based on the implementations. In some cases, manual operation is still required for system recovery. In the 6G era, in addition to the existing SON/MDT function, the combination of monitoring and management function using AI/ML and support of the resiliency function specified by 3GPP are required to restore and provide more automated, high-speed, and stable cellular communication. In addition, by supporting this function in the specification, operators can improve resiliency using a common HW pool without providing redundant equipment for each vendor.

Proposal 10:	Support functions for resiliency.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	RAN3 6G study should be based on the above 6G RAN architecture and migration requirements captured in TR 38.914.

Observation 1:	RAN will start the study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s). RAN will also task relevant RAN WGs for any specific technical analysis.

Proposal 2:	RAN3 should wait for liaison from RAN regarding additional migration options study.

Proposal 3:	Support both monolithic RAN node and split RAN node architecture in 6G. In which layer do we split the RAN node can be further discussed.

Proposal 4:	6G RAN should support virtualization and be cloud-native design.

Observation 2:	At least, co-site deployment for 6G/5G and 6G/4G would be needed in initial phase of 6G deployment.

Observation 3:	Co-site deployment for 4G/5G/6G would happen in initial phase of 6G deployment, however it can be considered as the combination of 6G/5G co-location and 6G/4G co-location in most cases.

Proposal 5:	Co-sited deployment with 6G-NR and with 6G-E-UTRAN should be supported. FFS on interworking between 6G and E-UTRAN pending on RAN plenary discussions.

Proposal 6:	RAN3 to discuss whether and how to specify interfaces and procedures for MRSS between RAN nodes.

Proposal 7:	Support functions for energy efficiency for both NW and UE.

Proposal 8:	Support both TN and NTN deployments.

Proposal 9:	Support automated RAN optimization functionalities, including SON/MDT and AI/ML assisted RAN optimizations.

Proposal 10:	Support functions for resiliency.

References

- RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo.

- RP-252870, “pCR for 38.914 on requirements for architecture and migration for 6G”, NTT Docomo.

- RP-252881, “Way Forward on 6G Fronthaul”

- TR 38.801, “Study on new radio access technology: Radio access architecture and interfaces”.