3GPP TSG-RAN WG3 #129bis	R3-257160

Prague - Czech, 13th – 17th Oct. 2025

Agenda Item:	10.3.1 General principles and requirements

Source:	NTT DOCOMO INC.

Title:	Requirements toward 6G RAN-CN functional split

Document for:	Discussion

Introduction

In this contribution, we will discuss the general requirements and principles for the interface between RAN and CN similar as 5G TR [1].

Discussion

C/U separation

In existing 4G/5G, C/U separation is very important to provide appropriate priority control within the limited bandwidth of the transport network. Even for virtualized RAN, the capacity of the transport network still varies depending on the deployment, and operators need to configure the network appropriately based on it. Similar to legacy generation, 6G also requires at least control-plane and user-plane separation. Regarding the new plane for new services (e.g. AI/ML data collection, sensing), we should wait for SA SID WT#5 [2] discussion in SA2 and RAN SID 3-e/8-b-ii [3] in RAN2.

Proposal 1:	RAN-CN interface in 6G shall support control plane and user plane separation. FFS on the new plane pending on the discussion in SA2/RAN2.

Open interface and multi-vendor operation

For operators, cross-vendor operability of NG interfaces is very important. In 4G/5G, we already deployed different vendors for CN and RAN, and considering the flexibility of vendor selection in RAN and the difference in granularity between CN and RAN, this requirement is also necessary for 6G.

Proposal 2:	RAN-CN interface in 6G shall be open and inter-operable across different vendors.

RAN-CN functional split

In 5G, functional separation between RAN and CN was defined based on RRC state and related procedures such as paging, mobility, UE context management, etc. The details of these functions are up to RAN2/SA2 discussion and should be defined based on coordination with the other WGs.

Proposal 3:	Functional split between RAN and CN should be defined based on coordination with RAN2/SA2 and the discussions for fundamental functionalities, e.g. paging, mobility, and UE context management.

Support of virtualization

As mentioned in our other contribution [4], virtualization has many advantages for operators. While 5GC already supports cloud native protocol design, RAN interfaces, including NG interface, follow legacy end-to-end connection protocol design. In 5G, implementation-based RAN virtualization has already been developed, however in some cases, P2P-based protocol design of 3GPP causes some difficulties. For example, from the point of view of virtualization, it is desirable for RAN nodes to be managed as distributed pods for scale/recovery, etc. On the other hand, since the existing 3GPP protocol does not assume dynamic interface termination points switching, it is required to rely on implementation-based solutions or to be integrated pods design that is inappropriate for micro services.

Observation 1:	Existing end-to-end based protocol design for RAN interfaces is one of the bottlenecks for flexible implementation for RAN virtualization.

Proposal 4:	RAN-CN interface in 6G shall allow virtualized RAN deployment.

TN and NTN integration

As mentioned in our other contribution [4], TN-NTN integration from day 1 is important for achieving perfect coverage. Suspend/resume/removal of NG interfaces are already supported in 5G, and these functions should be supported in 6G day 1. Further enhancement for NTN to RAN-CN interfaces should also be discussed for further smooth integration of TN and NTN.

Proposal 5:	RAN-CN interface in 6G shall allow both TN and NTN deployment

Resiliency

As described in our other contribution [4], resilience is also an important viewpoint for operators. For rapid recovery from RAN/CN failures, it is very useful to specify functions for resilience in 3GPP for RAN-CN interfaces that are expected to be cross-vendor operations. In particular, dynamic/fast endpoint switching of RAN-CN interfaces should be discussed in RAN3 in relation to virtualization and feeder link switch over in NTN.

Proposal 6:	RAN-CN interface in 6G shall support functionalities for enhanced resiliency.

Proposal 7:	RAN3 to discuss dynamic switch over of the RAN-CN interface endpoints both in CN and RAN side, e.g. for virtualization, NTN feeder link switch over, and fast recovery from failures on NW entities.

RAN-CN connectivity options

In 5G study, several options were discussed for EPC/5GC and E-UTRAN/NR connectivity. Similarly, for 6G, 5GC/6G RAN or 6GC/5G RAN connectivity should be discussed including backward compatibility of RAN-CN interface. This point is also affected by the migration option. If UE aggregates 6G and NR based on single registration to the CN, RAN-CN cross connection is required between 6G and 5G. However, as described in 6G RAN SID [3], the migration discussion is decided by June 2026 in RAN plenary. In addition, whether CN in 6G has NBC for AMF/UPF in 5GC from C-plane/U-plane perspective should be discussed coordinating with SA2.

We think that the problem with the current RAN-CN interface is the concentration of load on the AMF for all C-plane signalings toward the CN. At present, most of the main functions of 5GC are basic functions related to access and mobility. However, if all the signaling for new services involving CN in 6G is signaled through the RAN-CN interface to the AMF as the end point, the load will be concentrated on the AMF. We think that the design that these new services put additional load on the NF essential for communication should be avoided. On the other hand, since we have not found a critical problem that requires NBC for the existing NG-C/U signaling, reusing part of the existing NG-C/U in 6G, or changing only the transport network layer and reusing the overall procedure of legacy, can be also potential solutions. RAN3 should discuss whether to reuse the existing NG-C/U or specify a new RAN-CN interface based on coordination with SA2. In some cases, the details of the fundamental functions of RAN, such as paging, mobility, UE context management, and so on, which are involved in RAN2 discussions, may also affect it.

Proposal 8:	RAN3 to discuss whether and how to reuse existing NGAP or specify new RAN-CN interface, based on the discussions for overall architecture in SA2 and for fundamental functionalities, e.g. paging, mobility, and UE context management in RAN2/SA2.

Conclusions and proposals

Our proposals are summarized below.

Proposal 1:	RAN-CN interface in 6G shall support control plane and user plane separation. FFS on the new plane pending on the discussion in SA2/RAN2.

Proposal 2:	RAN-CN interface in 6G shall be open and inter-operable across different vendors.

Proposal 3:	Functional split between RAN and CN should be defined based on coordination with RAN2/SA2 and the discussions for fundamental functionalities, e.g. paging, mobility, and UE context management.

Observation 1:	Existing end-to-end based protocol design for RAN interfaces is one of the bottlenecks for flexible implementation for RAN virtualization.

Proposal 4:	RAN-CN interface in 6G shall allow virtualized RAN deployment.

Proposal 5:	RAN-CN interface in 6G shall allow both TN and NTN deployment

Proposal 6:	RAN-CN interface in 6G shall support functionalities for enhanced resiliency.

Proposal 7:	RAN3 to discuss dynamic switch over of the RAN-CN interface endpoints both in CN and RAN side, e.g. for virtualization, NTN feeder link switch over, and fast recovery from failures on NW entities.

Proposal 8:	RAN3 to discuss whether and how to reuse existing NGAP or specify new RAN-CN interface, based on the discussions for overall architecture in SA2 and for fundamental functionalities, e.g. paging, mobility, and UE context management in RAN2/SA2.

References

- TR 38.801, “Study on new radio access technology: Radio access architecture and interfaces”.

- SP-250806, “Study on Architecture for 6G System”, SA2.

- RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo.

- R3-257159, “Requirements and deployment scenarios toward 6G RAN”, NTT Docomo.