3GPP TSG-RAN WG3 Meeting # 129bis	R3-257168

Prague, Czech Republic, 13 – 17 October, 2025

Agenda Item:	8.1

Source:	ZTE Corporation

Title:	Discussion on  average window for alternative QoS

Document for:	Discussion and approval

1	Introduction

In previous meeting RAN3 sent LS(R3-252397) to SA2 and asked whether the Aervaging Window information shall be added in the Alternative QoS Parameters in RAN3 specification. Based on the SA2 replied LS(R3-256515), we prefer to further discuss this issue.

- 2	Discussion

- Companies found the mismatch description between SA2 specification and RAN3 specifications according to the detail info contained in the Alternative QoS Parameters. More specifically, the Averaging window information can be present in an Alternative QoS Profile in TS23.501 while RAN3 specifications can not achieve this. Based on the discussion outcome, RAN3 can not make consensus on how to fix this mismatch issue and preferred to ask SA2 to further clarify the introduction of this information.

- Based on the SA2 response in R3-252397 shown below, at least RAN3 can conduct the following observation:

- ==== content in SA2 reply LS R3-252397 ====

[SA2 Answer:] SA2 has discussed the LS and there are other understandings beyond Understanding 1 and 2. There is no consensus on how to select and the impacts of using the averaging window of current activated QoS Profile by NG-RAN. SA2 would like to ask RAN3 to decide how to select and use the averaging window when AQP are present in the NG-RAN.

- ==== content in SA2 reply LS R3-252397 ====

- Observation 1: SA2 has no agreement on how to use this averaging window information in AQP.

- Observation 2: RAN3 shall decide how to treat the averaging window issue.

- The SA2 definition for averaging window is shown below:

- ==== content in TS 23.501 ====

5.7.3.6	Averaging Window

Each GBR QoS Flow shall be associated with an Averaging window. The Averaging window represents the duration over which the GFBR and MFBR shall be calculated (e.g. in the (R)AN, UPF, UE).

Every standardized 5QI (of GBR and Delay-critical GBR resource type) is associated with a default value for the Averaging window (specified in QoS characteristics Table 5.7.4.1). The averaging window may also be signalled together with a standardized 5QI to the (R)AN and UPF and if it is received, it shall be used instead of the default value.

The Averaging window may also be signalled together with a pre-configured 5QI to the (R)AN and if it is received, it shall be used instead of the pre-configured value.

- ==== content in TS 23.501 ====

- Based on the current definition, the averaging window refers to a dedicated duration that the RAN (within the scope of RAN3) uses to calculate the Guaranteed Flow Bit Rate (GFBR) and Maximum Flow Bit Rate (MFBR) of a QoS flow.

- Observation 3: SA2 defines the averaging window as a dedicated duration for the RAN(RAN3 scope) to calculate the GFBR and MFBR of a QoS flow.

- Meanwhile, the Alternative QoS Profile is a defined mechanism aimed at maximizing the saving of existing QoS flows. If necessary, the RAN may utilize the lower-performance QoS profile provided by the CN to keep the processing of the QoS flow.Based on this understanding, RAN3 should first clarify the scenarios and benefits of applying the new averaging window in AQP, and then communicate RAN3’s decision on how to address the averaging window issue between SA2 and RAN3.

- Proposal 1: RAN3 shall first clarify the scenarios and benefits of applying the new averaging window in AQP, and then sends final decision to SA2 on how to address the averaging window issue.

- 3	Conclusion

- Observation 1: SA2 has no agreement on how to use this averaging window information in AQP.

- Observation 2: RAN3 shall decide how to treat the averaging window issue.

- Observation 3: SA2 defines the averaging window as a dedicated duration for the RAN(RAN3 scope) to calculate the GFBR and MFBR of a QoS flow.

- Proposal 1: RAN3 shall first clarify the scenarios and benefits of applying the new averaging window in AQP, and then sends final decision to SA2 on how to address the averaging window issue.