3GPP TSG-RAN WG3 Meeting #129bis	R3-257170

Prague, CZ, 13-17, Oct 2025

Agenda Item:	9.2.11

Source:	ZTE Corporation

Title:	Discussion on Reply LS to RAN2 about NW-side data collection

Document for:	Discussions & Approval

1	Introduction

RAN2 sent the LS R2-2506545 to RAN1 and RAN3 about NW side data collection, and attached the RAN2 latest agreements. In this contribution, we provide our understanding on the potential specification impacts to support NW side data collection, and prepare the reply LS to RAN2.

2	Discussion

1. The list of logging configurations is provided as a new list configurations within CSI-MeasConfig.

-	the data logging procedure based on receiving measurement quantities from lower layer  is captured in RRC spec.

2. The L3 event (i.e. Event A1 or Event A2) configuration is contained in the logging configuration to support the event based data logging.

3. The logged data is reported via UEInformationRequest/UEInformationResponse.

4. The UE can send a UAI that indicates:

•	Data is available

•	Reason for trigger (full buffer, threshold)

•	Low power indication

5. The NW side data collection configuration should be released if UE reports the lower power indication to the NW via UAI.

In the LS, RAN2 lists the potential impacts to support NW data collection that RAN3 should consider.

After reviewing the information listed by RAN2, the impacts mainly focus on the interaction between the lower layer and the higher layer. If considering split architecture, in order to support the above bullets, there needs to be coordination between CU and DU. And there is no specification impacts to support NW data collection in case of non-split gNB.

- The impacts to support NW data collection mainly focus on the interaction between lower layer and higher layer.

- There is no specification impacts to support NW data collection in case of non-split gNB.

Here, we further elaborate the impacts over F1 interface for following bullets.

1. The list of logging configurations is provided as a new list configurations within CSI-MeasConfig.

-	the data logging procedure based on receiving measurement quantities from lower layer  is captured in RRC spec.

2. The L3 event (i.e. Event A1 or Event A2) configuration is contained in the logging configuration to support the event based data logging.

3. The logged data is reported via UEInformationRequest/UEInformationResponse.

The logging configuration is provided within CSI-MeasConfig, and CSI-MeasConfig is generated by DU. However, the L3 event configuration is generated based on L3 measurements. Thus, there are two options for the interaction between CU and DU:

- Option 1 (CU handles the configuration): CU generates the L3 event configuration based on L3 measurements and notifies the L3 event configuration to DU.

- Option 2 (DU handles the configuration): DU requests the L3 measurements from CU, and DU generates the L3 event configurations.

The logged data is reported via UEInformationRequest/UEInformationResponse in RRC to CU. If DU generates the data collection configuration, CU has no knowledge about such configuration. F1 may be needed, that is, DU will inform CU of the generation of NW data collection configuration.

- RAN3 has to discuss the potential impact over F1 interface about the logging configuration:

    - Option 1 (CU handles the configuration): CU generates the L3 event configuration based on L3 measurements and notifies the L3 event configuration to DU.

    - Option 2 (DU handles the configuration): DU requests the L3 measurements from CU, and DU generates the L3 event configurations.

Regarding the 3rd bullet, CU needs to configure the UAI triggering condition to DU, that means, CU-DU interaction may be needed for CU to configure the UAI for the NW side data collection configured by DU. And also it was mentioned that the NW side data collection configuration should be released if UE reports the lower power indication to the NW via UAI. Therefore, CU need to inform the lower power indication to the DU for DU to release the NW side data collection configuration.

- For the UAI indication, the interaction between CU and DU also needs to be enhanced.

In Rel-19, RAN3 has not discussed standard solutions for NW data collection, and most importantly in the objectives of Rel-19, NW Data collection also does not include RAN3 scope. Considering the load of standardization discussions in Rel-20, it is not recommended to initiating the normative work for NW data collection on the F1 interface. Moreover, there is no negative impacts on RAN2’s progress if RAN3 doesn’t support NW data collection over F1 interface, since NW data collection can be supported in case of non-split gNB.

- Reply the LS to RAN2 that NW data collection can be supported in case of non-split gNB and NW data collection in split architecture is not supported.

Agree the LS in R3-256735 [2].

3	Conclusion

Following is our observations and proposals:

- The impacts to support NW data collection mainly focus on the interaction between lower layer and higher layer.

- There is no specification impacts to support NW data collection in case of non-split gNB.

- RAN3 has to discuss the potential impact over F1 interface about the logging configuration:

    - Option 1 (CU handles the configuration): CU generates the L3 event configuration based on L3 measurements and notifies the L3 event configuration to DU.

    - Option 2 (DU handles the configuration): DU requests the L3 measurements from CU, and DU generates the L3 event configurations.

- For the UAI indication, the interaction between CU and DU also needs to be enhanced.

- Reply the LS to RAN2 that NW data collection can be supported in case of non-split gNB and NW data collection in split architecture is not supported.

4	Reference

[1] R2-2506545 LS to RAN1 and RAN3 on NW side data collection, RAN2

[2] R3-256735 [Draft LS] Reply LS to RAN2 about NW-side data collection, ZTE Corporation