3GPP TSG-RAN WG3 Meeting #129-bis	R3-257179

Prague, Czech Republic, 13th ~17th Oct, 2025

Title:	Consideration on general requirements and principles for RAN architecture study in 6G

Agenda Item:	10.2.1

Source:	Huawei

Document for:	Discussion and decision

1	Introduction

This paper tries to have initial discussions on the general requirement and principles for RAN architecture, based on the common understandings reached in September plenary.

2	Discussion

2.1	Principles to be followed

During last RAN plenary, some agreements on requirements for architecture were reached for the 6G RAN architecture [1], as follows:

The RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

-	The 6G RAN architecture shall support standalone RAN architecture.

-	The 6G RAN shall support Multi-RAT Spectrum Sharing between 6GR and NR.

-	The 6G RAN architecture shall support inter-RAT mobility between the 6GR and NR.

-	The 6G RAN architecture shall support connectivity through multiple TRPs, either collocated or non-collocated.

-	The 6G RAT shall support Spectrum Aggregation (e.g. Carrier Aggregation) for both uplink and downlink, and for both co-located and non-co-located TRPs.

-	3GPP defined interfaces for 6G RAN shall be open for multi-vendor interoperability.

-	The 6G RAN architecture shall allow for control plane and user plane separation.

-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.

-	The 6G RAN architecture shall allow for the operation of network slicing.

-	The 6G RAN architecture shall be designed considering both terrestrial network and non-terrestrial network.

-	The 6G RAN architecture shall support enhanced service awareness in RAN.

-	The design of the 6G RAN shall allow enhanced resilience compared to NR if/where applicable.

-	The design of the 6G RAN shall enable lower CAPEX/OPEX with respect to current networks.

-	The 6G RAN architecture shall allow non-public networks.

- Based on the requirements agreement above, it is necessary, from RAN3 perspective, to focus on the study of functions, so that these requirements can be well fulfilled.

Proposal 1: RAN architecture study shall focus on the logical functions to be supported to meet the requirements achieved in RAN plenary.

On the other hand, when starting to study logical functions, we should note that there are new services to be introduced in 6G, see [2] below, which may have new or additional requirements comparing to 5G.

### 5.4.1	Mobile Broadband

### 5.4.2	Immersive Communication

### 5.4.3	Massive Communication (IoT)

### 5.4.4	Sensing

### 5.4.5	AI

### 5.4.6	Voice

### 5.4.7	Regulatory Services

Thus, 6G RAN architecture and function design shall allow for the support of such new services, which may bring new requirements to system design, and new requirements from these new services are expected to be clearer and clearer as the 6G RAN study discussion progresses. Meaning while, as RAN3 study on 6G RAN architecture and logical function gradually unfolds, RAN3 shall also consider the introduction of any potential new services and corresponding new requirements in an efficient, future proof way.

On the other hand, as we could see that the current 5G-A already supported similar services listed above, e.g. AI service for which L1/L2/L3 related use cases were also supported, XR which could be categorized as immersive communication, and sensing is just under discussion, i.e. 5G-A already set up a very good basis of RAN architecture and logical function for 6G design, any new ideas to restructure 5G-A design shall be well justified.

Proposal 2: 	RAN architecture and logical function design shall allow for the introduction of any potential new services in an efficient way, taking 5G-A as basis.

2.2	RAN Functions

As could be seen in 38.300, the 5G base station hosts the following functions:

The gNB and ng-eNB host the following functions:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and sidelink (scheduling);

-	IP and Ethernet header compression, uplink data decompression, encryption and integrity protection of data;

-	Selection of an AMF at UE attachment when no routing to an AMF can be determined from the information provided by the UE;

-	Routing of User Plane data towards UPF(s);

-	Routing of Control Plane information towards AMF;

-	Connection setup and release;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the AMF or OAM);

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Transport level packet marking in the uplink;

-	Session Management;

-	Support of Network Slicing;

-	QoS Flow management and mapping to data radio bearers;

-	Support of UEs in RRC_INACTIVE state;

-	Distribution function for NAS messages;

-	Radio access network sharing;

-	Dual Connectivity;

-	Tight interworking between NR and E-UTRA;

-	Maintain security and radio configuration for User Plane CIoT 5GS Optimisation, as defined in TS 23.501 [3] (ng-eNB only).

NOTE 1:	BL UE or UE in enhanced coverage is only supported by ng-eNB, see TS 36.300 [2].

NOTE 2:	NB-IoT UE is only supported by ng-eNB, see TS 36.300 [2].

In our understanding, some basic functions shall be kept as starting point, since they are fundamental functions which are necessary to basic operations for a RAN base station, no matter what types of new services may be introduced in the future. These functions shall at least include:

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink and side link (scheduling);

-	Measurement and measurement reporting configuration for mobility and scheduling;

RRM function is a fundamental and core function hosted by a base station since day 1 of 3GPP wireless network, while the measurement and reporting are basic parts of RRM function. Note that side link itself is a function should be discussed later.

-	Routing of User Plane data (towards user plane handling entity in core network);

-	Routing of Control Plane information (towards control plane handling entity in core network);

As agreed in [1], “The 6G RAN architecture shall allow for control plane and user plane separation”, so the above two functions are the natural consequence of this agreement.

-	Connection setup and release;

-	Session Management;

The conception of session and connection will anyway be there, since any service or feature will finally be based on the session and connection to bear the communication.

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the CN or OAM);

-	Distribution function for NAS messages;

In our understanding, the basic architecture a wireless network should still be composed of RAN and CN (and corresponding OAM), thus there will be paging messages coming from CN system information from CN or OAM, and NAS messages between CN and UE.

-	Radio access network sharing;

This should be a basic deployment requirement from operators, we think this should be supported from day 1, this is also in line with the agreed requirements in RAN#109 [1]: “-	The 6G RAN architecture shall support sharing of the RAN between multiple operators.”.

With these basic functions as starting point, RAN3 could continue to discuss if new logical functions are needed for new services or new requirements.

Proposal 3: RAN3 to discuss and agree the basic functions listed above as starting point, and capture them into TR.

Corresponding TP could be seen in the Annex part.

3	Conclusion

This document proposes:

Proposal 1: RAN architecture study shall focus on the logical functions to be supported to meet the agreements achieved in RAN plenary, taking 5G-A as basis.

Proposal 2: 	RAN architecture and logical function design shall allow for the introduction of any potential new services in an efficient way.

Proposal 3: RAN3 to discuss and agree the basic functions listed above as starting point, and capture them into TR.

4	Reference

[1] RP-252870, pCR for 38.914 on requirements for architecture and migration for 6G, NTT DOCOMO, INC. (Moderator)

[2]	RP-252960, pCR for 38.914 on Requirements of new and existing services for 6G, CMCC (moderator)

Annex1	Text Proposal (TR skeleton based on R3-255517)

<<<<<<<<<<<<<<<<<<<< Start of Change >>>>>>>>>>>>>>>>>>>>

5.3.2 	RAN Functions

Editor‘s Note:	RAN3 to take the following functions as starting point.

-	Functions for Radio Resource Management: Radio Bearer Control, Radio Admission Control, Connection Mobility Control, Dynamic allocation of resources to UEs in uplink, downlink;

-	Measurement and measurement reporting configuration for mobility and scheduling;

-	Routing of User Plane data (towards user plane handling entity in core network);

-	Routing of Control Plane information (towards control plane handling entity in core network);

-	Connection setup and release;

-	Session Management;

-	Scheduling and transmission of paging messages;

-	Scheduling and transmission of system broadcast information (originated from the CN or OAM);

-	Distribution function for NAS messages;

-	Radio access network sharing;

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>