3GPP TSG-RAN WG3 Meeting #129bis	R3-257189

Prague, Czech Republic, 13th – 17th October, 2025

Title: 	Discussion on the Reply LS from SA5 on Continuous MDT (C-MDT)

Source: 	Huawei, Jio Platforms, Ofinno

Agenda item:	9.2.2

Document Type:	Discussion and decision

# 1. Introduction

One of the objectives of the Rel-19 WI “New WID on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [1] was to introduce support of the leftovers in Rel-18 AI/ML for NG-RAN, one of which is related to Continuous MDT collection targeting the same UE across RRC states. The problem statement for Continuous MDT is reported in TR 38.743 [2] which summarizes the outcome of the preceding Rel-19 SI “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” [3].

In RAN3#128 meeting it was agreed to send a reply LS on Continuous MDT to SA5 in [4], which included additional guidance for SA5 needed to derive a solution for management based Continuous MDT as per the questions for clarification raised by SA5 in [5].

At this meeting RAN3 received the LS from SA5 in [6] (reply LS to [5]) where SA5 informs RAN3 that the specification work to introduce the OAM support for the Continuous MDT functionality in relevant SA5 Rel-19 specifications (TS 28.622, TS 28.623, TS 32.421, TS 32.422 and TS 32.423) has been concluded, and provides RAN3 with the CRs to such specifications.

This contribution discusses the Continuous MDT topic further, in light of the solution specified by SA5, and provides changes we think are needed to be introduced in NG-RAN in order to actually support the Continuous MDT functionality in Rel-19.

# 2. Background

SA5 introduced in the latest Rel-19 version of TS 32.422 (V19.4.0 / 2025-09) [7] the definition of Continuous management-based MDT (C-MDT) as follows:

## 3.1 Definitions For the purposes of the present document, the terms and definitions given in TR 21.905 [4], TS 23.501 [40], TS 38.300 [42] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [4], TS 23.501 [40] or TS 38.300 [42]. Continuous management-based MDT: a management-based MDT functionality that enables continuous collection of MDT data, including both Immediate MDT and Logged MDT, within a specified area. ## NOTE: This MDT data collection captures UE transitions between RRC states (RRC_IDLE, RRC_INACTIVE, RRC_CONNECTED), as well as UE mobility between participating NG-RAN nodes. The MDT data collection on selected UEs is stopped when both Immediate MDT and Logged MDT are deactivated. *** non-relevant parts are omitted ***

Moreover, still referring to the latest Rel-19 version of TS 32.422 (V19.4.0 / 2025-09), clauses 4.1.1.9.4 and 4.1.1.9.5 describe the NG-RAN activation mechanisms for C-MDT data collection for non-split and split architectures, respectively:

##### 4.1.1.9.4 NG-RAN activation mechanisms for C-MDT data collections for non-split RAN architecture For non-split RAN architecture, the C-MDT data collection procedure shall be same as specified in subclause 4.1.1.9.2 with following additions. 1) The management system shall either send two Trace Session activation requests (one for Immediate MDT, another for Logged MDT) with same TR to the gNB, or send one Trace Session activation request with Job Type (IMMEDIATE_MDT_AND_ LOGGED_MDT) to the gNB. In additional of the trace control and configuration parameters specified in subclause 4.1.1.9.2 step 1, the Trace Session activation request(s) shall contain a TRSR Prefix Configuration parameter, which reserves a TRSR range(s) for the gNB during TRSR assignment. This TRSR Prefix Configuration parameter is used to identify this is a C-MDT activation request. Furthermore, the management system shall ensure that Trace Session activation requests with the same TR are sent to all participating NR-RAN nodes involved in the same C-MDT job. 2) Upon receiving the Trace Session activation request(s), the gNB shall identify the Trace Session activation request(s) as a C-MDT job, start a C-MDT session, and store the associated configuration parameters. 3) The gNB and UE shall follow the steps specified in subclause 4.1.1.9.2, step 3 to 8, with following additions: - The gNB shall configure the selected UE with an Immediate MDT session and a Logged MDT session using the configurations from the Trace Session activation request(s). A unique TRSR is assigned to the UE for Immediate MDT and Logged MDT. The TRSR is allocated based on the received TRSR Prefix Configuration parameter. - Upon transitioning into RRC_CONNECTED, the UE may report Logged MDT measurements, including TR and TRSR, to the gNB, as specified in subclause 4.1.1.9.2 step 7 and 8. If the reported TR is same as the Trace Reference received in the Trace Session Activation request(s), the gNB shall identify the UE as part of the same C-MDT job. If the area-based selection and user consent conditions are satisfied, the gNB shall reconfigure the UE with an Immediate MDT and a Logged MDT. - In any case, if a new TRSR is assigned, the correlation between the old TRSR and the new TRSR shall be reported by gNB to the TCE. ##### 4.1.1.9.5 NG-RAN activation mechanisms for C-MDT data collections for split RAN architecture For split RAN architecture, the C-MDT data collection activation mechanism is only applicable if a gNB-CU-CP receives a C-MDT activation as specified in subclause 4.1.1.9.x step 2. In addition to the procedures specified in subclause 4.1.1.9.3, the gNB-CU-CP shall configure the selected UE with an Immediate MDT session and a Logged MDT session as specified in subclause 4.1.1.9.4 step 3. ##### *** non-relevant parts are omitted ***

From the above excerpted text from TS 32.422, the following aspects should be clear concerning the C-MDT functionality:

- How the OAM provides the C-MDT configuration to NG-RAN  OAM provides the C-MDT configuration to gNBs in a certain area (i.e., all participating NR-RAN nodes involved in the same C-MDT job) via either

  - two distinct Trace Session activation requests (one for Immediate MDT, another for Logged MDT) with the same Trace Reference (TR), or

  - a single Trace Session activation request with Job Type IMMEDIATE_MDT_AND_LOGGED_MDT;

- What is the content of the C-MDT configuration needed to configure the UE measurements  The Trace Session activation request(s) includes the legacy parameters specified in step 1 of subclause 4.1.1.9.2 in TS 32.422 (i.e., Job Type, area scope, list of measurements, TR, TCE IP address, etc.) as well as a new parameter (TRSR Prefix Configuration) whose purpose is twofold:

  - it identifies the reserved range of Trace Recording Session Reference (TRSR) in use for C-MDT only that the gNB shall consider when assigning the TRSR to the UE selected by the gNB itself for a C-MDT session, and

  - it is used to identify that the concerned Trace Session activation request(s) is(are) related to a C-MDT session;

- How the gNB makes use of the C-MDT configuration received from OAM  in addition to the gNB behaviour for the legacy MDT functionality as described in subclause 4.1.1.9.2 of TS 32.422 – i.e., UE selection (step 3), activation of MDT to the selected UEs (step 4), handling of the retrieved UE’s MDT report (step 6 and step 7) and transfer to TCE of the retrieved MDT report from the UE (step 8):

  - the gNB shall configure the selected UE with both Immediate MDT and Logged MDT using the configurations from the Trace Session activation request(s);

  - the gNB shall assign a unique TRSR to the selected UE – allocated based on the new parameter TRSR Prefix Configuration – which is valid for both Immediate MDT and Logged MDT;

  - at UE’s RRC state transition from idle/inactive to connected, the new serving gNB retrieves the logged MDT report from the UE which includes the TR and TRSR, and the gNB shall identify whether the UE has been previously selected for C-MDT by comparing the TR in the UE’s MDT report with the one included in the MDT configuration received from OAM: if this is the case, and if UE selection criteria are still met (i.e., user consent granted and new serving gNB is in the C-MDT area scope), the new serving gNB shall reconfigure the UE with both Immediate MDT and Logged MDT;

  - either after a UE handover (mobility in connected state) or a UE’s RRC state transition from idle/inactive to connected, if the new serving gNB needs to assign a new TRSR for this UE, the new serving gNB shall keep the correlation between the old TRSR and the new TRSR and report this correlation to the TCE.

Based on the above summary, we now discuss the specification impact in order to support C-MDT in NG-RAN.

# 3. Specification impact to support C-MDT in NG-RAN

## 3.1 C-MDT in non-split gNB scenario

It is our understanding that, as part of the problem statement for the C-MDT functionality as elaborated in TR 38.743 [2] clause 5.4.1, RAN3 has to work on the measurement continuity task (Problem A in TR 38.743) only – the trace correlation task (Problem B) is out of RAN3 scope.

### 5.4.1 Use Case Description The problem of continuous data collection for management-based MDT can be described as follows: a UE in the NG-RAN can be configured with management-based Logged MDT when in RRC_Idle and RRC_Inactive states and with management-based Immediate MDT when in RRC_Connected state. Differently from signalling-based MDT, in management-based MDT, a UE is not uniquely identified in the MDT activation. Therefore, when a UE transits to RRC_Connected state from RRC_Idle/RRC_Inactive (during which Logged MDT data have been collected) or when a UE is handed over between gNBs, the network does not have standardized means to select again the same UE for continuous MDT for subsequent MDT data collection. The Data Collection continuity in this scenario can be split into two tasks as below: - Problem A (measurement continuity): how to ensure that the same UE collecting MDT measurements during the same RRC state and across different RRC states. - Problem B (trace correlation): how to ensure that the TCE which eventually receives the MDT reports can associate the received logged and immediate MDT measurements to a continuous data collection period from the same UE.

To address the measurement continuity task, the standard impact to support the two scenarios of mobility in connected mode and UE’s RRC state transition from idle/inactive to connected shall be discussed for TS 38.423 [8].

Mobility in RRC_CONNECTED mode

In case the UE selected by a gNB for C-MDT is handed over to a target gNB, it is our understanding that an indication needs to be sent from the source gNB to the target gNB so that the target gNB is able to identify that the handed over UE is a C-MDT UE, and such indication can be provided within the HANDOVER REQUEST message.

Observation 1: In case of handover (mobility in connected mode) of the C-MDT UE from source gNB to target gNB, an indication needs to be provided to the target gNB in the HANDOVER REQUEST message so that the handed over UE is identified by the target gNB as a C-MDT UE.

The current HANDOVER REQUEST message includes the Trace Activation IE, which is defined in clause 9.2.3.55 of XnAP and it includes the NG-RAN Trace ID IE defined in clause 9.2.3.97 as follows:

#### 9.2.3.97 NG-RAN Trace ID This IE defines the NG-RAN Trace ID. IE/Group Name; Presence; Range; IE type and reference; Semantics description. NG-RAN Trace ID; M; OCTET STRING (SIZE(8)); This IE is composed of the following: Trace Reference defined in TS 32.422 [23] (leftmost 6 octets, with PLMN information encoded as in 9.2.2.4), and Trace Recording Session Reference defined in TS 32.422 [23] (last 2 octets).. a

Therefore, the target gNB already receives the TR (included in the MDT configuration as received from the OAM) and TRSR (allocated by the source gNB) of a MDT session encoded in the Trace Activation IE, if included by the source gNB. In case of C-MDT, the source gNB includes the Trace Activation IE in the HANDOVER REQUEST message which mandatorily includes the NG-RAN Trace ID IE, and hence the target gNB will be made aware not only of the TR (which it also received from the OAM, assuming the target gNB is within the C-MDT area scope) but also the TRSR allocated by the source gNB (based on the new TRSR Prefix Configuration parameter).

Proposal 1: RAN3 to agree to re-use the existing NG-RAN Trace ID IE included in the Trace Activation IE of the HANDOVER REQUEST message to allow the target gNB to identify that the handed over UE was previously selected by the source gNB for a C-MDT session.

The procedural text of the Trace Activation IE in XnAP needs to be enhanced to cover the C-MDT case, which encompasses both Immediate MDT and Logged MDT, by introducing proper changes in the MDT Activation IE included in the Trace Activation IE. Specifically, a new codepoint “Immediate MDT and Logged MDT” needs to be introduced in the MDT Activation IE, so that the target gNB is made aware that it needs to re-configure the C-MDT UE with both Immediate MDT and Logged MDT.

Proposal 2: RAN3 to agree that a new codepoint “Immediate MDT and Logged MDT” needs to be introduced in the MDT Activation IE included in the Trace Activation IE of the HANDOVER REQUEST message to allow the target gNB to re-configure the UE with both Immediate and Logged MDT.

The proposed new codepoint “Immediate MDT and Logged MDT” is actually linked to the new MDT mode that needs to be introduced in the CHOICE structure “MDT Mode” of the MDT Configuration-NR IE (clause 9.2.3.126 of XnAP). The new MDT Mode Immediate MDT and Logged MDT-NR includes the list of measurements that could be configured in a C-MDT session, and it is our understanding that in C-MDT only those measurements that are common in both Immediate MDT and Logged MDT should be considered.

Proposal 3: RAN3 to agree that measurements to be configured in a C-MDT session should be the ones which are common to be collected in both Immediate MDT and Logged MDT.

By checking the current specification, we believe that the measurements that could be commonly configured in both Immediate MDT and Logged MDT are M1 measurements (RSRP, RSRQ, SINR for the serving cell and for intra-frequency/inter-frequency/inter-RAT neighbour cells, including cell/beam level measurement for NR cells only), as well as Bluetooth, WLAN and sensor measurements. We think that at least M1 measurements should be considered, and RAN3 may discuss if other measurements are needed in the context of C-MDT.

Proposal 4: RAN3 to agree to include at least RSRP, RSRQ and SINR measurements as the list of measurements which can be configured in C-MDT. Whether additional measurements are to be considered needs further discussion.

UE’s RRC state transition from idle/inactive to connected mode

In case the UE selected by a gNB for C-MDT went to idle/inactive and, after a while, it transits back to connected mode under the coverage of a new serving gNB, it is our understanding that a similar indication as previously discussed for the handover case needs to be sent from the old serving gNB to the new serving gNB, so that the new serving gNB is able to identify that the UE is a C-MDT UE, and such indication can be provided within the RETRIEVE UE CONTEXT RESPONSE message.

Observation 2: In case of UE’s RRC state transition from idle/inactive to connected mode, an indication needs to be provided to the new serving gNB in the RETRIEVE UE CONTEXT RESPONSE message so that the UE is identified by the new serving gNB as a C-MDT UE.

Exactly as for the HANDOVER REQUEST message, the current RETRIEVE UE CONTEXT RESPONSE message includes the Trace Activation IE which, in turn, includes the NG-RAN Trace ID IE defined in clause 9.2.3.97. Therefore, the new serving gNB already receives the TR (included in the MDT configuration as received from the OAM) and TRSR (allocated by the old serving gNB) of a MDT session encoded in the Trace Activation IE, if included by the old serving gNB. In case of C-MDT, the old serving gNB includes the Trace Activation IE in the RETRIEVE UE CONTEXT RESPONSE message which mandatorily includes the NG-RAN Trace ID IE, and hence the new serving gNB will be made aware not only of the TR (which it also received from the OAM, assuming the new serving gNB is within the C-MDT area scope) but also the TRSR allocated by the old serving gNB (based on the new TRSR Prefix Configuration parameter).

Proposal 5: RAN3 to agree to re-use the existing NG-RAN Trace ID IE included in the Trace Activation IE of the RETRIEVE UE CONTEXT RESPONSE message to allow the new serving gNB to identify that the UE was previously selected by the old serving gNB for a C-MDT session.

In our understanding, with similar changes suggested in Proposal 2 (new codepoint “Immediate MDT and Logged MDT” to be introduced in the MDT Activation IE included in the Trace Activation IE of the RETRIEVE UE CONTEXT RESPONSE message) and Proposal 4 (at least RSRP, RSRQ and SINR measurements as the list of measurements which can be configured in C-MDT), the task of measurement continuity for the scenario of UE’s RRC state transition from idle/inactive to connected is addressed without further enhancements in the existing XnAP specification.

Proposal 6: RAN3 to agree that the changes in Proposal 2 and Proposal 4 allow to also address the measurement continuity for the scenario of UE’s RRC state transition from idle/inactive to connected.

## 3.2 C-MDT in split gNB scenario

In the latest version of Rel-19 TS 32.422 SA5 introduced a new clause related to NG-RAN activation mechanisms for C-MDT data collection for split RAN architecture:

##### 4.1.1.9.5 NG-RAN activation mechanisms for C-MDT data collections for split RAN architecture For split RAN architecture, the C-MDT data collection activation mechanism is only applicable if a gNB-CU-CP receives a C-MDT activation as specified in subclause 4.1.1.9.x step 2. In addition to the procedures specified in subclause 4.1.1.9.3, the gNB-CU-CP shall configure the selected UE with an Immediate MDT session and a Logged MDT session as specified in subclause 4.1.1.9.4 step 3.

From the above, it is our understanding that the OAM provides the C-MDT configuration activation only to the gNB-CU-CP, which will be in charge of selecting the UEs for C-MDT, and that will propagate the C-MDT configuration towards the gNB-DUs and/or gNB-CU-UPs over F1 and E1, respectively, in case the C-MDT activation involves measurements collected by multiple nodes controlled by the same gNB-CU-CP.

Observation 3: The C-MDT configuration activation can be sent by the OAM to the gNB-CU-CP only.

In light of Proposal 4 which pertains the set of measurements that could be configured in both Immediate MDT and Logged MDT for C-MDT, and considering that:

- over the F1 interface it is specified in TS 38.473 [9] that, as part of the MDT Configuration IE in the Trace Activation IE of the UE CONTEXT SETUP REQUEST message, only measurements M2 (Power Headroom measurement by UE), M5 (Average UE throughput measurement separately for DL and UL), M6 (Packet Delay measurement separately for DL and UL) and M7 (Packet loss rate measurement separately for DL and UL) can be configured, and these measurements can only be performed under an Immediate MDT session – in fact, the MDT Activation IE in the MDT Configuration IE has only two codepoints “Immediate MDT only” and “Immediate MDT and Trace”;

- over the E1 interface, it is specified in TS 37.483 [10] that, as part of the MDT Configuration IE in the Trace Activation IE of the BEARER CONTEXT SETUP REQUEST message, only measurements M4 (PDCP SDU Data Volume measurement separately for DL and UL), M6 (Packet Delay measurement separately for DL and UL) and M7 (Packet loss rate measurement separately for DL and UL) can be configured, and these measurements can only be performed under an Immediate MDT session – in fact, the MDT Activation IE in the MDT Configuration IE has only two codepoints “Immediate MDT only” and “Immediate MDT and Trace”,

our thinking is that C-MDT could actually be supported in split architecture without additional standard impacts.

Proposal 7: RAN3 to acknowledge that C-MDT could actually be supported in split gNB architecture without additional standard impacts.

Draft CRs for TS 37.320 and TS 38.300 are in [11] and [12], respectively, while a CR to TS 38.423 is in [13].

# 4. Conclusion

Based on the discussion in this paper, we provide the following proposals:

C-MDT in non-split gNB scenario

Observation 1: In case of handover (mobility in connected mode) of the C-MDT UE from source gNB to target gNB, an indication needs to be provided to the target gNB in the HANDOVER REQUEST message so that the handed over UE is identified by the target gNB as a C-MDT UE.

Proposal 1: RAN3 to agree to re-use the existing NG-RAN Trace ID IE included in the Trace Activation IE of the HANDOVER REQUEST message to allow the target gNB to identify that the handed over UE was previously selected by the source gNB for a C-MDT session.

Proposal 2: RAN3 to agree that a new codepoint “Immediate MDT and Logged MDT” needs to be introduced in the MDT Activation IE included in the Trace Activation IE of the HANDOVER REQUEST message to allow the target gNB to re-configure the UE with both Immediate and Logged MDT.

Proposal 3: RAN3 to agree that measurements to be configured in a C-MDT session should be the ones which are common to be collected in both Immediate MDT and Logged MDT.

Proposal 4: RAN3 to agree to include at least RSRP, RSRQ and SINR measurements as the list of measurements which can be configured in C-MDT. Whether additional measurements are to be considered needs further discussion.

Observation 2: In case of UE’s RRC state transition from idle/inactive to connected mode, an indication needs to be provided to the new serving gNB in the RETRIEVE UE CONTEXT RESPONSE message so that the UE is identified by the new serving gNB as a C-MDT UE.

Proposal 5: RAN3 to agree to re-use the existing NG-RAN Trace ID IE included in the Trace Activation IE of the RETRIEVE UE CONTEXT RESPONSE message to allow the new serving gNB to identify that the UE was previously selected by the old serving gNB for a C-MDT session.

Proposal 6: RAN3 to agree that the changes in Proposal 2 and Proposal 4 allow to also address the measurement continuity for the scenario of UE’s RRC state transition from idle/inactive to connected.

C-MDT in split gNB scenario

Observation 3: The C-MDT configuration activation can be sent by the OAM to the gNB-CU-CP only.

Proposal 7: RAN3 to acknowledge that C-MDT could actually be supported in split gNB architecture without additional standard impacts.

Draft CRs for TS 37.320 and TS 38.300 are in R3-257187 and R3-256984, respectively, while a CR to TS 38.423 is in R3-257188.

# 5. Reference

- RP-251245, “Revised WID Enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#108

- 3GPP TR 38.743 “Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN”, Rel-19

- RP-240323, “Revised SID on Study on enhancements for Artificial Intelligence (AI)/Machine Learning (ML) for NG-RAN” (ZTE, NEC), 3GPP RAN#103

- R3-253958, Reply LS on Continuous MDT, Response to R3-253024 (S5-252114), RAN3 (Ericsson)

- R3-253024 (S5-252114), Reply LS to LS on Continuous MDT, Response to R3-250891, SA5 (Huawei, Nokia)

- R3-256523 (S5-254110), Reply to LS on Continuous MDT, Response to R3-253958, SA5 (Ericsson, Deutsche Telekom, CATT, ZTE, Huawei)

- 3GPP TS 32.422, Telecommunication management; Subscriber and equipment trace; Trace control and configuration management

- 3GPP TS 38.423, NG-RAN; Xn Application Protocol (XnAP)

- 3GPP TS 38.473, NG-RAN; F1 Application Protocol (F1AP)

- 3GPP TS 37.483, E1 Application Protocol (E1AP)

- R3-257187, Introducing support for Continuous management-based MDT (C-MDT), Rel-19 Cat. F draftCR to TS 37.320, Huawei, Jio Platforms, Ofinno

- R3-256984, Correction on support for Continuous management-based MDT (C-MDT), Rel-19 Cat. F draftCR to TS 38.300, Huawei, Jio Platforms, Ofinno

- R3-257188, Introducing support for Continuous management-based MDT (C-MDT), Rel-19 Cat. F CR to TS 38.423, Huawei, Jio Platforms, Ofinno