3GPP TSG-RAN WG3 Meeting #129-bis	R3-257192

Prague, CZ, Oct 13-17, 2025

Agenda Item:	10.2.1

Source:	THALES, Echostar

Title:	6G RAN overall architecture to support NTN

Document for :	Discussion

# 1	Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting [1] and updated at RAN#109 [2] and states for an harmonized radio interface between TN and NTN:

(1) Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] a) Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. b) Energy efficiency and energy saving: both for network and device. c) Enhanced spectral efficiency. d) Enhanced overall coverage, focus on cell-edge performance and UL coverage. e) Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. f) Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. g) Target scalable and forward compatible design for diverse device types. h) Improved spectrum utilization and operations taking into account diverse spectrum allocations. i) Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. j) Aim at a harmonized 6G Radio design for TN and NTN, including their integration. k) System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

In this document we will discuss how to achieve an harmonized 6G radio access architecture, the different payload, orbits and splits that will help us to enable the 6G Scenarios and requirements mentioned on the 3GPP TR 38.914, “Study on 6G Scenarios and requirements” [3]

# 2	Discussion

## 2.1	Harmonized 6G Radio access design for TN and NTN

An harmonized 6G radio access architecture will enables seamless integration between terrestrial and non-terrestrial in 6G systems. By natively incorporating non-terrestrial networks (NTN) in 6G It will allow to deliver unprecedented levels of performance, resiliency, and coverage, hence fulfilling the evolving demands of the future mobile ecosystem. Achieving such integration requires a unified radio interface that seamlessly supports both terrestrial and satellite connections. This harmonization should address:

- Compatibility with diverse satellite orbits, including VLEO (Very LEO), LEO, MEO, and GSO (among which GEO).

- Robustness against challenges like Doppler shifts, significant and variable round-trip times (RTT), and delay spreads.

Consider support of a unified 6G radio access architecture that enables seamless integration between terrestrial and non-terrestrial networks

## 2.2	6G Radio Acces Network architecture for a native support of NTN

### 2.2.1	NTN Payload types and scenarios

Figure 1 : Proposal for a 6G NTN deployment scenario

Below are some considerations for practical NTN scenarios for 6G. The different scenarios are illustrated on the figure above. The following orbits should be considered:

- VLEO (very low Earth orbit) based scenario at minimum altitude of 300 km: Satellites operating at this altitude benefit from reduced latency due to their proximity to the surface of Earth, which can enhance communication and observation capabilities. At this altitude, the Round Trip Delay becomes closer to the one of terrestrial extended cells. Further, this closer proximity to Earth can offer more accurate ground metrics, and potentially lower cost for certain types of satellite operations. However, deployment at a VLEO orbit would require a very high number of satellites to generate overlapping footprint and hence service continuity in the targeted coverage. This is now made possible with the ever decreased launch cost. Typically, a maximum satellite beam footprint diameter of [250] km or lower should be supported.

- LEO (low Earth orbit) based scenarios (respectively 600 and 1200 km altitude): While these scenarios are already supported in the context of 5G, the maximum beam footprint can be reduced in the context of 6G since the targeted service rate is likely higher than 5G NTN capability and hence require satellites able to achieve high antenna gain and hence generate smaller beams. Typically, a maximum satellite beam footprint diameter of [250] km or lower should be supported.

- MEO (medium Earth orbit) based scenarios (10000 km altitude): In line with the above, typically, a maximum satellite beam footprint diameter of 250 km or lower should be supported. It offers a global coverage at a reduced cost.

- GSO (geostationary, including GEO) based scenarios (35786 km altitude): Offer a global coverage with more capacity embarked with typical beam footprint size of 250 up to 3500 km.

Additionally, the 6G radio access should also support multi orbit space segment combining at least a GSO with an NGSO based scenario, illustrated in Figure 1.

### 2.2.2	Types of cells supported on NTN :

The different types of cells supported in Non-Terrestrial Networks (NTN) are:

1. Quasi-Earth Fixed Cells: Quasi-Earth fixed cells are radio coverage areas on the ground that are almost, but not completely, stationary relative to the Earth’s surface. These cells are typically associated with non-geostationary satellites (such as those in LEO and MEO) whose orbits cause their coverage footprints to move slowly or only slightly shift on the ground.

2. Earth Moving Cells: Earth moving cells are coverage zones that move continuously across the Earth’s surface. These are commonly linked to satellites in orbits where the coverage area does not stay fixed to one location but sweeps across different regions as the satellite passes overhead.

3. Earth Fixed Cells: Earth fixed cells are coverage zones that remain fixed relative to a specific geographic location on the Earth’s surface. This is mainly possible with geostationary satellites, which orbit at the same rotational speed as the Earth and thus "hover" over one position. The coverage area of an Earth fixed cell does not change over time.

Table 1 Types of cells

| Cell Type | Coverage Movement | Typical Satellite Orbit |
| --- | --- | --- |
| Quasi-Earth fixed | Slightly moves | LEO/MEO (with tracking) |
| Earth moving | Continuously changes place | LEO/MEO |
| Earth fixed | Stays over same location | GEO |

### 2.2.3	NTN Scenarios

The table below provides the list of reference practical scenarios (single orbit) to be supported by the 6G radio access with the related parameters:

| NTN Scenarios | Very LEO | LEO-600 | LEO-1200 | MEO | GSO |
| --- | --- | --- | --- | --- | --- |
| Orbit type | circular around the earth | circular around the earth | circular around the earth | circular around the earth | station keeping around a nominally fixed position in terms of elevation/azimuth with respect to a given earth point (NOTE 6) |
| Altitude | 300 km (NOTE 1) | 600 km | 100 km | 2000 to ~35000 km | 35,786 km |
| Earth-fixed beams | Yes (Satellite generated steerable beams, NOTE 2) |  |  |  |  |
| Max beam footprint size (edge to edge) regardless of the elevation angle (NOTE 3) | [<250] km | [<250] km | [<250] km | [<250] km | [<250] km |
| Min Elevation angle for both sat-gateway and User Equipment | 10° for service link and 10° for feeder link |  |  |  |  |
| Max distance between satellite and User Equipment at min elevation angle | 1160 km | 1 932 km | 3 131 km | TBD km | 40 581 km |
| Max Round Trip Delay (propagation delay only NOTE 4) | 15.48 ms | 25.77 ms | 41.77 ms | 180 ms | 541.46 ms |
| Max Doppler shift (earth fixed user equipment) (NOTE 5) | 24.23ppm | 22.68ppm | 20.04ppm | TBD ppm | 0.93ppm |
| Max Doppler shift variation (earth fixed user equipment) (NOTE 4) | TBD ppm/s | 0.27 ppm/s | 0.13 ppm/s | TBD ppm/s | 0.000 045 ppm/s |
| NOTE 1: VLEO is characterized by altitudes ranging from approximately 300 to 600 kilometers above Earth. However, the lower the altitude, the higher the number of satellites is required to ensure service continuity and the higher the atmospheric drag which would need constant propulsion and re-boosting and hence reduced lifetime. NOTE 2: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite. NOTE 3: This beam size refers to the Nadir pointing of the satellite. NOTE 4: Assuming transparent payload with service and feeder links and return NOTE 5: Max Doppler shift and Max Doppler shift variation in the absence of any device pre-compensation of satellite Doppler shift on the service link. NOTE 6: In case of GSO, the inclination may be non-zero. This means the satellite’s ground track may trace a figure-eight pattern as seen from Earth. |  |  |  |  |  |

## 2.2.4	Inter-Satellite Link support

Regenerative satellite payloads enable routable front or backhaul traffic which can exploit Inter-satellite Links (ISLs) within the underlying transport network. ISLs can be deployed between two satellites of the same constellation (intra-/inter- orbital plane) or between two satellites of different constellations/orbits (e.g., between LEO and GSO satellites). ISL will allow to provide service in areas where there are no NTN-gateways and provide coverage of remote areas like the oceans. Inter-satellite links may operate in RF frequency or optical. The route taken between the GW and the satellite may thus change, adding or removing inter-satellite links to reach the GW.

Note that there are different types of inter satellite links:

- Between satellite of the same orbit which can further be distinguished between

  - Intra-plane ISL: between (adjacent) satellites of the same orbital plane

  - Inter-plane ISL: between satellites of (adjacent) orbital planes

- Between satellites of different orbits (e.g., GSO and NGSO, or LEO and MEO)

Take into account the predictability and the resiliency of the Inter-Satellite Link in the design of the NTN architecture

## 2.2.5	Multi-orbit architecture

Regenerative satellite payloads enable routable front or backhaul traffic which can exploit Intersatellite Links (ISLs) within the underlying transport network. ISLs can be deployed between two satellites of the same constellation (intra-/inter- orbital plane) or between two satellites of different constellations/orbits (e.g., between LEO and GSO satellites). ISL will allow to provide service in areas where there are no NTN-gateways and provide coverage of remote areas like the oceans. Inter-satellite links may operate in RF frequency or optical bands. The route taken between the GW and the satellite may thus change, adding or removing inter-satellite links to reach the GW.

Study architecture, interfaces adaptations and NGSO/GSO coordination in order to support multi-orbit architecture in 6G

## 2.3	6G RAN internal functional split and interfaces for NTN

The split architecture, initially developed for 5G networks, segments the radio access network into a centralized unit (CU) responsible for higher-layer processing and core network connectivity and a distributed unit (DU), which handles lower-layer radio functions. This separation, coupled via the F1 interface, offers significant deployment flexibility, especially advantageous in the NTN (Non-Terrestrial Network) context where satellites and ground stations can host different network functions.

We discussed in R3-25XXXX [4] multiple split scenarios for NTN, with each model distributing the RAN functions differently between the ground and space segments. These architectural options range from scenarios where only the radio unit (RU) is on the satellite with processing centralized on the ground, to more complex configurations where even part of the core network and edge computing can be deployed onboard satellites. Each scenario comes with its own performance, latency, energy, and implementation constraints, and targets different service types such as direct-to-device, broadband data, or critical low latency communication.

Supporting base station split architecture for NTN offers several significant benefits and deployment options. Some deployment scenarios are described on the table below :

|  | Scenario |  | Benefits | Constraints |
| --- | --- | --- | --- | --- |
|  | On board the satellite | On the ground |  |  |
| (1) | RU | DU + CU + Core Network | Reserve all the energy and computation resources to maximise the service link performances | Potential stringent latency requirements and important throughput needed across the feeder/Inter Satellite links |
| (2) | RU on one satellite DU and CU on another | Core network | Balance the computation and power requirements between satellites | High interactions requirements (ISL based) between both types of satellites Potential high rate of RU and DU+CU association |
| (3) | RU+DU | CU + Core network | Reduced computation and memory footprint requirements on board the satellite while limiting the feeder link throughput requirements | High change rate of association between DU and CU |
| (4) | RU+DU+CU | Core network | Ensure resilient RAN operations (except for authentification and AMF mobility) | Important memory sand computation footprint required on board the satellite payload, limiting the energy and computation allocated to the service link throughput performance |
| (5) | RU+DU+CU+ part of the core network + edge computing functions | Part of the core network | Ensure higher network resiliency and lower connection set-up delay | Fast changes of association between network functions to be mitigated |

Each scenario exhibits advantages and constraints and targets different types of services, e.g. direct to device, broadband, low latency services, UE-sat-UE connectivity. Scenarios (2) and (5) can also be implemented in a multi-orbit based architecture (between NGSO and GSO)..

For the scenarios (1) and (2) a fronthaul interface has to be specified to handle high latency and jitter but as endorsed at the RAN#109 plenary, the interface between RU and 6G NodeB is not specified in 3GPP and let to implementation and other standardization bodies (e.g. O-RAN) [RAN#109 chair’s notes].

For the scenario (3), the functional split takes place between a DU (low layers) and CU (high layers). The DU and CU split can be flexible according to the dynamic context (traffic load, service provided,…). The transport network layer should accommodate the latency of feeder link and possibly Inter-Satellite Links (ISL).

Note that scenario (4) corresponds to a regenerative architecture with fully gNB embarked and scenario (5) an architecture that can be described in SA WG2 for multi-orbit.

By leveraging the functional split between, NTN deployments can achieve greater flexibility and efficiency in network design and resource allocation. This separation allows part of the baseband processing to be centralized on ground stations or cloud infrastructure while distributing real-time radio functions closer to the satellite or airborne platform.

As a result, it reduces latency, optimizes bandwidth usage over satellite links, and enhances scalability by enabling dynamic allocation of processing resources depending on the traffic demand. Moreover, the split architecture facilitates easier upgrades and maintenance by isolating components, thus improving the overall operational resilience and adaptability of NTN systems to varying mission requirements and environments.

Consider the different NTN architectures (e.g. CU-DU, RU-DU, etc.) and related impacts in the 6G RAN internal functional split study

## 2.4	Support of OAM for TN-NTN orchestration

The integration of Operations, Administration, and Maintenance (OAM) support within TN-NTN orchestration is essential for achieving unified lifecycle management and end-to-end service assurance across hybrid terrestrial and non-terrestrial networks. OAM in this context encompasses a broad spectrum of technical functions, including real-time telemetry, automated fault detection and isolation, configuration management, software and firmware updates, as well as proactive performance and security monitoring. Such capabilities are particularly challenging in a multi-domain environment where reachability, latency, network topology, and management protocols may differ substantially between terrestrial and non-terrestrial segments.

- Proposal 4	RAN3 in coordination with SA5 define the necessary information to be provided by the OAM to the access nodes for providing the NTN access and the NTN-TN service continuity

# 3	Conclusion

In this document, the following proposals were made:

- Consider support of a unified 6G radio access architecture that enables seamless integration between terrestrial and non-terrestrial networks

- Take into account the predictability and the resiliency of the Inter-Satellite Link in the design of the NTN architecture

- Study architecture, interfaces adaptations and NGSO/GSO coordination in order to support multi-orbit architecture in 6G

- Consider the different NTN architectures (e.g. CU-DU, RU-DU, etc.) and related impacts in the 6G RAN internal functional split study

- RAN3 in coordination with SA5 define the necessary information to be provided by the OAM to the access nodes for providing the NTN access and the NTN-TN service continuity

# References

RP-252912, “Revised SID: Study on 6G Radio”, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN, September 2025.

RP-250810, “Revised SID: Study on 6G Scenarios and requirements”, CMCC, Verizon, NTT Docomo, Deutsche Telekom, Incheon, KR, March 2025.

3GPP TR 38.914, “	Study on 6G Scenarios and requirements”, v0.1.1, Beijing, CN, September 2025.

R3-257193 “6G RAN internal functional split and interfaces for NTN” THALES, October 2025