3GPP TSG-RAN WG3 Meeting #129	R3-255055

# Bangalore, India, 25-29 August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.300 | CR |  | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (BL CR to 38.300) Introduction of NR Femto Architecture and Protocol Aspects |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Nokia, TMO US, AT&T, Verizon Wireless, BT, Charter, Huawei, LG Electronics, Samsung, Lenovo, Baicells, ZTE, NEC, CATT, Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_WAB_5GFemto-Core |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduce NR Femto and NR Femto GW in NG-RAN architecture description |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce logical architecture and protocol stacks for NR Femto and NR Femto GW |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description of NR Femto and NR Femto GW in NG-RAN stage 2 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.413 CR 1232 TS 38.410 CR 0052 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R3-245864: endorsed as BL at RAN3 #125bis R3-247936: endorsed as BL at RAN3 #126 R3-250943: endorsed as BL at RAN3 #127 R3-252529: endorsed as BL at RAN3 #127bis R3-254017: endorsed as BL at RAN3 #128 |  |  |  |  |  |  |  |  |

START OF CHANGES

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 38.401: "NG-RAN; Architecture description".

[5]	3GPP TS 33.501: "Security Architecture and Procedures for 5G System".

[6]	3GPP TS 38.321: "NR; Medium Access Control (MAC) protocol specification".

[7]	3GPP TS 38.322: "NR; Radio Link Control (RLC) protocol specification".

[8]	3GPP TS 38.323: "NR; Packet Data Convergence Protocol (PDCP) specification".

[9]	3GPP TS 37.324: " E-UTRA and NR; Service Data Protocol (SDAP) specification".

[10]	3GPP TS 38.304: "NR; User Equipment (UE) procedures in Idle mode and RRC Inactive state".

[11]	3GPP TS 38.306: "NR; User Equipment (UE) radio access capabilities".

[12]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[14]	3GPP TS 22.168: "Earthquake and Tsunami Warning System (ETWS) requirements; Stage 1".

[15]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[16]	3GPP TS 38.410: "NG-RAN; NG general aspects and principles".

[17]	3GPP TS 38.420: "NG-RAN; Xn general aspects and principles".

[18]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[19]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[20]	3GPP TS 38.202: "NR; Physical layer services provided by the physical layer"

[21]	3GPP TS 37.340: "NR; Multi-connectivity; Overall description; Stage-2".

[22]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[23]	IETF RFC 4960 (2007-09): "Stream Control Transmission Protocol".

[24]	3GPP TS 26.114: "Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[25]	Void.

[26]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[27]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

[28]	3GPP TS 24.501: "NR; Non-Access-Stratum (NAS) protocol for 5G System (5GS)".

[29]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[30]	3GPP TS 38.415: "NG-RAN; PDU Session User Plane Protocol".

[31]	3GPP TS 38.340: "NR; Backhaul Adaptation Protocol (BAP) specification".

[32]	3GPP TS 38.470: "NG-RAN; F1 application protocol (F1AP) ".

[33]	3GPP TS 38.425: "NG-RAN; NR user plane protocol".

[34]	3GPP TS 23.216: "Single Radio Voice Call Continuity (SRVCC); Stage 2".

[35]	3GPP TS 38.101-2: "User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[36]	3GPP TS 38.101-3: "User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios".

[37]	3GPP TS 37.213: "Physical layer procedures for shared spectrum channel access".

[38]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[39]	3GPP TS 22.104 "Service requirements for cyber-physical control applications in vertical domains".

[40]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[41]	3GPP TS 23.285: "Technical Specification Group Services and System Aspects; Architecture enhancements for V2X services".

[42]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[43]	3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

[44]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[45]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[46]	3GPP TS 26.517: "5G Multicast-Broadcast User Services; Protocols and Formats".

[47]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[48]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[49]	3GPP TS 28.541: "5G Network Resource Model (NRM)".

[50]	3GPP TS 38.423: "NG-RAN; Xn Application Protocol (XnAP)".

[51]	NIMA TR 8350.2, Third Edition, Amendment 1, 3 January 2000: "DEPARTMENT OF DEFENSE WORLD GEODETIC SYSTEM 1984".

[52]	3GPP TS 38.211: "NR; Physical channels and modulation".

[53]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS)".

[54]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[55]	3GPP TS 24.554: "Technical Specification Group Core Network and Terminals; Proximity-services (ProSe) in 5G System (5GS) protocol".

[56]	3GPP TS 38.214: "Technical Specification Group Radio Access Network; NR; Physical layer procedures for data".

[57]	3GPP TR 38.835: "NR; Study on XR enhancements for NR".

[58]	3GPP TS 26.522: "5G Real-time Media Transport Protocol Configurations".

[59]	3GPP TS 38.215: "NR; Physical layer measurements".

[60]	3GPP TS 23.256: "Support of Uncrewed Aerial Systems (UAS) connectivity, identification and tracking; Stage 2".

[61]	IETF RFC 9330: "Low Latency, Low Loss, Scalable Throughput (L4S) Internet Service: Architecture".

[62]	IETF RFC 9331: "Explicit Congestion Notification (ECN) Protocol for Very Low Queuing Delay (L4S)".

[63]	IETF RFC 9332: "Dual-Queue Coupled Active Queue Management (AQM) for Low Latency, Low Loss, and Scalable Throughput (L4S)".

[64]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management".

[65]	3GPP TS 38.351: "NR; Sidelink Relay Adaptation Protocol (SRAP) Specification".

[xx]	3GPP TS 33.545: "Security aspects of NR Femto".

NEXT CHANGE

# 3	Abbreviations and Definitions

## 3.1	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], in TS 36.300 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 36.300 [2].

UNCHANGED PART OMITTED

SCS	SubCarrier Spacing

SD	Slice Differentiator

SDAP	Service Data Adaptation Protocol

SDT	Small Data Transmission

SD-RSRP	Sidelink Discovery RSRP

SeGW	Security Gateway

SFI-RNTI	Slot Format Indication RNTI

SHR	Successful Handover Report

SIB	System Information Block

SI-RNTI	System Information RNTI

SLA	Service Level Agreement

UNCHANGED PART OMITTED

NEXT CHANGE

# 4	Overall Architecture and Functional Split

UNCHANGED PART OMITTED

## 4.X	Support of NR Femto nodes

### 4.X.1	Architecture

Figure 4.X.1-1 shows a logical architecture for the NR Femto node that has a set of NG interfaces to connect the NR Femto node to the 5GC.

Figure 4.X.1-1: NR Femto node Logical Architecture

NOTE: the SeGW is out of RAN scope.

The NR Femto node may directly connect to the 5GC. The NG-RAN architecture may also deploy an NR Femto Gateway (NR Femto GW) to allow the concentration of the NG-C interface between the NR Femto node and the 5GC. Based on implementation, the transport of NG-U between the NR Femto node and the 5GC may be optionally concentrated in the NR Femto GW.

For the NR Femto node, the NG-C interface is defined as the interface:

-	Between the NR Femto GW and the Core Network;

-	Between the NR Femto node and the NR Femto GW;

-	Between the NR Femto node and the Core Network.

The NR Femto GW appears to the AMF as a gNB. The NR Femto GW appears to the NR Femto node as an AMF. The NG interface between the NR Femto node and the 5GC is the same regardless whether the NR Femto node is connected to the 5GC via an NR Femto GW or not.

The functions supported by the NR Femto node shall be the same as those supported by a gNB (with possible exceptions e.g. NNSF when the NR Femto node connects via the NR Femto GW) and the procedures run between an NR Femto node and the 5GC shall be the same as those between a gNB and the 5GC.

Xn-connectivity is supported between NR Femto nodes and between NR Femto nodes and gNBs, independent of whether any of the involved NR Femto nodes is connected to an NR Femto GW. One NR Femto node serves one or more cells.

NEXT CHANGE

### 4.X.2	Functional Split

#### 4.X.2.1	NR Femto node

An NR Femto node hosts the same functions as a gNB as described in clause 4.1, with the following additional specifics in case of connection to the NR Femto GW:

-	Discovery of a suitable Serving NR Femto GW;

NOTE: The NR Femto node may be configured with the identity and/or address of an appropriate NR Femto GW.

-	An NR Femto node shall only connect to a single NR Femto GW at one time:

-	The NR Femto node will not simultaneously connect to another NR Femto GW, or another AMF.

NOTE:	NR Femto nodes may be deployed without network planning. An NR Femto node may be moved from one geographical area to another and therefore it may need to connect to a different NR Femto GW depending on its location.

-		The TAC and PLMN ID(s) used by the NR Femto node shall also be supported by the NR Femto GW;

-	Selection of an AMF at UE attachment is performed by the NR Femto GW:

-	AMF selection is based on the GUAMI or requested NSSAI, if received from the UE, provided to the NR Femto GW in the INITIAL UE MESSAGE message.

-	At Xn handover:

-	Allowing the NR Femto GW to identify the serving AMF at path switch:

-	Including the GUAMI of the serving AMF in the PATH SWITCH REQUEST message.

-	Allowing the AMF to identify the termination of the source UE associated signalling connection:

-	Including the Source AMF UE NGAP ID to the NR Femto GW in the PATH SWITCH REQUEST message.

#### 4.X.2.2	AMF

In addition to functions specified in clauses 4.1 and 16.7, the AMF hosts the following functions:

-	Routing of handover messages and Downlink RAN Configuration Transfer message towards NR Femto GWs based on the Selected TAI contained in these messages.

-	In case of an NR Femto node directly connected to an AMF:

-	At NG Setup, verifying, as defined in TS 33.545 [xx], that the identity used by the NR Femto node is valid;

-	At Initial UE Message and NG Handover procedures, verifying, as defined in TS 33.545 [xx], that the reported CAG ID list is valid for the indicated cell of the NR Femto node;

-	At NG PWS Restart Indication and PWS Failure Indication, verifying, as defined in TS 33.545 [xx], that the indicated cell identity is valid.

#### 4.X.2.3	NR Femto GW

The NR Femto GW hosts the following functions:

-	Relaying UE-associated NGAP messages between the AMF and the NR Femto node serving the UE, applying the following additional functions:

-	Terminating the UE Context Release request procedure if an explicit GW Context Release Indication is included.  In this case, the NR Femto GW releases the UE context if it determines that the UE identified by the received UE NGAP IDs is no longer served by another NR Femto node attached to it.

-	At UE context establishment (Initial Context Setup or NG Handover) the NR Femto GW sends to the NR Femto node the serving AMF’s GUAMI as well as the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Path Switch, the NR Femto GW sends to the NR Femto node the AMF UE NGAP ID assigned by the AMF and the AMF UE NGAP ID assigned by the NR Femto GW for the UE.

-	At Initial UE Message and HG Handover procedures, the NR Femto GW verifies, as defined in TS 33.545 [xx], that the reported CAG ID list is valid for the indicated cell of the NR Femto node.

-	Terminating non-UE associated NGAP procedures towards the NR Femto node and towards the AMF, applying the following additional functions:

- 	At NG Setup, the NR Femto GW verifies, as defined in TS 33.545 [xx], that the identity used by the NR Femto node is valid.

-	In case of NG PWS Restart Indication and PWS Failure Indication, the NR Femto GW verifies, as defined in TS 33.545 [xx], that the indicated cell identity is valid and replaces the gNB ID of the NR Femto node with the NR Femto GW ID before sending the respective message to the AMF.

-	At Overload Start/Stop, the NR Femto GW should provide the NR Femto node with the identities of the affected AMF node(s). The NR Femto node uses the received information to identify the traffic and the AMF to which the overload indication applies. The NR Femto node shall apply the defined rejections until reception of an OVERLOAD STOP message applicable to this traffic, or until the NR Femto node receives a further OVERLOAD START message applicable to the same traffic, in which case it shall replace the ongoing overload action with the newly requested one.

-	Supporting TAC and PLMN ID used by the NR Femto node.

-	Relaying the PATH SWITCH REQUEST message towards the AMF indicated by the GUAMI of the source AMF received from the NR Femto node.

A TAI used in a NR Femto GW shall not be reused in another NR Femto GW.

NEXT CHANGE

### 4.X.3	Interfaces

#### 4.X.3.1	Protocol Stack for NG User Plane

NG-U is defined as specified in clause 4.3.1.1 regardless of whether it is concentrated in the NR Femto GW. The figure below shows the NG-U protocol stack between NR Femto node and UPF.

Figure 4.X.3.1-1: User plane for NG Interface between NR Femto node and UPF

#### 4.X.3.2	Protocol Stacks for NG Control Plane

The figure below shows the NG-C protocol stack with the NR Femto GW.

When the NR Femto GW is not present, NG-C is defined as specified in clause 4.3.1.2, and all the NGAP procedures are terminated at the NR Femto node and the AMF.

Figure 4.X.3.2-1: Control plane for NG Interface for NR Femto node to AMF with the NR Femto GW

NEXT CHANGE

### 4.x.4	Access Control

Cells served by an NR Femto node may be deployed as part of a PNI-NPN (see clause 4.8) in order to restrict access to UEs according to the respective subscription.

NOTE:		The NR Femto node may use the CAG mechanism for PNI-NPN (see clause 16.7.4) as follows:

-	The NR Femto node may activate a PLMN cell, which can be accessed by legacy UE without access control of CAG.

-	The NR Femto physical node may activate a physical cell shared by both PLMN and PNI-NPN, as specified in clause 4.6, through broadcasting both the plmn-IdentityInfoList and the npn-IdentityInfoList-r16 in the SIB1, but without the cellReservedForOtherUse.

-	The NR Femto node may activate an NPN-only cell by broadcasting the cellReservedForOtherUse IE with value “true”, then this cell can only be accessed by the UEs whose allowed CAG list includes a CAG-ID broadcasted by the cell.

END OF CHANGES