3GPP TSG-RAN WG3 Meeting #129	R3-255116

Bengaluru, India, 25th ~29th Aug, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 1220 | Rev | 4 | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of Multicast MBS session restoration procedure for N3mb path failure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CMCC, CBN, China Broadnet, Qualcomm Incorporated, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_Ph2, TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For unicast transport of multicast session, as there is only DL NG-U tunnel established, in case of N3mb path failure, the MB-SMF informs the N3mb path failure indication to the gNB in Multicast Session Update Request, and the RAN tiggers another Distribution Setup procedure towards the CN to re-establish the shared NG-U tunnel. On the other hand, it is noticed that in 8.17.2.2, the following was added for broadcast: If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to "N3mb path failure", … But the correct codepoint should be “NG-U path failure detected”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding N3mb path failure indication in the NGAP: Multicast Session Update Request Transfer IE, by including the existing MBS User Plane Failure Indication IE. Correct the description of the codepoint in 8.17.2.2, from “N3mb path failure” to “NG-U path failure detected”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support the Multicast MBS session restoration procedure for N3mb path failure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.17.2.2, 8.18.5.2, 9.3.2.18, 9.3.5.13, 9.3.2.xx (new), 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 37.483 CR 0146 TS 38.300 draft CR |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: fix 'ProtocolIE-ID ::= 433~~~ProtocolIE-ID~~' -> 'id-MBS-MC~~~~ Rev2: add co-source company, update the way to reuse existing MBS User Plane Failure Indication IE, updated WI code, updated linked CRs. Rev3: update WI code. Rev4: resubmission based on latest spec. |  |  |  |  |  |  |  |  |

------------------Start of the First Change------------------

### 8.17.2	Broadcast Session Modification

#### 8.17.2.1	General

The purpose of the Broadcast Session Modification procedure is to request the NG-RAN node to update the MBS session resources or the area related to a previously established broadcast MBS session. The procedure uses non-UE associated signalling.

#### 8.17.2.2	Successful Operation

Figure 8.17.2.2-1: Broadcast Session Modification, successful operation.

The AMF initiates the procedure by sending a BROADCAST SESSION MODIFICATION REQUEST message to the NG-RAN node.

If the MBS Service Area IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall update the MBS service area and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the MBS Session Modification Request Transfer IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall replace the previously provided information by the newly received one and update the MBS session resources and area as requested and send the BROADCAST SESSION MODIFICATION RESPONSE message.

If the Supported UE Type List IE is included in the BROADCAST SESSION MODIFICATION REQUEST message, the NG-RAN node shall, if supported, take it into account for configuring MBS session resources.

If the MBS NG-U Failure Indication IE is included in the BROADCAST SESSION MODIFICATION REQUEST message within the MBS Session Setup or Modification Request Transfer IE and set to " NG-U path failure detected ", the NG-RAN node may provide new NG-U transport layer information to replace the failed one(s) or switch the data delivery to another 5GC as per the Broadcast MBS session restoration procedure for N3mb Path Failure specified in TS 23.527 [57].

------------------Start of the Next Change------------------

### 8.18.5	Multicast Session Update

#### 8.18.5.1	General

The purpose of the Multicast Session Update procedure is to request the NG-RAN node to update the NG-RAN MBS session resources or area related to a multicast MBS session. The procedure uses non-UE associated signalling.

#### 8.18.5.2	Successful Operation

Figure 8.18.5.2-1: Multicast Session Update, successful operation.

The AMF initiates the procedure by sending a MULTICAST SESSION UPDATE REQUEST message to the NG-RAN node.

Upon receipt of the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall, if requested, update the MBS QoS profile and/or MBS Service Area for the multicast service and send the MULTICAST SESSION UPDATE RESPONSE message to the AMF.

For location dependent multicast session, the AMF shall include the MBS Area Session ID IE in the MULTICAST SESSION UPDATE REQUEST message, and the NG-RAN node shall provide the same value of the MBS Area Session ID IE in the MULTICAST SESSION UPDATE RESPONSE message.

In case the MBS Service Area Information IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall update the stored MBS Service Area Information for that service, as specified in TS 23.247 [44].

In case the MBS QoS Flows To Be Setup or Modify List IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall setup or modify the MBS QoS profile accordingly.

In case the MBS QoS Flows To Be Release List IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall release the indicated MBS QoS flows.

In case the MBS Session TNL Information 5GC IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message, the NG-RAN node shall take the information into account and update related NG-U resources.

Interactions with other procedures:

If the MBS User Plane Failure Indication IE is included in the Multicast Session Update Request Transfer IE in the MULTICAST SESSION UPDATE REQUEST message and set to " NG-U path failure detected ", the NG-RAN node may provide new NG-U transport layer information to replace the failed one via Distribution Setup procedure, or release the failed one via Distribution Release procedure, as per the Multicast MBS session restoration procedure for N3mb Path Failure specified in TS 23.527 [57].

------------------Start of the Next Change------------------

#### 9.3.2.18	MBS NG-U Failure Indication

This IE provides information that the 5GC has detected an NG-U failure.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE Session Type | M |  |  |  |
| >location independent |  |  |  |  |
| >>MBS User Plane Failure Indication | M |  | 9.3.2.xx |  |
| >location dependent |  |  |  |  |
| >>MBS UP Failure Indication List |  | 1 |  |  |
| >>>MBS UP Failure Indication Item |  | 1..<maxnoofMBSServiceAreaInformation> |  |  |
| >>>>MBS Area Session ID | M |  | 9.3.1.207 |  |
| >>>>MBS User Plane Failure Indication | M |  | 9.3.2.xx |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSServiceAreaInformation | Maximum no. of MBS Service Area Information. Value is 256. |

------------------Start of the Next Change------------------

#### 9.3.2.xx	MBS User Plane Failure Indication

This IE provides information that the 5GC has detected an NG-U failure for MBS session.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MBS User Plane Failure Indication | M |  | ENUMERATED (NG-U path failure detected, …) |  |

------------------Start of the Next Change------------------

#### 9.3.5.13	Multicast Session Update Request Transfer

This IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MBS Session ID | M |  | 9.3.1.206 |  | YES | reject |
| MBS Service Area | O |  | 9.3.1.208 |  | YES | reject |
| MBS QoS Flows To Be Setup or Modified List | O |  | MBS QoS Flows To Be Setup List 9.3.1.236 |  | YES | reject |
| MBS QoS Flow To Release List | O |  | QoS Flow List with Cause 9.3.1.13 | This IE indicates the MBS QoS flow Identifiers of the MBS QoS flows to be released. | YES | reject |
| MBS Session TNL Information 5GC | O |  | 9.3.2.15 |  | YES | reject |
| MBS User Plane Failure Indication | O |  | 9.3.2.xx |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxnoofMBSQoSFlows | Maximum no. of QoS Flows allowed within one MBS session. Value is 64. |

------------------Start of the Next Change------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

//skip unchanged part

id-MBS-NGUFailureIndication,

id-UserPlaneFailureIndication,

id-UserPlaneFailureIndicationReport,

id-QoERVQoEReportingPaths,

id-UserLocationInformationN3IWF-without-PortNumber,

id-ExtendedBackupAMFName,

id-MBS-UP-FailureIndication,

maxnoofAllowedAreas,

maxnoofAllowedCAGsperPLMN,

maxnoofAllowedS-NSSAIs,

//skip unchanged part

MulticastSessionUpdateRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {MulticastSessionUpdateRequestTransferIEs} },

...

}

MulticastSessionUpdateRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-MBS-SessionID						CRITICALITY reject	TYPE MBS-SessionID						PRESENCE mandatory	}|

{ ID id-MBS-ServiceArea						CRITICALITY reject	TYPE MBS-ServiceArea					PRESENCE	optional		}|

{ ID id-MBS-QoSFlowsToBeSetupModList		CRITICALITY reject	TYPE MBS-QoSFlowsToBeSetupList		PRESENCE	optional		}|

{ ID id-MBS-QoSFlowToReleaseList			CRITICALITY reject	TYPE QosFlowListWithCause				PRESENCE	optional		}|

{ ID id-MBS-SessionTNLInfo5GC				CRITICALITY reject	TYPE MBS-SessionTNLInfo5GC				PRESENCE	optional		}|

{ ID id-MBS-UP-FailureIndication			CRITICALITY reject	TYPE MBS-UP-FailureIndication			PRESENCE	optional		},

...

}

------------------Start of the Next Change------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

#### -- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM NGAP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-AMFConfigurationUpdate					ProcedureCode ::= 0

id-AMFStatusIndication						ProcedureCode ::= 1

id-CellTrafficTrace							ProcedureCode ::= 2

id-DeactivateTrace							ProcedureCode ::= 3

//skip unchanged part

id-SourceSN-to-TargetSN-QMCInfo							ProtocolIE-ID ::= 437

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-TAIMBSSupportList									ProtocolIE-ID ::= 441

id-ExtendedBackupAMFName    							ProtocolIE-ID ::= 442

id-ExtendedOldAMF			    						ProtocolIE-ID ::= 443

id-MBS-UP-FailureIndication 							ProtocolIE-ID ::= xxx

END

-- ASN1STOP

------------------Start of the Next Change------------------