3GPP TSG-RAN WG3 Meeting #129	R3-255153

Bengaluru, India, 25 – 29 August 2025

# Title: 	(TP to BL CR for TS 38.413) Leftover issues on AIoT

# Source: 	ZTE Corporation, China Telecom

Agenda item:	16.2

# Document for:	Discussion and Decision

- Introduction

This TP is based on the following observation and proposals.

Issue 1: Command Type

Proposal 1: Introduce Command Type (write, read, disable) in Command Request Transfer.

Issue 2: Time Interval

Observation 1: The NG-RAN node shall not send Inventory report message to the A-IoT CN until the time interval arrival.

Proposal 2: Introduce the Inventory Report interval in the Inventory request message. The detail value is FFS pending to SA2.

Issue 3: Mandatorily include the Expected D2R Message Size

Observation 2: Based on the RAN2 agreement, the D2R Upper Layer Data Transfer message is always included in the A-IoT Paging message.

Proposal 3: Mandatorily include the Expected D2R Message Size in Inventory Request Transfer.

- TP for TS38.413 (based on BLCR R3-253972)

===========================Start of the change====================================

9.3.x.1	Inventory Request Transfer

This IE provides the inventory request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| A-IoT Device Identification Requested | M |  | 9.3.3.xx5 |  | YES | reject |
| Requested Service Area Information | M |  | 9.3.3.xx3 |  | YES | reject |
| Inventory Assistance Information | O |  | 9.3.3.xx4 |  | YES | reject |
| Follow on Command Indication | O |  | ENUMERATED (true, ...) | This IE indicates that there is command(s) to be transmitted. | YES | reject |
| Expected D2R Message Size | M |  | FFS | Unit: FFS. This IE indicates the size of the A-IoT NAS PDU(s) carried in Inventory Report. | YES | reject |
| Inventory Report Interval | O |  | FFS | Unit: FFS, it is defined by SA2. |  |  |

===========================Next of the change====================================

9.3.x.5	Command Request Transfer

This IE provides the command request related information from the AIOTF to the NG-RAN node.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  | YES | reject |
| RAN A-IoT Device NGAP ID | M |  | 9.3.3.yy1 |  | YES | reject |
| A-IoT NAS PDU | M |  | OCTET STRING |  | YES | reject |
| Command Type | M |  | ENUMERATED (write, read, disable, ...) |  | YES | reject |
| Command Assistance Information | O |  | 9.3.3.yy3 |  | YES | reject |

===========================Next of the change====================================

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-AdditionalDLForwardingUPTNLInformation,

id-AdditionalULForwardingUPTNLInformation,

id-AdditionalDLQosFlowPerTNLInformation,

id-AdditionalDLUPTNLInformationForHOList,

id-AdditionalNGU-UP-TNLInformation,

id-AdditionalRedundantDL-NGU-UP-TNLInformation,

id-AdditionalRedundantDLQosFlowPerTNLInformation,

id-AdditionalRedundantNGU-UP-TNLInformation,

id-AdditionalRedundantUL-NGU-UP-TNLInformation,

id-AdditionalUL-NGU-UP-TNLInformation,

id-AIOTFIdentifier,

id-AIoT-CorrelationIdentifier,

id-AIoT-DeviceIdentificationRequested,

id-AIoT-RequestedServiceAreaInformation,

id-AIoT-InventoryAssistanceInformation,

id-AIoT-InventoryExpectedD2RMessageSize,

id-AIoT-FollowonCommandIndication,

id-RAN-AIOT-Device-NGAP-ID,

id-AIoT-CommandAssistanceInformation,

id-AIoT-CommandType,

id-AIoT-InventoryReportInterval,

id-AIoT-NASPDU,

id-AlternativeQoSParaSetList,

id-AssistanceInformationQoE-Meas,

id-AdditionalCancelledlocationReportingReferenceIDList,

id-BurstArrivalTimeDownlink,

id-Cause,

<Skip unchanged part>

AIoT-CommandAssistanceInformation ::= SEQUENCE {

estimateofExpectedD2RMsgSize		INTEGER (1..256, ...)					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-CommandAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-CommandAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-CommandType ::= ENUMERATED {write, read, disable, ...}

AIoT-DeviceIdentificationRequested ::= CHOICE {

singleDevice					OCTET STRING,

groupDevices					FFS,

allDevices						NULL,

choice-Extensions		ProtocolIE-SingleContainer { { AIoT-DeviceIdentificationRequested-ExtIEs} }

}

AIoT-DeviceIdentificationRequested-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

AIoT-DeviceReportList ::= SEQUENCE (SIZE(1..maxnoofDevices)) OF AIoT-DeviceReportItem

AIoT-DeviceReportItem ::= SEQUENCE {

aIoT-NASPDU					AIoT-NASPDU,

rAN-AIOT-Device-NGAP-ID		RAN-AIOT-Device-NGAP-ID									OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-DeviceReportItem-ExtIEs} }	OPTIONAL,

...

}

AIoT-DeviceReportItem-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIOTFIdentifier ::= OCTET STRING	--FFS--

AIoT-InventoryAssistanceInformation ::= SEQUENCE {

approximateNoofTargetDevices				INTEGER (1..2048, ...)				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { AIoT-InventoryAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

AIoT-InventoryAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

AIoT-InventoryExpectedD2RMessageSize ::= FFS

AIoT-InventoryReportInterval ::= FFS

AIoT-NASPDU  ::= OCTET STRING

AIoT-ReaderReportList ::= SEQUENCE (SIZE(1..maxnoofReaders)) OF AIoT-ReaderReportItem

<Skip unchanged part>

CommandRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { CommandRequestTransferIEs} },

...

}

CommandRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier				PRESENCE	mandatory	}|

{ ID id-RAN-AIOT-Device-NGAP-ID					CRITICALITY reject	TYPE RAN-AIOT-Device-NGAP-ID					PRESENCE	mandatory	}|

{ ID id-AIoT-NASPDU								CRITICALITY reject	TYPE AIoT-NASPDU								PRESENCE mandatory	}|

{ ID id-AIoT-CommandType						CRITICALITY reject	TYPE AIoT-CommandType							PRESENCE mandatory	}|

{ ID id-AIoT-CommandAssistanceInformation		CRITICALITY reject	TYPE AIoT-CommandAssistanceInformation	PRESENCE	optional		},

...

}

<Skip unchanged part>

InventoryRequestTransfer ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ { InventoryRequestTransferIEs} },

...

}

InventoryRequestTransferIEs NGAP-PROTOCOL-IES ::= {

{ ID id-AIoT-CorrelationIdentifier				CRITICALITY reject	TYPE AIoT-CorrelationIdentifier							PRESENCE	mandatory	}|

{ ID id-AIoT-DeviceIdentificationRequested		CRITICALITY reject	TYPE AIoT-DeviceIdentificationRequested				PRESENCE	mandatory	}|

{ ID id-AIoT-RequestedServiceAreaInformation	CRITICALITY reject	TYPE AIoT-RequestedServiceAreaInformation			PRESENCE mandatory	}|

{ ID id-AIoT-InventoryExpectedD2RMessageSize	CRITICALITY reject	TYPE AIoT-InventoryExpectedD2RMessageSize			PRESENCE mandatory	}|

{ ID id-AIoT-InventoryAssistanceInformation		CRITICALITY reject	TYPE AIoT-InventoryAssistanceInformation				PRESENCE	optional		}|

{ ID id-AIoT-FollowonCommandIndication			CRITICALITY reject	TYPE AIoT-FollowonCommandIndication					PRESENCE	optional		}|

{ ID id-AIoT-InventoryReportInterval			CRITICALITY reject	TYPE AIoT-InventoryReportInterval						PRESENCE	optional		},

...

}

===========================Next of the change====================================

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<Skip unchanged part>

id-MTCommunicationHandling					ProcedureCode ::= 78

id-RANPagingRequest							ProcedureCode ::= 79

id-BroadcastSessionTransport				ProcedureCode ::= 80

id-InventoryRequest							ProcedureCode ::= 193 --to be allocated

id-InventoryReport							ProcedureCode ::= 196 --to be allocated

id-CommandRequest							ProcedureCode ::= 197 --to be allocated

id-AIoT-CommandType 						ProcedureCode ::= 197 --to be allocated

id-AIoT-InventoryReportInterval				ProcedureCode ::= 199 --to be allocated

id-AIOTSessionRelease						ProcedureCode ::= 198 --to be allocated

id-AIOTSessionReleaseRequest				ProcedureCode ::= 199 --to be allocated

===========================End of the change====================================