3GPP TSG-RAN WG3 Meeting #129	R3-255162

Bengaluru, India, 25 – 29 August 2025

Agenda item:	12.3

Source:	Nokia

Title:	[TP for BL CR NR Femto TS 38.413] Completion of open points of NR Femto

WID/SID:	FS_WAB_5GFemto_NR - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN3#125 the study item of NR Femto was successfully completed and the outcome is visible in TR 38.799 which was agreed in R3-244849.

At the following RAN Plenary#105, the corresponding work item was agreed in [1] including the following objective:

The objectives of the 5G Femto work are as follows:

-	Specification to support NR Femto architecture with optional NR Femto GW for NG interface [RAN3].

-	Specification to support access control for NR Femtos operating in open, hybrid and closed modes reusing existing CAG functionality [RAN3].

NOTE 10: For NR Femto access control, only stage 2 impact is expected on this objective.

NOTE 11: Coordination with other WGs (e.g. SA2, SA3) when needed.

This paper investigates remaining open points of the work item.

# 2	Discussion

- Slicing information to NR Femto nodes

In tdoc [2], it is analysed that NR Femto node connected via GW doesn’t need to receive the list of slices supported by the GW in the NG Setup Response because it doesn’t perform NG flex function. In contrast, the same NR femto node connected directly to AMF would need to receive this list. As a consequence, tdoc [2] proposes to change the list of slices into conditional presence in the NG setup response.

Even though it is true that NR femto node connected via GW doesn’t need the list of slices we think that the list can be simply ignored when received and not needed. It is assumed that the NR Femto when connected via GW is aware through O&M and can ignore the list. We propose to just add some clarification to 38.413 to avoid issues for this case.

Proposal 1: clarify the (non)usage of the list of slices when received by the NR Femto connected to GW in TS 38.413.

- Paging femtos with limited Xn connectivity

NR Femtos will have very limited coverage. This creates special challenges with paging function because even if an end user is paged by an NR Femto near the last serving NR Femto where he made the last call these two new NR Femtos are likely to not be Xn connected unless they are immediate neighbors. Indeed, having Xn interface with neighbors of neighbors would lead to too many Xn connections.

As a result, a typical paging from AMF would first page in last serving NR Femto and because the end user is not there would escalade the paging at TA or RA level to a much bigger areas involving tens of hundreds of cells.

Observation 1: upon paging failure to last served NR Femto the paging escalation to TA or RA could lead to paging tens to hundreds of NR Femtos in the area whereas the end user is likely near the last serving NR cell.

Proposal 2: RAN3 is encouraged to solve the paging issue above in the context of NR Femtos. See tdoc [3].

- IP version selection

RAN3#127bis agreed that IP version selection is done in the NR Femto.

No need for duplication in the NR Femto GW. Indeed, duplication of function just lead to more interoperability issues.

Proposal 3: no need of IP version selection in the NR Femto GW.

- Remaining security issue

RAN3 has received at RAN3#128 the LS from SA3 in [5] in response to [4] saying:

SA3 has agreed that all the verifications listed in the LS need to be performed in NR Femto GW (or AMF if NR Femto GW is not deployed). SA3 agreed the attached CRs to update TS 33.545 to incorporate relevant verification requirements and mechanisms as well

The SA3 CR attached to this SA3 LS is in [6] and states:

The NR Femto GW shall verify that all UE associated messages from Femtos operating in closed access mode can be mapped to a specific CAG ID and that this CAG ID is allowed for the identity of the originating Femto. In the absence of a NR Femto GW, the AMF shall perform this CAG ID verification.

The above statement means that SA3 considers that an NR Femto may not be trusted depending on the environment, similar to 4g femto. This brings two points to be discussed:

- When the NR Femtos connect directly to AMF, the AMF shall check whether a gNB is an NR Femto or not at NG Setup Request. After this check, the AMF can then focus on only performing verification of the Initial UE Message from true NR Femtos and not for all cells of the network. In order to distinguish NR Femtos a new NR Femto IE is needed in the NG Setup Request.  Please notice that same solution did apply in 4g.

- Again, same as 4g, because 4g femto may not be trusted the MME does not send the Handover Restriction List to the HeNB but instead the MME performs the membership verification for closed cells itself. Similarly, some operators may prefer that AMF may or may not send the Allowed PNI NPN List to the NR Femtos depending on their deployment and trustability. Again, this requires an NR Femto IE in the NG Setup Request to identify NR Femtos, based on which the AMF may be configured to send or not send the Allowed PNI NPN List to gNB.

Proposal 4: send a specific Femto indication in the NG Setup Request message of TS 38.413 to identify NR Femtos and enable control of sending Allowed PNI NPN List or not.

# 3	Conclusion

This paper has explored the remaining open points and made following proposals:

Proposal 1: clarify the (non)usage of the list of slices when received by the NR Femto connected to GW in TS 38.413.

Proposal 2: RAN3 is encouraged to solve the paging issue above in the context of NR Femtos. See tdoc [3].

Proposal 3: no need of IP version selection in the NR Femto GW.

Proposal 4: send a specific Femto indication in the Initial UE message of TS 38.413 from NR Femto to enable control of sending Allowed PNI NPN List or not.

The above proposals are implemented in the TP in annex A.

# 4	References

- RP-242395, Additional topological enhancements for NR, NTT Docomo, AT&T

- R3-251786, Discussion on slice feature and access control for NR Femtos

- R3-255163, [TP for BL CR NR Femtos TS 38.413] Paging Issue with NR Femtos

- R3-250822, LS on Security Verification related to NR Femtos

- R3-253021, Reply LS on Security Verification related to NR Femtos

- S3-251699, Update NR Femto Gateway Functionality

# 4	TP for TS 38.413

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message. If the IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and use it for further AMF selection for the IAB-MT.

The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

If the RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node. If the Extended RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the NR Femto IE is included in the NG SETUP REQUEST message, the AMF may store it and decide to not include the Allowed PNI NPN List IE within the Mobility Restriction List IE in subsequent INITIAL CONTEXT SETUP REQUEST messages received.

If the Extended AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE.

If the NB-IoT Default Paging DRX IE is included in the NG SETUP REQUEST message, the AMF shall take it into account for paging.

If the RAT Information IE is included in the NG SETUP REQUEST message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the NID IE within the NPN Support IE is included within a PLMN Support Item IE in the NG SETUP RESPONSE message, the NG-RAN node shall consider that the AMF supports the SNPN identified by the PLMN Identity IE and the NID IE.

If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

If the gNB does not support NNSF it shall ignore the Slice Support List IE and the Extended Slice Support List IE if received in the NG SETUP RESPONSE message

#### 8.7.1.3	Unsuccessful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.6.1	NG SETUP REQUEST

This message is sent by the NG-RAN node to transfer application layer information for an NG-C interface instance.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Global RAN Node ID | M |  | 9.3.1.5 |  | YES | reject |
| RAN Node Name | O |  | PrintableString (SIZE(1..150, …)) |  | YES | ignore |
| Supported TA List |  | 1 |  | Supported TAs in the NG-RAN node. | YES | reject |
| >Supported TA Item |  | 1..<maxnoofTACs> |  |  | - |  |
| >>TAC | M |  | 9.3.3.10 | Broadcast TAC | - |  |
| >>Broadcast PLMN List |  | 1 |  |  | - |  |
| >>>Broadcast PLMN Item |  | 1..<maxnoofBPLMNs> |  |  | - |  |
| >>>>PLMN Identity | M |  | 9.3.3.5 | Broadcast PLMN | - |  |
| >>>>TAI Slice Support List | M |  | Slice Support List 9.3.1.17 | Supported S-NSSAIs per TAC, per PLMN or per SNPN. | - |  |
| >>>>NPN Support | O |  | 9.3.3.44 | If the NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>>>Extended TAI Slice Support List | O |  | Extended Slice Support List 9.3.1.191 | Additional Supported S-NSSAIs per TAC, per PLMN or per SNPN. | YES | reject |
| >>>>TAI NSAG Support List | O |  | 9.3.1.238 | NSAG information associated with the slices per TAC, per PLMN or per SNPN. | YES | ignore |
| >>Configured TAC Indication | O |  | 9.3.3.50 |  | YES | ignore |
| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| Default Paging DRX | M |  | Paging DRX 9.3.1.90 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| NB-IoT Default Paging DRX | O |  | 9.3.1.137 |  | YES | ignore |
| Extended RAN Node Name | O |  | 9.3.1.193 |  | YES | ignore |
| NR Femto | O |  | ENUMERATED (true, ...) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTACs | Maximum no. of TACs. Value is 256. |
| maxnoofBPLMNs | Maximum no. of Broadcast PLMNs. Value is 12. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for NGAP.

--

-- **************************************************************

NGAP-PDU-Contents {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-PDU-Contents (1) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

NotifySourceNGRANNode,

NPN-AccessInformation,

NR-A2X-ServicesAuthorized,

NR-CGI,

NR-Femto,

NR-PagingeDRXInformation,

NRPPa-PDU,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-NPN-AccessInformation,

id-NR-A2X-ServicesAuthorized,

id-NR-A2X-UE-PC5-AggregateMaximumBitRate,

id-NR-Femto,

id-NR-PagingeDRXInformation,

id-NRPPa-PDU,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

###### -- NG SETUP REQUEST

--

-- **************************************************************

NGSetupRequest ::= SEQUENCE {

protocolIEs		ProtocolIE-Container		{ {NGSetupRequestIEs} },

...

}

NGSetupRequestIEs NGAP-PROTOCOL-IES ::= {

{ ID id-GlobalRANNodeID				CRITICALITY reject	TYPE GlobalRANNodeID				PRESENCE mandatory	}|

{ ID id-RANNodeName					CRITICALITY ignore	TYPE RANNodeName					PRESENCE optional	}|

{ ID id-SupportedTAList				CRITICALITY reject	TYPE SupportedTAList				PRESENCE mandatory	}|

{ ID id-DefaultPagingDRX			CRITICALITY ignore	TYPE PagingDRX						PRESENCE mandatory	}|

{ ID id-UERetentionInformation		CRITICALITY ignore	TYPE UERetentionInformation			PRESENCE optional	}|

{ ID id-NB-IoT-DefaultPagingDRX		CRITICALITY ignore	TYPE NB-IoT-DefaultPagingDRX		PRESENCE optional	}|

{ ID id-Extended-RANNodeName		CRITICALITY ignore	TYPE Extended-RANNodeName			PRESENCE optional	}|

{ ID id-NR-Femto					CRITICALITY ignore	TYPE NR-Femto							PRESENCE optional		},

...

}

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

NR-CGIList ::= SEQUENCE (SIZE(1..maxnoofCellsingNB)) OF NR-CGI

NR-CGIListForWarning ::= SEQUENCE (SIZE(1..maxnoofCellIDforWarning)) OF NR-CGI

NR-Femto ::= ENUMERATED {

true,

...

}

NR-PagingeDRXInformation ::= SEQUENCE {

nR-paging-eDRX-Cycle		NR-Paging-eDRX-Cycle,

nR-paging-Time-Window		NR-Paging-Time-Window					OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {NR-PagingeDRXInformation-ExtIEs} }	OPTIONAL,

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

NGAP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-QoERVQoEReportingPaths								ProtocolIE-ID ::= 438

id-UserLocationInformationN3IWF-without-PortNumber		ProtocolIE-ID ::= 439

id-AUN3DeviceAccessInfo									ProtocolIE-ID ::= 440

id-AMF-UE-NGAP-ID2										ProtocolIE-ID ::= xxx

id-GUAMIList											ProtocolIE-ID ::= xxx

id-GW-Context-Release-Indication						ProtocolIE-ID ::= xxx

id-SourceAMF-GUAMI										ProtocolIE-ID ::= xxx

id-RequestedSNSSAI										ProtocolIE-ID ::= xxx

id-NR-Femto												ProtocolIE-ID ::= xxx

END

-- ASN1STOP