3GPP TSG-RAN WG3 Meeting #129	R3-255169

Bengaluru, India, Aug. 25th – Aug. 29th, 2025

Agenda Item:	12.2

Source: 	Canon Research Centre France

Title:	Remaining aspects for the support of WAB

Document for:	Discussion

# Introduction

In this paper, we discuss some remaining aspects related to Wireless Access Backhaul (WAB), based on the TR 38.799 on WAB [1], the WID on additional topological enhancements [2] and on agreements/notes from the previous RAN3 meetings [3] [4] [5] listed below:

RAN3#127 Xn connection - Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication. NG connection - FFS whether to Introduce a “WAB-gNB” indication in the NG SETUP REQUEST message. RAN3#127bis User location information - RAN3 to agree the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures. Resource coordination - For WAB resource coordination, introduce a new class-1 Xn procedure between WAB-gNB and BH gNB. The message content can be based on relevant parts of the XnAP IEs defined in clauses 9.2.2.94-97 of TS 38.423. Multi-hop avoidance - To be further discussed whether Solution 1 or Solution 3 (or both) need to be selected 1. For initial access, WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells. - 3. For initial access, WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator. RAN3#129 Resource coordination - The specifications shall not define any priority between the WAB-gNB or the BH-gNB on how to split resources. - There is no consensus on supporting resource coordination by specification in Rel-19. Xn connection - The “WAB-MT ID” sent from the WAB-gNB to the BH-gNB consists of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell id of BH-gNB´s cell serving the WAB MT. - The BH-gNB can provide the TNL information of neighbour gNBs to the WAB node. - The WAB-gNB should be notified about the target BH-gNB before the WAB-MT HO. Satellite backhaul - If non-terrestrial link is used between WAB MT and BH gNB and/or between BH gNB and BH CN, the WAB-gNB informs UE’s CN that the BH includes a non-terrestrial link. - FFS how a WAB node know the BH-gNB is using a non-terrestrial link. Possible options include BH-gNB informs WAB-gNB via Xn.

# Discussion

## NG interface

NG Connection Setup

A WAB-gNB may advertise to the Core Network that it actually belongs to a WAB node by sending some WAB indication to AMFs during the NG Setup procedure, e.g., by adding a WAB indication in the NG SETUP REQUEST message.

At RAN3#127bis, RAN3 agreed the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures. When the Additional ULI IE is included in the NG SETUP REQUEST message, this WAB specific IE can thus be used as a WAB indication for the AMF. Consequently, if the Additional ULI IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, consider that the transmitting NG-RAN node is a WAB-gNB.

If the AMF does not support WAB, it will not be able to process the additional ULI sent by the WAB-gNB. In order to properly handle the UEs served by a WAB-node, it is necessary to select an AMF that supports WAB. This selection can be performed when the WAB-gNB requests an NG connection with an AMF.

Thus, a WAB-gNB may add a WAB indication in the NG SETUP REQUEST message. An AMF may confirm, during the NG setup procedure, that it actually supports WAB features by sharing some WAB support information with the WAB-gNB, e.g., by adding some WAB support information in the NG SETUP RESPONSE message.

Proposal 1a: If the Additional ULI IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, consider that the transmitting NG-RAN node is a WAB-gNB.

Proposal 1b: A WAB support information is added to the NG SETUP RESPONSE message.

A corresponding TP for 38.413 is provided in Annex A.

NTN satellite backhaul link notification

When NTN satellites are involved to serve satellite-enabled UEs, the core network establishes the PDU session(s) of the UEs taking into account this configuration. In particular, the QoS flows transmitted with a PDU session via an NTN satellite can be configured with a large Packet Delay Budget (PDB) when long propagation delays are expected to transmit radio signals between the NTN satellite and the UEs and between the NTN satellite and the Earth station.

In order to ensure that the core network is aware of long propagation delays introduced by NTN satellite, the NTN satellite base station shall inform the AMF(s) managing the served UE(s) when an NG connection is established between the AMF(s) and the NTN satellite base station. For this purpose, the information element RAT Information may be included in the message NG SETUP REQUEST with a value indicating the type of orbit. This IE may also be included in the RAN CONFIGURATION UPDATE message.

Moreover, the User Location Information providing location information of a served UE embeds NR NTN TAI Information when the UE is served by an NTN satellite base station.

When an NTN satellite is involved in the backhaul link for a WAB node, the WAB-gNB is not an NTN entity. Hence, the RAT Information in an NG SETUP REQUEST message or in a RAN CONFIGURATION UPDATE message sent by the WAB-gNB to an AMF would not indicate the presence of NTN satellite and would not indicate a potential long propagation delay to reach the WAB-gNB. Therefore, an indication that NTN satellite is used for the backhaul link should be sent by the WAB-gNB to the serving AMF(s).

It can be noted that the format of the additional ULI to be introduced for WAB is not specifically adapted in case of backhaul link using NTN satellite. Thus, a network function receiving additional ULI from a WAB node would not deduce any information related to the NTN nature of the backhaul link.

Therefore, an indication that NTN satellite is used for the backhaul link may be included in the NG SETUP REQUEST message and in the RAN CONFIGURATION UPDATE message sent by the WAB-gNB to the serving AMF(s). The indication may be in the form of a new information element BH RAT Information.

Proposal 1c: An indication that NTN satellite is used for the backhaul link may be included in the NG SETUP REQUEST message and in the RAN CONFIGURATION UPDATE message sent by the WAB-gNB to the serving AMF(s).

When NTN is used for the backhaul connection of a WAB node, the WAB-gNB may be first informed by the BH-gNB serving the WAB-MT co-located with the WAB-gNB in the WAB node. For this purpose, the information element BH RAT Information may be introduced in various Xn messages as proposed in the section 2.2.

NG Connection Removal

A WAB-gNB may, at some point, decide to remove an NG connection with the Core Network (e.g., due to a change of AMF, a change of authorization status, the co-located WAB-MT entering RRC_IDLE / RRC_INACTIVE state …). When initiating NG removal process, a WAB-gNB may further include in the NG Removal Request message sent to the AMF a cause value indicating the reason for requesting the NG connection removal.

Proposal 1d: a WAB-gNB may include in the NG Removal Request message a cause value indicating the reason for requesting the NG connection removal.

## Xn Interface

RAN3 agreed that the establishment of Xn connections between a WAB-gNB and the BH RAN node serving the WAB-MT or between a WAB-gNB and surrounding RAN nodes is supported. It can follow legacy procedures, but addition of new information in these legacy procedures is not precluded.

Sharing of neighbour NG-RAN nodes

In order to ease up some further Xn connection establishment, a WAB-gNB as other NG-RAN nodes, can share neighbour information to update the Neighbour Cell Relation Table (NCRT). In particular, the shared neighbour information may include the identification of neighbour WAB cells / WAB-gNBs.

Proposal 2a: The neighbour information shared between NG-RAN nodes may include the identification of neighbour WAB cells / WAB-gNBs.

Xn Connection Rejection

A WAB-gNB may, for some WAB specific reasons (e.g., preventing Xn connection between WAB-gNBs, mobility constraints …), reject an Xn setup request received from another NG-RAN node, by issuing an XN SETUP FAILURE message, which includes the cause of rejection.

Proposal 2b: A new cause value is introduced, to be used by a WAB-gNB in the XN SETUP FAILURE message to reject a Xn setup request for WAB specific reason.

Xn Connection Removal

In case the benefit of maintaining an Xn connection is considered as not sufficient (e.g., Xn link quality is too low, WAB-gNB is moving away, WAB-node authorization status has changed …) by a WAB-gNB or a BH RAN node or a surrounding NG-RAN node, this WAB-gNB / BH RAN node / surrounding NG-RAN node may initiate the removal of the Xn connection by sending an XN REMOVAL REQUEST message. In such case, a cause value information indicating the reason for initiating the Xn connection removal may be included in the XN REMOVAL REQUEST message. The existing cause value “Action Desirable for Radio Reasons” may be appropriate when the Xn link quality is too low or when the WAB-node is moving away, but a new cause value may be introduced, for instance “WAB not authorized”, when the reason is that the WAB-node authorization has changed.

Proposal 2c: The XN REMOVAL REQUEST message may include a cause value indicating the reason for requesting the Xn connection removal.

NTN satellite backhaul link notification

As mentioned in the section 2.1, when NTN is used for the backhaul connection of a WAB node, the WAB-gNB may be first informed by the BH-gNB serving the WAB-MT co-located with the WAB-gNB in the WAB node. For this purpose, the information element BH RAT Information may be introduced in various Xn messages: XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK.

Proposal 2d: An indication that NTN satellite is used in the backhaul may be included in various Xn messages (XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK), sent to a WAB-gNB by the BH-gNB serving the WAB-MT co-located with the WAB-gNB.

Besides, to allow a BH-gNB to provide BH RAT Information to the WAB-gNB co-located to a served WAB-MT, the BH-gNB shall first discover the WAB-MT/WAB-gNB co-location, as further discussed in the following paragraph.

Co-location discovery

For the purpose of NTN satellite backhauling notification to a WAB node, the WAB-gNB may be informed via Xn by the BH-gNB serving the co-located WAB-MT. Hence, the BH-gNB needs to discover the WAB-MT/WAB-gNB co-location.

Co-location information (i.e., a WAB-MT identifier) may be provided by the WAB-gNB via the XN SETUP REQUEST or XN SETUP RESPONSE message at the establishment of a Xn connection with the BH-gNB, or via NG-RAN NODE CONFIGURATION UPDATE or NG-RAN NODE CONFIGURATION UPDATE ACK if the Xn connection is already established.

Besides, at the handover of the WAB-MT to a target BH-gNB involving the use of NTN for the backhaul link, the WAB node should be informed as soon as possible, with the objective to inform AMF(s) and to correctly handle the QoS flows. Therefore, the target BH-gNB may inform the WAB-gNB via Xn once the WAB-MT’s handover is accepted. To speed up the process, the target BH-gNB may be informed by the source BH-gNB of the identifier of the WAB-gNB co-located with the WAB-MT to be handed over.

Observation 2.a: To allow a BH-gNB to provide NTN backhaul link information to a WAB-gNB, it is beneficial to specify:

- a WAB-MT identifier optionally included in the messages XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK),

- a WAB-gNB identifier optionally included in the HANDOVER REQUEST message related to the handover of the co-located WAB-MT.

Considering now WAB resource coordination, there is no consensus on supporting WAB resource coordination by specification in Rel-19, and it may be left to implementation for this release. In any case, some aspects may still be specified to facilitate WAB resource coordination implementation when needed.

For WAB resource coordination with a BH-gNB serving a WAB-MT, it is also necessary for the BH-gNB to discover the WAB-gNB co-located with the WAB-MT. For this purpose, a WAB-MT identifier (known by the BH-gNB) may be provided by the WAB-gNB in various Xn messages: XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK.

In order to minimize radio interference when a WAB-MT is handed over from a source BH-gNB to a target BH-gNB involving in-band operation, it is beneficial to quickly establish WAB resource coordination between this target BH-gNB and the WAB-gNB co-located with the WAB-MT that is being handed over.

The WAB-gNB could initiate the WAB resource coordination procedure but this involves the WAB-gNB providing co-location information to the target BH-gNB. Thus, the WAB-gNB has to wait to be informed of an identifier of the WAB-MT assigned by the target BH-gNB.

A faster solution is to allow the target BH-gNB to initiate the WAB resource coordination procedure, assuming that the identifier of the WAB-gNB is provided to the target BH-gNB by the source BH-gNB, e.g. through the HANDOVER REQUEST message.

Observation 2.b: To allow a smooth implementation of WAB resource coordination between a WAB-gNB and the BH-gNB serving the co-located WAB-MT, it is beneficial to specify:

- a WAB-MT identifier optionally included in the messages XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK,

- a WAB-gNB identifier optionally included in the HANDOVER REQUEST message related to the handover of the co-located WAB-MT.

As a conclusion, for the purpose of informing that NTN is used for a WAB backhaul link, or for enabling the implementation of WAB resource coordination, a BH-gNB has to discover the WAB-MT/WAB-gNB co-location at a WAB node.

Proposal 2.e: RAN3 to keep the agreements already taken for co-location discovery:

- Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication.

- The “WAB-MT ID” sent from the WAB-gNB to the BH-gNB consists of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell id of BH-gNB´s cell serving the WAB MT.

Proposal 2.f: At handover of a WAB-MT, the HANDOVER REQUEST message may include the identifier of the WAB-gNB co-located with the WAB-MT.

## Backhaul link degradation management

Unlike the typical 5G NR scenario where the NG and Xn interfaces are operated over wired links, the WAB architecture would imply that part of the NG/Xn links is wireless (i.e., the BH link between the WAB-MT and the BH RAN node is operated over the wireless Uu interface).

Considering that the NG/Xn protocols have been designed to be operated over wired links, some countermeasures should be considered to guarantee the reliability of these interfaces when operated over a versatile wireless link, which is likely to experience link degradation or congestion.

The WAB-gNB can be aware of the backhaul link quality status. However, the AMFs having established a NG connection with the WAB-gNB, and the NG-RAN nodes (other than the BH-gNB) having established a Xn connection with the WAB-gNB, are not aware of the backhaul link quality status. Therefore, to help the mitigation of backhaul link degradation, the WAB-gNB should inform the connected AMFs and (WAB aware) NG-RAN nodes about any change in the status of the wireless backhaul link.

For this purpose, the NG procedure RAN Configuration Update and the Xn procedure NG-RAN node Configuration Update may be enhanced to report the backhaul link quality status.

Proposal 3: The NG procedure RAN Configuration Update and the Xn procedure NG-RAN node Configuration Update may be enhanced to report the backhaul link quality status.

## Multiple hop prevention

For multi-hop avoidance during WAB-MT initial access, two solutions are still considered:

Solution 1: A WAB-gNB may use dedicated frequencies and/or PCIs and potential other legacy OTA parameters (e.g. NR CGI), to ensure that the WAB-MTs of other WAB-nodes avoid (re)selecting the WAB-gNB cells.

Solution 3: WAB-gNB-cells broadcast a new indicator in SIB to bar WAB-MT, and the WAB-MT avoids (re)selection of cells broadcasting this indicator.

If solution 1 is adopted and in order to ensure WAB implementation consistency, it is essential that RAN3 reaches a consensus on the OTA parameters to be used for multiple hop prevention.

Proposal 4: If solution 1 is adopted, and for compliance with the WID objectives, the OTA parameters used for multiple hop prevention shall be added to the specifications.

# Conclusion

In this contribution, we have discussed some remaining WAB-related subjects about Xn connection management, NG connection management, multiple hop prevention, and resource coordination. Finally, we have made the following proposals:

Proposal 1a: If the Additional ULI IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, consider that the transmitting NG-RAN node is a WAB-gNB.

Proposal 1b: A WAB support information is added to the NG SETUP RESPONSE message.

Proposal 1c: An indication that NTN satellite is used for the backhaul link may be included in the NG SETUP REQUEST message and in the RAN CONFIGURATION UPDATE message sent by the WAB-gNB to the serving AMF(s).

Proposal 1d: a WAB-gNB may include in the NG Removal Request message a cause value indicating the reason for requesting the NG connection removal.

Proposal 2a: The neighbour information shared between NG-RAN nodes may include the identification of neighbour WAB cells / WAB-gNBs.

Proposal 2b: A new cause value is introduced, to be used by a WAB-gNB in the XN SETUP FAILURE message to reject a Xn setup request for WAB specific reason.

Proposal 2c: The XN REMOVAL REQUEST message may include a cause value indicating the reason for requesting the Xn connection removal.

Proposal 2d: An indication that NTN satellite is used in the backhaul may be included in various Xn messages (XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK), sent to a WAB-gNB by the BH-gNB serving the WAB-MT co-located with the WAB-gNB.

Proposal 2.e: RAN3 to keep the agreements already taken for co-location discovery:

- Include a WAB-MT Identifier in the XN SETUP REQUEST, XN SETUP RESPONSE, NG-RAN NODE CONFIGURATION UPDATE and NG-RAN NODE CONFIGURATION UPDATE ACK e.g. for colocation discovery for resource multiplexing or for WAB node indication.

- The “WAB-MT ID” sent from the WAB-gNB to the BH-gNB consists of the WAB-MT’s C-RNTI assigned by the BH-gNB and the cell id of BH-gNB´s cell serving the WAB MT.

Proposal 2.f: At handover of a WAB-MT, the HANDOVER REQUEST message may include the identifier of the WAB-gNB co-located with the WAB-MT.

Proposal 3: The NG procedure RAN Configuration Update and the Xn procedure NG-RAN node Configuration Update may be enhanced to report the backhaul link quality status.

Proposal 4: If solution 1 is adopted, and for compliance with the WID objectives, the OTA parameters used for multiple hop prevention shall be added to the specifications.

# References

[1] TR 38.799 V19.0.0

[2] RP-242395 - New WID on additional topological enhancements for NR

[3] Chair notes at RAN3#127 meeting

[4] Chair notes at RAN3#127bis meeting

[5] Chair notes at RAN3#128 meeting

# Annex A: TP for TS 38.413

-------------------------------------------Start of changes-------------------------------------------

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: NG setup: successful operation

The NG-RAN node initiates the procedure by sending an NG SETUP REQUEST message including the appropriate data to the AMF. The AMF responds with an NG SETUP RESPONSE message including the appropriate data.

If the Configured TAC Indication IE set to "true” is included for a Tracking Area contained in the Supported TA List IE in the NG SETUP REQUEST message, the AMF may take it into account to optimise NG-C signalling towards this NG-RAN node.

If the UE Retention Information IE set to “ues-retained“ is included in the NG SETUP REQUEST message, the AMF may accept the proposal to retain the existing UE related contexts and signalling connections by including the UE Retention Information IE set to “ues-retained“ in the NG SETUP RESPONSE message.

If the AMF supports IAB, the AMF shall include the IAB Supported IE in the NG SETUP RESPONSE message. If the IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and use it for further AMF selection for the IAB-MT.

The AMF shall include the Backup AMF Name IE, if available, in the Served GUAMI List IE in the NG SETUP RESPONSE message. The NG-RAN node shall, if supported, consider the AMF as indicated by the Backup AMF Name IE when performing AMF reselection, as specified in TS 23.501 [9].

If the GUAMI Type IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall store the received value and use it for further AMF selection as defined in TS 23.501 [9].

If the RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node. If the Extended RAN Node Name IE is included in the NG SETUP REQUEST message, the AMF may use this IE as a human readable name of the NG-RAN node and shall ignore the RAN Node Name IE if also included.

If the Extended AMF Name IE is included in the NG SETUP RESPONSE message, the NG-RAN node may use this IE as a human readable name of the AMF and shall ignore the AMF Name IE.

If the NB-IoT Default Paging DRX IE is included in the NG SETUP REQUEST message, the AMF shall take it into account for paging.

If the RAT Information IE is included in the NG SETUP REQUEST message, the AMF shall handle this information as specified in TS 23.502 [10].

If the NID IE within the NPN Support IE is included within a Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall consider that the NG-RAN node supports the indicated S-NSSAI(s) for the corresponding tracking area code for the SNPN identified by the PLMN Identity IE and the NID IE.

If the NID IE within the NPN Support IE is included within a PLMN Support Item IE in the NG SETUP RESPONSE message, the NG-RAN node shall consider that the AMF supports the SNPN identified by the PLMN Identity IE and the NID IE.

If the Onboarding Support IE is also included within the same PLMN Support Item IE, the NG-RAN node shall, if supported, consider that the AMF supports UE onboarding for the identified SNPN, as specified in TS 23.501 [9].

If the TAI NSAG Support List IE is included in the Broadcast PLMN Item IE in the NG SETUP REQUEST message, the AMF shall, if supported, use this information as specified in TS 23.501 [9].

If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

If the AMF supports mobile IAB, the AMF shall include the Mobile IAB Supported IE in the NG SETUP RESPONSE message. If the Mobile IAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection for the mobile IAB-MT.

If the Additional ULI IE is included in the NG SETUP REQUEST message, the AMF shall, if supported, store this information, consider that the transmitting NG-RAN node is a WAB-gNB, and take it into account for determining the location of the UEs served by the NG-RAN node, as specified in TS 23.501 [9].

If the AMF supports WAB, the AMF shall include the WAB Supported IE in the NG SETUP RESPONSE message. If the WAB Supported IE is included in the NG SETUP RESPONSE message, the NG-RAN node shall, if supported, store this information and further use it for AMF selection.

-------------------------------------------Next change-------------------------------------------

#### 9.2.6.2 NG SETUP RESPONSE

This message is sent by the AMF to transfer application layer information for an NG-C interface instance.

Direction: AMF  NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| AMF Name | M |  | 9.3.3.21 | This IE is ignored if the Extended AMF Name IE is present. | YES | reject |
| Served GUAMI List |  | 1 |  |  | YES | reject |
| >Served GUAMI Item |  | 1..<maxnoofServedGUAMIs> |  |  | - |  |
| >>GUAMI | M |  | 9.3.3.3 |  | - |  |
| >>Backup AMF Name | O |  | AMF Name 9.3.3.21 |  | - |  |
| >>GUAMI Type | O |  | ENUMERATED (native, mapped, …) |  | YES | ignore |
| Relative AMF Capacity | M |  | 9.3.1.32 |  | YES | ignore |
| PLMN Support List |  | 1 |  |  | YES | reject |
| >PLMN Support Item |  | 1..<maxnoofPLMNs> |  |  | - |  |
| >>PLMN Identity | M |  | 9.3.3.5 |  | - |  |
| >>Slice Support List | M |  | 9.3.1.17 | Supported S-NSSAIs per PLMN or per SNPN. | - |  |
| >>NPN Support | O |  | 9.3.3.44 | If NID IE is included, it identifies a SNPN together with the PLMN Identity IE. | YES | reject |
| >>Extended Slice Support List | O |  | 9.3.1.191 | Additional Supported S-NSSAIs per PLMN or per SNPN. | YES | reject |
| >>Onboarding Support | O |  | ENUMERATED (true, ...) | Indication of onboarding support. | YES | ignore |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| UE Retention Information | O |  | 9.3.1.117 |  | YES | ignore |
| IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for IAB. | YES | ignore |
| Extended AMF Name | O |  | 9.3.3.51 |  | YES | ignore |
| Mobile IAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for mobile IAB. | YES | ignore |
| WAB Supported | O |  | ENUMERATED (true, ...) | Indication of support for WAB. | YES | ignore |

-------------------------------------------End of changes-------------------------------------------