3GPP TSG-RAN WG3 Meeting #129                                                                R3-255184

Bangaluru, India, 25th – 29th August, 2025

Agenda Item:	20.2

Source:	CATT

Title:	(TP to BLCR for TS 38.305) Support of AI Positioning

Document for:	other

1	Introduction

In the last RAN3 meetings, we discussed the overall procedures for AI Positioning Case 3a and 3b, corresponding agreements and open issues on the data collection procedures are shown below:

Define the Positioning Data Collection Needed IE as a new bitmap. The requested information in the bitmap is timing measurement (UL-TDOA, gNB Rx-Tx Time Difference) together with optional LoS/NLoS indicator.

FFS if UE location can be requested.

The Positioning Data Collection Needed IE is introduced in the MEASUREMENT REPORT message.

Introduce POSITIONING DATA COLLECTION REPORT message as class 2 procedure including Positioning Data Information IE and RAN/LMF Measurement IDs

Introduce an optional Positioning Data Unavailable IE (FFS on encoding) when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. The codepoints indicating the reason.

RAN3 confirms that Performance monitoring is done in the RAN in case 3a.

RAN3 confirms the WA: For case 3a data collection, Part A is known internally by the gNB and is not signalled over any network interface.

Introduce the new flag “Inferred measurement” in the TRP Measurement Result to align with RAN1 conclusion

FFS: if LMF should specifically request for AIML inferred measurement.

In the BL CR, there’re still a few Editor’s notes in the stage 2 BL CR. In this contribution, we would try to finalize the stage 2 TP.

2	Discussion

2.1 Data collection procedures

As we discussed and agreed that gNB could indicate LMF data collection is needed for the UE being positioned in NRPPa MEASUREMENT RESPONSE or MEASUREMENT REPORT messages, it’s better to clarify that in the stage 2 BL CR.

Proposal 1: Clarify that gNB could indicate LMF data collection is needed in NRPPa MEASUREMENT RESPONSE or MEASUREMENT REPORT messages.

In the stage 2 BL CR, it’s said that the POSITIONING DATA COLLECTION REPORT message includes information related to UE location and correlation information… Currently, the IE defined in the Stage 3 is named as Positioning Data Information. It’s beneficial to use the general name “Positioning Data Information” in the stage 2 procedure texts.

Proposal 2: It’s beneficial to change the “information related to UE location” to “Positioning Data Information” in the stage 2 procedure texts.

According to the agreement of the last RAN3 meeting, an optional Positioning Data Unavailable IE should be included when the Positioning Data Information IE is absent in the POSITIONING DATA COLLECTION REPORT message. But the current stage 2 texts could not reflect the case, it’s beneficial to revise the wording accordingly.

Proposal 3: Beneficial to mention the successful case and failure case of the Positioning Data Collection Report.

As above, the data collection procedures could be revised as below:

And we understand that the two Editor’s Notes in section 7.x as below could be removed.

Editor’s Note: The texts above could be further refined, if needed.

Editor’s Note: FFS on details of the signallings and IEs.

Proposal 4: Remove the below Editor’s Notes in section 7.x from BL CR for TS38.305.

2.2 Other stage 2 open issues

For the LoS/NLoS information for each measurement, it’s FFS on whether it could be inferred by using a trained AI/ML model hosted by the NG-RAN node.

LoS/NLoS information for each measurement NOTE 2 (FFS)

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be inferred by using a trained AI/ML model hosted by the NG-RAN node,

In RAN1, they also discussed whether the field of LOS/NLOS indicator should be removed or kept in the measurement report from gNB to LMF.

Note: It is up to RAN3 to decide whether the field of LOS/NLOS indicator should be removed or kept.

Our understanding is when the timing information is obtained via AI/ML, it is an estimation of ‘ideal UL RTOA or DL RSTD of the LOS ray’. In other words, the AI/ML model provides the estimation of UL RTOA or DL RSTD by assuming there is a (virtual) LOS propagation path. In this way, inferred LOS/NLOS indicator is useless and even misleading.

Proposal 5: Remove the NOTE 2 from the LoS/NLoS information for each measurement.

For the “Time domain channel measurements”, there’s a FFS on the NOTE3, as below:

NOTE 3: 	These measurements may also be used by an LMF to directly infer the UE location using a trained AI/ML model hosted by the LMF.

We understand it’s correct as for Case 3b, the AI model is located in the LMF, it’s trained by LMF, and corresponding measurement result reported from gNB to LMF could be used to infer the UE location by using a trained AI model. Thus, the FFS could be removed.

Proposal 6: “Time domain channel measurements” may also be used by LMF to directly infer the UE location using a trained AI/ML model hosted by the LMF, the FFS could be removed.

As above, we provide the TP to BL CR for TS38.305 in the Annex, refining the procedure texts, removing the remaining FFS.

Proposal 7: Discuss and agree the TP to BL CR for TS 38.305.

3	Conclusion

Base on the discussion above, we provide the following observations and proposal:

Proposal 1: Clarify that gNB could indicate LMF data collection is needed in NRPPa MEASUREMENT RESPONSE or MEASUREMENT REPORT messages.

Proposal 2: It’s beneficial to change the “information related to UE location” to “Positioning Data Information” in the stage 2 procedure texts.

Proposal 3: Beneficial to mention the successful case and failure case of the Positioning Data Collection Report.

Proposal 4: Remove the below Editor’s Notes in section 7.x from BL CR for TS38.305.

Proposal 5: Remove the NOTE 2 from the LoS/NLoS information for each measurement.

Proposal 6: “Time domain channel measurements” may also be used by LMF to directly infer the UE location using a trained AI/ML model hosted by the LMF, the FFS could be removed.

Proposal 7: Discuss and agree the TP to BL CR for TS 38.305.

4	Reference

[1] RAN3#128bis Chair Notes

[2] R3-253977 (BL CR to 38.305) Introduction of AI/ML air

# Annex: TP to BLCR for TS 38.305

START OF CHANGES

## 5.4	Functional Description of Elements Related to UE Positioning in NG-RAN

[…]

### 5.4.2	gNB

The gNB is a network element of NG-RAN that may provide measurement information for a target UE and communicates this information to an LMF.

To support NR RAT-Dependent positioning, the gNB may make measurements of radio signals for a target UE, and provide measurement results for position estimation. A gNB may serve several TRPs, including for example remote radio heads, and UL-SRS only RPs and DL-PRS-only TPs. For NTN, a TRP may be located on board the satellite.

A gNB may host AI/ML models to infer the measurements from radio signals transmitted by a target UE. The AI/ML model that is used for measurement inference by the gNB may have been trained by the gNB.

A gNB may broadcast assistance data information, received from an LMF, in positioning System Information messages.

### 5.4.3	ng-eNB

The ng-eNB is a network element of NG-RAN that may provide measurement results for position estimation and makes measurements of radio signals for a target UE and communicates these measurements to an LMF.

The ng-eNB makes its measurements in response to requests from the LMF (on demand or periodically).

An ng-eNB may serve several TPs, including for example remote radio heads and PRS-only TPs for PRS-based TBS positioning for E-UTRA.

An ng-eNB may broadcast assistance data information, received from an LMF, in positioning System Information messages.

### 5.4.4	Location Management Function (LMF)

The LMF manages the support of different location services for target UEs, including positioning of UEs and delivery of assistance data to UEs. The LMF may interact with the serving gNB or serving ng-eNB for a target UE in order to obtain position measurements for the UE, including uplink measurements made by an NG-RAN and downlink measurements made by the UE that were provided to an NG-RAN as part of other functions such as for support of handover.

The LMF may interact with a target UE in order to deliver assistance data if requested for a particular location service, or to obtain a location estimate if that was requested.

The LMF may interact with multiple NG-RAN nodes to provide assistance data information for broadcasting. The assistance data information for broadcast may optionally be segmented and/or ciphered by the LMF. The LMF may also interact with AMFs to provide ciphering key data information to the AMF as described in greater detail in TS 23.273 [35].

For positioning of a target UE, the LMF decides on the position methods to be used, based on factors that may include the LCS Client type, the required QoS, UE positioning capabilities, gNB positioning capabilities and ng-eNB positioning capabilities. The LMF then invokes these positioning methods in the UE, serving gNB and/or serving ng-eNB. The positioning methods may yield a location estimate for UE-based position methods and/or positioning measurements for UE-assisted and network-based position methods. The LMF may combine all the received results and determine a single location estimate for the target UE (hybrid positioning). Additional information like accuracy of the location estimate and velocity may also be determined.

The LMF may interact with the AMF to provide (updated) UE Positioning Capability to AMF and to receive stored UE Positioning Capability from AMF as described in TS 23.273 [35].

For NTN, the LMF is configured by the OAM with satellite related information (described in TS 38.300 [52]), as well as the association between TRP(s) and satellite(s), the association between gNB and TRP(s).

A LMF may host AI/ML models to infer the target UE location from measurement information received from gNBs. The AI/ML model that is used for UE location inference by the LMF may have been trained by the LMF.

### 5.4.5	Positioning Reference Unit (PRU)

A Positioning Reference Unit (PRU) at a known location can perform positioning measurements (e.g., RSTD, RSRP, UE Rx-Tx Time Difference measurements, DL-RSCPD, DL-RSCP, etc.) and report these measurements to a location server. In addition, the PRU can transmit SRS to enable TRPs to measure and report UL positioning measurements (e.g., RTOA, UL-AoA, gNB Rx-Tx Time Difference, UL-RSCP, etc.) from PRU at a known location. The PRU measurements can be compared by a location server with the measurements expected at the known PRU location to determine correction terms for other nearby target devices. The DL- and/or UL location measurements for other target devices can then be corrected based on the previously determined correction terms.

PRU measurements may also be provided to the target device in the assistance data as described in clause 8.12.

From a location server perspective, the PRU functionality is realized by a UE with known location.

Next Change

## 6.3	NG-RAN Node terminated protocols

### 6.3.1	NR Positioning Protocol A (NRPPa)

The NR Positioning Protocol A (NRPPa) carries information between the NG-RAN Node and the LMF. It is used to support the following positioning functions:

-	E-CID for E-UTRA where measurements are transferred from the ng-eNB to the LMF.

-	Data collection from ng-eNB's and gNB's for support of OTDOA positioning for E-UTRA.

-	Cell-ID and Cell Portion ID retrieval from gNB's for support of NR Cell ID positioning method.

-	Exchange of information between LMF and NG-RAN node for the purpose of assistance data broadcasting.

-	NR E-CID where measurements are transferred from the gNB to the LMF.

-	NR Multi-RTT where measurements are transferred from the gNB to the LMF.

-	NR UL-AoA where measurements are transferred from the gNB to the LMF.

-	NR UL-TDOA where measurements are transferred from the gNB to the LMF.

-	Data collection from gNBs for support of DL-TDOA, DL-AoD, Multi-RTT, UL-TDOA, UL-AoA.

-	Measurement Preconfiguration Information Transfer which allows the LMF to request the NG-RAN node to pre-configure and activate/deactivate measurement gap and/or PRS processing window.

-	Area-specific SRS Information Transfer which allows the LMF to notify the NG-RAN node about area-specific SRS configuration information.

-	Transfer of data collection information from LMF to gNB to facilitate AI/ML model training and performance monitoring at the gNB.

The NRPPa protocol is transparent to the AMF. The AMF routes the NRPPa PDUs transparently based on a Routing ID corresponding to the involved LMF over NG-C interface without knowledge of the involved NRPPa transaction. It carries the NRPPa PDUs over NG-C interface either in UE associated mode or non-UE associated mode.

In case of a split gNB architecture, the NRPPa protocol is terminated at the gNB-CU.

Next Change

## 7.x	AI/ML assisted positioning with gNB side model

### 7.x.1	General

NG-RAN support for AI/ML positioning requires inputs of ground truth and related data from LMF and measurements by the gNB for the purpose of management of the corresponding gNB side AI/ML model.

### 7.x.2	Positioning Data Collection Procedure

Figure 7.x.2-1 shows the Positioning Data Collection procedure used to retrieve positioning data information for a UE that is undergoing a positioning process.

Figure 7.x.2-1: Positioning Data Collection Procedure

1.	The LMF sends the NRPPa MEASUREMENT REQUEST message to one or more gNBs according to UL related positioning procedures described in Clause 8.

2.	The gNB(s) determines that data collection is needed for the UE being positioned.

3/4.	The gNB(s) indicates the LMF that data collection is needed for the UE being positioned in NRPPa MEASUREMENT RESPONSE or MEASUREMENT REPORT message.

Note:	Steps 1 to 4 may occur while the LMF performs one or more of the positioning procedures described in clause 8.

5.	The LMF sends an NRPPa POSITIONING DATA COLLECTION REPORT message to the gNB(s) which indicated in step 3/4 that positioning data collection is needed. The message includes Positioning Data Information or Positioning Data Unavailable indication. The message also includes  correlation information that enables the gNB to correlate with  the UL measurement(s).

Next Change

## 8.10	Multi-RTT positioning

### 8.10.1	General

In the Multi-RTT positioning method, the UE position is estimated based on measurements performed at both, UE and TRPs. The measurements performed at the UE and TRPs are UE/gNB Rx-Tx time difference measurements (and optionally DL-PRS-RSRP, DL-PRS-RSRPP, UL-SRS-RSRP, UL-SRS-RSRPP, and/or DL-RSCP/UL-RSCP) of DL-PRS and UL-SRS, which are used by an LMF to determine the RTTs.

The gNB Rx-Tx time difference measurements may also be inferred by using a trained AI/ML model hosted by the NG-RAN node.

For network verification of UE location in NTN, the measurements can be performed at a single TRP at different time instances. The additional measurements performed at UE are the UE Rx – Tx time difference subframe offset in unit of subframe and the DL timing drift due to Doppler in service link between UE and satellite as defined in TS 38.215 [37].

The UE may require measurement gaps to perform the Multi-RTT measurements from NR TRPs. The UE may request measurement gaps from a gNB using the procedure described in clause 7.4.1.1. The UE may also request to activate pre-configured measurement gaps as described in clause 7.7.2.

NOTE: Multi-RTT positioning with aperiodic or semi-persistent SRS is not supported for a U2N Remote UE.

[…]

#### 8.10.2.3	Information that may be transferred from the gNB to LMF

[…]

The measurement results that may be signalled from gNBs to the LMF is listed in Table 8.10.2.3-3.

Table 8.10.2.3-3: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

gNB Rx-Tx time difference measurement NOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information of the measurement

LoS/NLoS information for each measurement

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be inferred by using a trained AI/ML model hosted by the NG-RAN node

[…]

### 8.10.3	Multi-RTT Positioning Procedures

#### 8.10.3.0	General

The procedures described in this clause support Multi-RTT positioning measurements obtained by the UE and TRPs/gNB.

The procedures for data collection to support AI/ML model training and performance monitoring at the NG-RAN node for Multi-RTT positioning measurement inference are described in Clause 7.x.

[…]

Next Change

## 8.13	UL-TDOA positioning

### 8.13.1	General

In the UL-TDOA positioning method, the UE position is estimated based on UL-RTOA (and optionally UL-SRS-RSRP and/or UL-SRS-RSRPP and/or UL-RSCP) measurements taken at different TRPs of uplink radio signals from UE, along with other configuration information.

The UL-RTOA measurements taken at different TRPs of uplink radio signals from UE may also be inferred by using a trained AI/ML model hosted by the NG-RAN node.

The LMF may also use a trained AI/ML model hosted by the LMF to infer directly the UE location using the TRP measurements.

The specifics of any UL-TDOA positioning methods or techniques used to estimate the UE's location from these measurements are beyond the scope of this specification.

In order to obtain uplink measurements, the TRPs need to know the characteristics of the SRS signal transmitted by the UE for the time period required to perform uplink measurement. These characteristics should be static over the periodic transmission of SRS during the uplink measurements. Hence, the LMF will indicate to the serving gNB the need to direct the UE to transmit SRS signals for uplink positioning. It is up to the serving gNB to make the final decision on resources to be assigned and to communicate this SRS configuration information back to the LMF so that LMF can forward the SRS configuration to the TRPs. The gNB may decide (e.g., in case no resources are available) to configure no resources for the UE and report the empty resource configuration to the LMF.

NOTE: UL-TDOA positioning with aperiodic or semi-persistent SRS is not supported for a U2N Remote UE.

[…]

### 8.13.2	Information to be transferred between NG-RAN/5GC Elements

This clause defines the information that may be transferred between LMF and gNB/TRPs.

[…]

#### 8.13.2.2	Location Information that may be transferred from the gNBs to LMF

The information that may be transferred from gNBs to the LMF include measurement results listed in Table 8.13.2.2-1. The individual measurements are defined in TS 38.215 [37].

Table 8.13.2.2-1: Measurement results that may be transferred from gNBs to the LMF

Measurement results

NCGI and TRP ID of the measurement

UL-RTOA NOTE 2

UL-SRS-RSRP

UL-SRS-RSRPP

UL-RSCP measurement

UL Angle of Arrival (azimuth and/or elevation) NOTE 1

Multiple UL Angle of Arrival (azimuth and/or elevation) NOTE 1

SRS Resource Type

Time stamp of the measurement

Quality for each measurement

Beam Information for each measurement

LoS/NLoS information for each measurement

ARP ID of the measurement

Mobile TRP Location Information

Measured frequency hops

Aggregated positioning SRS resource ID list

Measurement based on aggregated resources indication

Time domain channel measurements NOTE 3

NOTE 1: When used with UL-AoA for hybrid positioning. NOTE 2: These measurements may also be inferred by using a trained AI/ML model hosted by the NG-RAN node. NOTE 3: These measurements may also be used by an LMF to directly infer the UE location using a trained AI/ML model hosted by the LMF.

#### 8.13.2.3	Information that may be transferred from the LMF to gNBs

The requested UL-SRS transmission characteristics information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-1.

Table 8.13.2.3-1: Requested UL-SRS transmission characteristics information that may be transferred from LMF to gNB.

Information

Number Of Transmissions/duration for which the UL-SRS is requested

Bandwidth

Resource type (periodic, semi-persistent, aperiodic)

Pathloss reference: - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID

Spatial relation info - PCI, SSB Index, SSB configuration (time/frequency occupancy of SSBs) - DL-PRS ID, DL-PRS Resource Set ID, DL-PRS Resource ID - NZP CSI-RS Resource ID - SRS Resource ID - Positioning SRS Resource ID

SSB Information

Periodicity of the SRS for each SRS resource set

Carrier frequency of SRS transmission bandwidth

Bandwidth aggregation request indication

Positioning validity area cell list

Validity area specific SRS information

The TRP measurement request information that may be signalled from the LMF to the gNB is listed in table 8.13.2.3-2.

Table 8.13.2.3-2: TRP Measurement request information that may be transferred from LMF to gNB.

Information

TRP ID, cell ID of the TRP to receive UL-SRS

UE-SRS configuration

UL timing information together with timing uncertainty, for reception of SRS by candidate TRPs

Report characteristics for the measurements

Measurement Quantities

Measurement periodicity and amount

Measurement beam information request

Search window information

Expected UL AoA/ZoA and uncertainty range

Number of TRP Rx TEGs

Response time

Measurement characteristics request indicator

Measurement time occasions for a measurement instance

Time window information for measurements

Information on time window size and number of channel response samples for channel measurements

The Positioning Activation/Deactivation request information that may be signalled from the LMF to the gNB is listed in Table 8.13.2.3-3.

Table 8.13.2.3-3: Requested positioning activation/deactivation information that may be transferred from LMF to gNB.

Information

SP UL-SRS: - Activation or Deactivation request - Positioning SRS Resource Set ID which is to be activated/deactivated - Spatial relation for Resource IDi - Activation Time

Aperiodic UL-SRS: - Aperiodic SRS Resource Trigger List - Activation time

UL-SRS: - Release all

### 8.13.3	UL-TDOA Positioning Procedures

#### 8.13.3.0	General

The procedures described in this clause support UL-TDOA positioning measurements obtained by the gNB and provided to the LMF using NRPPa.

The procedures for data collection to support AI/ML model training and performance monitoring at the NG-RAN node for UL-TDOA measurement inference are described in Clause 7.x.

The procedures for data collection to support AI/ML model training and performance monitoring at the LMF for direct UE location inference are described in TS 23.273 [35].

[…]

END OF CHANGES