3GPP TSG-RAN WG3 Meeting #129														       R3-255270

Bengaluru, India, 25th – 29th August 2025

Agenda Item:	13.3

Source:	Ericsson

Title:	(TP to CLTM BL CR for TS 38.473, TS 38.401) – Completion of Intra-CU Conditional LTM

Document for:	Discussion, Agreement

# Introduction

At the last RAN3 meeting, the following progress was made for intra-CU conditional LTM:

RAN3 confirms that the network can trigger a LTM Cell Switch Command MAC CE towards a candidate cell with C-LTM candidate configuration.

The candidate (initial source) DU need to know the L1 C-LTM and the L3 C-LTM.

The DU needs to know to which other candidate cell(s) to generate the L1 execution condition for L1 C-LTM.

WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

WA: To introduce a new IE with a list of candidate cells for L1 execution condition.

It is acknowledged that the C-LTM indicator needs to be sent to the initial source DU for the very first C-LTM.

The candidate DU generates the TAT for its own candidate cells.

In this contribution, we further discuss network signaling impact to support CLTM.

# Discussion

## Transfer of TAT value from candidate to source

Based on the discussions at the last RAN3 meeting, in the current F1AP baseline, an additional “C-LTM” codepoint has been added to the existing LTM Indicator IE included in the UE CONTEXT SETUP/MODIFICATION REQUEST message. This new codepoint, together with the new Request for L1 Execution Condition IE can be used to address one CLTM-specific need, namely that if the source CU decides that CLTM is based on L1 measurements, a DU needs to generate and send to the CU the L1 execution conditions.

The following agreement: “The candidate DU generates the TAT for its own candidate cells.” highlights a second CLTM-specific need, which applies to CLTM based on L1 measurements as well as to CLTM based on L3 measurements. The TAT value generated by a candidate DU needs to be sent to the UE, and therefore needs to be sent to the source DU. Since the candidate DU does not signal the TAT value in case of non-Conditional LTM (intra-CU or inter-CU), it needs to receive an explicit request to do that.

A possibility is that if the candidate DU receives an LTM Indicator IE set to “C-LTM”, the candidate DU generates the TAT value and sends it to the source CU (in UE CONTEXT SETUP RESPONSE message). The source CU forwards the TAT value to the source DU (in the UE CONTEXT MODIFICATION REQUEST message), and the source DU sends it to the UE.

In summary, for CLTM the network signaling needs to support:

- For CLTM in general, the CU requests a candidate DU (using the CLTM codepoint) to provide the TAT value of the candidate cell. The candidate DU sends the generated TAT value to the CU in the UE CONTEXT SETUP RESPONSE message. The CU sends the collected TAT values to the source DU in the UE CONTEXT MODIFICATION REQUEST message.

- (already agreed) Only for CLTM based on L1 measurement, the CU requests the source/candidate DU (considering both the LTM Indicator set to “C-LTM” and the Request for L1 Execution Condition IE) to provide L1 execution conditions to the CU. The source/candidate DU sends L1 execution conditions in the UE CONTEXT MODIFICATION/SETUP RESPONSE.

Based on the presented analysis, we propose to confirm the Working Assumptions:

WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

WA: To introduce a new IE with a list of candidate cells for L1 execution condition.

and to capture in stage 3 the new behaviour related to the signaling of the TAT value. Furthermore, RAN2 agreed that one additional TimeAlignment timer is introduced to support 2TA, and therefore both TAT values should be included in the messages. Similar to RRC, the new IE can be named as TAT Value Tag2.

ltm-TimeAlignmentTimer-r19         TimeAlignmentTimer  OPTIONAL, -- Need R

ltm-TimeAlignmentTimerTag2-r19     TimeAlignmentTimer  OPTIONAL  -- Cond 2TA

- Confirm the WA to introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

- Confirm the WA to introduce a new IE with a list of candidate cells for L1 execution condition.

- For CLTM, the CU requests a candidate a DU (using the CLTM codepoint) to provide the TAT value of the candidate cell.

- The candidate DU sends the generated TAT value to the CU in the UE CONTEXT SETUP RESPONSE message.

- The CU sends the collected TAT values to the source DU in the UE CONTEXT MODIFICATION REQUEST message.

## Layer 2 Reset

In this section, we discuss the aspect of L2 Reset and how to make the solution for CLTM consistent with the solution already adopted for non-CLTM.

In Rel-18, during the LTM preparation phase, the CU can send to source/candidate DU the intra-DU L2 reset configuration in the LTM Reset Information IE within the UE CONTEXT MODIFICATION REQUEST message. The DU shall take this into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch:

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch as specified in TS38.331 [8].

After a non-conditional LTM cell switch, or after a CLTM cell switch, the new serving DU needs to understand whether the UE applied L2 reset when moving to the new serving cell, and act accordingly. The new serving DU can do this by checking whether the L2 reset ID of the old serving cell and the L2 reset ID of the new serving cell are the same (the UE did not perform L2 reset) or different (UE performed L2 reset).

For the case of intra-DU non conditional LTM cell switch execution, the source DU coincides with the target DU. Therefore, the DU knows the identities of the old serving cell and target cell, can compare the L2 reset ID of the two cells and understands whether the UE has performed L2 reset or not.

For the case of inter-DU non conditional LTM cell switch execution, the target DU cannot compare the L2 Reset ID of the old serving cell with the L2 Reset ID of the new cells. Considering the current signaling design, the target DU receives the CU-DU CELL SWITCH NOTIFICATION F1AP message, containing the target cell ID, but the old serving cell ID is not included. Due to that, the target DU cannot understand if the UE performed L2 reset or not, and there can be a mismatch in terms of L2 reset handling between the network and the UE.

- In case of inter-DU non conditional LTM, the new serving DU does not receive the old serving cell ID, and it cannot understand whether the UE performed L2 reset or not.

According to the current baselines for CLTM (38.473 and 38.401), for the case of inter-DU Conditional LTM cell switch execution, the target DU does not receive any CU-DU CELL SWITCH NOTIFICATION message, and the same problem just described for inter-DU LTM also exists for CLTM in general.

- In case of inter-DU CLTM, the new serving DU does not receive the old serving cell ID, and it cannot understand whether the UE performed L2 reset or not.

To resolve this issue, we propose that:

- during the preparation phase, source and candidate DUs are informed about the LTM Reset Information. This effectively means that the LTM Reset Information IE contained in the UE CONTEXT MODIFICATION REQUEST is used for CLTM as well.

- during the execution phase, the CU sends the CU-DU CELL SWITCH NOTIFICATION message to the new serving DU, including the old serving cell ID.

- Update stage 2 (38.401) to include a new step, where the CU sends to the new serving DU a CU-DU CELL SWITCH NOTIFICATION message including the Old Serving Cell ID.

With this information, the new serving DU can compare the L2 reset ID of the old serving cell with the L2 reset ID of the target cell, understand whether the UE performed L2 reset or not, and take this into account.

- The LTM Reset Information IE contained in the UE CONTEXT MODIFICATION REQUEST is used for CLTM as well.

- Add a new Old Serving Cell ID IE in the CU-DU CELL SWITCH NOTIFICATION message to indicate the cell serving the UE before initiating the CLTM cell switch execution.

- Update stage 2 (38.401) to include a new step where the CU sends to the new serving DU a CU-DU CELL SWITCH NOTIFICATION message.

## TAT values for candidate

At RAN2#129-bis the following was agreed:

RAN2#129bis agreement: Upon C-LTM execution, UE performs the following for received TA values and TA timers for other candidate cells than the target candidate cell: Maintain the TA value and Keep the early TA timer running.

For non-CLTM, the TA values of the candidate cells are transferred to the new serving DU. A serving DU, by implementation, maintains the validity of the TA. Therefore, the new serving DU, based on the TA values of the other candidate cells, can decide if/when it is appropriate to trigger a new early TA acquisition (before the next cell switch execution), to ensure RACH-less LTM. A similar approach can be used for CLTM. The candidate DU decides the TAT value of the candidate cell, and this is sent to the UE. Therefore, the source DU knows the TAT values of all the candidate cells (for all candidate DUs).

When a CLTM is executed, the new serving DU does not receive the Cell Switch Notification, so it cannot assume that the TA values sent to the UE are valid, and to prepare for a subsequent RACH-less CLTM, it needs to start a new early UL sync, which increases network signaling.

- In CLTM, the new serving DU cannot assume that the TA values at the UE are valid and to prepare for the next RACH-less CLTM it needs to trigger early UL synch.

One option to favor RACH-less CLTM without the need for a new round of early UL sync in the new serving DU is that, when the old serving DU receives from the CU the UE CONTEXT MODIFICATION REQUEST, indicating to stop the data transmission for the UE (i.e., that the UE has left due to CLTM), the old serving DU forwards the TA values and the TAT values received from the other candidates DUs to the new serving DU (via the CU).

In principle, another option could be that the synchronization of the TAT values between candidate DUs is done during the LTM preparation. For example, after the source DU has received the TAT value generated by for the candidate cells in other DUs, it forwards the TA values and TAT value(s) to the other candidate DUs (via the CU). However, this option looks quite complex and expensive in terms of signaling, and it is not our preference.

- The old serving DU forwards the TAT values received from the other candidates to the new serving DU (via the CU).

## Other remaining FFS

In our current F1 baseline, the DU can provide L1 execution conditions via the L1 Execution Condition List IE. It is FFS how to encode the Execution Condition IE:

#### 9.3.1.XXX		L1 Execution Condition List

This IE indicates the list of conditional LTM L1 execution conditions to be used by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| L1 Execution Condition Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Execution Condition | M |  | OCTET STRING | Includes the LTM-CSI-ReportConfigId-r18 IE as defined in subclause 6.3.2 in TS 38.331 [8]. FFS:to be checked in RRC running CR. | - |  |

In the running RRC CR, LTM execution conditions are encoded as follows:

LTM-ExecutionConditionList-r19 ::= SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-ExecutionCondition-r19

LTM-ExecutionCondition-r19 ::= SEQUENCE {

ltm-CandidateId-r19                            LTM-CandidateId-r18,

executionCondition-r19                         CHOICE {

l1-Conditions-r19                              LTM-CSI-ReportConfigId-r18,

l3-Conditions-r19                              SEQUENCE (SIZE (1..2)) OF MeasId

}                                                                                                    OPTIONAL,   -- Need R

...

}

While, on one hand, the current F1 baseline, points to the exact IE encoding the L1 Condition (LTM-CSI-ReportConfigId-r18), this does not take into account that for one cell, different L1 execution conditions can be defined for different candidate cell (different ltm-CandidateId). Moreover, we think that, to be future proof, the signal should consider that future releases might define more than one L1 execution conditions for the same candidate cell. Considering these two aspects, we prefer that the Execution Condition IE points to the RRC LTM-ExecutionCondition IE instead. Since the LTM-ExecutionCondition IE includes either L1 execution conditions, or L3 execution conditions, there is a need to clarify (e.g., in the semantics), that only L1 execution conditions are relevant.

- For the Execution Condition IE, refer to the RRC LTM-ExecutionCondition IE and indicates in the semantics that it only refers to L1 execution conditions.

- Agree to the TPs for TS 38.473 and TS 38.401, respectively in Appendix A and B, that capture the above proposals.

# Conclusions and Proposals

In this contribution we made observations and proposals for different LTM related topics, as listed below.

Observation 1	In case of inter-DU non conditional LTM, the new serving DU does not receive the old serving cell ID, and it cannot understand whether the UE performed L2 reset or not.

Observation 2	In case of inter-DU CLTM, the new serving DU does not receive the old serving cell ID, and it cannot understand whether the UE performed L2 reset or not.

Observation 3	In CLTM, the new serving DU cannot assume that the TA values at the UE are valid and to prepare for the next RACH-less CLTM it needs to trigger early UL synch.

Proposal 1	Confirm the WA to introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

Proposal 2	Confirm the WA to introduce a new IE with a list of candidate cells for L1 execution condition.

Proposal 3	For CLTM, the CU requests a candidate a DU (using the CLTM codepoint) to provide the TAT value of the candidate cell.

Proposal 4	The candidate DU sends the generated TAT value to the CU in the UE CONTEXT SETUP RESPONSE message.

Proposal 5	The CU sends the collected TAT values to the source DU in the UE CONTEXT MODIFICATION REQUEST message.

Proposal 6	The LTM Reset Information IE contained in the UE CONTEXT MODIFICATION REQUEST is used for CLTM as well.

Proposal 7	Add a new Old Serving Cell ID IE in the CU-DU CELL SWITCH NOTIFICATION message to indicate the cell serving the UE before initiating the CLTM cell switch execution.

Proposal 8	Update stage 2 (38.401) to include a new step where the CU sends to the new serving DU a CU-DU CELL SWITCH NOTIFICATION message.

Proposal 9	The old serving DU forwards the TAT values received from the other candidates to the new serving DU (via the CU).

Proposal 10	For the Execution Condition IE, refer to the RRC LTM-ExecutionCondition IE and indicates in the semantics that it only refers to L1 execution conditions.

Proposal 11	Agree to the TPs for TS 38.473 and TS 38.401, respectively in Appendix A and B, that capture the above proposals.

# Appendix A – TP for TS 38.473

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

[49]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[50]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[51]	3GPP TS 33.501: " Security architecture and procedures for 5G system".

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.2.12	DU-CU TA Information Transfer

#### 8.2.12.1	General

The purpose of the DU-CU TA Information Transfer procedure is to to enable the gNB-DU to send the TA related information to the gNB-CU. The procedure uses non-UE-associated signalling.

#### 8.2.12.2	Successful Operation

Figure 8.2.12.2-1: DU-CU TA Information Transfer procedure. Successful operation.

The gNB-DU initiates the procedure by sending a DU-CU TA Information Transfer message.

Upon reception of the DU-CU TA Information Transfer message, the gNB-CU shall, if supported, consider that the received information is the TA information from the candidate cell(s) that is indicated by the included Candidate Cell ID IE.

If the Tag ID Pointer IE is included, the gNB-CU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

#### 8.2.12.3	Unsuccessful Operation

Not applicable.

#### 8.2.12.4	Abnormal Conditions

Not applicable.

### 8.2.13	CU-DU TA Information Transfer

#### 8.2.13.1	General

The purpose of the CU-DU TA Information Transfer procedure is to to enable the gNB-CU to send the TA related information to the gNB-DU. The procedure uses non-UE-associated signalling.

#### 8.2.13.2	Successful Operation

Figure 8.2.13.2-1: CU-DU TA Information Transfer procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU TA Information Transfer message.

Upon reception of the CU-DU TA Information Transfer message, the gNB-DU shall, if supported, consider that the received information is the TA information from the candidate cell(s) that is indicated by the included Candidate Cell ID IE.

If the Tag ID Pointer IE is included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

#### 8.2.13.3	Unsuccessful Operation

Not applicable.

#### 8.2.13.4	Abnormal Conditions

Not applicable.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.1	UE Context Setup

#### 8.3.1.1	General

The purpose of the UE Context Setup procedure is to establish the UE Context including, among others, SRB, DRB, BH RLC channel, Uu Relay RLC channel, PC5 Relay RLC channel, and SL DRB configuration. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

<skip unchanged part>

If the LTM Indicator IE set to "true" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8].

If the LTM Indicator IE set to "C-LTM" is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider that the request concerns conditional LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT SETUP RESPONSE message.

If the LTM Indicator IE is set to "C-LTM" and the Request for L1 Execution Condition IE is present in the LTM Information Setup IE in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall generate the conditional LTM L1 execution condition(s) for the candidate cell(s) included in the Request for L1 Execution Condition IE and provide the L1 Execution Condition List IE, the TA Value IE and the TAT Value IE within the LTM Configuration IE in the UE CONTEXT SETUP RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, provide the lower layer configuration in the Reference Configuration Information IE in the LTM Configuration IE in the UE CONTEXT SETUP RESPONSE message for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration(s) in the CellGroupConfig IE for the requested LTM candidate cell.

If the Request for CSI Resource Configuration IE is contained in the LTM Information Setup IE included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the CSI Resource Configuration IE in the LTM Configuration IE in the UE CONTEXT SETUP RESPONSE message.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, include the Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation), in the UE CONTEXT SETUP RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the LTM Configuration IE is included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the candidate gNB-DU.

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT SETUP RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

<skip unchanged part>

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification (gNB-CU initiated)

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to modify the established UE Context, e.g., establishing, modifying and releasing radio resources or sidelink resources. This procedure is also used to command the gNB-DU to stop data transmission for the UE for mobility (see TS 38.401 [4]). The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

<skip unchanged part>

If the LTM Indicator IE set to "true" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. The gNB-DU shall regard it as a reconfiguration with sync as defined in TS 38.331 [8]. If the gNB-DU accepts the request for LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the LTM Indicator IE set to "C-LTM" is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider that the request concerns conditional LTM for the included SpCell ID IE and shall include it as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message. If the gNB-DU accepts the request for conditional LTM for that SpCell, the gNB-DU shall generate and include the CellGroupConfig IE for the accepted LTM candidate cell in the UE CONTEXT MODIFICATION RESPONSE message.

If the LTM Indicator IE is set to "C-LTM" and the Request for L1 Execution Condition IE is present in the LTM Information Modify IE in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall generate the conditional LTM L1 execution condition(s) for the candidate cell(s) as included in the Request for L1 Execution Condition IE and provide the L1 Execution Condition List IE within the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the Request for Lower Layer Configuration IE set to "true" is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the Reference Configuration Information IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message to provide lower layer configuration for the gNB-CU to generate the LTM reference configuration.

If the Reference Configuration Information IE is contained within the Reference Configuration IE in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take it into account for generating the LTM lower layer configuration.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message and the SpCell ID IE is also included, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the requested LTM candidate cell identified by the SpCell ID IE.

If the CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message while the SpCell ID IE is absent, the gNB-DU shall, if supported, use it to generate the LTM CSI reporting configuration in the CellGroupConfig IE for the serving cell.

If the Request for CSI Resource Configuration IE is contained in the LTM Information Modify IE included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include the CSI Resource Configuration IE in the LTM Configuration IE in the UE CONTEXT MODIFICATION RESPONSE message.

If the LTM Configuration ID Mapping List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, consider this as the mapping information for the LTM candidate cell(s).

If the Early Sync Information Request IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, include Early Sync Information IE of the accepted candidate cell for early TA acquisition (early UL synchronisation) in the UE CONTEXT MODIFICATION RESPONSE message. If the Early UL Sync Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information from the accepted candidate cell in the gNB-DU. If the Early UL Sync Configuration for SUL IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated early UL sync information for SUL from the accepted candidate cell in the gNB-DU.

If the Early Sync Candidate Cell Information List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Candidate Cell Information List IE for some candidate cell, the gNB-DU shall, if supported, take them into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the Early Sync Serving Cell Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the UE Based TA Measurement Configuration IE is contained in the Early Sync Serving Cell Information IE, the gNB-DU shall, if supported, take it into account for UE based TA measurement during LTM cell switch as specified in TS 38.331 [8].

If the LTM CFRA Resource Config List IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use it for the LTM cell switch command as specified in TS 38.321 [16].

If the LTM Configuration IE is included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider it as the generated configuration for LTM from the accepted candidate cell in the gNB-DU.

If the LTM Cells to be Released List IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, release the configured candidate cells in the list.

If the LTM Reset Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, take them into account for L2 reset (i.e., RLC re-establishment) during an intra-DU LTM cell switch, or an inter-DU CLTM cell switch, as specified in TS38.331 [8].

If the Complete Candidate Configuration Indicator IE set to "complete" is contained in the LTM Configuration IE included in the UE CONTEXT MODIFICATION RESPONSE message, the gNB-CU shall, if supported, consider that the LTM candidate configuration is a complete candidate configuration.

If the LTM Security Information IE is included in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store it and take it into account for supporting the UE’s AS security continuation during an inter-CU LTM cell switch and act as specified in TS 38.401 [4] and TS 38.321 [16].

<skip unchanged part>

If the Ranging and Sidelink Positioning Service Information IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, update its service information for the UE accordingly. If the Ranging and Sidelink Positioning Authorized IE within the Ranging and Sidelink Positioning Service Information IE is set to "not authorized", the gNB-DU shall, if supported, initiate actions to ensure that the UE is no longer accessing the Ranging and Sidelink Positioning service.

Interaction with UE Inactivity Notification procedure

If the SDT Volume Threshold IE is contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, use the information during an SDT transaction to inform the gNB-CU via the UE INACTIVITY NOTIFICATION message as specified in TS 38.401 [4].

Interaction with UE Context Setup or UE Context Modification (gNB-CU initiated) procedures

If the UE CONTEXT MODIFICATION REQUEST message is sent for a UE context set up for S-CPAC and contains the Transmission Action Indicator IE set to "stop", the gNB-DU shall, if supported, reset the UE context for the included SpCell ID IE, prepare for subsequent CPAC. The gNB-DU shall include the SpCell ID IE as the Requested Target Cell ID IE in the UE CONTEXT MODIFICATION RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.3.10	CU-DU Cell Switch Notification

#### 8.3.10.1	General

The purpose of the CU-DU Cell Switch Notification procedure is to enable the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE. This procedure is also used to transfer the selected TCI state from the gNB-CU to the gNB-DU. This procedure is also used to inform the gNB-DU about the initiation of an inter-gNB-DU conditional LTM cell switch. The procedure uses UE-associated signalling.

#### 8.3.10.2	Successful Operation

Figure 8.3.10.2-1: CU-DU Cell Switch Notification procedure. Successful operation.

The gNB-CU initiates the procedure by sending a CU-DU CELL SWITCH NOTIFICATION message.

Upon reception of the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, consider that a cell switch command was sent to the UE where the target cell is indicated by the included Cell ID IE.

If the Old Serving Cell ID IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the LTM Cell Switch Information IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4].

If the TA Information List IE is included in the CU-DU CELL SWITCH NOTIFICATION message, the gNB-DU shall, if supported, use it as specified in TS 38.401 [4]. If the Tag ID Pointer IE is also included, the gNB-DU shall, if supported, consider it to determine the TAG corresponding to the received TA value.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

/*********************Next change***********************/

### 9.2.1	Interface Management messages

<skip unchanged part>

#### 9.2.1.24	DU-CU TA INFORMATION TRANSFER

This message is sent by the gNB-DU to inform the gNB-CU about TA information.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| DU to CU TA Information List |  | 0..1 |  |  | YES | ignore |
| >DU to CU TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |
| >>Source gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | - |  |
| >>Source gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| >>TAT Value | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |
| >>TAT Value TAG2 | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

#### 9.2.1.25	CU-DU TA INFORMATION TRANSFER

This message is sent by the gNB-CU to inform the gNB-DU about TA information.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| CU to DU TA Information List |  | 1 |  |  | YES | ignore |
| >CU to DU TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Preamble Index | M |  | INTEGER (0..63) |  | - |  |
| >>RA-RNTI | M |  | INTEGER (0..65535, ...) | RA-RNTI as defined in TS 38.321 [16]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | - |  |
| >>TAT Value | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |
| >>TAT Value TAG2 | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |

### 9.2.2	UE Context Management messages

(skip unchanged)

#### 9.2.2.1	UE CONTEXT SETUP REQUEST

This message is sent by the gNB-CU to request the setup of a UE context.

Direction: gNB-CU  gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | O |  | 9.3.1.5 |  | YES | ignore |
| SpCell ID | M |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | reject |
| ServCellIndex | M |  | INTEGER (0..31,...) |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM InformationSetup |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …, C-LTM) |  | - |  |
| >LTM Configuration ID | M |  | INTEGER (1..8) | Corresponds to the LTM-CandidateId IE, as defined in TS 38.331 [8]. | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >Request for CSI Resource Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >Request for L1 Execution Condition | O |  | 9.3.1.x |  | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs List |  | 0..1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  |  |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  |  |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Path Addition Information | O |  | 9.3.1.296 | This IE contains either the Indirect Path Addition IE or the N3C Indirect Path Addition IE. | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |

/*********************Next change***********************/

#### 9.2.2.2	UE CONTEXT SETUP RESPONSE

This message is sent by the gNB-DU to confirm the setup of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| DU To CU RRC Information | M |  | 9.3.1.26 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell. | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition List | O |  | 9.3.1.XXX |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.x1 |  | - |  |
| >TAT Value | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |
| >TAT Value TAG2 | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

/*********************Next change***********************/

#### 9.2.2.7	UE CONTEXT MODIFICATION REQUEST

This message is sent by the gNB-CU to provide UE Context information changes to the gNB-DU.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| SpCell ID | O |  | NR CGI 9.3.1.12 | Special Cell as defined in TS 38.321 [16]. For handover case, this IE is considered as target cell. | YES | ignore |
| <skip unchanged part> |  |  |  |  |  |  |
| LTM Information Modify |  | 0..1 |  |  | YES | reject |
| >LTM Indicator | M |  | ENUMERATED (true, …, C-LTM) |  | - |  |
| >Reference Configuration | O |  | 9.3.1.292 |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.330 |  | - |  |
| >Request for CSI Resource Configuration | O |  | ENUMERATED (true, …) |  | - |  |
| >Request for L1 Execution Condition | O |  | 9.3.1.x |  | - |  |
| >TAT Value | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |
| >TAT Value TAG2 | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |
| LTM CFRA Resource Config List |  | 0..1 |  |  | YES | ignore |
| >LTM CFRA Resource Config Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >>LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| LTM Configuration ID Mapping List | O |  | 9.3.1.294 |  | YES | reject |
| Early Sync Information Request |  | 0..1 |  |  | YES | ignore |
| >Request for RACH Configuration | M |  | ENUMERATED (true, …) |  | - |  |
| >LTM gNB-DUs ID List |  | 1 |  | This IE contains the IDs of the source gNB-DU and candidate gNB-DU(s). | YES | reject |
| >>LTM gNB-DUs Item IEs |  | 1..< maxnoofLTMgNBDUs> |  |  | - |  |
| >>>LTM gNB-DU ID | M |  | gNB-DU ID 9.3.1.9 |  | - |  |
| >>>LTM gNB ID | O |  | Global gNB ID 9.3.1.305 |  | - |  |
| Early Sync Candidate Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Early Sync Candidate Cell Information Item IEs |  | 1 .. <maxnoofLTMCells> |  |  | EACH | ignore |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >>Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >>Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| >>TA Assistance Information | O |  | ENUMERATED (zero, …) | The value "zero" corresponds to TA value of the cell being equal to zero. | - |  |
| >>UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-UE-MeasuredTA-ID contained in the LTM-Candidate IE, as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. | - |  |
| >>SSB Positions In Burst | C-ifEarlyUL |  | 9.3.1.138 | This IE applies to early TA acquisition. | YES | ignore |
| Early Sync Serving Cell Information |  | 0..1 |  |  | YES | ignore |
| >UE Based TA Measurement Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellUE-MeasuredTA-ID contained in the LTM-Config IE, as defined in TS 38.331 [8], for the current serving cell. | - |  |
| LTM Cells To Be Released List | O |  | 9.3.1.291 |  | YES | reject |
| Path Addition Information | O |  | 9.3.1.296 |  | YES | reject |
| NR A2X Services Authorized | O |  | 9.3.1.323 |  | YES | ignore |
| LTE A2X Services Authorized | O |  | 9.3.1.324 |  | YES | ignore |
| NR UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | NR UE Sidelink Aggregate Maximum Bit Rate 9.3.1.119 | This IE applies only if the UE is authorized for NR A2X services. | YES | ignore |
| LTE UE Sidelink Aggregate Maximum Bit Rate for A2X | O |  | LTE UE Sidelink Aggregate Maximum Bit Rate 9.3.1.118 | This IE applies only if the UE is authorized for LTE A2X services. | YES | ignore |
| DL LBT Failure Information Request | O |  | ENUMERATED (inquiry, …) |  | YES | ignore |
| Ranging and Sidelink Positioning Service Information | O |  | 9.3.1.331 | This IE applies only if the UE is authorized for NR V2X services and/or 5G ProSe services. | YES | ignore |
| Non-Integer DRX Cycle | O |  | 9.3.1.344 |  | YES | ignore |
| LTM Reset Information | O |  | 9.3.1.346 |  | YES | ignore |
| LTM Security Information | O |  | 9.3.1.XX |  | YES | reject |

/*********************Next change***********************/

#### 9.2.2.8	UE CONTEXT MODIFICATION RESPONSE

This message is sent by the gNB-DU to confirm the modification of a UE context.

Direction: gNB-DU  gNB-CU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| <skip unchanged part> |  |  |  |  |  |  |
| Early Sync Information |  | 0..1 |  |  | YES | ignore |
| >TCI States Configurations List | M |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [8]. | - |  |
| >Early UL Sync Configuration | O |  | 9.3.1.328 |  | - |  |
| >Early UL Sync Configuration for SUL | O |  | Early UL Sync Configuration 9.3.1.328 | This IE applies for SUL carrier. | - |  |
| LTM Configuration |  | 0..1 |  |  | YES | ignore |
| >SSB Information | M |  | 9.3.1.202 | Includes the SSB Information for the requested target cell | - |  |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |
| >LTM CFRA Resource Configuration | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. | - |  |
| >LTM CFRA Resource Configuration for SUL | O |  | OCTET STRING | Includes the RACH-ConfigDedicated IE, as defined in TS 38.331 [8]. This IE applies for SUL carrier. | - |  |
| >L1 Execution Condition List | O |  | 9.3.1.XXX |  | - |  |
| >CSI Resource Configuration | O |  | 9.3.1.x1 |  | - |  |
| >TAT Value | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |
| >TAT Value TAG2 | O |  | ENUMERATED (ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,…) | This IE indicates the value of the TA timer of the cell. | - |  |
| S-CPAC Configuration |  | 0..1 |  |  | YES | ignore |
| >Reference Configuration Information | O |  | OCTET STRING | Includes the CellGroupConfig IE, as defined in TS 38.331 [8]. | - |  |
| >Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | - |  |

/*********************Next change***********************/

#### 9.2.2.16	CU-DU CELL SWITCH NOTIFICATION

This message is sent by the gNB-CU to inform the gNB-DU about the initiation of the cell switch command to the UE.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| gNB-DU UE F1AP ID | M |  | 9.3.1.5 |  | YES | reject |
| Cell ID | M |  | NR CGI 9.3.1.12 | Indicates the target cell ID. | YES | reject |
| LTM Cell Switch Information |  | 0..1 |  |  | YES | ignore |
| >Joint or DL TCI State ID | M |  | OCTET STRING | Includes the TCI-StateId IE, as defined in TS 38.331 [8]. | - |  |
| >UL TCI State ID | O |  | OCTET STRING | Includes the TCI-UL-StateId IE, as defined in TS 38.331 [8]. | - |  |
| TA Information List |  | 0..1 |  |  | YES | ignore |
| >TA Information Item IEs |  | 1 .. <maxnoofTAList> |  |  | EACH | ignore |
| >>Candidate Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >>TA Value | M |  | INTEGER (0..4095) | Indicates the TA value as defined in TS 38.213 [31]. | - |  |
| >>Tag ID Pointer | O |  | OCTET STRING | Includes the tag-Id-ptr contained in the TCI-UL-State IE or the TCI-State IE, as defined in TS 38.331 [8]. | YES | ignore |
| Old Serving Cell ID | O |  | NR CGI 9.3.1.12 | Indicates the cell serving the UE before conditional LTM cell switch execution. | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofTAList | Maximum no. of TA values to be sent, the maximum value is 8. |

/*********************Next change***********************/

#### 9.3.1.XX	LTM Security Information

This IE contains the security related information for LTM candidate cell(s) to support the UE in generating the key material for AS security during an inter-CU LTM cell switch.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Next Hop Chaining Count | M |  | INTEGER (0..7) | Next Hop Chaining Count (NCC) defined in TS 33.501 [51] |
| Security Change Serving Cell Configuration | O |  | OCTET STRING | Includes the ltm-ServingCellNoSecurityChangeID IE as defined in TS 38.331 [8], for the current serving cell. |
| Security Change Candidate Cell Information List |  | 0..1 |  |  |
| >Security Change Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |
| >>Cell ID | M |  | NR CGI 9.3.1.12 |  |
| >>Security Change Candidate Cell Configuration | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE as defined in TS 38.331 [8], for the LTM candidate cell identified by the Cell ID IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

/*********************Next change***********************/

/******************Next change*******************************/

#### 9.3.1.XXX		L1 Execution Condition List

This IE indicates the list of conditional LTM L1 execution conditions to be used by the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| L1 Execution Condition Item IEs |  | 1..< maxnoofLTMCells> |  |  | - |  |
| >LTM Cell ID | M |  | NR CGI 9.3.1.12 |  | - |  |
| >Execution Condition | M |  | OCTET STRING | Includes the LTM-ExecutionCondition IE as defined in TS 38.331 [8]. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured LTM allowed towards one UE, the maximum value is 8. |

/*********************Next change***********************/

<<<<<<<<<<<<<<<<<<<< Start of ASN.1 changes >>>>>>>>>>>>>>>>>>>>

### 9.4.4	PDU Definitions

(Skip unchanged)

(Skip unchanged)

id-AggregatedPosSRSResourceSetList,

id-RANSharingAssistanceInformation,

id-F1U-PathFailure,

id-LTMResetInformation,

id-PreconfiguredSRSInformation,

id-MobilityInitiation,

id-PLMNIndexNRAssistanceInfoForNetShar,

id-LTMSecurityInformation,

id-OldServingCell,

maxCellingNBDU,

maxnoofCandidateSpCells,

maxnoofDRBs,

maxnoofIndividualF1ConnectionsToReset,

maxnoofPotentialSpCells,

maxnoofSCells,

maxnoofSRBs,

maxnoofPagingCells,

maxnoofTNLAssociations,

maxCellineNB,

maxnoofUEIDs,

maxnoofBHRLCChannels,

maxnoofRoutingEntries,

maxnoofTLAsIAB,

maxnoofULUPTNLInformationforIAB,

maxnoofUPTNLAddresses,

maxnoofSLDRBs,

maxnoofTRPInfoTypes,

maxnoofTRPs,

maxnoofMRBs,

maxnoofUEIDforPaging,

maxnoofMRBsforUE,

maxnoofServingCellMOs

FROM F1AP-Constants;

(Skip unchanged)

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

-- **************************************************************

--

#### -- CU-DU CELL SWITCH NOTIFICATION ELEMENTARY PROCEDURE

--

-- **************************************************************

-- **************************************************************

--

##### -- CU-DU Cell Switch Notification

--

-- **************************************************************

CUDUCellSwitchNotification ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ CUDUCellSwitchNotificationIEs}},

...

}

CUDUCellSwitchNotificationIEs F1AP-PROTOCOL-IES ::= {

{ ID id-gNB-CU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-CU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-gNB-DU-UE-F1AP-ID		CRITICALITY reject	TYPE GNB-DU-UE-F1AP-ID				PRESENCE mandatory	}|

{ ID id-NRCGI					CRITICALITY reject	TYPE NRCGI							PRESENCE mandatory	}|

{ ID id-LTMCellSwitchInformation	 CRITICALITY ignore	TYPE LTMCellSwitchInformation	PRESENCE optional	}|

{ ID id-TAInformation-List			CRITICALITY ignore	TYPE TAInformation-List			PRESENCE optional		}|

{ ID id-OldServingCell				CRITICALITY ignore	TYPE NRCGI						PRESENCE optional		},

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

(skip unchanged)

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-candidatePSCellsToCancel,

id-ValidityAreaSpecificSRSInformationExtended,

id-LTMgNB-ID,

id-L1ExecutionConditionList,

id-RequestforCSIResourceConfig,

id-CSIResourceConfig,

id-RequestforL1ExecutionCondition,

id-TATValue,

id-TATValueTAG2,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

maxnoofAdditionalSIBs,

maxnoofUACPLMNs,

maxnoofUACperPLMN,

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### -- C

(skip unchanged)

CUtoDUTAInformation-List ::= SEQUENCE (SIZE(1.. maxnoofTAList)) OF CUtoDUTAInformation-Item

CUtoDUTAInformation-Item	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

preambleIndex			PreambleIndex,

rA-RNTI					RA-RNTI,

tagIDPointer			TagIDPointer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { CUtoDUTAInformation-Item-ExtIEs} }	OPTIONAL,

...

}

CUtoDUTAInformation-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-TATValue		CRITICALITY ignore	EXTENSION TATValue	PRESENCE optional }|

{ ID id-TATValueTAG2	CRITICALITY ignore	EXTENSION TATValue	PRESENCE optional },

...

}

CSIResourceConfiguration ::= SEQUENCE  {

cSIResourceConfigToAddModList 			OCTET STRING		OPTIONAL,

cSIResourceConfigToReleaseList			OCTET STRING		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { CSIResourceConfiguration-ExtIEs} }	OPTIONAL

}

CSIResourceConfiguration-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### -- D

(skip unchanged)

DUtoCUTAInformation-Item	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

preambleIndex			PreambleIndex,

rA-RNTI					RA-RNTI,

sourceGNB-DU-ID 		GNB-DU-ID,

tagIDPointer			TagIDPointer		OPTIONAL,

iE-Extensions			ProtocolExtensionContainer { { DUtoCUTAInformation-Item-ExtIEs} }	OPTIONAL,

...

}

DUtoCUTAInformation-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-LTMgNB-ID		CRITICALITY ignore	EXTENSION GlobalGNB-ID	PRESENCE optional }|

{ ID id-TATValue		CRITICALITY ignore	EXTENSION TATValue	PRESENCE optional }|

{ ID id-TATValueTAG2	CRITICALITY ignore	EXTENSION TATValue	PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### -- L

(skip unchanged)

LTMInformation-Setup ::= SEQUENCE {

lTMIndicator						LTMIndicator,

referenceConfiguration				ReferenceConfiguration								OPTIONAL,

cSIResourceConfiguration			CSIResourceConfiguration							OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMInformation-Setup-ExtIEs} }	OPTIONAL,

...

}

LTMInformation-Setup-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RequestforCSIResourceConfig		CRITICALITY		reject		EXTENSION		RequestforCSIResourceConfig		PRESENCE		optional	} |

{ ID id-RequestforL1ExecutionCondition		CRITICALITY		ignore		EXTENSION		RequestforL1ExecutionCondition		PRESENCE		optional	},

...

}

LTMConfigurationIDMappingList	::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF LTMConfigurationIDMapping-Item

LTMConfigurationIDMapping-Item::= SEQUENCE{

lTMCellID			NRCGI,

lTMConfigurationID	LTMConfigurationID,

iE-Extensions		ProtocolExtensionContainer {{ LTMConfigurationIDMapping-Item-ExtIEs}}		OPTIONAL

}

LTMConfigurationIDMapping-Item-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMInformation-Modify	::= SEQUENCE {

lTMIndicator						LTMIndicator,

referenceConfiguration				ReferenceConfiguration									OPTIONAL,

cSIResourceConfiguration 			CSIResourceConfiguration								OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMInformation-Modify-ExtIEs} }	OPTIONAL,

...

}

LTMInformation-Modify-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-RequestforCSIResourceConfig	CRITICALITY		reject	EXTENSION		RequestforCSIResourceConfig		PRESENCE		optional	}|

{ ID id-RequestforL1ExecutionCondition	CRITICALITY		ignore		EXTENSION		RequestforL1ExecutionCondition		PRESENCE		optional	}|

{ ID id-TATValue						CRITICALITY ignore	EXTENSION TATValue					PRESENCE optional }|

{ ID id-TATValueTAG2					CRITICALITY ignore	EXTENSION TATValue					PRESENCE optional },

...

}

RequestforCSIResourceConfig ::= ENUMERATED {true, ...}

RequestforL1ExecutionCondition ::= SEQUENCE (SIZE(1.. maxnoofLTMCells)) OF RequestforL1ExecutionConditionCandidateCellList-Item

RequestforL1ExecutionConditionCandidateCellList-Item ::= SEQUENCE {

candidateCellID			NRCGI,

iE-Extensions			ProtocolExtensionContainer { { RequestforL1ExecutionConditionCandidateCellList-ExtIEs } }	OPTIONAL,

...

}

RequestforL1ExecutionConditionCandidateCellList-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMIndicator ::= ENUMERATED {true, ..., c-ltm}

CompleteCandidateConfigurationIndicator	::= ENUMERATED {complete, ...}

LTMConfigurationID ::=  INTEGER (1..8)

ReferenceConfigurationInformation ::= OCTET STRING

LTMConfiguration	::= SEQUENCE {

sSBInformation					SSBInformation,

referenceConfigurationInformation 	ReferenceConfigurationInformation			OPTIONAL,

completeCandidateConfigurationIndicator		CompleteCandidateConfigurationIndicator 				OPTIONAL,

lTMCFRAResourceConfig				LTMCFRAResourceConfig						OPTIONAL,

lTMCFRAResourceConfigSUL			LTMCFRAResourceConfig						OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { LTMConfiguration-ExtIEs } }	OPTIONAL,

...

}

LTMConfiguration-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-L1ExecutionConditionList		CRITICALITY ignore	EXTENSION L1ExecutionConditionList	PRESENCE optional }|

{ ID id-CSIResourceConfig			CRITICALITY ignore	EXTENSION CSIResourceConfiguration		PRESENCE optional }|

{ ID id-TATValue						CRITICALITY ignore	EXTENSION TATValue					PRESENCE optional }|

{ ID id-TATValueTAG2					CRITICALITY ignore	EXTENSION TATValue					PRESENCE optional },

...

}

LTMCellSwitchInformation	::= SEQUENCE {

jointorDLTCIStateID			JointorDLTCIStateID,

uLTCIStateID					ULTCIStateID 				OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { LTMCellSwitchInformation-ExtIEs } }	OPTIONAL,

...

}

LTMCellSwitchInformation-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMgNB-DU-IDsList	::= SEQUENCE (SIZE(1..maxnoofLTMgNB-DUs)) OF LTMgNB-DU-IDs-Item

LTMgNB-DU-IDs-Item	::= SEQUENCE{

lTMgNB-DU-ID			GNB-DU-ID,

iE-Extensions			ProtocolExtensionContainer {{ LTMgNB-DU-IDs-Item-ExtIEs}}		OPTIONAL

}

LTMgNB-DU-IDs-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-LTMgNB-ID		CRITICALITY reject	EXTENSION GlobalGNB-ID	PRESENCE optional },

...

}

LTMgNB-DU-IDs-PreambleIndexList	::= SEQUENCE (SIZE(1..maxnoofLTMgNB-DUs)) OF LTMgNB-DU-IDs-PreambleIndex-Item

LTMgNB-DU-IDs-PreambleIndex-Item	::= SEQUENCE{

lTMgNB-DU-ID			GNB-DU-ID,

preambleIndexList		PreambleIndexList																OPTIONAL,

iE-Extensions			ProtocolExtensionContainer {{ LTMgNB-DU-IDs-PreambleIndex-Item-ExtIEs}}		OPTIONAL

}

LTMgNB-DU-IDs-PreambleIndex-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMCFRAResourceConfig-List ::= SEQUENCE (SIZE (1.. maxnoofLTMCells)) OF LTMCFRAResourceConfig-Item

LTMCFRAResourceConfig-Item ::= SEQUENCE {

nRCGI				NRCGI,

lTMCFRAResourceConfig			LTMCFRAResourceConfig							OPTIONAL,

lTMCFRAResourceConfigSUL		LTMCFRAResourceConfig							OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { LTMCFRAResourceConfig-Item-ExtIEs } }	OPTIONAL,

...

}

LTMCFRAResourceConfig-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMCFRAResourceConfig ::= OCTET STRING

L1ExecutionConditionList	::= SEQUENCE (SIZE(1..maxnoofL1Conditions)) OF L1ExecutionCondition-Item

L1ExecutionCondition-Item	::= SEQUENCE{

ltmCellID				NRCGI,

executionCondition		OCTET STRING,

iE-Extensions			ProtocolExtensionContainer {{ L1ExecutionCondition-Item-ExtIEs}}		OPTIONAL

}

L1ExecutionCondition-Item-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

LTMSecurityInformation	::= SEQUENCE{

nextHopChainingCount	INTEGER (0..7),

securityChangeServCellConfig		OCTET STRING,

securityChangeCandidateCellInfoList	SecurityChangeCandidateCellInfoList,

iE-Extensions			ProtocolExtensionContainer {{ LTMSecurityInformation-ExtIEs}}		OPTIONAL

}

LTMSecurityInformation-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

SecurityChangeCandidateCellInfoList ::= SEQUENCE (SIZE(1..maxnoofLTMCells)) OF SecurityChangeCandidateCellInfo-Item

SecurityChangeCandidateCellInfo-Item ::=	SEQUENCE{

cellID								NRCGI,

securityChangeCandidateCellConfig	OCTET STRING,

iE-Extensions						ProtocolExtensionContainer { { SecurityChangeCandidateCellInfo-Item-ExtIEs} } OPTIONAL

}

SecurityChangeCandidateCellInfo-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

#### -- T

(skip unchanged)

TAInformation-List	::= SEQUENCE (SIZE(1.. maxnoofTAList)) OF TAInformation-Item

TAInformation-Item	::= SEQUENCE {

nRCGI					NRCGI,

tAValue					TAValue,

iE-Extensions			ProtocolExtensionContainer { { TAInformation-Item-ExtIEs} }	OPTIONAL,

...

}

TAInformation-Item-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

{ ID	id-TagIDPointer	CRITICALITY ignore	EXTENSION	TagIDPointer	PRESENCE optional},

...

}

TATValue ::= ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity,...}	d

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

(skip unchanged)

maxnoofUEsInQMCTransferControlMessage	INTEGER ::= 512

maxnoofUEsforRAReportIndications		INTEGER ::= 64

maxnoofSuccessfulPSCellChangeReports	INTEGER ::= 64

maxnoofPeriodicities					INTEGER ::= 8

maxnoofThresholdMBS-1					INTEGER ::= 7

maxMBSSessionsinSessionInfoList			INTEGER ::= 1024

maxnoofLBTFailureInformation			INTEGER ::= 64

maxnoofRSPPQoSFlows						INTEGER ::= 2048

maxnoVACell								INTEGER ::= 32

maxnoAggregatedSRS-Resources			INTEGER ::= 3

maxnoAggregatedPosSRSResourceSets		INTEGER ::= 3

maxnoAggregatedPosPRSResourceSets		INTEGER ::= 3

maxnoofTimeWindowSRS					INTEGER ::= 16

maxnoofTimeWindowMea					INTEGER ::= 16

maxnoPreconfiguredSRS					INTEGER ::= 16

maxnoHopsMinusOne						INTEGER ::= 5

maxnoAggCombinations					INTEGER ::= 2

maxnoAggregatedPosSRSCombinations		INTEGER ::= 32

maxnoofCandidateCells					INTEGER ::= 8

maxnoofSSBIndices						INTEGER ::= 64

maxnoofL1Conditions						INTEGER ::= 8	--TBD

<<<<<<<<<<<<<<<<<<<< Next change >>>>>>>>>>>>>>>>>>>>

id-candidatePSCellsToCancel							ProtocolIE-ID ::= 858

id-MobilityInitiation								ProtocolIE-ID ::= 859

id-ValidityAreaSpecificSRSInformationExtended		ProtocolIE-ID ::= 860

id-PLMNIndexNRAssistanceInfoForNetShar						ProtocolIE-ID ::= 861

id-LTMgNB-ID										ProtocolIE-ID ::= 862	–- To be assigned by MCC

id-L1ExecutionConditionList							ProtocolIE-ID ::= 863

id-LTMSecurityInformation							ProtocolIE-ID ::= 864

id-RequestforCSIResourceConfig					ProtocolIE-ID ::= 865

id-CSIResourceConfig								ProtocolIE-ID ::= 866

id-RequestforL1ExecutionCondition					ProtocolIE-ID ::= 867

id-TATValue											ProtocolIE-ID ::= 868

id-OldServingCell									ProtocolIE-ID ::= 869

id-TATValueTAG2                                 	ProtocolIE-ID ::= 870

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>

# Appendix B – TP for TS 38.401

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

#### 8.2.1.x Conditional intra-CU LTM (Intra-gNB-DU)

This procedure is used for the case when the UE moves within the same gNB-DU during NR operation for conditional LTM. Figure 8.2.1.x-1 shows the intra-gNB-DU conditional LTM procedure for intra-NR.

Figure 8.2.1.x-1: Conditional intra-CU LTM (Intra-gNB-DU)

1.	The UE sends a MeasurementReport message (L3 measurement result) to the gNB-DU containing measurements of neighbouring cells. The gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate conditional LTM configuration.

3.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU for each candidate cell, containing conditional LTM indication, one candidate cell ID and the CSI resource configuration for subsequent LTM. The gNB-CU may provide the LTM configuration ID mapping list to the gNB-DU. The gNB-CU may request PRACH resources from the gNB-DU. The gNB-CU may request the gNB-DU to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer reference configuration to the gNB-DU. The gNB-CU may inform the gNB-DU about intra-DU L2 reset configuration. If the gNB-CU decides to initiate the L1 event-triggered conditional LTM, it also provides a list of candidate cells to which the L1 event-triggered conditional LTM is applied and requests the gNB-DU to generate the corresponding L1-based execution condition(s).

4.	If the gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT MODIFICATION RESPONSE message including the generated lower layer RRC configurations for the accepted candidate cell. If the L1-based execution conditions are requested, the gNB-DU also provides a list of execution conditions generated for other candidate cells (detail FFS).

NOTE 1:	Steps 3 and 4 may be initiated multiple times for conditional LTM candidate cell preparation of multiple cells including the source cell.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the gNB-DU which may include the LTM configuration ID mapping list and/or the updated CSI resource configuration. The gNB-CU may inform the gNB-DU about intra-DU L2 reset configuration.

6.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell. If the L1-based execution conditions are requested, the source gNB-DU also provides a list of execution conditions generated for the candidate cells.

NOTE 2:	In case of subsequent conditional LTM, the CU-initiated UE Context Modification procedure may be invoked per each candidate cell to transfer to the gNB-DU the updated CSI resource configuration.

7.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the gNB-DU, which includes the generated RRCReconfiguration message with the conditional LTM configuration.

8.	The gNB-DU forwards the received RRCReconfiguration message to the UE.

9.	The UE responds to the gNB-DU with an RRCReconfigurationComplete message.

10.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

11.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

12.	The gNB-DU sends the MAC CE to the UE.

13.	The execution condition(s) to trigger initiation of conditional LTM is fulfilled in the UE.

NOTE 3: The gNB-DU may decide to trigger an LTM Cell Switch Command MAC CE to the UE towards a candidate cell with conditional LTM candidate configuration.

14. The gNB-DU detects the UE access as specified in TS 38.300 [2].

15. The gNB-DU sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed. The gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source cell, are sent from the gNB-CU to the gNB-DU.

16.	The UE sends an RRCReconfigurationComplete message to the gNB-DU.

17.	The gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

18.	The gNB-CU may send the UE CONTEXT MODIFICATION REQUEST message to the gNB-DU to release the resources of prepared cells.

19.	The gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message.

#### 8.2.1.y Conditional intra-CU LTM (Inter-gNB-DU)

This procedure is used for the case when the UE moves from one gNB-DU to another gNB-DU within the same gNB-CU during NR operation for Conditional LTM. Figure 8.2.1.y-1 shows the inter-gNB-DU Conditional LTM procedure for intra-NR.

Figure 8.2.1.y-1: Conditional intra-CU LTM (Inter-gNB-DU)

1.	The UE sends a MeasurementReport message (L3 measurement result) to the source gNB-DU containing measurements of neighbouring cells. The source gNB-DU sends an UL RRC MESSAGE TRANSFER message conveying the received MeasurementReport message to the gNB-CU.

2.	The gNB-CU determines to initiate conditional LTM configuration.

3.	The gNB-CU sends a UE CONTEXT SETUP REQUEST message to the candidate gNB-DU(s) for each candidate cell, containing conditional LTM indication, one candidate cell ID and the CSI resource configuration for subsequent conditional LTM. The gNB-CU may provide the LTM configuration ID mapping list to the candidate gNB-DU(s). The gNB-CU may request PRACH resources from the candidate gNB-DU(s). The gNB-CU may request the candidate gNB-DU(s) to provide the lower layer configuration for the purpose of generating the reference configuration or provide the lower layer part of the reference configuration to the candidate gNB-DU(s). If the gNB-CU decides to initiate the L1 event-triggered conditional LTM, it also provides a list of candidate cells to which the L1 event-triggered conditional LTM is applied and requests the gNB-DU to generate the corresponding L1-based execution condition(s).

4.	If the candidate gNB-DU accepts the request of LTM configuration, it responds with a UE CONTEXT SETUP RESPONSE message including the generated lower layer RRC configurations for the accepted target candidate cell. If the L1-based execution conditions are requested, the candidate gNB-DU also provides a list of execution conditions and a list of TA timers generated for other candidate cells.

NOTE 1:	The CU-initiated UE Context Modification procedure may be initiated for preparing candidate cells in the source gNB-DU as specified in step 3 and 4 in 8.2.1.4 Intra-gNB-DU conditional LTM.

5.	The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to the source gNB-DU including the information related to early sync and the LTM configuration ID mapping list for the accepted target candidate cell(s). The gNB-CU may send the updated CSI resource configuration to the source gNB-DU. The gNB-CU may inform the source gNB-DU about intra-DU L2 reset configuration.

6.	The source gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message which includes an updated lower layer configuration, e.g., containing the updated CSI report configuration of the source cell. If the L1-based execution conditions are requested, the source gNB-DU also provides a list of execution conditions generated for the candidate cells.

7.	The gNB-CU may send a UE CONTEXT MODIFICATION REQUEST message for each candidate cell accepted in the candidate gNB-DU(s), containing the information for subsequent conditional LTM or for updating the configurations of candidate cells. The gNB-CU may also provide the lower layer part of the reference configuration to the candidate gNB-DU(s). The gNB-CU may inform the candidate gNB-DU(s) about intra-DU L2 reset configuration.

8.	The candidate gNB-DU responds with a UE CONTEXT MODIFICATION RESPONSE message including the updated lower layer configuration, e.g., containing the updated CSI report configuration of the requested candidate cell. If the L1-based execution conditions are requested, the candidate gNB-DU also provides a list of execution conditions generated for other candidate cells.

NOTE 2: Step 7 may also be triggered after step 18 for subsequent conditional LTM.

9.	The gNB-CU sends a DL RRC MESSAGE TRANSFER message to the source gNB-DU, which includes the generated RRCReconfiguration message with the conditional LTM configuration.

10.	The source gNB-DU forwards the received RRCReconfiguration message to the UE.

11.	The UE responds to the source gNB-DU with an RRCReconfigurationComplete message.

12.	The source gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

13.	Early TA acquisition to the candidate cell(s) may be performed as specified in TS 38.300 [2].

14. The candidate gNB-DU sends a DU-CU TA INFORMATION TRANSFER message to the gNB-CU, which includes the TA values, and the associated PRACH resource information.

15.	The gNB-CU forwards the TA value and the associated PRACH resource information to the source gNB-DU in the CU-DU TA INFORMATION TRANSFER message.

Editor’s Note: Details are FFS on step 14 and 15 on how to handle the TAT.

16.	The source gNB-DU sends the MAC CE to the UE.

17. The execution condition(s) to trigger initiation of conditional LTM is fulfilled in the UE.

NOTE 3: The source gNB-DU may decide to trigger a LTM Cell Switch Command MAC CE to the UE towards a candidate cell with conditional LTM candidate configuration.

18. The target gNB-DU detects the UE access as specified in TS 38.300 [2].

19. The target gNB-DU sends an ACCESS SUCCESS message to inform the gNB-CU of which cell the UE has successfully accessed. The target gNB-DU also sends a Downlink Data Delivery Status frame to inform the gNB-CU.

20. The gNB-CU sends a CU-DU CELL SWITCH NOTIFICATION message to inform the target gNB-DU of which cell served the UE before the conditional LTM cell switch execution. The target gNB-DU may use the information for L2 reset handling.

21.	The UE sends an RRCReconfigurationComplete message to the target gNB-DU.

22.	The target gNB-DU forwards the RRCReconfigurationComplete message to the gNB-CU via an UL RRC MESSAGE TRANSFER message.

23. The gNB-CU sends a UE CONTEXT MODIFICATION REQUEST message to source gNB-DU to inform that the UE executed an inter-DU Conditional LTM and indicate to stop the data transmission for the UE. The source gNB-DU sends a Downlink Data Delivery Status frame to inform the gNB-CU about the unsuccessfully transmitted downlink data to the UE. Downlink packets, which may include PDCP PDUs not successfully transmitted in the source gNB-DU, are sent from the gNB-CU to the target gNB-DU.

NOTE 4: The step 23 may happen before step 22, as soon as the gNB-CU knows which cell the UE has successfully accessed.

24. The source gNB-DU responds to the gNB-CU with a UE CONTEXT MODIFICATION RESPONSE message.

25.	The gNB-CU may send the UE CONTEXT RELEASE COMMAND message to the source gNB-DU to release the resources of prepared cells.

26.	The source gNB-DU responds with a UE CONTEXT RELEASE COMPLETE message.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>