3GPP TSG-RAN WG3 Meeting #129	R3-255272

Bengaluru, India, 25 – 29 August 2025

Agenda Item:		11.3

Source:		Ofinno

Title:	(TP for BLCR to 38.473) AI/ML enabled Coverage and Capacity Optimization: Cause value in the Neighbour Future Coverage Modification Notification

Document for:		Text proposal

# Introduction

In the RAN3 #128 meeting, Opton2 was agreed: gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU.

However, there is cause value missing in the Neighbour Future Coverage Modification Notification.

This TP to BL CR for 38.473 proposes to add Cause for Neighbour Future Coverage Modification to the Neighbour Future Coverage Modification Notification.

Also, some editorial clarifications are proposed.

# Text proposal

#### 9.3.1.C	Neighbour Future Coverage Modification Notification

This IE includes a list of cells and/or SS/PBCH block indexes with the corresponding future coverage configuration selected by one or more neighbour node(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Neighbour Future Coverage Modification Notification List |  | 1 |  |  |
| >Neighbour Future Coverage Modification Notification Item |  | 1..< maxNeighbourCellingReport > |  |  |
| >>NR CGI | M |  | 9.3.1.12 |  |
| >>Neighbour Future Cell Coverage State | M |  | INTEGER (0..63, ...) | Value ‘0’ indicates that the cell will be inactive. Other values indicate that the cell will be active and also indicate the future coverage configuration of the concerned cell. |
| >>Neighbour Future SSB Modification Notification List |  | 0..1 |  |  |
| >>>Neighbour Future SSB Modification Notification Item |  | 1..<maxnoofSSBAreas> |  |  |
| >>>>SSB Index | M |  | INTEGER (0..63) |  |
| >>>>Neighbour Future SSB Coverage State | M |  | INTEGER (0..15, ...) | Value ‘0’ indicates that the SSB beam will be inactive. Other values indicate that the SSB beams will be active and also indicate the future coverage configuration of the concerned SSB beams. |
| >> Cause for Neighbour Future Coverage Modification | O |  | ENUMERATED (coverage, cell edge capacity, ...) | Indicates the reason for the neighbour future coverage modification by the neighbour node(s). |
| >>Time for Neighbour Future Coverage Modification | O |  | INTEGER (1..60, ...) | Indicates the time when the Neighbour Future Cell Coverage State(s) and/or the Neighbour Future SSB Coverage State(s) will be applied by the neighbour node(s) relative to the time of receiving this information, in seconds. |

| Range bound | Explanation |
| --- | --- |
| maxNeighbourCellingReport | Maximum no. neighbour cells for which the future neighbour state can be reported. Value is 512. |
| maxnoofSSBAreas | Maximum numbers of SSB Areas that can be served by a NG-RAN node cell. Value is 64. |

# Reference

[1] R3-255001, RAN3#129 Meeting Agenda

[2] R3-255002, RAN3#128 Meeting Report

[3] R3-255046, (BL CR to TS 38.473) Support of enhancements on AI/ML for NG-RAN