3GPP TSG-RAN WG3 #129	R3-255399

Bangaluru, India, 25th – 29th August 2025

Agenda Item:	11.4

Source:	Lenovo, CATT, ZTE Corporation

Title: 		TP to 38.401 AIML for RAN in split architecture

Document for:	Discussion and Approval

# Introduction

In this paper, text is proposed to capture the aspects related procedures in split architecture to support AIML for RAN.

# TP to 38.401

***************************************** START OF CHANGE ****************************************************

# 7.11	Support of AI/ML for NG-RAN

### 7.11.1	General

The support of AI/ML for NG-RAN is specified in TS 38.300 [2].

In case of CU-DU split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU.

For a split gNB, the Energy Cost (EC) of the gNB is the sum of the Energy Cost of its gNB-DUs.

In case of CU-CP and CU-UP split architecture, the following scenarios may be supported:

-	AI/ML Model Training is located in the OAM and AI/ML Model Inference is located in the gNB-CU-CP.

-	AI/ML Model Training and Model Inference are both located in the gNB-CU-CP.

gNB-CU-CP may request from gNB-CU-UP the reporting of information to support AI/ML for NG-RAN.

### 7.11.2	OAM requirements

OAM configures the following on gNB-DU:

-	the minimum and maximum energy consumption values corresponding to the minimum and maximum EC index values respectively, based on an implementation-specific mapping rule, which is unified within a defined area.

-	the recommended time interval within which an NG-RAN node selects an implementation-specific time window for averaging of the measurements of the NG-RAN node's consumed energy.

***************************************** END OF CHANGE ****************************************************