3GPP TSG-RAN WG3 Meeting #129	R3-255483

Bengaluru, India, 25th ~29th Aug, 2025

Title:	Usage of root cause in MRO

Agenda Item:	9.2

Source:	Huawei, Vodafone, Nokia, Qualcomm, BT, Jio Platforms

Document for:	discussion

# 1	Introduction

The term root cause analysis was used in initial discussion for SON in LTE and was used throughout all MRO related discussions. It is however not so often captured in the specifications. When it is used, it is representing different meaning. This paper discuss how to clarify this.

# 2	Background

For LTE, TS 36.300 include the wording “root cause” in the description for Radio Link Failure Indication procedure as follows:

The previously serving eNB may then match the correct context, or use the information available in the RLF Report, if included in the RLF INDICATION message, to analyze the possible root cause of the failure. If the previous serving eNB matches the correct context, it may also trigger the Handover Preparation procedure towards the eNB that initiated the Radio Link Failure Indication procedure.

The text here relates to a more general description of the usage of the procedure, but is not included in the MRO definition part. Hence, the details on the analysis is captured in a different part of the specification where the term root cause is not used. In this text, the term root cause could be replaced with “cause”.

The term root cause is not used for NG-RAN in 38.300.

The term root cause (without the word “analysis”) exist is XnAP and 37.340, but is used more as a general description i.e. not necessarily pointing to a specific action performed by the node. This is excluded from the analysis below since it does not appear to be contradicting. It would however be possible to remove the "root" and just keep "cause" without impacting any functionality.

Root cause analysis is used in 37.340 in the following places:

1)	Conditional PSCell addition or change failure

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final root cause analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs root cause analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

In the two sentences above, the root cause seems to encompass the analysis to either decide where to send forward information or the final analysis itself.

2)	PSCell change failure

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final root cause analysis.

In this section, the term initial analysis is used to determine where to send information and root cause is used to define the final analysis, i.e. whether/how to modify parameters. This is however not spelled out.

3)	Successful PSCell Addition/Change Report

For analysis of such sub-optimal successful PSCell change/CPC and successful PSCell addition/CPA, the UE may collect SPR based on the triggers configured by the network and makes the SPR available to the network as specified in TS 38.331 [4]. If the SPR available indication via SN RRCReconfigurationComplete is received by SN, SN should inform MN that an SPR is available at the UE via S-NODE MODIFICATION REQUIRED message.

For PSCell addition/CPA and PSCell change/CPC (MN or SN initiated), the target SN always decides the T304 trigger for SPR and performs root cause analysis.

In this section root cause is used to define the final analysis, i.e. whether/how to modify parameters.

## 3	Discussion

Even if the term root cause was used in the discussions since LTE, there is no clear definition of what is included in root cause and what it entails. In the discussions it was often used to perform an analysis to identify the problem and decide what to do. The result may be to change the parameters for this node or to identify which node was causing the problem and inform that node.

At the moment the term root cause is used in different places with different meaning. In some cases the behaviour is defined by a following sentence but in other cases the behaviour is not defined but can be understood.

Therefore one proposal to clarify this is to avoid using the term root cause but rather talk about:

- Initial analysis: The first analysis performed to determine where to forward information

- Final analysis: the final analysis determining how to optimize the parameters.

This approach is illustrated below for 37.340.

1)	Conditional PSCell addition or change failure

The MN performs the initial analysis when SCGFailureInformation is received from the UE. In the first step, MN verifies whether intra-SN PSCell change has been triggered in the last serving SN. In case the intra-SN PSCell change has been triggered in the last serving SN, the MN forwards the SCG Failure Information Report message to this last serving SN, which performs the final analysis. In case of no intra-SN PSCell change, the MN determines the type of PSCell addition/change, e.g., whether it is CPA or CPC in case of conditional mobility, if CPC whether it is MN initiated

For SN initiated CPC, the MN sends the SCG Failure Information Report message to source SN, and source SN performs an initial analysis. If the suitable PSCell is one of the candidate PSCells provided by the source SN, but not one of the candidate PSCells selected by the candidate or target SN, the source SN indicates to MN that the root cause of the SCG failure may have occurred in the other nodes. MN then sends the SCG Failure Information Report message to the candidate or target SN. Otherwise, the source SN performs the final MRO related optimisation.

2)	PSCell change failure

MN performs initial analysis to identify the node that caused the failure. The MN may use the SCG Failure Information Report procedure to verify whether intra-SN PSCell change has been triggered in the last serving SN and stores the SCG Failure Information for the time needed to receive possible response from the last serving SN. If the failure is caused by a source SN, the MN forwards then the SCG Failure Information to the source SN. The node responsible for the last PSCell change (the source SN, the last serving SN or the MN) performs the final analysis.

3)	Successful PSCell Addition/Change Report

For analysis of such sub-optimal successful PSCell change/CPC and successful PSCell addition/CPA, the UE may collect SPR based on the triggers configured by the network and makes the SPR available to the network as specified in TS 38.331 [4]. If the SPR available indication via SN RRCReconfigurationComplete is received by SN, SN should inform MN that an SPR is available at the UE via S-NODE MODIFICATION REQUIRED message.

For PSCell addition/CPA and PSCell change/CPC (MN or SN initiated), the target SN always decides the T304 trigger for SPR and performs the final analysis.

Based on the above discussion, we propose:

Reword the usage of root cause in TS 37.340

# 3	Conclusion

This document proposes:

Proposal 1:	Reword the usage of root cause in TS 37.340

The proposed change is captured in the associated CR for Rel18.