3GPP TSG-RAN WG3 Meeting #129	R3-255581

Bengaluru, India, August 25th–29th, 2025

Agenda Item:	17.4

Source:	Ericsson

Title:	Adaptation of common signal/channel transmissions

Document for:	Discussion, Decision

# 1	Introduction

The revised WID for “Enhancements of network energy savings for NR” [1] in Rel-19 states the following objectives:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

This paper discusses the “adaptation of common signal/channel transmissions” related topics (Objective 3).

A complete list of RAN1 and RAN2 agreements on this objective is provided at the end of the paper.

# 2	Discussion

## 2.1	Adaptation of SSB transmission

It has been concluded by RAN1 and confirmed by RAN2 that SSB adaptation is not for RRC Idle/Inactive mode UEs and not for Rel-19 NES-capable UEs’ PCell. We thus observe that adaptation of SSB transmission is only for Rel-19 NES-capable UEs’ SCell(s). Also note that, a cell is an SCell once it is configured as an SCell; before that, it is a neighbor cell.

According to RAN1 and RAN2 conclusions, adaptation of SSB transmission is only for Rel-19 NES-capable UEs’ configured SCell(s).

RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

Send response LS to RAN4 (Cc: RAN1) to inform RAN2 preference and to ask if RAN4 has any concern.

For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

The agreement above will be also included in the response LS.

In addition, RAN2 has discussed whether to follow the SMTC (SSB Measurement Timing Configuration) based L3 RRM framework for SCell with SSB adaptation, and for neighbor cell with SSB adaptation. The following agreements were captured at RAN2#129:

We observe that RAN2 intends to keep the SMTC based L3 RRM framework for both cases, SCell with SSB adaptation and neighbor cell with SSB adaptation.

For the case of SCell with SSB adaptation, RAN2 has identified the need to introduce additional SMTC configuration according to which the UE performs L3 measurements upon SSB adaptation. For the case of neighbor cell with SSB adaptation, however, RAN2 has concluded that there is no need for enhancement, i.e., the UE performs L3 measurements according to the legacy SMTC configuration.

According to RAN2 preference, for SSB adaptation on SCell, the SMTC based L3 RRM framework is applied and is enhanced by additional SMTC configuration.

According to RAN2 preference, for SSB adaptation on neighbor cell, the legacy SMTC based L3 RRM framework is applied without enhancement.

Considering this (namely, Observation 1 and Observation 3), we conclude that SSB adaptation on neighbor cell has no RAN3 impact.

For SSB adaptation, RAN3 to conclude that SSB adaptation on neighbor cell has no RAN3 impact.

For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

For SSB adaptation on SCell, RAN2 agreed the following in the RAN2#129bis meeting:

Stage 3 details are not yet decided, but we assume that RAN2 will add the foreseen additional SMTC configurations (as an enhancement) to the existing MeasurementTimingConfiguration message, which is defined in TS 38.331 (RRC specification) and signaled over F1 and Xn as a container (as an OCTET STRING). In that case, the legacy signaling over F1 and Xn is still sufficient and there is again no RAN3 impact. We propose RAN3 to wait for further RAN2 progress regarding the SMTC enhancement (that was agreed in RAN2).

For SSB adaptation, RAN3 to wait for further RAN2 progress regarding the SMTC enhancement.

Since SSB adaptation is only for Rel-19 NES-capable UEs’ SCell(s), as per Observation 1, it is not visible to any other UEs and cannot impact them, thus cannot benefit them. SSB adaptation is hence not for neighbor cell. This is well aligned with the RAN2 agreement that, for neighbor cell measurements, the legacy SMTC based L3 RRM framework is applied without enhancement (see Observation 3).

Besides, in CA operation, all serving cells of a UE belong to one gNB, more specifically, one gNB-DU.

SSB adaptation in a cell of a gNB(-DU) may only be visible to Rel-19 NES-capable UEs served by that cell/gNB(-DU) as an SCell.

Consequently, for SSB transmissions according to the additional SMTC configuration, there is no need to inform a neighbor node about such SSB transmissions or request a neighbor node to perform SSB transmissions (since UEs served by other nodes cannot benefit from them). Similarly, the gNB-DU does not need to inform the gNB-CU when such SSB transmissions take place, and the gNB-CU has no reason to request the gNB-DU to perform SSB transmissions. We conclude that SSB adaptation can be, and therefore should be, a gNB-DU internal process. However, note that, independent of the triggering of SSB adaptation, the additional SMTC configuration may be sent to the gNB-CU for configuring a UE for SSB adaptation.

For SSB adaptation, RAN3 to conclude there is no need to inform the gNB-CU or neighbor nodes about the triggering of SSB adaptation and no need for the gNB-CU or neighbor nodes to request/trigger SSB adaptation.

On the other hand, SSB transmissions according to the legacy SMTC configuration must not be affected by SSB adaptation. This is because the legacy SMTC information is semi-statically exchanged over F1 and Xn and is semi-statically configured at the UEs. Those SSB transmissions must occur in order not to distort, e.g., L3 measurements taken by UEs in neighbor cells. For practical purposes, this may mean that SSB adaptation can only be used to trigger additional SSB transmissions (on top of legacy SSB transmissions), when needed.

In legacy, the SMTC configuration (including ARFCN frequency, subcarrier spacing, periodicity, time offset, transmit power, etc.) is decided by the gNB-DU, e.g., based on OAM configuration.

In legacy, it is the gNB-DU that determines the SMTC configuration including ARFCN frequency, subcarrier spacing, periodicity, time offset and transmit power; the gNB-CU is not involved.

As suggested above, we believe the same principle can be applied to the additional SMTC configuration. We see no reason to limit the use of SSB adaptation at the gNB-DU. The gNB-DU should be free to apply SSB adaptation (i.e., perform additional SSB transmission) as it sees fit, since the gNB-DU is the entity that is in control of SCell operation and that observes the effect of SSB adaptation on CA performance and NW energy consumption. We thus believe that the gNB-DU should not require permission from the gNB-CU for using the SSB adaptation functionality in a cell.

For SSB adaptation, RAN3 to agree that the gNB-DU does not require permission from the gNB-CU for using SSB adaptation in a cell.

## 2.2	Adaptation of PRACH

According to various RAN1 agreements (see Section 5), adaptation of PRACH in time domain can be summarized as follows:

Additional PRACH resources (in addition to the PRACH resources available to legacy UEs) can be provided to Rel-19 NES-capable UEs via semi-static signaling. This is simply an additional PRACH configuration. Moreover, the additional PRACH resources can be indicated as available or not available via DCI-based signaling, thus dynamically adapted.

Adaptation of PRACH in time domain relies on semi-statically configured additional PRACH resources that can be dynamically de-/activated.

To avoid excessive inter-cell interference on PRACH, it must be assured that intra-frequency neighbor cells use different PRACH resources in time, frequency, or code domain, e.g., by using different root sequences for deriving preamble sequences.

To automate this, RAN3 has introduced the exchange of information on configured PRACH resources per cell over F1 and Xn. A gNB-DU can share this information (a list of PRACH configurations) to its gNB-CU via F1, which can forward the information to neighbor gNB-CUs via Xn, which can in turn distribute the information to their gNB-DUs via F1. With knowledge of the PRACH configurations of cells of neighbor gNB-DUs, a gNB-DU can choose the PRACH configurations of its cells accordingly.

In current RAN3 specifications, gNB-DUs can, indirectly via their gNB-CU(s), exchange up to 16 PRACH configurations per cell. Hence the additional PRACH configuration that is used for (dynamic) adaptation of PRACH can be coordinated in the same way. The existing signaling is still futureproof and can be reused. We currently see no need to discuss new signaling.

The existing signaling for coordinating PRACH resources among gNBs/gNB-DUs can be reused for coordinating dynamically adaptable PRACH resources in Rel-19.

For adaptation of PRACH, close the discussion unless further progress in RAN2 motivates to revisit it.

## 2.3	Adaptation of paging occasions

It has been observed in RAN3 that, upon paging, a gNB, and, in case of split architecture, the gNB-DU needs to know whether the UE (that is to be paged) supports adaptation of paging occasions (POs).

Considering the NG and Xn interface, this was solved by an earlier RAN2 agreement to add a new UE capability for Rel-19 NES paging enhancement (the pagingAdaptation-r19 field in the IE UE-RadioPagingInfo) to the “paging container” (the UERadioPagingInformation message, defined in TS 38.331), which can already be signaled in the PAGING messages over NG and Xn. Refer to [2] for further details.

For the F1 interface however, this is not the case. The “paging container” as such is not sent in the F1AP PAGING message. Due to this, the following agreement was captured at RAN3#127bis:

Introduce a new indication for NES paging adaptation in UE Paging Capability over F1.

This means that, the UE Paging Capability IE, which may be signaled in the PAGING message over F1, can comprise the above-mentioned information required at the gNB-DU.

In summary: For non-split architecture, RAN2 has addressed the UE capability issue by adding a new capability to the “paging container”. In addition to that, for split architecture, RAN3 has solved the issue by introducing a new indication in UE Paging Capability over F1. We thus consider this issue solved.

For indicating that a UE supports adaptation of paging occasions, RAN2 has addressed the UE capability issue by adding a new capability to the “paging container”, and in addition to that, for split architecture, RAN3 has solved the issue by introducing a new indication in UE Paging Capability over F1.

No other issues regarding adaptation of POs have been raised in RAN3 so far; thus, we see no further impact on RAN3 for now. Given that RAN1 has finished the work on this topic, and RAN2 will finish during this meeting, we propose RAN3 to close the discussion unless further progress in RAN2 motivates to revisit it.

Unless needed due to further agreements in RAN2, adaptation of paging occasions has no further RAN3 specification impact.

For adaptation of paging occasions, close the discussion unless further progress in RAN2 motivates to revisit it.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	According to RAN1 and RAN2 conclusions, adaptation of SSB transmission is only for Rel-19 NES-capable UEs’ configured SCell(s).

Observation 2	According to RAN2 preference, for SSB adaptation on SCell, the SMTC based L3 RRM framework is applied and is enhanced by additional SMTC configuration.

Observation 3	According to RAN2 preference, for SSB adaptation on neighbor cell, the legacy SMTC based L3 RRM framework is applied without enhancement.

Observation 4	SSB adaptation in a cell of a gNB(-DU) may only be visible to Rel-19 NES-capable UEs served by that cell/gNB(-DU) as an SCell.

Observation 5	In legacy, it is the gNB-DU that determines the SMTC configuration including ARFCN frequency, subcarrier spacing, periodicity, time offset and transmit power; the gNB-CU is not involved.

Observation 6	Adaptation of PRACH in time domain relies on semi-statically configured additional PRACH resources that can be dynamically de-/activated.

Observation 7	The existing signaling for coordinating PRACH resources among gNBs/gNB-DUs can be reused for coordinating dynamically adaptable PRACH resources in Rel-19.

Observation 8	For indicating that a UE supports adaptation of paging occasions, RAN2 has addressed the UE capability issue by adding a new capability to the “paging container”, and in addition to that, for split architecture, RAN3 has solved the issue by introducing a new indication in UE Paging Capability over F1.

Observation 9	Unless needed due to further agreements in RAN2, adaptation of paging occasions has no further RAN3 specification impact.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For SSB adaptation, RAN3 to conclude that SSB adaptation on neighbor cell has no RAN3 impact.

Proposal 2	For SSB adaptation, RAN3 to wait for further RAN2 progress regarding the SMTC enhancement.

Proposal 3	For SSB adaptation, RAN3 to conclude there is no need to inform the gNB-CU or neighbor nodes about the triggering of SSB adaptation and no need for the gNB-CU or neighbor nodes to request/trigger SSB adaptation.

Proposal 4	For SSB adaptation, RAN3 to agree that the gNB-DU does not require permission from the gNB-CU for using SSB adaptation in a cell.

Proposal 5	For adaptation of PRACH, close the discussion unless further progress in RAN2 motivates to revisit it.

Proposal 6	For adaptation of paging occasions, close the discussion unless further progress in RAN2 motivates to revisit it.

# 4	References

RP-251678, Revised WID on enhancements of network energy savings for NR

R3-252047, Adaptation of common signal/channel transmissions, Ericsson, RAN3#127bis, Wuhan, China, 7th – 11th April 2025

# 5	RAN1 and RAN2 agreements for Objective 3

  - RAN1 agreements: Adaptation of common channels/signals

    - RAN1#116

Agreement

For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study

- Adaptation of SSB burst periodicity

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration

- Adapting the transmitted number of SSBs within a SSB burst

- Cell DTX for SSB adaptation

- Whether to support new SSB burst periodicity value(s)

- Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst)

  - New compact SSB burst(s)

  - Adapting the position of SSBs within a SSB burst

- Other mechanisms/combinations are not precluded

Agreement

For adaptation of PRACH in time-domain, consider the following adaptation mechanisms for further study

- Adaptation based on configuration of additional[/different] PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

- For the additional PRACH resources,

  - Adaptation of PRACH resource periodicity/PRACH occasion

  - Adaptation at PRACH configuration/association period/association pattern period level and SSB to RO mapping cycle

  - Adaptation based on extending cell DRX operation for PRACH

  - Concentrating ROs in time domain

- Other options are not precluded

Agreement

For adaptation of paging,

- Study further from RAN1 perspective, techniques for adaptation of paging occasions in time-domain and achievable network energy savings

- Note: Specification details for PO/PF determination and paging-related configuration/procedures to be handled by RAN2

Agreement

For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following:

- Applicability to UE in idle/inactive and/or connected mode

- Applicability to PCell and/or SCell(s)

Agreement

For the adaptation mechanisms of SSB in time-domain, study further following mechanisms:

- Adaptation mechanism indicated or configured by gNB without UE trigger

- Adaptation triggered by UE (if any)

FFS: Details of associated signaling/indication/configuration

Agreement

For the adaptation mechanisms of PRACH in time-domain

- Support at least PRACH adaptation provided by gNB without UE trigger

  - FFS: PRACH adaptation with UE trigger

  - Note: UE trigger means UE requests adaptation of PRACH

- Study at least the following,

  - Dynamic signaling and/or semi-static signaling of PRACH adaptation

  - Adaptation of PRACH transmission according to certain condition

  - Applicability to idle/inactive and/or connected mode UEs

  - Which scenarios the adaptation mechanism is applicable to (e.g. cell with both legacy and Rel-19 UE, cell with only Rel-19 UEs)

    - RAN1#116bis

Agreement

For indication of adaptation of SSB in time-domain,

- Support at least SSB adaptation provided by gNB without UE trigger

Agreement

For adaptation of PRACH in time-domain, support at least the following:

- Adaptation based on additional PRACH resources for NES-capable UEs in addition to PRACH resources for legacy UEs (if any)

  - Note: NES-capable UEs can use both additional PRACH resources and PRACH resources for legacy UEs

  - Configuration of additional PRACH resources is provided by semi-static signalling

    - FFS: details including whether there is overlap of additional PRACH resources and PRACH resources for legacy UEs

  - FFS: adaptation mechanism for additional PRACH resources

  - Note: No change to the existing PRACH configuration tables in 38.211

Agreement

For adaptation of PRACH in time-domain, support the following:

- SSB-RO mapping for the additional PRACH resources is separate from the SSB-RO mapping of the PRACH resources for legacy UEs (if any)

  - FFS: whether/how to handle SSB-RO mapping if the additional PRACH resources overlap in both time and frequency with the PRACH resources for legacy UEs

  - Note: SSB-RO mapping of the PRACH resources for legacy UEs is not impacted if Rel-19 UE uses these PRACH resources

  - FFS: SSB-RO mapping for the additional PRACH resources

Agreement

Support adaptation mechanisms of PRACH in time-domain for following:

- UE in idle/inactive mode

- UE in connected mode

Agreement

Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s):

- For cell with both legacy UEs and Rel-19 NES-capable UEs

  - Rel-19 NES-capable UE’s PCell (Connected mode)

    - Study from the following options:

      - Option A1: adaptation for CD-SSB

      - Option A2: adaptation for SSB that is not CD-SSB

      - Option A3: adaptation for SSB not on sync raster

  - Rel-19 NES-capable UE’s SCell

    - Study from the following options:

      - Option B1: adaptation for CD-SSB

      - Option B2: adaptation for SSB that is not CD-SSB

      - Option B3: adaptation for SSB not on sync raster

  - FFS: Rel-19 NES-capable UE in idle/inactive mode

- Note: Impact to idle/inactive UEs shall be minimized

Agreement

For adaptation of PRACH in spatial domain,

- Study possibility of scenarios with non-uniform distribution of UEs in different beams

    - Note 6: Companies are encouraged to provide details on how they map UEs to different beams

- Study network energy savings gain achieved by non-uniform PRACH resource allocation across SSBs for scenarios with non-uniform distribution of UEs in different beams (if any),

    - Assume the following framework for network energy evaluation in FR1 and companies to report at least the below settings used in the evaluation/simulation

      - 20ms SSB period

      - 30kHz SCS, DDDSU TDD pattern

      - Setting A: SIB1 period (20ms/40ms/160ms)

      - Setting B1: Cell load (Empty/low/medium)

      - Setting B2: Traffic model

      - Setting C: SIB1 PDSCH time domain resource index in 38.214 Table 5.1.2.1.1-2

      - Setting D: CORESET0/SSB multiplexing pattern including controlResourceSetZero (index) in 38.213 Table 13-6, and searchSpaceZero (index) in 38.213 Table 13-11

      - Setting E1: PRACH configurations

        - (legacy) PRACH resources according to the following PRACH configuration for all transmitted SSBs

      - Case A1-1: PRACH configuration #5 (20ms)

      - Case A1-2: PRACH configuration #17 (10ms)

      - Case A2-1: PRACH configuration #0 (160ms)

        - (time-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration for all transmitted SSBs

      - Case B1: PRACH configuration #17 (10ms)

      - Case B2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed time domain adaptation mechanism

        - (spatial-domain PRACH adaptation) Additional and legacy PRACH resources yielding total PRACH resources that are according to one of the following PRACH configuration

      - Case C1: PRACH configuration #17 (10ms)

      - Case C2: PRACH configuration #0 (160ms)

      - Companies to report details of assumed spatial domain adaptation mechanism, including details of non-uniform PRACH resource allocation across SSBs

      - Setting F: Cat 1/Cat 2 BS as defined in TR38.864

      - Setting G1: Number of SSB beams: 4,8 SSBs in a SSB burst with SSB pattern case C

      - Note 1: Baseline to compare is Case C1 vs Case B1/A1-1/A1-2, Case C2 vs Case B2/A2-1

      - Note 2: It is up to company to report the SSB-RO mapping ratio and FDMed RO number, etc

      - Note 3: Other PRACH configuration index with different PRACH format other than format 0 is not precluded

      - Note 4: Other SSB/SIB1/RACH periodicity/PRACH resource/configuration assumptions are not precluded (up to companies to report)

    - Other frameworks for network energy evaluation are not precluded, e.g. including for FR2

    - RAN1#117

Agreement

For the study of adaptation of PRACH in spatial domain, following network energy savings gains were reported by sources based on the evaluation framework agreed in RAN1#116bis:

- Two sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs A1-1, zero load [R1-2404409, R1-2405107]

  - -4% ~ -45%

- Seven sources showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1/A1-2, zero load [R1-2404225, R1-2404185, R1-2404334, R1-2404123, R1-2404562, R1-2405107, R1-2405163]

  - 0% ~ 31%

  - Note: Five sources assumed that case B1 has same PRACH resources as case A1-2. Remaining two sources evaluated only A1-2.

  - Note: Three sources showed NES gains 0% ~ 10% [R1-2404225, R1-2404185, R1-2404334]

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 1.0%~8.8%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes.

- One source showed following NES gain for TDD, CAT1 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -48.41%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, CAT1 BS power model, for case C1 vs A1-2, zero load [R1-2404626]

  - 4.59%~38.04%

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Four sources showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1/A1-2, zero load [R1-2404562, R1-2404225, R1-2403943, R1-2404626]

  - 0% ~ 3.5%

  - Note: Three sources assumed that case B1 has same PRACH resources as case A1-2. One source evaluated only A1-2.

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404464]

  - 0%~0.2%

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed following NES gain for TDD, CAT2 BS power model, case C1 vs B1, zero load [R1-2404626]

  - -1.19%~0%

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- Two sources showed following NES gain for TDD, CAT1 or CAT2 BS power model, case C2 vs B2, zero load [R1-2403943, R1-2405107]

  - Less than 0.2%

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs A1-2 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 13.7%/8.7%/4.9%/2.6% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, CAT1 BS power model, (C1 vs B1 with changed PRACH format), PRACH format A, 10ms PRACH periodicity, different loads [R1-2403980]

  - 8.03%/5.1%/3.06%/1.74% for zero/low/light/medium cell load

- One source showed following NES gain for TDD, C1 vs B1/A1-2, different loads [R1-2404562]

  - 16%/4.78% for light/medium cell load for CAT1 BS power model

  - 0.65%/0.29% for light/medium cell load for CAT2 BS power model

- One source showed following NES gain for TDD, C1 vs B1, different loads [R1-2404626]

  - -18.57%~0%/-2.52%~0% for low /medium cell load for CAT1 BS power model

  - -0.81%~0%/-0.42%~0% for low /medium cell load for CAT2 BS power model

  - Note: For B1, it was assumed that periodicity of PRACH resources can be adapted. For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed following NES gain for TDD, C1 vs A1-2, different loads [R1-2404626]

  - 3.67%~19.88%/2.29%~5.22% for low /medium cell load for CAT1 BS power model

  - 0.67%~1.75%/0.39%~0.91% for low /medium cell load for CAT2 BS power model

  - Note: For C1, it was assumed that periodicity of PRACH resources is not adapted and some ROs within a periodicity can be deactivated.

- One source showed NES gain for FDD, C1 vs B1, zero load [R1-2404464]

  - 1.4%~7% for CAT1 BS power model

  - 0%~0.3% for CAT2 BS power model

  - Note: The evaluation results provide the extra NES gain of spatial domain PRACH adaptation compared to time domain PRACH adaptation, where spatial domain and time domain PRACH adaptations are based on dynamic switching between PRACH resources according to two PRACH configuration indexes

- One source showed NES gain for FR2, CAT1 BS power model, spatial domain adaptation of PRACH configuration index 75 vs a time domain adaptation of PRACH configuration index 75, zero load [R1-2405163]

  - 4%~7%

- Note 1: About possibility of scenarios with non-uniform distribution of UEs in different beams

  - Several companies indicated (and three companies showed data/analysis) that there can be scenarios with non-uniform distribution of UEs in different beams.

  - Several companies mentioned that for non-uniform UE distribution, it can be addressed by gNB implementation e.g. by adjusting SSB beamwidth, etc. Several companies also mentioned that it is not clear how gNB can predict the distribution of UEs in different beams, especially for Idle/Inactive UEs.

- Note 2: Most sources that showed the NES gains (if any) for adaptation of PRACH in spatial domain compared to A1-2/B1 observed that the gain would be due to reduction in the number of overall ROs in time domain in their evaluations. Most of these companies only accounted for ROs in time domain.

- Note 3: The evaluation results assumed the non-uniform distribution of UE is static during the evaluation time period.

Conclusion

There is no consensus in RAN1 on the support of PRACH adaptation in spatial domain

Agreement

For adaptation of PRACH in time-domain, support at least the following case(s)

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

- Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains

- FFS: whether additional conditions are needed to support the above cases

- FFS: Additional case whether full/partial overlap in both time and frequency is allowed

- Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources

Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For adaptation of SSB in time-domain, Option 1 is supported

- Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s)

- Note: Using Option 2 to realize Option 1 is not precluded

  - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active]

    - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities

- FFS: Details including applicable scenarios

- FFS: Support of Cell DTX for connected mode UEs for SSB

Agreement

For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least:

- a PRACH configuration index

- FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources

Study further the following

- When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource,

  - Additional parameter(s) for determining the additional PRACH resources e.g.

    - Scaled/adjusted PRACH configuration period

    - Additional timing offset

    - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Muting/masking ROs

- When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource

  - Additional mechanisms (if any) for determining the additional PRACH resources e.g.

    - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

- Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

Agreement

For the adaptation mechanism for additional PRACH resources, study further the following:

- Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation

- Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

  - Strive to re-use existing DCI format(s)

- Option 3: Adaptation of PRACH transmission according to predefined condition(s)

  - FFS: details

- Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation

  - Strive to re-use existing DCI format(s)

- Option 5: Enhanced cell DRX

    - RAN1#118

Agreement

For adaptation of PRACH in time-domain, select at least one from the following alternatives for configuration of the additional PRACH resources

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources and

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 1-1: Scaled/adjusted PRACH configuration period

    - Opt 1-2: Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration

    - Opt 1-3: Muting/masking ROs

    - Opt 1-4: additional timing offset(s)

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources,

  - Discuss further additional mechanism(s) for determining the additional PRACH resources, e.g.

    - Opt 2-1: Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources)

    - Opt 2-2: Additional timing offset(s)

- FFS: Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed)

- FFS: Additional frequency domain parameter(s) (e.g., freq. starting offset)

Agreement

Extend the RAN1#117 agreement on SSB-RO mapping rule for additional PRACH resources to Case 1

- Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs

RAN1#117 Agreement

At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule.

- Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes.

  - Note: This mapping is not impacted by time domain PRACH adaptation

- Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs.

Agreement

For SSB-RO mapping rule for additional PRACH resources for Case 2.

- Extend the RAN1#117 and RAN1#118 agreements on SSB-RO mapping

Agreement

For the adaptation mechanism for additional PRACH resources (for CONNECTED mode UE and IDLE/INACTIVE mode UE),

- At least DCI based adaptation is supported. No introduction of new DCI format.

Agreement

For adaptation mechanism(s) of SSB in time-domain,

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported

  - FFS: Adaptation for SSB that is not CD-SSB is supported (A2)

  - FFS: Adaptation for SSB not on sync raster is supported (A3)

- For Rel-19 NES-capable UE’s SCell

  - Adaptation of SSB configured for the SCell is supported for the following cases

    - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell

    - Adaptation for SSB that is not CD-SSB on sync raster (B2’)

    - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Agreement

For DCI-based adaptation for additional PRACH resources,

- Select from the following DCI format(s) to carry the adaptation indication.

  - DCI format 1_0

  - DCI format 2_7

  - DCI format 2_9

- FFS: existing (P-RNTI, SI-RNTI, CellDTRX-RNTI, PEI-RNTI, C-RNTI) or new RNTI used for detecting the DCI format

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options

- Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period

- Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period

- Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

- Alt 3: DCI-based Enhanced/new Cell DRX to indicate whether the enhanced/new Cell DRX is activated or deactivated.

  - If activated, the additional configured PRACH provided by semi-static signalling within non-active period are not available.

  - FFS: whether Alt 1 and/or Alt 2 can be applied to the active period

  - FFS: details

    - RAN1#118bis

Agreement

For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

Agreement

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

Conclusion

There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19

Agreement

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).

- No spec impact

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- For Alt 1 (same PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

- For Alt 2 (different PRACH configuration index for additional and legacy PRACH resources), select one or more of the following additional mechanism(s),

  - Opt 1-2a: up to N1 additional value(s) of (x, y)

    - FFS: value of N1(>=1)

  - Opt 1-2b: up to N2 additional value(s) of y

    - FFS: value of N2 (>=1)

  - Opt 1-3: Muting/masking ROs

  - Opt 1-4:

    - up to N4_1 additional timing offset(s) at frame-level

    - up to N4_2 additional timing offset(s) at slot-level

    - FFS: values of N4_1(>=1) and N4_2(>=1)

  - Opt 1-5: No additional mechanism is selected.

  - Note: x and y refer to the parameters from the random-access configuration tables (from 38.211)

    - RAN1#119

Agreement

Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?):

- RAN1 agreed that at least SSB burst periodicity is adapted.

- There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options.

Agreement

Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?):

- The frequency location is same before and after SSB adaptation.

Agreement

Reply to Q3(What is the spatial relation before and after SSB adaptation?):

- There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation.

- At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported.

Further update to be made based on RAN1#119 progress if any.

Agreement

At least msg1-FrequencyStart can be configured separately for the additional PRACH resources at least for 4-step RACH.

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: (PRACH resource configuration level) DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: details

- Alt 2: (subset of PRACH resource level) DCI-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not

  - FFS: Maximum number of subsets of the additional PRACH resources= [2 or 3 or 4 or 16]

  - FFS: whether the subset of the additional PRACH resources is in

    - Alt 2-1: RO level per SSB

    - Alt 2-2: SSB-to-RO mapping cycle level

    - Alt 2-3: PRACH association period level

    - Alt 2-4: PRACH association pattern period level

    - Alt 2-5: SFN level

    - Alt 2-6: Network configured time period

Conclusion

There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

Working Assumption

For DCI-based adaptation for additional PRACH resources,

- Select from the following options for carrying the adaptation indication in DCI format 1_0 with P-RNTI

- Option 1: Use reserved bits in the DCI format

  - FFS: relation (if any) to TRS availability bits / short message indicator in the DCI format

- Option 2: Use Bits 5-8 within the Short Message (from upper layers)

  - Note: Availability should be confirmed by checking with RAN2.

- Option 3: Use bits available for both Option 1 and Option 2

FFS: Payload size for adaptation for additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, select only from the following alternatives:

- Alt 1: DCI-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not

  - [DCI payload size = 1 bit]

  - FFS: A single PRACH mask provided by semi-static signalling is used to identify the subset of the additional PRACH resources

  - FFS: details

Agreement

Confirm the following working assumption from RAN1#118bis.

Working Assumption

For DCI-based adaptation for additional PRACH resources, at least DCI format 1_0 can carry the adaptation indication for UEs in idle/inactive and connected mode.

- P-RNTI is used

    - RAN1#120

Agreement

For adaptation of PRACH in time-domain, for determining the additional PRACH resources in time-domain,

- When an additional RO is overlapped with legacy valid RO in both time and frequency domain, the additional RO is invalid before the SSB-RO mapping

  - Note: the overlapped RO for legacy resource is not impacted

  - FFS: Clarification on configuration of legacy ROs

Conclusion

There is no RAN1 consensus to support the following in Rel-19

- New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms).

- New UE trigger

- Adapting the transmitted number of SSBs within a SSB burst

- Adaptation of the time domain positions of SSBs within a burst

Conclusion

There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell

Agreement

For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell

- UE is configured with SSB burst periodicity using legacy signalling for the SCell

- UE is configured with X additional SSB burst periodicity for the SCell

  - FFS: Value of X

- SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity

  - FFS: Whether there is specification impact

  - Note: This does not impact the discussion on OD-SSB

- For switching the periodicity, down-select between

  - (MAC-CE)

    - MAC-CE details for SSB burst periodicity adaptation is up to RAN2.

  - (DCI)

    - Alt 1-3: DCI based signalling is used

Agreement

For adaption of PRACH in time-domain, for a connected mode UE, support a 1-bit field in DCI 1_0 with C-RNTI used to trigger PRACH (i.e. PDCCH order) to indicate whether the additional PRACH resource(s) is available for the triggered PRACH.

- FFS: UE behaviour (e.g. applicable resources for PRACH mask index) when it is indicated of additional PRACH resource(s)

- FFS: Details on how to reuse existing bit for the 1-bit indication

Agreement

For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration

- FFS: Validity time duration for availability is configured by higher layer signaling or predefined

- FFS: Location of the reference point defined in the specification

- FFS: Value/granularity of the validity time duration.

- FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources

Agreement

For DCI-based adaptation for additional PRACH resources, support optional semi-static signalling of a single PRACH mask to identify the subset of the additional PRACH resources

- The mask is applicable at unit of

  - Alt 1: PRACH association period

  - Alt 2: PRACH association pattern period

  - Alt 3: SFN level

- The PRACH association period is determined based on valid additional ROs only.

- The mask is applied after valid RO determination and SSB-RO mapping.

- Note: The existing behaviour in TS 38.213 "An association pattern period includes one or more association periods and is determined so that a pattern between PRACH occasions and SS/PBCH block indexes repeats at most every 160 msec." is not impacted due to application of the mask.

- This is applicable at least for adaptation for DCI 1_0 with P-RNTI

- The DCI does not indicate PRACH mask selection

- FFS: how the mask is identified

  - Option 1: The PRACH mask is from a PRACH mask table

    - Pre-defined table with N=[4 or 8 or 16] rows

    - The semi-static signalling indicates a PRACH mask index

  - Option 2: The PRACH mask is based on configuration parameters e.g. bitmap at SFN-level, periodic time domain window, …

Agreement

- Separate configuration of Msg1-FDM for the additional PRACH resources at least for 4-step RACH is supported

  - UE is not expected to be configured such that there are more than 8 FDM-ed valid ROs (legacy + additional ROs)

  - FFS: When there is no configuration of Msg1-FDM

- Separate configuration of number of SSB per RO is supported

Agreement

Study the following options for the reference point (for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI in a PF) for RRC idle/inactive mode UE and RRC connected mode UE,

- Option 1: SFN of the first PF from the next I-DRX cycle

- Option 2: SFN of the first PF from the current I-DRX cycle

- Option 3: From the first frame of the first PRACH association period after UE receives the DCI

- Option 4: From the first frame of the current SI modification period

- Option 5: From the first frame of the next SI modification period

Agreement

For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell

- Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI]

- FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1)

Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

    - RAN1#120bis

Agreement

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

Agreement

For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI

- Use one bit from the Bits 5-8 within the Short Message field (from upper layers)

- Send LS to RAN2 to confirm the use of this bit.

Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs

Agreement

For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI

- the validity duration is configured via higher layer signalling.

Agreement

For DCI-based adaptation for additional PRACH resources, PRACH mask that identifies the subset of the additional PRACH resources is applicable at unit of

- PRACH association period

- This PRACH mask applies to every [configured] K SSB RO association pattern period(s)

Agreement

For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell,

- The DCI is scrambled a new RNTI,

  - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell

- the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter

- the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell

Agreement

For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell.

- Xmax=2

Agreement

Separate configuration of the following parameters for the additional PRACH resources at least for 4-step RACH is supported

- CB-PreamblesPerSSB

Agreement

LS on DCI-based PRACH adaptation endorsed with the ACTION part modified compared to draft LS in R1-2503085 as follows:

- ACTION: RAN1 respectfully asks RAN2 to confirm whether the use of above bit is feasible.

Final LS in R1-2503086.

Agreement

For adaptation of PRACH in time-domain, for a connected mode UE,

- One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources.

Working Assumption

When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213].

- FFS: how to determine the first [possible] SSB burst

Agreement

For DCI-based adaptation for additional PRACH resources, the PRACH mask to identify the subset of the additional PRACH resources is given by:

- Option 1-2:  Semi-static signalling of a PRACH mask index and a value of K (number of association pattern periods)

  - For K: one from up to four candidate values {2,4,8, [1 or 16]}

| Mask index | Indication of association periods (AP) for subset of additional PRACH resources within every K association pattern periods (APP) |
| --- | --- |
| 0 | 1st half of the APs in K APPs |
| 1 | 1st quarter of the APs in K APPs |
| 2 | 1st eighth of the APs in K APPs |
| 3 | 1st sixteenth of the APs in K APPs |

Agreement

- For DCI-based adaptation for additional PRACH resources, the reference point for the availability of additional PRACH resources indicated by DCI 1_0 with P-RNTI is the start of first frame of the current SI modification period where UE receives the DCI

- The validity duration configured by higher layer signalling for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI is

  - (Option 4) Multiple of SI modification period ({[1],2,[3],4,8,[],[],..})

    - RAN1#121

Agreement

For DCI 2_9-based SSB burst periodicity adaptation for an SCell for the case when cell DTX/DRX is not configured, reuse existing search space configuration parameter for DCI 2_9-based monitoring and existing DCI 2_9 size configuration parameter and update in specification that these are also applicable to SSB burst periodicity adaptation (when configured)

Agreement

Value d (in the WA from RAN1#120bis) is the number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213.

Agreement

Update the agreement from RAN1#120bis as shown below (i.e. updates in red).

Agreement (from RAN1#120bis)

For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI,

- Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources

  - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon

Note: Whether the additional PRACH configuration can be from RRC other than SIB1 is up to RAN2.

Agreement

The fourth candidate value for K (for PRACH subset mask) is 16. K=1 is default value (if parameter is not configured).

Agreement

Supported values for validity duration configured by higher layers are

- {2,4,8,16} x SI modification period.

Agreement

Value range for PRACH-Config Index parameter for additional RACH configuration is same as legacy, i.e. INTEGER (0...255).

- Note: Final decision on the value range is up to RAN2

Conclusion

Using DCI 1_0 with P-RNTI to explicitly deactivate the additional PRACH resources is NOT supported.

Conclusion

There is no consensus to support adaptation of RACH in time domain for 2-step RA in Rel-19.

Agreement

‘PRACH resource indicator’ field is present in DCI 1_0 with C-RNTI for PDCCH order when the configuration of the additional RACH resources is provided in SIB1(i.e. addl-RACH-Config-Adaptation).

- Above applies for PCell

Agreement

When the SSB burst periodicity is switched from periodicity value P1 to periodicity value P2 based on DCI format 2_9 indication,

- Alt 1: SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value.

  - the first SSB burst according to the periodicity value P2 is determined as the first SSB burst according to the SSB burst periodicity value P2 and associated SFN offset and half-frame index that is no earlier than slot m+d.

SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

Agreement

Both CBRA and CFRA based on additional PRACH resources is supported for PDCCH order via DCI 1_0 with C-RNTI.

- For CFRA,

  - The indicated SSB index and PRACH mask index are applied to both legacy PRACH resources and additional PRACH resources.

    - Note: The PRACH mask applies to either additional resources and/or legacy resources, depending on which one satisfies the conditions for the mask to be applicable.

Agreement

Adopt the below TP for Clause 8.1 of TS 38.213, as per Editor CR available in R1-2503167.

*** Unchanged text omitted ***

For valid PRACH occasions associated with addl-RACH-Config-Adaptation [in RACH-ConfigCommon], the UE can be additionally provided a PRACH mask index, by prach-SubsetMask-Index-Adaptation that, if provided, indicates one or more association periods per K_mask association pattern periods according to Table 8.1-0, where K_mask is provided by KforAPPForPRACHsubsetMask.

Table 8.1-0: Mapping of mask index to association periods per Kmask association pattern periods

| Mask Index | Association periods (APs) perKmask association pattern periods (APPs) |
| --- | --- |
| 0 | First half of APs in Kmask APPs |
| 1 | First quarter of APs in Kmask APPs |
| 2 | First eighth of APs in Kmask APPs |
| 3 | First sixteenth of APs in Kmask APPs |

Valid PRACH occasions associated with addl-RACH-Config-Adaptation, and additionally associated with in association periods indicated by prach-SubsetMask-Index-Adaptation, if provided, are activated indicated as available for PRACH transmission based on an indication in a DCI format 1_0 with CRC scrambled by a P-RNTI [or a C-RNTI] [5, TS 38.212]. For activation indication by DCI format 1_0 with CRC scrambled by the P-RNTI, the PRACH occasions are available for a duration provided by validity-DurationForAddlRACHAdaptation, starting from the first frame of the SI modification period [12, TS 38.331] that includes a PDCCH monitoring occasion where the UE receives a PDCCH providing the DCI format 1_0 with CRC scrambled by the P-RNTI.

*** Unchanged text omitted ***

Agreement

Additional PRACH availability indication can be carried by a DCI 1_0 with P-RNTI with Short Messages Indicator set to 00, 01,10,11.

- Note: Above is already reflected in the endorsed editor CR 38.212

## RAN2 agreements: Adaptation of common channels/signals

### RAN2#125bis

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

- For adaptation of paging occasions in time domain, RAN2 to study a) bundle paging frames and b) extend the values of N to have increased interval between PFs (e.g. T/64, T/128 ...) and compensating decrease in number of PFs by increasing POs per PF.

- For Paging adaptation, R2 discusses the following options on compatibility of legacy RRC_IDLE/RRC_INACTIVE UE:

  - Option 1: Prevent the access of legacy UE via barring;

  - Option 2: Separate paging resources for legacy UEs and Rel-19 NES UEs (assuming there are legacy UEs)

### RAN2#126

RACH adaptation in spatial domain

- RAN2 counts on RAN1 evaluation and conclusion.

### RAN2#127

Agreements on paging adaptation

- Option-a) is about the bundling of PF for R19 NES UEs.

- R2 observe that the option-a) and option-b) can be designed to configure the PO:s at same time position.

### RAN2#127bis

Agreements on paging adaptation

- Select option-b as baseline for R19 NES paging enhancement.

- R2 should aim at signaling overhead minimization (e.g., Ns=8 and FFS for other larger values)

- Allowing legacy and R19 UEs to co-ex in the same PF/PO is possible, based on NW configuration.

- Legacy UE is not barred.

- Rel-19 UEs only monitor the PO(s) according to Rel-19 paging configuration.

### RAN2#128

Agreements on paging adaptation

- Introduce value for Ns=8. FFS on 16.

- Introduce value for N= T/32. FFS on T/64, T/128, T/256.

### RAN2#129

Agreements on paging adaptation

- For N, values smaller than T/32 are not supported.

- The maximum possible value for Ns is 8.

- Introduce a separate PEI configuration.

- Paging adaptations are configured semi-statically and updated via system information update notification.

- A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

Agreements on SSB adaptation

- RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell.

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation.

- For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

- RAN2 starts CBRA for RACH adaptation.

- RAN2 starts 4-step RACH adaptation.

- From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

- From R2 perspective, RACH adaptation is not modelled as RA feature(s).

- From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

### RAN2#129bis

Agreements on common channel adaptation

- For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17.

- For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17.

- Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED.

- Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1.

- For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration.

- Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

    - RAN2#130

Agreements on common channel adaptation

- Regarding RA type for PRACH adaptation, support only 4-step RA in Rel-19 unless RAN1 decides supporting 2-step RACH this meeting.

- addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon

- Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1. addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

- Agree that the following information is included in additional RO configuration (see the agreed table in the discussion of R2-2504579)

- If adaptation for additional PRACH resources is supported in active UL BWP, network can configure that paging search space in active DL BWP if network want to activate additional RO to the corresponding UEs.

- RRC connected UE monitors the DCI-based adaptation indication at least once every once every SI modification period.

- RAN2 will not do anything to handle the UEs that missed the short message.

- No need to extend the RA-RNTI space for additional PRACH occasions, i.e. no RAN2 impact for RA-RNTI calculation. RAN2 assumes that f_id for the additional PRACH resources is reinterpreted as the index of the PRACH occasion in the frequency domain plus the number of FDMed legacy ROs. RAN2 understand there is no MAC impact.

- RAN2 de-prioritizes the discussion of NES (including all NES scopes) and SBFD co-existence.

- BI is commonly applied to both legacy RO and additional RO.

- For SSB adaptation, R2 concludes the support of L3 measurement configuration based on a single MO. No RAN2 spec impact is foreseen.

- RACH adaptation is not applied to the following cases (for non-serving cell case): 1) PDCCH order for additional RACH, 2) L3 HO command, 3) LTM CSC MAC-CE, 4) PDCCH order for LTM early UL sync.

- RACH adaptation is not applied to BFR.

- RAN2 confirms the newly introduced 1-bit indication in DCI 1_0 for C-RNTI, i.e., PDCCH order, applies to both CBRA and CFRA.

- RAN2 will select one of the following options. (CE here means MSG1 repetition).

  - Option-1b: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, allow mixing of CE feature and non-CE feature in the same RACH-ConfigCommon, and clarify in the MAC spec that if CE is applicable for random access procedure and RACH-ConfigCommon associated with selected random access resource set includes additional RO, UE does not use additional RO configured in this RACH-ConfigCommon.

  - Option-1c: A same featureCombinationPreamblesList-r17 applies to both legacy and additional RACH, and R2 keep the conclusion, but rely on network implementation to avoid mixing CE feature and non-CE feature in the same RACH-ConfigCommon. (i.e., if [feature combination including CE / CE-only feature] is included in featureCombinationPreamblesList-r17 as one of the associated feature(-combination):s, the additional RACH is not supposed to be included in this RACH-ConfigCommon for this featureCombinationPreamblesList-r17).

Organizational

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging

- The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

- The maximum offset value is extended to 32 radio frames.

- The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

- A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging

- The maximum number of PEI occasion per paging frame is extended to 8.

- The maximum offset value is extended to 32 radio frames.