3GPP TSG-RAN WG3 Meeting #129	R3-255696

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	11.4

Source:	CMCC

Title:	Discussion on AI/ML based Mobility Optimization for NR-DC

Document for:	Discussion

# 1 Introduction

In the previous RAN3 meetings, the following agreements were achieved for AI/ML based Mobility Optimization for NR-DC [1].

RAN3#123bis

Agree to take as baseline for the Mobility Optimization for NR-DC that the use case is studied assuming inference at the MN. The main use case is limited to dual connectivity only (e.g. no conditional procedures are in scope).

RAN3#124

Enhance the Dual Connectivity procedure (e.g., MN-initiated SN addition, MN-initiated SN change) to trigger the collection of measured UE performance.

RAN3#125bis

As a starting point, SN Addition and MN-initiated SN Change procedures are considered to be enhanced to trigger the collection of UE performance from SN to MN. This assumes that the involved gNBs have an existing Data Collection Reporting Initiation procedure in place configuring the collection of UE performance.

RAN3#126

Using the Data Collection Reporting Initiation procedure to configure the collection of UE performance at SN side.

Introduce the Data Collection ID IE into the SN Addition Request message to identify a Data Collection Reporting context.

RAN3#127bis

The reporting of legacy UE performance from SN to MN is performed via the Data Collection Reporting procedure (within the DATA COLLECTION UPDATE message). The details of measurement metrics and any other additional information need to be further checked.

The following list of exit conditions to stop the collection of UE performance at SN side:

the time since NR-DC was successfully setup with the S-NG-RAN node is equal to the value of the Collection Time Duration for UE Performance IE;

the NR-DC with the S-NG-RAN node for the UE is released.

The open issues were captured in RAN2#127bis as below and need to be continued in next meeting:

UE performance measurements in NR-DC based on the outcome of the UE performance discussion in split architecture?

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

Whether additional conditions (e.g., SCG activation) for starting the collection of UE performance are needed, pending the outcome of the discussion on UE performance in split architecture?

In this contribution, we would continue the discussion on the solutions for NR-DC mobility optimization and provide some proposals.

# 2 Discussion

2.1 Prediction Information of Mobility Optimization for NR-DC

Whether to introduce assistance information (predicted PSCell ID) from MN to SN?

Regarding to AI/ML based Mobility Optimization, we think the benefit is mainly reflected in the prediction information provided by AI/ML model. However, there is still no consensus on the type of prediction information for NR-DC mobility optimization.

In the previous meetings, whether to introduce predicted PSCell ID in the DC procedure (e.g., SN Addition, MN-initiated SN change) is the focus of discussion for AI/ML based NR-DC Mobility Optimization. From our point of view, in SN Addition and MN-initiated SN Change procedures, the key operations for the target SN are to choose the PSCell and configure the SCG cell(s). If the predicted PSCell ID could be introduced as the assistance information, it could help MN select the appropriate target SN node and help the target SN choose a suitable PSCell, configure the SCG cell(s) and reserve resource in advance.

Observation 1: The predicted PSCell could help MN select the appropriate target SN node. And the target SN could further use the prediction information to choose a suitable PSCell, configure the SCG cell(s) and reserve resource in advance.

We observe from the legacy SN Addition and MN-initiated SN Change procedures in TS 37.340 [2] that MN could provide measurement results of PSCells to the target SN for SN to choose and configure the SCG cell(s). That means the measurement results related to the target SN could be used as the input data for AI/ML model to generate the predicted PSCell(s). Furthermore, some other assistance information (e.g., resource status of neighbour nodes) maybe used by MN in the legacy target SN node selection. The assistance information in the legacy target SN node selection could also be used as the input data for AI/ML model. This means that MN has enough input data for AI/ML model to generate the predicted PSCell(s).

Observation 2: It is feasible for MN to generate the predicted PSCell ID, since MN has enough input data (e.g., the measurement results related to the target SN and other assistance information for target SN node selection) for AI/ML model to generate the predicted PSCell ID.

In Rel-18, for AI/ML based Mobility Optimization of single connectivity, the UE Trajectory Prediction is transferred from source node to target node via the HANDOVER REQUEST message. UE Trajectory Prediction is provided as a list of cells where the UE is predicted to connect after handover, each of which is indicated together with an expected time of stay into the cell (optional). The similar solution could also apply to NR-DC Mobility Optimization.

In NR-DC procedures, MN could generate the predicted PSCell ID and use the prediction to select the appropriate target SN node. And MN could also transfer the predicted PSCell ID as one of the assistance information to target SN which use it to choose and configuration the SCG cell(s).

Observation 3: In Rel-18, the UE Trajectory Prediction is transferred from source node to target node for AI/ML based Mobility Optimization of single connectivity. UE Trajectory Prediction is provided as a list of cells where the UE is predicted to connect after handover. The similar solution could also apply to NR-DC Mobility Optimization.

Based on the above analysis, we propose that the predicted PSCell ID should be considered as the baseline of NR-DC Mobility Optimization.

Proposal 1: Predicted PSCell ID should be introduced in the DC procedure (e.g., SN Addition, MN-initiated SN change) as the assistance information from MN to SN.

In the case that AI/ML model is deployed in MN, the MN could generate the prediction information of NR-DC in advance. So, we think on one hand, the prediction information generated by the MN could be used by MN itself as the assistance information for target SN selection for SN Addition and MN-initiated SN Change. On the other hand, the MN could send the prediction information to the target SN for resource reservation. Similar with the signaling procedure of AI/ML based mobility optimization in Rel-18 that the prediction is transferred to the target gNB via the HANDOVER REQUEST message, we think that MN could send the prediction information to target SN via S-NODE ADDITION REQUEST message for NR-DC.

Proposal 2: After MN generate the prediction information of NR-DC (e.g., the predicted PSCell ID),

- The MN could use the prediction information as the assistance information for target SN selection for SN Addition and MN-initiated SN Change.

- The MN could send the prediction information to the target SN for resource reservation via S-NODE ADDITION REQUEST message.

2.2 Measured information collection of Mobility Optimization for NR-DC

Whether to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the actual PSCell ID?

We think the measured PSCell ID is necessary for measured information (e.g., UE performance) of NR-DC, and there are mainly two reasons:

1) The measured PSCell ID is necessary for MN to identify the correlation between the cell and the measured UE performance reported by target SN.

2) If the predicted PSCell ID in proposal 1 is agreed as the assistance information for NR-DC, the measured PSCell which target SN select could be used for AI/ML model performance testing and model retraining.

Observation 4: The measured PSCell ID is necessary for measured information of NR-DC, since it is helpful for MN to identify the correlation between the cell(s) and the measured UE performance reported by target SN and could also be used for AI/ML model performance testing and model retraining.

Proposal 3: The measured PSCell ID should be introduced into the measured information collection as the feedback for NR-DC Mobility Optimization.

In the contributions from many companies submitted to the previous meetings, we observe there are two potential solutions to include the measured PSCell ID in the DATA COLLECTION UPDATE message:

Option 1: To enhance the UE Performance IE to include the measured PSCell ID;

Option 2: To enhance the Measured UE Trajectory IE to include the measured PSCell ID;

As we mentioned above, the measured PSCell ID is needed to identify the correlation between the cell and the measured UE performance reported by target SN. For this purpose, it seems Option 1 is a straightforward solution to include the measured PSCell ID. On the other hand, Option 2 is a solution to follow the principle in Rel-18 since the Measured UE Trajectory IE is used to contain information about cells that a UE has connected to. Therefore, we think both options are reasonable. Considering there is no consensus to transfer the predicted/measured UE trajectory over NR-DC in SI phase, we sightly prefer to use Option 1 to include the measured PSCell ID in the DATA COLLECTION UPDATE message.

Proposal 4: RAN3 to consider to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the measured PSCell ID.

# 3 Conclusion

In this paper, we discuss on the solutions for NR-DC mobility optimization, we have the following observations and proposals:

Prediction Information of Mobility Optimization for NR-DC

Observation 1: The predicted PSCell could help MN select the appropriate target SN node. And the target SN could further use the prediction information to choose a suitable PSCell, configure the SCG cell(s) and reserve resource in advance.

Observation 2: It is feasible for MN to generate the predicted PSCell ID, since MN has enough input data (e.g., the measurement results related to the target SN and other assistance information for target SN node selection) for AI/ML model to generate the predicted PSCell ID.

Observation 3: In Rel-18, the UE Trajectory Prediction is transferred from source node to target node for AI/ML based Mobility Optimization of single connectivity. UE Trajectory Prediction is provided as a list of cells where the UE is predicted to connect after handover. The similar solution could also apply to NR-DC Mobility Optimization.

Proposal 1: Predicted PSCell ID should be introduced in the DC procedure (e.g., SN Addition, MN-initiated SN change) as the assistance information from MN to SN.

Proposal 2: After MN generate the prediction information of NR-DC (e.g., the predicted PSCell ID),

- The MN could use the prediction information as the assistance information for target SN selection for SN Addition and MN-initiated SN Change.

- The MN could send the prediction information to the target SN for resource reservation via S-NODE ADDITION REQUEST message.

Measured information collection of Mobility Optimization for NR-DC

Observation 4: The measured PSCell ID is necessary for measured information of NR-DC, since it is helpful for MN to identify the correlation between the cell(s) and the measured UE performance reported by target SN and could also be used for AI/ML model performance testing and model retraining.

Proposal 3: The measured PSCell ID should be introduced into the measured information collection as the feedback for NR-DC Mobility Optimization.

Proposal 4: RAN3 to consider to enhance the UE Performance IE in the DATA COLLECTION UPDATE message to include the measured PSCell ID.

# Reference

- RAN3_127bis_agenda_20250415_EOM1, Apr 2025.

- 3GPP TS 37.340: NR; Multi-connectivity; Overall Description; Stage 2.

- R3-247895, (TP to BLCR of 38.423) Mobility Optimization for NR-DC, Nov 2024.