3GPP TSG-RAN WG3 Meeting #129	R3-255850

Bangalore, India, 25 – 29 August 2025

Agenda Item:	19.2

Source:	CATT, Ericsson, ZTE, Huawei, China telecom

Title:	(TP to TS 38.470 BL CR) Introduction of SBFD

Document for:	Approval

- Introduction

This paper capture the following changes.

- Add abbreviation for CLI

- Add UE to UE CLI mitigation

- TP for BLCR to TS 38.470

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BH	Backhaul

CLI	Cross Link Interference

DRB	Data Radio Bearers

eDRX	extended Discontinuous Reception

eRedCap	Enhanced Reduced Capability

F1-C	F1 Control plane interface

<<<Unchanged part omitted>>>

## 5.2	F1-C functions

<<<Unchanged part omitted>>>

### 5.2. x	CLI indication function

The CLI indication function enables the transfer of gNB-to-gNB CLI measurements, gNB-to-gNB CLI mitigation request and SRS Resource indication between gNB-DU and gNB-CU.

<<<Next change>>>

6.1	Control plane procedures

<<<Unchanged part omitted>>>

### 6.1. x	CLI indication procedure

The following procedure is used to report the result of gNB-to-gNB CLI measurements, to request mitigation of gNB-to-gNB CLI and SRS Resource configuration:

-	CLI INDICATION

<<<< End of change >>>