3GPP TSG-RAN WG3 Meeting #129	R3-255866

Bengaluru , IN, 25 – 29 August, 2025

Agenda Item:	21.3

Source:	ZTE Corporation, Nokia, Nokia Shanghai Bell, Ericsson, CATT, Qualcomm, Meta, Samsung

Title:	(TP to BL CR for TS 38.473) Support of DL PDU Set Information Marking Support Indication

Document for:	Other

1	Introduction

The WID of Rel-19 XR has been updated with the following objective:

-	Support of PDU set based QoS handling enhancement [RAN3]:

-	Support of DL PDU Set marking without PDU Set QoS

-	Support of Alternative PDU Set QoS, which may contain UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

This paper provides the F1AP TP to capture the support for DL PDU Set Information Marking Support Indication. Detail explanation can be found in our other contribution in agenda 21.3.

2	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: UE Context Setup Request procedure: Successful Operation

The gNB-CU initiates the procedure by sending UE CONTEXT SETUP REQUEST message to the gNB-DU. If the gNB-DU succeeds to establish the UE context, it replies to the gNB-CU with UE CONTEXT SETUP RESPONSE. If no UE-associated logical F1-connection exists, the UE-associated logical F1-connection shall be established as part of the procedure. Except for RACH based SDT and UE configured with BWP specific ServingCellMO, the gNB-CU shall perform RRC Reconfiguration or RRC connection resume to send UE to the RRC_CONNECTED state as described in TS 38.331 [8], and in this case, the CellGroupConfig IE shall transparently be signaled to the UE as specified in TS 38.331 [8]. In the cases of RACH based SDT procedure and UE configured with BWP specific ServingCellMO, the CellGroupConfig IE shall be ignored by the gNB-CU.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the PDU Set QoS Parameters IE or the DL PDU Set Information Marking Support Indication IE is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT SETUP REQUEST message, the gNB-DU shall, if supported, store this information and use it as specified in TS 23.501 [21].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE Context Modification procedure. Successful operation

The UE CONTEXT MODIFICATION REQUEST message is initiated by the gNB-CU.

Upon reception of the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall perform the modifications, and if successful reports the update in the UE CONTEXT MODIFICATION RESPONSE message.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the PDU Set QoS Parameters IE or the DL PDU Set Information Marking Support Indication IE  is included in the QoS Flow Level QoS Parameters IE contained in the UE CONTEXT MODIFICATION REQUEST message, the gNB-DU shall, if supported, store this information and use it as specified in TS 23.501 [21].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.45	QoS Flow Level QoS Parameters

This IE defines the QoS to be applied to a QoS flow, or to a DRB, or to a BH RLC channel, or to a Uu Relay RLC channel, or to a PC5 Relay RLC channel, or to a MRB.

NOTE:	For a BH RLC channel, the listed mandatory IEs and the GBR QoS Flow Information IE are applicable, where GBR QoS Flow Information IE may be present if BH RLC channel conveys the traffic belonging to a GBR QoS Flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | - |  |
| >Non-dynamic 5QI |  |  |  |  | - |  |
| >>Non Dynamic 5QI Descriptor | M |  | 9.3.1.49 |  | - |  |
| >Dynamic 5QI |  |  |  |  | - |  |
| >>Dynamic 5QI Descriptor | M |  | 9.3.1.47 |  | - |  |
| NG-RAN Allocation and Retention Priority | M |  | 9.3.1.48 |  | - |  |
| GBR QoS Flow Information | O |  | 9.3.1.46 | This IE shall be present for GBR QoS Flows only and is ignored otherwise. | - |  |
| Reflective QoS Attribute | O |  | ENUMERATED (subject to, ...) | Details in TS 23.501 [21]. This IE applies to non-GBR flows only and is ignored otherwise. | - |  |
| PDU Session ID | O |  | INTEGER (0 ..255) | As specified in TS 23.501 [21]. | YES | ignore |
| UL PDU Session Aggregate Maximum Bit Rate | O |  | Bit Rate 9.3.1.22 | The PDU session Aggregate Maximum Bit Rate Uplink which is associated with the involved PDU session. | YES | ignore |
| QoS Monitoring Request | O |  | ENUMERATED (UL, DL, Both, …, stop) | Indicates to measure UL, or DL, or both UL/DL delays for the associated QoS flow or stop the corresponding QoS monitoring. | YES | ignore |
| PDCP Terminating Node DL Transport Layer Address | O |  | Transport Layer Address 9.3.2.3 | DL Transport Layer Address of node terminating PDCP. Included for MN-terminated SCG bearers and SN-terminated MCG bearers. | YES | ignore |
| PDU Set QoS Parameters |  | 0..1 |  |  | YES | ignore |
| >UL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.319 |  | - |  |
| >DL PDU Set QoS Information | O |  | PDU Set QoS Information 9.3.1.319 |  | - |  |
| DL PDU Set Information Marking Support Indication | O |  | ENUMERATED (true, …) |  | YES | ignore |
| MMSID | O |  | OCTET STRING (SIZE (1)) | Multi-modal service ID from the application, used to indicate QoS flows are related to a multi-modal service, as specified in TS 23.501 [21]. | YES | ignore |
| Indication of Bitrate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the QoS Flow allows rate adaptation in the indicated direction. | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

F1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

------ Unchanged part skipped ------

id-Transmission-Bandwidth-asymmetric,

id-TagIDPointer,

id-LocalOrigin,

id-SRSPosPeriodicConfigHyperSFNIndex,

id-PSIbasedSDUdiscardDL,

id-PduSetDelayBudgetDownlink,

id-PduSetDelayBudgetUplink,

id-PduSetErrorRateDownlink,

id-PduSetErrorRateUplink,

id-MonitoringRequestonAvailableBitrate,

id-MMSID,

id-Indication-of-Bitrate-Adaptation,

id-DLPDUSetInformationMarkingSupportIndication,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

maxnoofBPLMNsNR,

maxnoofDLUPTNLInformation,

maxnoofNrCellBands,

maxnoofULUPTNLInformation,

maxnoofQoSFlows,

maxnoofSliceItems,

maxnoofSIBTypes,

maxnoofSITypes,

maxCellineNB,

maxnoofExtendedBPLMNs,

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- D

------ Unchanged part skipped ------

DRB-Information	::=	SEQUENCE {

dRB-QoS		QoSFlowLevelQoSParameters,

sNSSAI		SNSSAI,

notificationControl		NotificationControl		OPTIONAL,

flows-Mapped-To-DRB-List	Flows-Mapped-To-DRB-List,

iE-Extensions	ProtocolExtensionContainer { { DRB-Information-ItemExtIEs } }	OPTIONAL

}

DRB-Information-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-ECNMarkingorCongestionInformationReportingRequest		CRITICALITY ignore	EXTENSION ECNMarkingorCongestionInformationReportingRequest		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardUL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardUL		PRESENCE optional	}|

{	ID id-PSIbasedSDUdiscardDL		CRITICALITY ignore	EXTENSION PSIbasedSDUdiscardDL		PRESENCE optional	},

...

}

DRBs-Modified-Item	::= SEQUENCE {

dRBID							DRBID,

lCID								LCID		OPTIONAL,

dLUPTNLInformation-ToBeSetup-List		DLUPTNLInformation-ToBeSetup-List,

iE-Extensions	ProtocolExtensionContainer { { DRBs-Modified-ItemExtIEs } }	OPTIONAL,

...

}

DLPDUSetInformationMarkingSupportIndication ::= ENUMERATED { true,...}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### -- Q

------ Unchanged part skipped ------

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation				OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{ ID id-PDUSessionID								CRITICALITY ignore	EXTENSION PDUSessionID				PRESENCE optional}|

{ ID id-ULPDUSessionAggregateMaximumBitRate			CRITICALITY ignore	EXTENSION BitRate						PRESENCE optional}|

{ ID id-QosMonitoringRequest						CRITICALITY ignore	EXTENSION QosMonitoringRequest		PRESENCE optional}|

{ ID id-PDCPTerminatingNodeDLTNLAddrInfo			CRITICALITY ignore	EXTENSION TransportLayerAddress	PRESENCE optional	}|

{ ID id-PDUSetQoSParameters							CRITICALITY	ignore	EXTENSION PDUSetQoSParameters		PRESENCE optional}|

{ ID id-MMSID										CRITICALITY ignore	EXTENSION MMSID							PRESENCE optional}|

{ ID id-Indication-of-Bitrate-Adaptation			CRITICALITY	ignore	EXTENSION Indication-of-Bitrate-Adaptation		PRESENCE optional}|

{ ID id-DLPDUSetInformationMarkingSupportIndication	CRITICALITY ignore	EXTENSION DLPDUSetInformationMarkingSupportIndication	PRESENCE optional}|

,

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

------ Unchanged part skipped ------

id-EarlySyncServingCellInformation					ProtocolIE-ID ::= 843

id-RANSharingAssistanceInformation					ProtocolIE-ID ::= 844

id-LTMCFRAResourceConfig-List						ProtocolIE-ID ::= 845

id-F1U-PathFailure									ProtocolIE-ID ::= 846

id-MeasBasedOnAggregatedResources					ProtocolIE-ID ::= 847

id-SIB23-message									ProtocolIE-ID ::= 848

id-BarringExemptionforEmerCallInfo					ProtocolIE-ID ::= 849

id-SIB17bis-message									ProtocolIE-ID ::= 850

id-ReportingIntervalIMs								ProtocolIE-ID ::= 851

id-Transmission-Bandwidth-asymmetric				ProtocolIE-ID ::= 852

id-TagIDPointer										ProtocolIE-ID ::= 853

id-LocalOrigin										ProtocolIE-ID ::= 854

id-LTMResetInformation								ProtocolIE-ID ::= 855

id-SRSPosPeriodicConfigHyperSFNIndex				ProtocolIE-ID ::= 856

id-PreconfiguredSRSInformation						ProtocolIE-ID ::= 857

id-PSIbasedSDUdiscardDL								ProtocolIE-ID ::= xxx

id-PduSetDelayBudgetDownlink						ProtocolIE-ID ::= xxx1

id-PduSetDelayBudgetUplink							ProtocolIE-ID ::= xxx2

id-PduSetErrorRateDownlink							ProtocolIE-ID ::= xxx3

id-PduSetErrorRateUplink							ProtocolIE-ID ::= xxx4

id-MonitoringRequestonAvailableBitrate 				ProtocolIE-ID ::= xxx5

id-MMSID											ProtocolIE-ID ::= xxx6

id-Indication-of-Bitrate-Adaptation					ProtocolIE-ID ::= xxx7

id-DLPDUSetInformationMarkingSupportIndication		ProtocolIE-ID ::= xxx8

END

-- ASN1STOP

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>