3GPP TSG-RAN WG3 Meeting #129	R3-25xxxx

Bengaluru, India, 25th ~29th Aug, 2025

Agenda Item:	19.2

Source:	Huawei (moderator)

Title:	Summary of SBFD offline discussion

Document for:	Discussion and decision

# Introduction

This is a summary of discussions for the following CB, based on the submitted contributions [1]~[21].

CB: # 9_DuplexOperation

- Reflect above agreements in TPs

- Discuss exchange of SBFD RACH configuration and agree (if possible) on solution

- Whether and how to clarify in Stage 2 to avoid simultaneous configuration of L1 and L3 based inter-UE CLI measurements.

- Cleanup BL CRs as needed

(Huawei)

(moderator - HW)

Summary of offline disc R3-25xxxx

# For the Chairman’s Notes

Issue 1: IE structure for SBFD configuration info

- Already concluded during online session

Issue 2: Further clean-ups

- Already concluded during online session

Issue 3: Other

- For SBFD RACH configuration, RAN3 will check this after RAN2’s running CR is agreed and introduce it as Rel-19 maintenance in October meeting.

- To capture in 38.401: For inter-UE CLI for SBFD operation in split architecture, simultaneous configuration of L1 and L3 measurements shall be avoided.

TP

- BL CR authors to prepare corresponding TPs, based on latest ongoing RAN3 BL CRs.

# Discussion

## IE structure for SBFD configuration info

Basically, there are three options:

Option 1: Just keep the current design, i.e. still separate the timing and frequency info with two IEs with reference to RAN2 respectively, it looks clean and direct, like in [16], but the timing info is duplicated since with current RAN2 design, timing info was already reflected with in current spec.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SBFD time Info | M |  | OCTET STRING | Includes the tdd-UL-DL-ConfigurationCommon IE as defined in TS 38.331 [10]. If included, the TDD UL-DL Configuration Common NR IE shall be ignored. |
| SBFD frequency Info | M |  | OCTET STRING | Includes the scs-SpecificCarrierList IE as defined in TS 38.331 [10]. If included, the Transmission Bandwidth IE shall be ignored. |

Option 2: Just to introduce SBFD frequency info, like in [8], as explained above,  :

| >>>SBFD Configuration | O |  | OCTET STRING | Includes the SBFD-Subband-Allocation-r19 IE of TS38.331 | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |

Option 3: on top of option 1, tries to explicitly list all the detailed parameters, like in [13]:

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| SBFD Starting Slot Index | O |  | INTEGER (0..maxNrofSlots-1) | This IE configures the starting slot index of SBFD subbands within a TDD-UL-DL period, as specified in TS38.331[8] |
| SBFD Starting Symbol Index | O |  | INTEGER (0..maxNrofSymbols-1) | This IE configures the starting symbol index within the starting slot of SBFD subbands within a TDD-UL-DL period, as specified in TS38.331[8] |
| SBFD Ending Slot Index | O |  | (0..maxNrofSlots-1) | This IE configures the ending slot index of SBFD subbands within a TDD-UL-DL period, as specified in TS38.331[8] |
| SBFD Ending Symbol Index | O |  | (0..maxNrofSymbols-1) | This IE configures the ending symbol index within the ending slot of SBFD subbands within a TDD-UL-DL period, as specified in TS38.331[8] |

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UL-subband Location and Bandwidth | O |  | INTEGER (0..37949) | This IE configures frequency domain location and bandwidth of UL subband, as specified in TS38.331[8]. |
| First DL-subband Location and Bandwidth | O |  | INTEGER (0..37949) | This IE configures frequency domain location and bandwidth of first DL subband, as specified in TS38.331[8]. |
| Second DL-subband Location and Bandwidth | O |  | INTEGER (0..37949) | This IE configures frequency domain location and bandwidth of second DL subband, as specified in TS38.331[8]. |

While in [11], it listed all the parameters but keep the IE type as OCTET STRING.

Please note that, technically, according to the latest RAN2 discussion, the wording “SBFD-Subband-Allocation-r19 within SCS-SpecificCarrier IE, as specified in TS38.331” is more precise.

Conclusion:

- Option 2: Agree that the semantics description of the SBFD Frequency Configuration IE added to the Served Cell Information NR IE in the BLCR to TS38.423 reference the SBFD-Subband-Allocation-r19 IE defined in TS38.331.

## Further clean-ups

Many contributions tried to do some clean-ups, like in [16]

- Remove editor’s note with the update to Fi

- Update the semantics description for the “NZP CSI-RS Resource Indication” as: “Strongest DL NZP CSI-RS beam information. The value is relative index of the CSI-RS resources within a set of resources.”

Conclusion:

- Remove the Editor’s Note in section 8.4.y CLI Indication procedure about the procedure text, and update the message name in figure to CLI INDICATION.

- Update the description of NZP CSI-RS Resource Indication IE in XnAP CR and CRI IE in F1AP CR to clarify the value is a relative index of the CSI-RS resources within the set of resources signalled.

## Others:

We also see some other proposals, including:

- Discuss exchange of SBFD RACH configuration and agree (if possible) on solution

- Whether and how to clarify in Stage 2 to avoid simultaneous configuration of L1 and L3 based inter-UE CLI measurements.

For exchange of SBFD RACH configuration, it was discussed and the majority shared the view that it might be out of WID scope, not sure if we should still go for it or not.

For the second one, at least the current spec already specified the mechanism for the UE to handle the case where if both L1 and L3 measurement were configured, it seems that there is no need to introduce additional coordination mechanism at network side.

Conclusion

- For SBFD RACH configuration, RAN3 will check this after RAN2’s running CR is agreed and introduce it as Rel-19 maintenance in October meeting.

- To capture in 38.401: For inter-UE CLI for SBFD operation in split architecture, simultaneous configuration of L1 and L3 measurements shall be avoided.

## Stage 2&3 TPs

38.300: There are some stage 2 TPs capturing the inter-UE CLI mitigation function [2] [9] [19], whose intention was correct, but it seems that RAN1 already endorsed some TPs to 38.300, see the attachment in LS [22].

Other stage 2 TPs, including 38.420, 38.470 and 38.401 [6] [19], some polishes could be considered.

TP to 38.423 is needed to capture agreements in 3.1&3.2.

Conclusion

- BL CR authors to prepare corresponding TPs, based on latest ongoing RAN3 BL CRs.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-255267, UE-to-UE CLI mitigation support (Charter Communications, Inc)	other

- R3-255276, [TP for BL CR TS 38.300] Introduction of SRS resource configurations for Evolution of NR duplex operation Sub-Band Full Duplex (Charter Communications, Inc)	other

- R3-255277, [TP for BL CR 38.423] Introduction of UE-to-UE CLI mitigation support signaling (Charter Communications, Inc)	other

- R3-255278, [TP for BL CR 38.473] Introduction of UE-to-UE CLI mitigation support signaling (Charter Communications, Inc)	other

- R3-255324, Signaling design to enable NR SBFD operation (Qualcomm Incorporated)	discussion

- R3-255438, (TP for BLCRs to TS 38.420, 38.470) Finalizing stage 2 for Sub-band Full Duplex operation (Nokia)	other

- R3-255439, (TP for BLCRs to TS 38.423, 38.473) Finalizing stage 3 for Sub-band Full Duplex operation (Nokia)	other

- R3-255480, (TP to BLCR to TS38.423 for SBFD) Discussion on SBFD WI completion (Ericsson)	other

- R3-255481, (TP to BLCR to TS38.300 for SBFD) Introduction of SBFD (Ericsson)	other

- R3-255645, Discussion on Evolution of Duplex Operation (CATT)	discussion

- R3-255646, (TP for 38.423 and 38.473) Support on Evolution of Duplex Operation (CATT)	other

- R3-255667, Further discussion on information exchange for CLI handling (Samsung)	discussion

- R3-255668, Text proposals to BLCRs of TS38.473 on CLI handling (Samsung)	other

- R3-255679, Further discussion on remaining open issues (Huawei, China Telecom, China Unicom)	discussion

- R3-255680, Discussion on the exchange of SRS resource configurations (Huawei, China Telecom, China Unicom)	discussion

- R3-255681, (TP to BL CR for 38.423) miscellaneous clean-ups for Evolution of NR duplex operation Sub-band full duplex (SBFD) (Huawei, China Telecom, China Unicom)	other

- R3-255682, (TP to BL CR for 38.423) introduction of SRS resource configurations for Evolution of NR duplex operation Sub-band full duplex (SBFD) (Huawei)	other

- R3-255683, (TP to BL CR for 38.473) introduction of SRS resource configurations forEvolution of NR duplex operation Sub-band full duplex (SBFD) (Huawei)	other

- R3-255691, (TP to BLCR/draft CR for 38.300, 38.401, 38.423 and 38.473) SBFD (ZTE Corporation)	other

- R3-255692, Discussion on information exchange for SBFD (ZTE Corporation)	discussion

- [21]	R3-255703, (TPs to BLCR for TS 38.423 and TS 38.473) Discussion on supporting CLI handling in SBFD (CMCC)	other

- [22]	R3-255010, LS on TP for TS38.300 on Rel-19 SBFD (RAN1(Huawei))	LS in