3GPP TSG-RAN WG3 Meeting #129                                                                  R3-255953

Bengaluru, India, August 25th – 29th, 2025

Source:	CATT, Qualcomm, Huawei, NEC, Ericsson, Deutsche Telekom, ZTE, Rakuten, Verizon, Samsung

Title:	TP to BLCR 38.300 for OD-SIB1

Agenda Item:	17.3

Document for:	Discussion and Approval

1 Introduction

This TP is to capture/realize following agreements made in RAN3#128[1] and RAN3#129[2].

RAN3#128: One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M) + one NES Cell ID (presence M ) + one Cell-A ID (presence O ) Cell A gNB-CU encoding the SIBxx. The NES gNB-CU sends the indication to NES gNB-DU. The NES gNB-DU MAY go to OD-SIB 1 operation up to gNB-DU decision. RAN3#129: Change the procedure and message name from “UL WUS Configuration Provision” to “OD-SIB1 Configuration Provision” Change from NR CGI at NG-RAN node1 (Naming FFS) to NES Cell ID Change from NR CGI at NG-RAN node2 (Naming FFS) to Cell A ID UL WUS CONFIGURATION PROVISION STATUS UPDATE => OD-SIB1 Configuration Provision Status Update In the class 2 message “OD-SIB1 Configuration Provision Status Update”, it is agreed to include “NES Cell ID” as Mandatory presented. Cell A ID is optionally presented. If Cell A ID IE is included in the provision request, then Cell A gNB shall, if support, broadcast the OD-SIB1 configuration in the indicated Cell A. If Cell A ID IE is present, this message indicates that the indicated Cell A has stopped the broadcasting. If Cell A ID IE is not present, then all the Cell A(s) have stopped the broadcasting for the given NES Cell. If Cell A ID is included in the OD-SIB1 Configuration Provision Request message, then it shall be included in the OD-SIB1 Configuration Provision Response message. Capture this in specifications.

3 Reference

- RAN3#128 Meeting Minutes, RAN3 chair

- RAN3#129 Meeting Minutes, RAN3 chair

5 Annex (TP to BLCR for TS 38.300)

-------------------------------------Start of TP---------------------------------------------

#### 15.4.2.x2	On-demand SIB1

To facilitate reducing gNB downlink transmissions, instead of always periodically transmitting SIB1, the gNB can provide on-demand SIB1, i.e., upon receiving an OD-SIB1 request from a UE supporting OD-SIB1. OD-SIB1 is supported for UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED when T311 is running. A request for SIB1 triggers a random access procedure, where MSG1 is used for indicating OD-SIB1 request and the gNB acknowledges the request in MSG2. OD-SIB1 request configurations of one or more cells which support OD-SIB1 are included in SIBxx, which can be broadcasted in any cell, including cell’s own OD-SIB1 request configuration. UE may request SIB1 based on the OD-SIB1 request configuration from SIBxx in order to determine the suitability of a cell during and after cell reselection.

A gNB may request over the Xn interface neighbour gNB(s) to transmit or stop transmitting the OD-SIB1 configuration for a cell supporting OD-SIB1. The neighbour gNB(s) deciding to stop the OD-SIB1 configuration transmission informs the requesting gNB over Xn. The inter-gNB coordination procedures are defined in TS 38.401 [4].

-------------------------------------End of TP---------------------------------------------