3GPP TSG-RAN WG3 Meeting #129	R3-255177

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	11.4

Source:	ZTE Corporation

Title:	(TP to BL CR to 37.483) Remaining issues in E1AP for AI/ML for NG-RAN

Document for:	Discussions & Approval

- Introduction

During the last meeting, we received the LS from SA5 regarding feedback on UL/DL throughputs at the UE level, as well as the LS on how SA5 defines UL/DL packet loss at the PDCP level. Therefore, we agreed to CU-CP can request the packet delay and packet loss UL/DL measurement via DATA COLLECTION INITIATION procedure. And the semantic description of packet loss UL/DL and the range of packet loss UL/DL are to be continued pending on SA5 clarification.

As this is the final meeting for the Rel-19 AI/ML for NG-RAN topic, in this contribution, we present the solution to address the remaining issues in E1AP and remove the relevant FFS.

- Discussion

2.1 Packet Loss

9.3.1.aa	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.bb | Corresponds to Average Packet Delay per-UE as specified in TS 28.558 [XX] clause 6.3.1.1. |
| Average Packet Loss DL | O |  | 9.3.1.cc | Corresponds to DL PDCP SDU Drop Rate in gNB-CU-UP as specified in TS 28.552 [XX1] clause 5.1.3.2.1 |
| Average Packet Loss UL | O |  | 9.3.1.cc | Corresponds to UL PDCP SDU Loss Rate as specified in TS 28.552 [XX1] clause 5.1.3.1.1 |

Editor’s note: the semantics description could be further refined.

Following is the current situation for UL/DL packet loss in the E1 BLCR:

Regarding the semantic description of packet loss, we previously sent the LS (R3-253816) [2] to SA5 in the last meeting. RAN3 would like to further inquire whether it is feasible to specify the above Packet Loss Rate metrics with a per-DRB per-UE granularity at the UL and DL levels at the PDCP layer, which can also be aggregated to reflect a per-UE Packet Loss Rate metric. If SA5 confirms that Packet Loss Rate metrics at the per-DRB per-UE level can be specified and informs RAN3, then RAN3 can directly add reference for average packet loss UL/DL.

Because the semantic description of packet loss at UL and DL is pending clarification from SA5, we propose two actions to resolve the issue:

- If SA5 provides the necessary clarification in time, incorporate their input to finalize the packet loss description in TS 37.483.

- If SA5 does not respond immediately, RAN3 has two options:

  - Remove the packet loss from the current discussion and plan to address it in Rel-20 through TEI correction, along with related issues from Rel-18 and Rel-19.

  - Retain the packet loss but keep its semantic description general until SA5 provides the required details. This would mean removing specific references and simply noting that this IE corresponds to either DL PDCP SDU Drop Rate or UL PDCP SDU Loss Rate.

Proposal 1a	If SA5 provides the necessary clarification in time, incorporate their input to finalize the packet loss description in TS 37.483.

Proposal 1b	If SA5 does not respond immediately, retain the packet loss but keep its semantic description general until SA5 provides the required details.

5.1.3.1.1	UL PDCP SDU Loss Rate

a)	This measurement provides the fraction of PDCP SDU packets which are not successfully received at gNB-CU-UP. It is a measure of the UL packet loss including any packet losses in the air interface, in the gNB-CU and on the F1-U interface.  Only user-plane traffic (DTCH) and only PDCP SDUs that have entered PDCP (and given a PDCP sequence number) are considered.  The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC), and subcounters per supported S-NSSAI.

b)	SI.

c)	This measurement is obtained as:  1000000* Number of missing UL PDCP sequence numbers, representing packets that are not delivered to higher layers, of a data radio bearer, divided by Total number of UL PDCP sequence numbers (also including missing sequence numbers) of a bearer, starting from the sequence number of the first packet delivered by UE PDCP to gNB-CU-UP until the sequence number of the last packet. If transmission of a packet might continue in another cell, it shall not be included in this count. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the loss rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurements are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

5.1.3.2.1	DL PDCP SDU Drop rate in gNB-CU-UP

a)	This measurement provides the fraction of PDCP SDU packets which are dropped on the downlink, due to high traffic load, traffic management etc in the gNB-CU-UP. Only user-plane traffic (DTCH) is considered. A dropped packet is one whose context is removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface. The measurement is optionally split into subcounters per QoS level (mapped 5QI or QCI in EN-DC), and subcounters per supported S-NSSAI.

NOTE:	this measurement may include packets that were supposed to be sent via the eUtran air interface if using NR split bearer option 3, 4 or 7.

b)	SI.

c)	This measurement is obtained as: 1000000*Number of dropped DL PDCP SDU packets whose contexts are removed from the gNB-CU-UP without any part of it having been transmitted on the F1-U or Xn-U or X2-U interface, of a data radio bearer, divided by Number of DL PDCP SDU packets for data radio bearers that have entered PDCP-SAP after being decoded from GTP-U packets. Separate counters are optionally maintained for mapped 5QI (or QCI for EN-DC) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the drop rate multiplied by 1E6. The number of measurements is equal to one. If the optional QoS and S-NSSAI level measurement are performed, the measurements are equal to the number of mapped 5QIs and the number of supported S-NSSAIs.

Regarding the range of packet loss UL/DL, the same issue also exists in the Rel-18 correction. A more granular value range is needed to accurately represent packet loss, rather than the legacy range of from 0 to 100. In 28.552 [3], both UL PDCP SDU Loss Rate and DL PDCP SDU Drop Rate are described as each measurement being an integer value representing the drop rate multiplied by 1E6. Therefore, we believe that using the range from 0 to 1,000,000 is more appropriate and aligns with SA5's definition.

Proposal 2	Using the range (0…1000000) to represent the Average Packet Loss DL IE and Average Packet Loss UL IE.

2.2 Exit conditions in E1AP

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and one or more of the UE performance metrics are requested, the UE Performance Collection Configuration IE shall be included. The NG-RAN node2 shall take the UE Performance Collection Configuration IE into account for the configuration of UE performance collection and reporting. NG-RAN node2 shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since UE was successfully handed over to NG-RAN node2 is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	UE is handed over to another cell.

For the data collection of UE performance over Xn interface, the exit conditions to terminate the collection process were captured in the text procedural of Data Collection Initiation procedure.

Therefore, the exit conditions to terminate the collection of UE performance on the CU-UP side should also be described in the procedural text. Firstly, the expiration of the time duration is one of the exit conditions. Moreover, when the Bearer context changes, the UE performance collection on the CU-UP side must also be terminated.

Proposal 3	Capture the following exit conditions of termination of data collection in E1AP:

-	the time since the bearer context was successfully established is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	the established bearer context is modified or released

UE performance configuration, i.e., UE performance collection duration, is missing in the DATA COLLECTION REQUEST message.

Proposal 4	Introduce UE performance Collection Configuration IE in the DATA COLLECTION REQUEST message.

3	Conclusion

Following is our proposals and observations:

Proposal 1a	If SA5 provides the necessary clarification in time, incorporate their input to finalize the packet loss description in TS 37.483.

Proposal 1b	If SA5 does not respond immediately, retain the packet loss but keep its semantic description general until SA5 provides the required details.

Proposal 2	Using the range (0…1000000) to represent the Average Packet Loss DL IE and Average Packet Loss UL IE.

Proposal 3	Capture the following exit conditions of termination of data collection in E1AP:

-	the time since the bearer context was successfully established is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	the established bearer context is modified or released

Proposal 4	Introduce UE performance Collection Configuration IE in the DATA COLLECTION REQUEST message.

4	Reference

[1] R3-253023, Reply LS on per-UE UE performance metrics, SA5

[2]	R3-253824, Reply LS on clarification on UE performance measurement at PDCP level, SA5

[3] R3-253816, Reply LS on per-UE UE performance metrics, RAN3

[4] TS28.552

5	Text Proposal to 37.483

### 8.2.AA	Data Collection Reporting Initiation

#### 8.2.AA.1	General

This procedure is used by a gNB-CU-CP to request from a gNB-CU-UP the reporting of information to support, e.g., AI/ML in NG-RAN.

The procedure uses non UE-associated signalling.

#### 8.2.AA.2	Successful Operation

Figure 8.4.AA.2-1: Data Collection Reporting Initiation, successful operation

The gNB-CU-CP initiates the procedure by sending the DATA COLLECTION REQUEST message to gNB-CU-UP to start information reporting and stop information reporting. Upon receipt, the gNB-CU-UP:

-	shall initiate the requested information reporting according to the parameters given in the request in case the Registration Request for Data Collection IE is set to “start”; or

-	shall stop all information reporting and terminate the reporting in case the Registration Request for Data Collection IE is set to “stop”.

If the gNB-CU-UP is capable to provide all of the requested information, it shall initiate the information reporting as requested by the gNB-CU-CP and respond with the DATA COLLECTION RESPONSE message.

If the Reporting Periodicity for Data Collection IE in the DATA COLLECTION REQUEST is present, this indicates the periodicity for the reporting of periodic information. The gNB-CU-UP shall report only once, unless otherwise requested within the Reporting Periodicity for Data Collection IE.

If the Registration Request for Data Collection IE is set to "start" in the DATA COLLECTION REQUEST message and one or more of the UE performance metrics are requested, the UE Performance Collection Configuration IE shall be included. The gNB-CU-UP shall take the UE Performance Collection Configuration IE into account for the configuration of UE performance collection and reporting. The gNB-CU-UP shall terminate the collection when at least one of the following conditions is fulfilled:

-	the time since the bearer context was successfully established is equal to the value of the Collection Time Duration for UE Performance IE;

-	UE moves to RRC_INACTIVE or RRC_IDLE state;

-	the established bearer context is modified or released.

Interaction with other procedures

When starting a measurement, the Report Characteristics for Data Collection IE in the DATA COLLECTION REQUEST indicates the type of objects gNB-CU-UP shall perform measurements on. The gNB-CU-UP shall include in the DATA COLLECTION UPDATE message:

-	the Average Packet Delay IE, if the first bit, "Average Packet Delay" of the Report Characteristics for Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP;

-	the Average Packet Loss DL IE, if the second bit, "Average Packet Loss DL" of the Report Characteristics for   Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP;

-	the Average Packet Loss UL IE, if the third bit, "Average Packet Loss UL" of the Report Characteristics for   Data Collection IE included in the DATA COLLECTION REQUEST message is set to "1" and if the measurement object is admitted by gNB-CU-UP.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

## 8.3	Bearer Context Management procedures

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

*** skip unmodified parts ***

If the PDU Set QoS Parameters IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store it and use the information as specified in TS 23.501 [20].

Interactions with DL Data Notification procedure:

If the MT-SDT Information Request IE is included in the BEARER CONTEXT SETUP REQUEST message and the value is set to 'true', the gNB-CU-UP shall, if supported, store it and report the MT-SDT Information IE in the DL DATA NOTIFICATION message as specified in TS 38.401 [2].

If the SDT Data Size Threshold IE is included in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, store it and act as specified in TS 38.401 [2].

Interaction with the Data Collection Reporting and the Data Collection Reporting Initiation procedures:

If the Data Collection ID IE is contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall, if supported, report to the gNB-CU-CP after successful bearer context setup via the Data Collection Reporting procedure the requested information configured via the previous Data Collection Reporting Initiation procedure corresponding to the gNB-CU-CP Measurement ID IE, allocated by the gNB-CU-CP, and the gNB-CU-UP Measurement ID IE, allocated by the gNB-CU-UP.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.CC	DATA COLLECTION REQUEST

This message is sent by gNB-CU-CP to gNB-CU-UP to initiate the requested information reporting according to the parameters given in the message.

Direction: gNB-CU-CP  gNB-CU-UP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | C-ifRegistrationRequestStop |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| Registration Request for Data Collection | M |  | ENUMERATED(start, stop, …) | Type of request for which the AI/ML related information is required. | YES | reject |
| Report Characteristics for Data Collection | C-ifRegistrationRequestStart |  | BITSTRING (SIZE(32)) | Each position in the bitmap indicates the object the gNB-CU-UP is requested to report. First Bit = Average Packet Delay, Second Bit = Average Packet Loss DL, Third Bit = Average Packet Loss UL Other bits are ignored by the gNB-CU-UP. | YES | reject |
| Reporting Periodicity for Data Collection | O |  | ENUMERATED (500ms, 1000ms, 2000ms, 5000ms, 10000ms, …) | Periodicity that can be used for reporting of requested objects. Also used as the averaging window length for all objects if supported. | YES | ignore |
| UE Performance Collection Configuration | O |  | 9.3.1.MM |  | YES | ignore |

| Condition | Explanation |
| --- | --- |
| ifRegistrationRequestStop | This IE shall be present if the Registration Request IE is set to the value "stop". |
| ifRegistrationRequestStart | This IE shall be present if the Registration Request IE is set to the value "start". |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.1.FF	DATA COLLECTION UPDATE

This message is sent by gNB-CU-UP to gNB-CU-CP to report the requested information.

Direction: gNB-CU-UP  gNB-CU-CP.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.53 |  | YES | reject |
| gNB-CU-CP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-CP | YES | reject |
| gNB-CU-UP Measurement ID | M |  | INTEGER (1..4095,...) | Allocated by gNB-CU-UP | YES | ignore |
| UE Associated Info Result List |  | 0..1 |  |  |  |  |
| >UE Associated Info Result Item |  | 1..<maxnoofUEReports> |  |  |  |  |
| >>UE Assistant Identifier | M |  | gNB-CU-CP UE E1AP ID 9.3.1.4 | gNB-CU-CP UE E1AP ID allocated by gNB-CU-CP. |  |  |
| >>DRB To Report List |  | 0..1 |  |  |  |  |
| >>>DRB To Report Item |  | 1..<maxnoofDRBs> |  |  |  |  |
| >>>>DRB ID | M |  | 9.3.1.16 |  |  |  |
| >>>>UE Performance | O |  | 9.3.1.aa |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofUEReports | Maximum no. UE s for which information can be reported by a NG-RAN node. Value is 16. |
| maxnoofDRBs | Maximum no. of DRBs allowed towards one UE. Value is 32. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.2.2.1	BEARER CONTEXT SETUP REQUEST

This message is sent by the gNB-CU-CP to request the gNB-CU-UP to setup a bearer context.

Direction: gNB-CU-CP  gNB-CU-UP

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| gNB-CU-CP UE E1AP ID | M |  | 9.3.1.4 |  | YES | reject |
| *** skip unmodified parts *** |  |  |  |  |  |  |
| SDT Data Size Threshold | O |  | INTEGER (1.. 192000, ...) | Unit: byte. | YES | Ignore |
| Data Collection ID | O |  | 9.3.1.dd |  | YES | ignore |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.aa	UE Performance

This IE indicates the UE performance measurements metrics.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Average Packet Delay | O |  | 9.3.1.bb | Corresponds to Average Packet Delay per-UE as specified in TS 28.558 [XX] clause 6.3.1.1. |
| Average Packet Loss DL | O |  | 9.3.1.cc | Corresponds to DL PDCP SDU Drop Rate in gNB-CU-UP as specified in TS 28.552 [XX1] clause 5.1.3.2.1 |
| Average Packet Loss UL | O |  | 9.3.1.cc | Corresponds to UL PDCP SDU Loss Rate as specified in TS 28.552 [XX1] clause 5.1.3.1.1 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.bb	Average Packet Delay

This IE indicates the RAN part of the average packet delay in the UL and DL directions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Delay UL | M |  | INTEGER (0.. 10000) | Defined in TS 38.314 [XX2]. Unit: 0.1 millisecond |
| Average Packet Delay DL | M |  | INTEGER (0.. 10000) | Defined in TS 38.314 [XX2]. Unit: 0.1 millisecond |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.cc	Average Packet Loss

This IE indicates the Average Packet Loss in the UL and DL directions.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Average Packet Loss | M |  | INTEGER (0..100000, ...) | Ratio of lost packets per number of packets sent, expressed in units of 10-6 percent. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.3.1.MM	UE Performance Collection Configuration

This IE indicates the configuration for UE performance measurement collection.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Collection Time Duration for UE Performance | M |  | INTEGER(1..5000, ...) | Time duration starting at successfully bearer context establishment within which the UE performance measurements are collected. Unit: millisecond |

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>