3GPP TSG-RAN WG3 #129                                           R3-255181

Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	16.2

Source:	CATT

Title:	Discussion on A-IoT leftover issues

Document for:	Decision

- Introduction

On support of A-IoT, a study item has been completed in the last RAN WG meetings. Corresponding solutions on the architecture, signallings, locating, etc. are captured in the TR38.769 [1]. In RAN#106, a new work item for Ambient IoT has been approved(WID in [2]), aimed to standardize RAN aspects of Ambient IoT. In the WID, we will only focus on Topology 1, the major RAN3 scope are highlighted below:

- RAN3 scope:

  - Specify necessary architectural aspects, and signaling and procedures between A-IoT RAN and A-IoT CN to support the A-IoT functions, assuming an architecture of aggregated gNB, including:

    - Inventory and command operations

    - Device location reporting at reader ID granularity

Note: The above A-IoT functions are supported over the existing NG interface, based on architecture(s) defined by RAN3/SA2.

In the last RAN3 meeting, we discussed the inventory and command procedure for AIoT Topology 1, corresponding agreements and working assumptions are listed below:

The new A-IoT Area is encoded as an A-IoT Area ID.

A-IoT Area ID = PLMN ID +NID(optional) + A-IoT Area Code (OCTET STRING (SIZE(3)))

One reader only belongs to one gNB.

One reader can map to one or multiple A-IoT Area ID(s).

One A-IoT Area may include readers belong to the same or different gNBs.

One gNB may serve multiple readers which belong to the same or different A-IoT Areas.

OAM configures in the AIOTF the mapping relationships among gNBs, readers and A-IoT areas, as needed.

Non-UE associated signalling principle and requirements applies to A-IoT related signalling.

Introduce new CN triggered Class 1 Session Release procedure, which includes A-IoT SESSION RELEASE REQUEST message and A-IoT SESSION RELEASE RESPONSE message.

Turn WA to agreement “NGAP: Command Request procedure is a per single device procedure, and no need to have a Command Report procedure.”

In case of indirect connectivity, allow parallel sessions between gNB and AMF.

In case of indirect connectivity, allow parallel Command procedures for different devices between gNB and AMF within the same session.

CN A-IoT Device NGAP ID is not needed.

Define the Expected D2R Message Size IE in Inventory Request Transfer IE. FFS on the detail encoding.

Remove the FFSs on the presence of Cause IE in the INVENTORY FAILURE message and the COMMAND FAILURE message.

Define the A-IoT Device Identification Requested IE as CHOICE type, with three branches:

-	single device inventory:		OCTECT STRING	refer to Device Identifier

-	group devices inventory:		FFS on the encoding

-	all devices inventory:		NULL

Define the Estimate of Expected D2R Message Size IE in the Command Assistance Information IE as INTEGER (1..128, …) unit: byte. Add the semantic description that this IE refers to NAS PDU size.

Details of the Reader Location is out the scope of RAN3.

Reader Selection refers to SA2 TS 23.369 clause 5.3.3. Further check is needed.

RAN A-IoT Information will not be provided over NGAP from NG-RAN node to AIoT CN.

Turn WAs to agreement “Include the Correlation Identifier IE outside of the Inventory related Transfer IEs in all the Inventory related messages.” “Include the Correlation Identifier IE in both inside and outside of the Command related Transfer IEs in all the Command related messages.”

Include RAN NGAP Device ID outside of the containers in the Command related messages.

Mandatorily include the Expected D2R Message Size in Inventory Request Transfer? Check progress in SA2.

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message.

Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN.

WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …)

In this contribution, we will further discuss the remaining issues on support of Ambient IoT. Base on the discussion, we will provide the proposals and TPs accordingly.

- Discussion

2.1. IE encoding

In last RAN3 meeting, Expected D2R Message Size IE is introduced and encoding is FFS as below:

Define the Expected D2R Message Size IE in Inventory Request Transfer IE. FFS on the detail encoding.

We believe it should be the same type as the Estimate of Expected D2R Message Size IE in the Command Assistance Information IE as below.

Define the Estimate of Expected D2R Message Size IE in the Command Assistance Information IE as INTEGER (1..128, …) unit: byte. Add the semantic description that this IE refers to NAS PDU size.

For inventory procedure, AIoT Device Permanent Identifier may be transferred from device to reader. The structure is defined in C4-252464[4] captured below:

YY.2	Structure of AIoT Device Permanent Identifier

The AIoT Device Permanent Identifier shall be composed as shown in Figure YY.2-1:

The maximum size of AIOT device permanent identifier is 256 bits which is the same as the size of Filtering Information. For security aspect, if LS from SA3 received later, RAN3 can make further discussion then.

Proposal 1: Expected D2R Message Size IE in Inventory Request Transfer IE should be defined as INTEGER type, the SIZE is 256 as defined in CT4.

For the encoding of Identifier IE, the FFS captured in last meeting is as below:

FFS on the detail encoding of the AIOTF Iden	tifier IE? Refer to the NfInstanceId IE defined in TS 29.571?

FFS on the detail encoding of the A-IoT Correlation Identifier IE? Wait for further inputs from CT4.

These identifiers are still under discussion in SA2/CT4. In our opinion, it could be defined as OCTET STRING now, and the length is variable.

Proposal 2: AIOTF Identifier IE and A-IoT Correlation Identifier IE should be defined as OCTET STRING, the length is variable.

On group devices inventory, encoding is still FFS.

Define the A-IoT Device Identification Requested IE as CHOICE type, with three branches:

-        single device inventory:             OCTECT STRING         refer to Device Identifier

-        group devices inventory:           FFS on the encoding

-        all devices inventory:                  NULL

On triggering inventory for group devices, filtering information is used and the structure of filtering information is defined in C4-252464[4] as below:

aa.3	Structure of Filtering Information

The Filtering Information is used to identify or filter multiple AIoT Devices, the structure is shown in Figure aa.3.1

Figure aa.3.1: Structure of Filtering Information

Therefore, the encoding of group devices inventory IE could refer to Filtering Information.

Proposal 3: The “group devices inventory” could be defined as BIT STRING, SIZE is 256.

2.2. Complete indication

In last RAN3 meeting, we have agreed to introduce Inventory Complete indication. The related agreement and FFS is captured below:

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message.

In our opinion, introducing indication IE in current Inventory Report message is feasible and straightforward, not necessary to define a new class 2 message to indicate the completion of the Inventory procedure.

Proposal 4: Introduce an Inventory Complete Indication IE in current Inventory Report message.

On introducing complete indication for the case of Command after Inventory scenario, the FFS is captured below:

In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

For Command after Inventory scenario, completion of the inventory could also be indicated to AIoTF via the new indicator in the Inventory report as proposed above.

For the command procedure, it’s performed per device. Thus, the per device completion of the command procedure could be implicitly indicated by the COMMAND RESPONSE/FAILURE. For per session complete, the new introduced A-IoT Session Release procedure could also be used to ask the A-IoT CN node to complete A-IoT session. Therefore, introducing the complete indication for command after inventory scenario is not needed.

Proposal 5: No need to introduce extra indication to indicate completion of the Command procedure.

2.3. A-IoT Support Indicator in the NG SETUP RESPONSE message?

In last RAN3 meeting, we captured following WA:

WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …)

Moreover, whether introducing A-IoT Support Indicator in the NG SETUP RESPONSE message is still under discussion as below:

Whether to have A-IoT Support Indicator in the NG SETUP RESPONSE message?

We agree to define the A-IoT Support IE in the NG Setup Request. With this IE, A-IoT CN could only use A-IoT related IE when receiving legacy NG Setup Request message. As for response message, A-IoT RAN can be aware of the capability of A-IoT CN based on configuration, i.e. whether it is an A-IoT CN or a legacy CN. So, it is not necessary to introduce A-IoT Support Indicator in the NG SETUP RESPONSE message.

Proposal 6: No need to introduce A-IoT Support Indicator in the NG SETUP RESPONSE message.

In case A-IoT RAN is directly connected with AIOTF, when receiving NG SETUP RESPONSE message, A-IoT RAN should ignore the not applicable IEs with mandatory presence. Some clarification should be added in tabular and procedure.

Proposal 7: Some description should be added in tabular and procedure text to clarify that the mandatory IEs in NG SETUP RESPONSE message should be ignored by A-IoT RAN if directly connected with AIOTF.

2.4. Assistance information

For introducing time interval as assistance information in the Inventory Request message, the following FFS is captured.

It is FFS on the Time Interval as assistance information in the Inventory Request message. Clarify with SA2 on the usage of this information in NG-RAN node.

5.9	AIoT Service Operation Result Aggregation

An AIoT service operation may involve many AIoT Devices. The NG-RAN may perform AIoT service operation result aggregation with a specific Correlation ID based on the aggregation assistance information received from the AIOTF for a service operation request as specified in clause 5.4.

The AIOTF determines the aggregation assistance information based on the request from the AF or local configuration, which includes:

-	Time interval: the fixed time interval for which NG-RAN collects multiple AIoT Devices’ operation responses before reporting the aggregated AIoT response to the AIOTF. The reporting based on time interval may potentially happen multiple times until the NG-RAN completes the request operation.

If the AF has provided a time interval, then the AIOTF should signal a time interval to the NG-RAN that is equal or shorter than the time interval received from the AF.

NOTE:	Based on local configuration, the AIOTF can reject the AF request, e.g. if the AF provided time interval is shorter than a locally configured minimum interval.

If the AIOTF does not provide aggregation assistance information, the aggregation process in the NG-RAN may be determined by implementation.

The AIOTF may also aggregate the results of a requested service operation before sending them to the NEF or trusted AF.

In TS23.369 [3], time interval is used for aggregation. So, Time Interval should be introduced in assistance information and the usage can refer to SA2 specification.

Proposal 8: Beneficial to introduce Time Interval in assistance information and the usage in NG-RAN can refer to TS23.369 [3].

As for command type, the FFS is captured below:

It is FFS to introduce Command Type (write, read, disable) in Command Request Transfer.

For SA2, Command Type is not included in the assistance information provided to NG-RAN node according to TS23.369 [3]. We understand it’s beneficial to provide this information to RAN as the assistance information, gNB may use it for better resource management for different command types. Even though, there’s no consensus on the usage of this command type in RAN2 and RAN3, we still think it’s no harm to add it in the assistance info, how to use it could be left to implementation.

Proposal 9: Beneficial to introduce Command Type (write, read, disable) in Command Request Transfer.

Base on the discussion above, we provide a stage2 TP for TS38.300 BL CR [5], and also a stage 3 TP to clean-up the FFS for TS38.413 BL CR [6]. RAN3 is requested to discuss and agree the TPs.

Proposal 10: Discuss and agree the TP for TS38.300 BL CR in [5], and the TP for TS38.413 BL CR in [6].

- Conclusions

In this contribution, we discussed RAN3 remaining issues on support of Ambient IoT. Based on the discussion, the following proposals are provided:

Proposal 1: Expected D2R Message Size IE in Inventory Request Transfer IE should be defined as INTEGER type, the SIZE is 256 as defined in CT4.

Proposal 2: AIOTF Identifier IE and A-IoT Correlation Identifier IE should be defined as OCTET STRING, the length is variable.

Proposal 3: The “group devices inventory” could be defined as BIT STRING, SIZE is 256.

Proposal 4: Introduce an Inventory Complete Indication IE in current Inventory Report message.

Proposal 5: No need to introduce extra indication to indicate completion of the Command procedure.

Proposal 6: No need to introduce A-IoT Support Indicator in the NG SETUP RESPONSE message.

Proposal 7: Some description should be added in tabular and procedure text to clarify that the mandatory IEs in NG SETUP RESPONSE message should be ignored by A-IoT RAN if directly connected with AIOTF.

Proposal 8: Beneficial to introduce Time Interval in assistance information and the usage in NG-RAN can refer to TS23.369 [3].

Proposal 9: Beneficial to introduce Command Type (write, read, disable) in Command Request Transfer.

Proposal 10: Discuss and agree the TP for TS38.300 BL CR in [5], and the TP for TS38.413 BL CR in [6].

- Reference

[1] TR 38.769 Study on solutions for ambient IoT (Internet of Things)

[2] RP-243326 New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[3] TS23.369 V0.3.0 (2025-04) “Ambient power-enabled Internet of Things”

[4] C4-252464 “Definition of AIoT Device Permanent Identifier” 3GPP TSG-CT WG4 Meeting #129

[5] R3-255182 (TP to BL CR for TS38.300) Support of A-IoT, CATT

[6] R3-255183 (TP to BL CR for TS38.413) on A-IoT leftover issues, CATT