3GPP TSG-RAN WG3 Meeting #129	R3-255187

Bengaluru, India, 25-29, August, 2025

Agenda item:	9.2

Source: 	Samsung, CATT, ZTE

Title: 	Discussion on positioning activation and deactivation procedure

Document for:	Discussion and decision

Introduction

The procedure of area-specific SRS configuration activation is captured in TS 38.305, 7.14.4 as below,

Figure 7.14.4-1: Area-specific SRS Configuration Activation Procedure

0.	The UE in RRC_INACTIVE state is (pre-)configured with one or more area-specific SRS for positioning configurations as specified in clause 7.14.2 and receives a request from upper layers to activate an area-specific (pre-)configured SRS for positioning when the UE is camped on one of the cells indicated in the validity area(s) (e.g., when a configured periodic or triggered event is detected).

1.	The UE sends a RRC Resume Request message with the resume cause "srs-PosConfigOrActivationReq" to request activation of the area-specific (pre-)configured SRS.

2.	The receiving gNB which receives the request from the UE triggers the Retrieve UE Context procedure towards the last serving gNB and includes the area-specific (pre-)configured SRS activation request in the Retrieve UE Context Request message.

3.	The last serving gNB sends a NRPPa Positioning Information Update message to the LMF including the Cell ID of the receiving gNB where the UE resumes from, together with an indication to notify the LMF that the area-specific (pre-)configured SRS is activated in the UE.

4.	The last serving gNB relocates the full UE context to the receiving gNB and includes the (pre-)configured SRS for positioning configuration(s), the Routing ID and the NRPPa Transaction ID in the Retrieve UE Context Response message.

NOTE:	If the last serving gNB decides not to relocate the full UE context, the last serving gNB provides a Retrieve UE Context Failure message to the receiving gNB, including a RRC Release message with the area-specific (pre-)configured SRS for positioning configuration and steps 5, 6, and 8 below are not performed.

5.	The receiving gNB triggers the Path Switch Request procedure towards the AMF.

6.	The AMF responds with the Path Switch Request Acknowledge.

7.	The receiving gNB activates the area-specific (pre-)configured SRS for positioning and releases the UE to RRC_INACTIVE state.

8.	The receiving gNB indicates the last serving gNB to release the UE context.

According to the above text, we can obtain that the activation procedure supports both anchor relocation case and no anchor relocation case.

However, the previous RAN3 discussion did not touch too much on the SRS resource type (i.e. periodic, semi-persistent and aperiodic), and it seems from RAN3 perspective that the above text should cover all supported SRS resource types situations; on the contrary, RAN2 discussed whether to support each SRS resource type in detail, and had the following agreements,

Agreements:

Periodic SRS is supported to be configured with validity area.  This agreement does not affect preconfigured SRS.

Activation/deactivation is not required for periodic SRS.  This agreement does not affect preconfigured SRS.

Aperiodic SRS is not supported to be configured with validity area.

Semi-persistent SRS is supported to be configured with validity area, and RAN2 agree to reuse legacy mechanism to deactivate the SP SRS

According to RAN2 agreement, Periodic and Semi-Persistent (SP) SRS are supported for area-specific SRS configuration.

Observation 1: According to latest TS 38.305, the area-specific SRS activation procedure supports both anchor relocation case and no anchor relocation case.

Observation 2: According to RAN2 agreement, both Periodic and Semi-Persistent (SP) resource types are supported for area-specific SRS configuration.

Observation 3: The procedure of area-specific SRS activation should cover both SP SRS and periodic SRS cases.

However, it seems that the current procedure can cover the periodic SRS activation, but how to cover the SP SRS activation needs further investigation.

In addition, how to support SP SRS deactivation also needs further investigation.

Observation 4: How to support SP SRS activation and deactivation procedure needs further investigation.

In this contribution, we discuss how to support the SP SRS activation and deactivation procedure.

Discussion

In legacy mechanism, for SP SRS activation over NRPPa, the LMF is able to trigger UE-associated Positioning Activation procedure to activate ONE SRS Resource Set as follows,

9.1.1.17	POSITIONING ACTIVATION REQUEST

This message is sent by the LMF to cause the NG RAN node to activate/trigger UL SRS transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE SRS type | M |  |  |  | YES | reject |
| >Semi-persistent |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 | This IE is ignored if the Aggregated Positioning SRS Resource Set List IE below is present. | - |  |

In addition, for SP SRS deactivation over NRPPa, the LMF is also able to trigger UE-associated Positioning Deactivation procedure to deactivate ONE SRS Resource Set as follows,

9.1.1.20	POSITIONING DEACTIVATION

This message is sent by the LMF to cause the NG RAN node to deactivate UL SRS transmission or release all the transmission by the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| CHOICE Abort Transmission | M |  |  |  | YES | ignore |
| >Deactivate SRS Resource Set |  |  |  |  |  |  |
| >>SRS Resource Set ID | M |  | 9.2.33 |  | - |  |

Observation 5: For SP SRS activation and deactivation over NRPPa, the LMF is able to activate and deactivate SRS transmission on a SRS Resource Set basis, which is different from Periodic SRS activation (i.e. the whole periodic SRS configuration including ALL configured SRS Resource Sets are activated for Periodic SRS activation).

In addition, it is RAN2 common understanding to reuse the legacy mechanism to activate/deactivate the SP SRS by means of SP SRS Activation/Deactivation MAC CE as defined in TS 38.321.

Observation 6: It is RAN2 common understanding to reuse the existing SP SRS Activation/Deactivation MAC CE for area-specific SP SRS activation/deactivation.

As a result, RAN3 needs to discuss which node decides the Activated/Deactivated SRS Resource Set(s) and how to perform the corresponding coordination at the network side for area-specific SP SRS.

Observation 7: RAN3 needs to discuss which node decides the Activated/Deactivated SRS Resource Set(s) and how to perform the corresponding coordination at the network side for area-specific SP SRS, also taking split architecture into account.

2.1 SP SRS Activation procedure

Generally speaking, there are three options for activation decision for area-specific SP SRS,

- Option 1: LMF makes the activation decision

- Option 2: Last Serving gNB makes the activation decision

- Option 3: Receiving gNB makes the activation decision

And both anchor relocation case and no anchor relocation case should be considered.

For anchor relocation case,

The activation procedure for Option 1 is as follows,

Under this case,

- For Stage 2, two additional messages for NRPPa Positioning Activation procedure is needed between LMF and Receiving gNB after the Receiving gNB becomes the new Anchor gNB.

- No Stage 3 impact is identified.

The activation procedure for Option 2 is as follows,

Under this case,

- For Stage 2, no additional message is needed.

- For Stage 3, Activated SRS Resource Set ID(s) is needed to be added in both NRPPa POSITIONING INFORMATION UPDATE message and XnAP RETRIEVE UE CONTEXT RESPONSE message.

The activation procedure for Option 3 is as follows,

Under this case,

- For Stage 2, an additional NRPPa POSITIONING INFORMATION UPDATE message is needed to be added between Step 6&7.

- For Stage 3, Activated SRS Resource Set ID(s) is needed to be added in NRPPa POSITIONING INFORMATION UPDATE message.

For no anchor relocation case,

The activation procedure for Option 1 is as follows,

Under this case,

- For Stage 2, two additional messages for NRPPa Positioning Activation procedure is needed between LMF and Receiving gNB after the Receiving gNB becomes the new Anchor gNB.

- For Stage 3, Activated SRS Resource Set ID(s) is needed to be added in XnAP RETRIEVE UE CONTEXT FAILURE message.

The activation procedure for Option 2 is as follows,

Under this case,

- For Stage 2, no additional message is needed.

- For Stage 3, Activated SRS Resource Set ID(s) is needed to be added in both NRPPa POSITIONING INFORMATION UPDATE message and XnAP RETRIEVE UE CONTEXT FAILURE message.

The activation procedure for Option 3 is as follows,

In order to enable the receiving gNB to make activation decision in case of no anchor relocation, the receiving gNB needs to understand the SRS (pre-)configuration configured for a UE.

On how to let receiving gNB understand the SRS (pre-)configuration for a UE, basically there are two approaches,

Approach 1: The UE ID is provided in advance during area-specific SRS configuration procedure

The activation procedure for Option 3, Approach 1 is as follows,

Under this case,

- For Stage 2, no additional message is needed.

- For Stage 3,

  - During configuration procedure, I-RNTI is needed to be added in SRS INFORMATION RESERVATION NOTIFICATION message.

  - During activation procedure, Activated SRS Resource Set ID(s) is needed to be added in both NRPPa POSITIONING INFORMATION UPDATE message and XnAP RETRIEVE UE CONTEXT REQUEST message.

Approach 2: The SRS (pre-)configuration is provided during Partial UE Context Transfer procedure

The activation procedure for Option 3, Approach 2 is as follows,

Under this case,

- For Stage 2, two additional messages for XnAP Partial UE Context Transder procedure is needed.

- For Stage 3, Activated SRS Resource Set ID(s) is needed to be added in XnAP PARTIAL UE CONTEXT TRANSFER ACKNOWLEDGE message and NRPPa POSITIONING INFORMATION UPDATE message, and Area-specific SRS (pre-)configuration is needed to be added in XnAP PARTIAL UE CONTEXT TRANSFER message.

The following table gives a primary analysis on these options,

| Activation decision by | Anchor relocation | No anchor relocation |
| --- | --- | --- |
| LMF | Follows similar logic as for non-area-specific SRS configuration. Two additional messages during activation procedure. No stage 3 impact. | Follows similar logic as for non-area-specific SRS configuration. Two additional messages during activation procedure. Small Stage 3 impact on XnAP. |
| Last Serving gNB | No additional message during activation procedure. Small Stage 3 impact on XnAP. Small Stage 3 impact on NRPPa. | No additional message during activation procedure. Small Stage 3 impact on XnAP. Small Stage 3 impact on NRPPa. |
| Receiving gNB | One additional message during activation procedure. Small Stage 3 impact on NRPPa. | No/Two additional message(s) during activation procedure. Big Stage 3 impact on XnAP. Big Stage 3 impact on NRPPa. |

According to the primary discussion last meeting, companies seemed to have different flavours on which option to go. More specifically, some company commented that Option3 (Receiving gNB activation) is more compliant to the original intention in R18; some other company preferred to follow Option1 (LMF activation) which may have the minimum spec impact; while some other company suggested to adopt Option2 (Last Serving gNB activation) since it may be the simplest approach.

In order to move forward, we suggest companies to take the above table into account when down selecting options. Considering this is a late stage, we prefer the option with minimum spec impact (i.e. Option1).

Proposal 1: LMF makes the activation decision for area-specific SP SRS.

Proposal 2: Introduce SP SRS activation related information (i.e. Activated SRS Resource Set ID) in XnAP RETRIEVE UE CONTEXT FAILURE message in case of no anchor relocation.

2.2 SP SRS Deactivation procedure

Generally speaking, there are two options for deactivation decision for area-specific SP SRS,

- Option 1: LMF makes the deactivation decision

- Option 2: Anchor gNB makes the deactivation decision

In our understanding, compared to Option 2, it is more proper to follow Option 1 because LMF has more knowledge on the progress of the measurement; while if the anchor gNB makes the deactivation decision, the anchor gNB has to make the decision blindly which is not an appropriate behaviour.

Proposal 3: LMF makes the deactivation decision for area-specific SP SRS.

And the basic call flow for deactivation procedure is as follows (considering the LMF makes the deactivation decision),

It can be seen from the above figure that,

- Before Step 6, the Anchor gNB1 will decide whether to perform anchor relocation. If gNB1 decides to relocate the anchor, it will send XnAP RETRIEVE UE CONTEXT RESPONSE message to gNB2 in Step 6, and after Step 6 the gNB2 will become the new anchor gNB; else if gNB1 decides not to relocate the anchor, it will send XnAP RETRIEVE UE CONTEXT FAILURE message to gNB2 in Step 6.

- No matter which XnAP message is transmitted in Step 6, the information on Deactivated SRS Resource Set ID should be included in the message.

Proposal 4: Introduce SP SRS deactivation related information (e.g. Deactivated SRS Resource Set ID) in XnAP RETRIEVE UE CONTEXT RESPONSE/FAILURE message to support SP SRS Deactivation procedure.

2.3 F1AP impact

It can be observed from above figures that for SP SRS activation/deactivation, the gNB-DU needs to send SP SRS Activation/Deactivation MAC CE to the UE. As a result, the gNB-CU needs to send SP SRS activation/deactivation related information to gNB-DU via F1AP.

Observation 8: The gNB-CU needs to send SP SRS activation/deactivation related information to gNB-DU via F1AP.

In current TS 38.401, when the UE goes from RRC inactive to other RRC states, the following figure is captured,

From the perspective of SP SRS transmission activation/deactivation, it can be observed from the above figure that, after the receiving gNB-CU obtains the RRC Release message (either generated by itself in case of anchor relocation, or obtained from the last serving gNB in case of no anchor relocation), the RRC Release message is sent from receiving gNB-CU to gNB-DU via F1AP DL RRC MESSAGE TRANSFER message.

In order to support SP SRS activation/deactivation which requires the receiving gNB-DU to send SP SRS Activation/Deactivation MAC CE to the UE, the SP SRS activation/deactivation related information should also be included in F1AP DL RRC MESSAGE TRANSFER message.

Proposal 5: In case of no anchor relocation, introduce SP SRS activation/deactivation related information (e.g. SRS Resource Set ID) in F1AP DL RRC MESSAGE TRANSFER message.

Proposal 6: Agree CRs in R3-255186, R3-255188, R3-255189, R3-255190.

Conclusion

In this paper, we provides considerations on SP SRS activation and deactivation. The following observations and proposals are provided,

Observation 1: According to latest TS 38.305, the area-specific SRS activation procedure supports both anchor relocation case and no anchor relocation case.

Observation 2: According to RAN2 agreement, both Periodic and Semi-Persistent (SP) resource types are supported for area-specific SRS configuration.

Observation 3: The procedure of area-specific SRS activation should cover both SP SRS and periodic SRS cases.

Observation 4: How to support SP SRS activation and deactivation procedure needs further investigation.

Activation Procedure

Observation 5: For SP SRS activation and deactivation over NRPPa, the LMF is able to activate and deactivate SRS transmission on a SRS Resource Set basis, which is different from Periodic SRS activation (i.e. the whole periodic SRS configuration including ALL configured SRS Resource Sets are activated for Periodic SRS activation).

Observation 6: It is RAN2 common understanding to reuse the existing SP SRS Activation/Deactivation MAC CE for area-specific SP SRS activation/deactivation.

Observation 7: RAN3 needs to discuss which node decides the Activated/Deactivated SRS Resource Set(s) and how to perform the corresponding coordination at the network side for area-specific SP SRS, also taking split architecture into account.

Proposal 1: LMF makes the activation decision for area-specific SP SRS.

Proposal 2: Introduce SP SRS activation related information (i.e. Activated SRS Resource Set ID) in XnAP RETRIEVE UE CONTEXT FAILURE message in case of no anchor relocation.

Deactivation Procedure

Proposal 3: LMF makes the deactivation decision for area-specific SP SRS.

Proposal 4: Introduce SP SRS deactivation related information (e.g. Deactivated SRS Resource Set ID) in XnAP RETRIEVE UE CONTEXT RESPONSE/FAILURE message to support SP SRS Deactivation procedure.

F1AP Impact

Observation 8: The gNB-CU needs to send SP SRS activation/deactivation related information to gNB-DU via F1AP.

Observation 9: In case of anchor relocation, reusing the existing F1AP Positioning Activation/Deactivation procedure is enough for SP SRS Activation/Deactivation procedure.

Proposal 5: In case of no anchor relocation, introduce SP SRS activation/deactivation related information (e.g. SRS Resource Set ID) in F1AP SRS INFORMATION RESREVATION NOTIFICATION message.

Summary

Proposal 6: Agree CRs in R3-255186, R3-255188, R3-255189, R3-255190.