3GPP TSG-RAN WG3 #129	R3-255198

Bengaluru, India, 25th – 29th August 2025

Agenda item:		13.2 Support of inter-CU LTM

Title:		(TP to BL CR for TS 38.423 and 37.340 on Inter-CU LTM with DC) Remaining issues of Rel-19 inter-CU LTM in DC scenario

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

Last RAN3#128 meeting for the inter-CU LTM with DC configuration,

- TP to baseline CR of 38.423 (R3-253850, R3-253904) were agreed, and after the meeting in e-mail checking of Post_Meeting_Merging_TPs, the merging of TP into the new baseline CR of 38.423 (R3-254010).

- also TP to baseline CR of 37.340 (R3-253922) was agreed, and after the meeting in e-mail checking of Post_Meeting_Merging_TPs, the merging agreed TP into the based CR of 37.340 (R3-254006) which was endorsed.

The agreed TP was reflecting the following agreement:

From RAN3#128: Inter-CU LTM in DC: Update the text description of CELL SWITCH NOTIFICATION message to capture the RAN3#127bis agreement: Include the target PSCell ID and corresponding TCI State ID(s) in the Cell Switch Notification message, and reusing the existing IEs. Update the TA INFORMATION TRANSFER message to provide a list of TA values of multiple candidate cells. The SCG reference configuration is provided by an implicit way in the CG-Config RRC container in the SN Change Required message. FFS on the SN Addition Request Acknowledge message from the source SN/candidate SN to the MN. The SCG reference configuration is provided by an implicit way in the CG-ConfigInfo RRC container in the SN Addition Request message from the MN to the candidate SN. A per cell indicator indicating complete candidate configuration is included in the SN Addition Request Acknowledge message. The SN Change Required message and the SN Modification Request message from MN to source SN should design a mechanism to support multiple candidate SNs, i.e., SN can include multiple candidate SNs information in a single SN Change Required message, and MN includes multiple candidate SNs information in a single SN Modification Request message. The SN Addition Request message should design a mechanism to support single candidate SN, i.e., MN sends SN addition request towards only one candidate SN. The yellow colour highlighted does not concern inter-CU LTM in DC: Update the online agreement to: CU can request Candidate DU to provide CSI-RS configuration in UE CONTEXT SETUP REQUEST and UE CONTEXT MODIFICATION REQUEST message, and Candidate DU signals the CSI-RS configuration in UE CONTEXT SETUP RESPONSE and UE CONTEXT MODIFICATION RESPONSE message. Reuse the CSI-RS coordination procedure over F1AP and XnAP for source gNB/gNB-DU to activate or deactivate the SP CSI-RS resource for CSI acquisition in candidate cell. In Xn interface, candidate gNB provides the LTM CFRA Resource Configuration of each candidate cell to source gNB for LTM cell switch command generation. Source gNB generate the UE Based TA Measurement Configuration, and transfer it to all candidate gNB(s) via LTM Configuration Update message. Include the Rel-19 set IDs of source cell and each candidate cell(s) in UE Context Modification Request message. Introduce Rel-19 set IDs into LTM Security Information IE. FFS on the Rel-19 Set ID(s) assignment among CUs, down select from Option1 and Option2: Option 1: Source gNB sends the Rel-19 Set ID(s) or Rel-19 set ID range assigned to the candidate gNB in the HANDOVER REQUEST message, then candidate gNB assigns Rel-19 set ID(s) to its own candidate cells and feedback via HANDOVER REQUEST ACKNOWLEDGE message. Option 2: Candidate gNB provides Rel-19 set ID per candidate cell in HANDOVER REQUEST ACKNOWLEDGE message, then source gNB may update the Rel-19 set ID to ensure that the Rel-19 set IDs under different candidate gNB-CU are different. FFS on the procedure to be used for source gNB to transfer Rel-19 set ID per candidate cell to the candidate gNB. FFS on whether gNB-DU/gNB provides the report type (periodic or semi-persistent) of the CSI-RS resources in both F1AP and XnAP. FFS on whether the TCI State/QCL-info List needs to be included in CSI-RS COORDINATION procedure. FFS on whether to include the SP CSI-RS and SSB mapping in the HANDOVER REQUEST ACKKNOLEDGE message and UE Context Modification Request message. FFS on whether to add a new IE for SP CSI-RS resource for CSI acquisition in the corresponding procedure of SP CSI-RS resource for L1 RSRP measurement in F1AP and XnAP. The suggested PSCell list should be explicitly included in the SN Change Required message. The maximum number of PSCells that can be prepared by each candidate SN is included in the SN Change Required message. The LTM Configuration ID Mapping List IE may be included in the SN Change Required message and the SN Addition Request message. The LTM Security Configuration List IE including a list of pair of {security key, sk-counter} is included in the SN Addition Request message and the SN Modification Request message to support subsequent inter-CU SCG LTM. The SN Security Key IE included in the SN Addition Request message should be ignored if the procedure is triggered for the LTM. WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM. If the source SN has the SCG reference configuration, the source SN will provide the SCG reference configuration in the SN Change Required message and thus the MN will not request source SN to provide the SCG reference configuration. Early data forwarding is supported in inter-CU LTM in DC. Normal data forwarding can be initiated after the source SN decides to trigger the LTM cell switch for the UE. The exact timing of its initiation is left to implementation. Enhance XN-U ADDRESS INDICATION message and define IE to cover Inter-CU MCG LTM case. FFS on Inter-SN LTM case. MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN. The source SN generates the common LTM CSI resource configuration for inter-CU SCG LTM and then provides the common LTM CSI resource configuration to the MN via the SN Modification Request ACK message. For the SSB based L1 measurement, the common CSI resource configuration may be included in the SN Change Required message and the SN Addition Request message. The CSI Resource Configuration IE is the common CSI resource configuration, which refers to the ltm-CSI-ResourceConfig-r18 IE in the RRC spec. The L1 Configuration IE is the L1 RS configuration per candidate PSCell, which refers to the ltm-SSB-Config-r18 IE in the RRC spec for the SSB based L1 measurement, or the ltm-NZP-CSI-RS-ResourceConfigToAddModList-r19 IE associated with the ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 IE in the RRC spec for the CSI-RS based L1 measurement. For the avoidance of simultaneous configurations of inter-CU MCG LTM and inter-CU SCG LTM, no RAN3 impact is foreseen. FFS on whether LTM modification/cancel related procedures are needed. The SP CSI-RS activation/(de)activation for the inter-CU LTM in DC is deprioritized in rel-19.

This contribution we continue to discuss the remaining issues.

WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.

FFS on whether LTM modification/cancel related procedures are needed.

2. Discussion

2.1	Security related issue for SCG LTM – R19 Set ID (ltm-NoSecurityChangeID)

For the transferring of security information (list of pair if {security key, sk-counter}), last RAN3#128 meeting has discussed and agreed:

The LTM Security Configuration List IE including a list of pair of {security key, sk-counter} is included in the SN Addition Request message and the SN Modification Request message to support subsequent inter-CU SCG LTM.

The SN Security Key IE included in the SN Addition Request message should be ignored if the procedure is triggered for the LTM.

It was also reached an WA with an FFS:

WA: Different candidate PSCells in the same SN can have different Rel-19 set IDs. FFS on the solutions. Try to reuse the same solution for inter-CU LTM.

There are some possible ways to assign the R19 Set ID: (quoted from SOD in R3-253787)

- MN allocates the Rel-19 set IDs for candidate PSCell

- Source SN allocates the Rel-19 set IDs for candidate PSCell

- MN sends Rel-19 set IDs to the S-SN and C-SN via SN modification message (sync-up info)

The simple way is for the MN to allocate the R19 Set ID for the candidate PSCell and send it to the S-SN and C-SN via the SN Modification Request message. This will then guarantee no duplicated assigned the same R19 Set ID unintentionally from different candidate SN.

Proposal 1: for the assignment of the R19 Set ID (ltm-NoSecurityChangeID), it is proposed the MN to allocate the R19 Set IDs or range for the candidate PSCell and send it to the S-SN and C-SN via the SN Modification Request message, the C-SN assign R19 Set ID(s) to its own candidate cells and feedback via SN Addition Modification Acknowledge message. This will then guarantee no duplicated assigned the same R19 Set ID unintentionally from different candidate SN.

2.2	Access Success

Last RAN3#128 meeting discussed the situation that after the UE has successfully accessed to the target SN, in order for the MN to inform the S-SN to at least stop sending DL Data to the UE, it has been agreed to use SN modification request message. But still with an FFS:

MN uses SN modification request message to notify the Source SN that UE has successfully accessed to the target SN. FFS whether Handover Success is used from the target-SN to the MN to notify that UE has successfully accessed to the target SN.

The FFS is because there are two ways:

- Option 1: Reuse HO Success (T-SN to MN) + SN modification request message (MN to S-SN)

- Option 2: Follow CPAC, reuse RRC Reconfiguration (UE to MN) + SN modification request message (MN to S-SN)

So the difference is whether to reuse HO Success (T-SN to MN) or reuse RRC ReconfigurationComplete (UE to MN) for the MN to trigger sending the SN Modification Request message to S-SN. Both are workable. Considering the RRCReconfigurationComplete message will always be there after the UE has successfully accessed to the target SN, then the option 2: reuse RRCReconfigurationComplete message(UE to MN) is enough.

The use of SN Modification Request message may need an indicator to let the S-SN to stop sending DL Data to the UE.

Proposal 2: In order for the MN to inform the S-SN to at least stop sending DL Data to the UE, the MN after receiving Cell Switch Notification from S-SN and later receiving RRCReconfigurationComplete message from UE, the MN send SN Modification Request message with an indicator to the S-SN, the S-SN then stop sending DL data to the UE.

2.3	LTM modification/cancel

Last RAN3#128 meeting discussed whether to have LTM modification and LTM cancel from source SN to candidate SN (via MN) and from candidate SN to source SN (via MN). FFS remain as “FFS on whether LTM modification/cancel related procedures are needed.”

We have an observation that it is needed for at least the C-SN to trigger the cancelation of the resource of the already prepared PSCell for the LTM e.g. due to resource limitation in C-SN. This will be similar to the Rel-18 S-CPAC case. The C-SN can send the SN Release Required message to the MN, the MN then send SN Modification Request message (like the Conditional PSCell Change Cancel message in S-CPAC) to the S-SN to indicate the C-SN to be cancelled.

Whether the exiting Conditional PSCell Change Cancel procedure can be used for this Rel-19 inter-CU LTM with DC configuration? Since the content (Target S-NG-RAN node ID IE) can be reused, one way is to change the procedure name and message name. However, this is not allowed as other spec may refer to this subclause name so change of the procedure name and signalling message name is not appropriate. The other way is to introduce new procedure for example LTM Preparation Cancel procedure with LTM Preparation Cancel message, this may be too much. Then final way is to use the SN Modification Request message by introducing new IE e.g. Cancel Target S-NG-RAN node ID. This can be discussed further.

The S-SN may also need to cancel already prepared SN LTM e.g. due to any internal reason. The S-SN can send the SN Change Required message to the MN, then the MN send SN Release Request message to the C-SN to cancel the prepared resource for LTM.

If the MN want to cancel a C-SN the already prepared SN LTM, then can send SN Release Request to the C-SN. Upon reception of SN Release Request Ack from the C-SN, the MN can then send SN Modification Request to the S-SN to indicate the cancelled C-SN.

It can be discussed whether partially cancellation of prepared PScells is applicable. We think that similar to the S-CPAC, in order to simplify the procedure, it is better to limit to cancel all the prepared PScells of a SN. For example, if a C-SN trigger to cancel the prepared LTM, then all the prepared PScells in that C-SN are cancelled.

Proposal 3: it is proposed that:

- if the C-SN want to cancel the prepared LTM, then send the SN Release Required message to the MN, the MN then send SN Modification Request message (like the Conditional PSCell Change Cancel message in S-CPAC) to the S-SN to indicate the C-SN to be cancelled.

- if the S-SN want to cancel the prepared LTM of a C-SN, then can send the SN Change Required message to the MN, then the MN send SN Release Request message to the candidate C-SN to cancel the prepared resource for LTM.

- If the MN want to cancel a C-SN, then can send SN Release Request to the C-SN. Upon reception of SN Release Request Ack from the C-SN, the MN can then send SN Modification Request to the S-SN to indicate the cancelled C-SN.

3. Conclusion and proposals

Proposal 1: for the assignment of the R19 Set ID (ltm-NoSecurityChangeID), it is proposed the MN to allocate the R19 Set IDs or range for the candidate PSCell and send it to the S-SN and C-SN via the SN Modification Request message, the C-SN assign R19 Set ID(s) to its own candidate cells and feedback via SN Addition Modification Acknowledge message. This will then guarantee no duplicated assigned the same R19 Set ID unintentionally from different candidate SN.

Proposal 2: In order for the MN to inform the S-SN to at least stop sending DL Data to the UE, the MN after receiving Cell Switch Notification from S-SN and later receiving RRCReconfigurationComplete message from UE, the MN send SN Modification Request message with an indicator to the S-SN, the S-SN then stop sending DL data to the UE.

Proposal 3: it is proposed that:

- if the C-SN want to cancel the prepared LTM, then send the SN Release Required message to the MN, the MN then send SN Modification Request message (like the Conditional PSCell Change Cancel message in S-CPAC) to the S-SN to indicate the C-SN to be cancelled.

- if the S-SN want to cancel the prepared LTM of a C-SN, then can send the SN Change Required message to the MN, then the MN send SN Release Request message to the candidate C-SN to cancel the prepared resource for LTM.

- If the MN want to cancel a C-SN, then can send SN Release Request to the C-SN. Upon reception of SN Release Request Ack from the C-SN, the MN can then send SN Modification Request to the S-SN to indicate the cancelled C-SN.

[1] RP-250339 Revised WID: NR mobility enhancements Phase 4 (WID [NR_Mob_Ph4-Core])

[2] R3-253787 Summary of offline discussion on CB: # MobilityEhn_LTMDC

[3] R3-254006 -> R3-255060 (BL CR to 37.340) stage 2 for inter-CU LTM in NR-DC

[4] R3-253085 -> R3-255057 (BL CR to 37.483) Introducing Rel-19 Mobility Enhancement

[5] R3-254007 -> R3-255061 (BL CR to 38.300) Inter-CU LTM

[6]R3-254008 -> R3-255062 (BL CR to 38.401) on conditional intra-CU LTM

[7] R3-253088 -> R3-255058 (BL CR to 38.420) Support for Inter-CU LTM

[8] R3-254009 -> R3-255063 (BL CR to 38.423) Xn Support for Inter-CU LTM

[9] R3-253091 -> R3-255059 (BL CR to 38.470) Introducing Rel-19 Mobility Enhancement

[10] R3-254010 -> R3-255064 (BL CR to 38.423 for DC) Xn support for inter-CU LTM in DC

[11] R3-254011 -> R3-255065 (BL CR to 38.473) Inter-CU LTM and intra-CU conditional LTM

[12] R3-253009 (==R2-2503012) LS on RAN2 agreements for security key handling in inter-CU LTM

[13] R3-253850 (TP for TS 38.423) Data Forwarding for LTM DC scenario

[14] R3-253904 [TP to BLCR for TS 38.423] Inter-CU LTM in DC

[14a] R3-253922 (TP for 37.340) stage2 CR for Inter-CU LTM in DC

[15] R2-2504125 Running RRC CR for inter-CU and conditional LTM (RAN2#130)

[16] R2-250xxxx (For RAN2#131) Running RRC CR for inter-CU and conditional LTM

[17] Draft_RAN2_130_Meeting_Report_v2

(TP to TS 38.423 on Inter-CU LTM with DC) Inter-CU LTM on top of R3-254010

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

### 8.2.6	XN-U Address Indication

#### 8.2.6.1	General

For the retrieval of a UE context, the Xn-U Address Indication procedure is used to provide forwarding addresses from the new NG-RAN node to the old NG-RAN node for all PDU session resources successfully established at the new NG-RAN node for which forwarding was requested, and/or all MBS session resources successfully established at the new NG-RAN node for which forwarding was requested.

For MR-DC with 5GC, the Xn-U Address Indication procedure is used to provide data forwarding related information, and Xn-U bearer address information for completion of setup of SN terminated bearers from the M-NG-RAN node to the S-NG-RAN node as specified in TS 37.340 [8].

The procedure uses UE-associated signalling.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Xn-U Address Indication, successful operation for UE context retrieval

Figure 8.2.6.2-2: Xn-U Address Indication, successful operation for MR-DC with 5GC

UE Context Retrieval

The Xn-U Address Indication procedure is initiated by the new NG-RAN node. Sending the XN-U ADDRESS INDICATION message, the new NG-RAN node informs the old NG-RAN node of successfully established PDU Session Resource contexts, or MBS session resource contexts, or both, to which user data pending at the old NG-RAN node can be forwarded.

Skip unchanged part

If the XN-U ADDRESS INDICATION message includes the CPC Data Forwarding indicator IE set to "triggered", the S-NG-RAN node shall, if supported, consider that the XN-U ADDRESS INDICATION message concerns a Conditional PSCell Change, and act as specified in TS 37.340 [8]. If the CPC Data Forwarding indicator IE is present and value set to "early data transmission stop", the S-NG-RAN node shall, if supported and if already initiated, stop early data forwarding for the provided Data Forwarding Address information.

If the XN-U ADDRESS INDICATION message includes the LTM DC Data Forwarding Indicator IE, the S-NG-RAN node shall, if supported, consider that the XN-U ADDRESS INDICATION message concerns an Inter-CU MCG LTM, and act as specified in TS 37.340 [8].

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3	Procedures for Dual Connectivity

8.3.1	S-NG-RAN node Addition Preparation

8.3.1.1	General

The purpose of the S-NG-RAN node Addition Preparation procedure is to request the S-NG-RAN node to allocate resources for dual connectivity operation for a specific UE. Possible parallel requests are identified by the PCell ID when the initiating NG-RAN node UE AP IDs are the same.

The procedure uses UE-associated signalling.

8.3.1.2	Successful Operation

Figure 8.3.1.2-1: S-NG-RAN node Addition Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE ADDITION REQUEST message to the S-NG-RAN node.

Skip unchanged part

If the Source M-NG-RAN node ID IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node may use it to deduce direct data path availability with the source M-NG-RAN node, and if the direct data forwarding path is available, may include the Direct Forwarding Path Availability with source M-NG-RAN node IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

If the S-NODE ADDITION REQUEST message contains the IAB Authorization status IE, the S-NG-RAN node shall, if supported, store it and use it as defined in TS 38.401[2].

If the LTM Candidate PSCell Addition Information Request IE is included in the S-NODE ADDITION REQUEST message, the S-NG-RAN node shall, if supported, consider that the M-NG-RAN node has requested the LTM information for the UE in the S-NG-RAN node and include the LTM Candidate PSCell Addition Information Acknowledge IE in the S-NODE ADDITION REQUEST ACKNOWLEDGE message.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits at least one PDU session resource, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE ADDITION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NODE RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Interaction with the Activity Notification procedure

Upon receiving an S-NODE ADDITION REQUEST message containing the Desired Activity Notification Level IE, the S-NG-RAN node shall, if supported, use this information to decide whether to trigger subsequent Activation Notification procedures according to the requested notification level.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3.3	M-NG-RAN node initiated S-NG-RAN node Modification Preparation

8.3.3.1	General

This procedure is used to enable an M-NG-RAN node to request an S-NG-RAN node to either modify the UE context at the S-NG-RAN node or to query the current SCG configuration for supporting delta signalling in M-NG-RAN node initiated S-NG-RAN node change, or to provide the S-RLF-related information to the S-NG-RAN node.

The procedure uses UE-associated signalling.

8.3.3.2	Successful Operation

Figure 8.3.3.2-1: M-NG-RAN node initiated S-NG-RAN node Modification Preparation, successful operation

The M-NG-RAN node initiates the procedure by sending the S-NODE MODIFICATION REQUEST message to the S-NG-RAN node.

Skip unchanged part

If the S-NODE MODIFICATION REQUEST message contains the IAB Authorization status IE, the S-NG-RAN node shall, if supported, store it and use it as defined in TS 38.401[2].

If the LTM Candidate PSCell Information Update Request IE is included in the S-NODE MODIFICATION REQUEST message, the S-NG-RAN node shall, if supported, consider that the included information concerns the Inter-CU SCG LTM preparation of the other candidate S-NG-RAN node(s), and include the LTM Candidate PSCell Information Update Acknowledge IE in the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message.

Interactions with the S-NG-RAN node Reconfiguration Completion procedure:

If the S-NG-RAN node admits a modification of the UE context requiring the M-NG-RAN node to report about the success of the RRC connection reconfiguration procedure, the S-NG-RAN node shall start the timer TXnDCoverall when sending the S-NODE MODIFICATION REQUEST ACKNOWLEDGE message to the M-NG-RAN node except for a request for conditional configuration. The reception of the S-NG-RAN node RECONFIGURATION COMPLETE message shall stop the timer TXnDCoverall if TXnDCoverall is running.

Skip unchanged part

Interaction with the Path Switch Request procedure as specified in TS 38.413 [5]:

For a split PDU session, if the Integrity Protection Indication IE and/or the Confidentiality Protection Indication IE included in the PATH SWITCH REQUEST ACKNOWLEDGE message is set to "preferred", the M-NG-RAN node may keep the current UP integrity protection and ciphering policy.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

8.3.5	S-NG-RAN node initiated S-NG-RAN node Change

8.3.5.1	General

This procedure is used by the S-NG-RAN node to trigger the change of the S-NG-RAN node.

The procedure uses UE-associated signalling.

8.3.5.2	Successful Operation

Figure 8.3.5.2-1: S-NG-RAN node initiated S-NG-RAN node Change, successful operation.

The S-NG-RAN node initiates the procedure by sending the S-NODE CHANGE REQUIRED message to the M-NG-RAN node including the Target S-NG-RAN node ID IE. When the S-NG-RAN node sends the S-NODE CHANGE REQUIRED message, it shall start the timer TXnDCoverall.

Skip unchanged part

If the Source SN to Target SN QMC Information IE is contained in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, use it for QoE measurements handling, as specified in TS 37.340 [8].

If the LTM Candidate PSCell Change Information Required IE is included in the S-NODE CHANGE REQUIRED message, the M-NG-RAN node shall, if supported, consider that the S-NG-RAN node has requested the LTM information, as specified in TS 37.340 [8]. Accordingly, the M-NG-RAN node may include the LTM Candidate PSCell Change Information Confirm IE in the S-NODE CHANGE CONFIRM message (FFS).

Interaction with M-NG-RAN node initiated S-NG-RAN node Release:

If the M-NG-RAN node receives the S-NODE CHANGE REQUIRED message indicating releasing target S-NG-RAN node(s) and cancelling all prepared PSCells in the target S-NG-RAN node(s), the M-NG-RAN shall, if supported, trigger the M-NG-RAN node initiated S-NG-RAN node release procedure to the target S-NG-RAN node(s) and cancel all the prepared PSCells at the target S-NG-RAN node(s).

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.1.11	XN-U ADDRESS INDICATION

This message is either sent by the new NG-RAN node to transfer data forwarding information to the old NG-RAN node, or by the M-NG-RAN node to provide either data forwarding or Xn-U bearer address related information for SN terminated bearers to the S-NG-RAN node.

Direction: new NG-RAN node  old NG-RAN node, M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| New NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the new NG-RAN node or at the M-NG-RAN node. | YES | ignore |
| Old NG-RAN node UE XnAP ID reference | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the old NG-RAN node or at the S-NG-RAN node. | YES | ignore |
| Xn-U Address Information per PDU Session Resources List |  | 1 |  | This IE is ignored if the CHO DC Indicator IE is included and set to "coordination-only", or if the CPC Data Forwarding indicator IE is included and set to "coordination-only", or if the MBS Data Forwarding Indicator IE is included and set to "MBS-only". | YES | reject |
| Skip unchanged part |  |  |  |  |  |  |
| MBS Session Information Response List | O |  | 9.2.1.38 |  | YES | ignore |
| PDU Set based Handling Indicator | O |  | 9.2.3.206 |  | YES | ignore |
| LTM DC Data Forwarding Indicator | O |  | ENUMERATED (triggered, ...) | Indicating that the XN-U ADDRESS INDICATION message is for Inter-CU MCG LTM. | YES | reject |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2	Messages for Dual Connectivity Procedures

9.1.2.1	S-NODE ADDITION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to request the preparation of resources for dual connectivity operation for a specific UE.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| UE Security Capabilities | M |  | 9.2.3.49 |  | YES | reject |
| S-NG-RAN node Security Key | M |  | 9.2.3.51 | This IE is ignored if the S-CPAC Request Information IE is present in the Conditional PSCell Addition Information Request IE or the LTM Candidate PSCell Addition Information Request IE is present. | YES | reject |
| S-NG-RAN node UE Aggregate Maximum Bit Rate | M |  | UE Aggregate Maximum Bit Rate 9.2.3.17 | The UE Aggregate Maximum Bit Rate is split into M-NG-RAN node UE Aggregate Maximum Bit Rate and S-NG-RAN node UE Aggregate Maximum Bit Rate which are enforced by M-NG-RAN node and S-NG-RAN node respectively. | YES | reject |
| Skip unchanged part |  |  |  |  |  |  |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | ignore |
| LTM Candidate PSCell Addition Information Request | O |  |  |  | YES | reject |
| >Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >LTM Request Indication | M |  | ENUMERATED (request, ...) |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.1.xx8 |  | – |  |
| >Suggested LTM Candidate PSCell List | O |  | 9.2.3.xx3 |  | – |  |
| >SCG Reference Configuration Request | O |  | ENUMERATED (request, …) | Indicates that the reference configuration for LTM is requested. | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  | – |  |
| >LTM Security Configurations Information | O |  | 9.2.3.xx1 | Indicates the security configurations for LTM. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |

| Condition | Explanation |
| --- | --- |
| ifSNterminated | This IE shall be present if there is at least one PDU Session Resource Setup Info – SN terminated in the PDU Session Resources To Be Added List IE. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.2	S-NODE ADDITION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node about the S-NG-RAN node addition preparation.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Skip unchanged part |  |  |  |  |  |  |
| Direct Forwarding Path Availability with source M-NG-RAN node | O |  | ENUMERATED (direct path available, …) | Indicates direct forwarding path is available between the target S-NG-RAN node and source M-NG-RAN node. | YES | ignore |
| LTM Candidate PSCell Addition Information Acknowledge | O |  |  |  | YES | ignore |
| >LTM Candidate PSCell List | O |  | 9.2.3.xx4 |  | – |  |
| >LTM Security Configurations Information Response | O |  | 9.2.3.xxY | Indicates the security configurations response for LTM. | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.5	S-NODE MODIFICATION REQUEST

This message is sent by the M-NG-RAN node to the S-NG-RAN node to either request the preparation to modify S-NG-RAN node resources for a specific UE, or to query for the current SCG configuration, or to provide the S-RLF-related information to the S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Skip unchanged part |  |  |  |  |  |  |
| IAB Authorization Status | O |  | ENUMERATED (authorized, not authorized, …) | Indicates the IAB node´s authorization status. | YES | Ignore |
| LTM Candidate PSCell Information Update Request | O |  |  |  | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>LTM Candidate PSCell List | O |  | 9.2.3.xx4 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.1.xx8 |  | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  | – |  |
| SN LTM succeeded | O |  | ENUMERATED (true, …) |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.6	S-NODE MODIFICATION REQUEST ACKNOWLEDGE

This message is sent by the S-NG-RAN node to confirm the M-NG-RAN node’s request to modify the S-NG-RAN node resources for a specific UE.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| Skip unchanged part |  |  |  |  |  |  |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | Ignore |
| LTM Candidate PSCell Information Update Acknowledge | O |  |  |  | YES | ignore |
| >CSI Resource Configuration | O |  | 9.2.1.xx8 |  | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUSessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofPSCellCandidate | Maximum no. of PSCell candidates. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.1.2.11	S-NODE CHANGE REQUIRED

This message is sent by the S-NG-RAN node to the M-NG-RAN node to trigger the change of the S-NG-RAN node.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 | This IE shall be ignored if the Conditional PSCell Change Information Required IE is present or the LTM Candidate PSCell Change Information Required IE is present. | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| Skip unchanged part |  |  |  |  |  |  |
| Source SN to Target SN QMC Information | O |  | QMC Configuration Information 9.2.3.156 | This IE contains S-NG-RAN node-related QMC Configuration Information to be forwarded to the target S-NG-RAN node. | YES | ignore |
| LTM Candidate PSCell Change Information Required | O |  |  |  | YES | reject |
| >LTM Request Indication | M |  | ENUMERATED (request, Cancellation,...) |  | – |  |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Maximum Number of PSCells To Prepare | M |  | INTEGER (1..8, …) | Indicates the maximum number of PSCells that the target SN may prepare. | – |  |
| >>>S-NG-RAN node to M-NG-RAN node Container | M |  | OCTET STRING | Includes the CG-Config message as defined in subclause 11.2.2 of TS 38.331 [10]. | – |  |
| >>>Suggested LTM Candidate PSCell List | O |  | 9.2.3.xx3 |  | – |  |
| >CSI Resource Configuration | O |  | 9.2.1.xx8 |  | – |  |
| >LTM Configuration ID Mapping List | O |  | 9.2.1.xx5 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.1.2.12	S-NODE CHANGE CONFIRM

This message is sent by the M-NG-RAN node to inform the S-NG-RAN node that the preparation of the S-NG-RAN node initiated S-NG-RAN node change was successful.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | ignore |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | ignore |
| Skip unchanged part |  |  |  |  |  |  |
| M-NG-RAN node to S-NG-RAN node Container | O |  | OCTET STRING | Includes the RRCReconfigurationComplete message as defined in subclause 6.2.2 of TS 38.331 [10]. | YES | ignore |
| LTM Candidate PSCell Change Information Confirm | O |  |  | FFS whether this IE is needed | YES | ignore |
| >Multiple Target S-NG-RAN Node List |  | 1 |  |  | – |  |
| >>Multiple Target S-NG-RAN Node Item |  | 1 .. <maxnoofTargetSNs> |  |  | – |  |
| >>>Target S-NG-RAN node ID | M |  | Global NG-RAN Node ID 9.2.2.3 |  | – |  |
| >>>Candidate PSCell List | M |  | 9.2.3.xx4 |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPDUsessions | Maximum no. of PDU sessions. Value is 256 |
| maxnoofTargetSNs | Maximum no. of the target S-NG-RAN nodes. Value is 8 |
| maxnoofPSCellCandidate | Maximum no, of PSCell candidate. Value is 8 |
| maxnoofTargetSNsMinusOne | Maximum no. of the target S-NG-RAN nodes minus 1. Value is 7 |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

9.2.3.xx1	LTM Security Configurations Information

This IE is used to apply security in the S-NG-RAN node for inter-SN SCG LTM as defined in TS 37.340 [8].

Editor Note: FFS how to include Rel-19 set ID.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| LTM Security Configurations Information |  | 1 |  |  |
| >LTM Security Configurations Item |  | 1 .. <maxnoofSecurityConfigurations> |  |  |
| >>S-NG-RAN node Security Key | M |  | BIT STRING (SIZE(256)) | The S-KSN which is provided by the M-NG-RAN node, see TS 33.501 [28]. |
| >>SK-counter | M |  | INTEGER (0..65535) |  |
| >>Security Change ID range can be assigned |  | 0..1 |  |  |
| >>>Low bound of ID | O |  | INTEGER (1.. maxNrofLTM-Configs-plus1) |  |
| >>>Upper bound of ID | O |  | INTEGER (1.. maxNrofLTM-Configs-plus1) |  |
| Security Change ID Candidate Cell Information List |  | 0..1 |  |  |
| >Security Change Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |
| >>Cell ID | M |  | NR CGI 9.2.2.7 |  |
| >>Security Change ID Candidate Cell Configuration | M |  | OCTET STRING | Includes the ltm-NoSecurityChangeID IE as defined in TS 38.331 [10], for the LTM candidate cell identified by the Cell ID IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofSecurityConfigurations | Maximum no. of LTM security configurations. Value is 8. |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |
| maxNrofLTM-Configs-plus1 | Maximum no. of Cells +1 configured for LTM allowed towards one UE, the maximum value is 9. |

9.2.3.xxY	LTM Security Configurations Information Response

This IE is used to apply security in the S-NG-RAN node for inter-SN SCG LTM as defined in TS 37.340 [8].

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| LTM Security Configurations Information Response |  | 1 |  |  |
| >Security Change Candidate Cell Information Item |  | 1.. <maxnoofLTMCells> |  |  |
| >>Cell ID | M |  | NR CGI 9.2.2.7 |  |
| >>Assigned Security Change ID | M |  | INTEGER (1.. maxNrofLTM-Configs-plus1) |  |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

    - 9.2.3.xx2	Layer 1 Configuration

This IE contains information related to the L1 reference signal configuration.

Editor Note: FFS on the choice type.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE L1 Configuration | M |  |  |  |
| >SSB |  |  |  |  |
| >>SSB Configuration | M |  | OCTET STRING | Contains the ltm-SSB-Config-r18 as defined in TS 38.331 [10]. FFS the coding of this IE needs further check. |
| >CSI-RS |  |  |  |  |
| >>CSI-RS Resource Configuration | M |  | OCTET STRING | Contains the ltm-NZP-CSI-RS-ResourceConfigToAddModList-r19 as defined in TS 38.331 [10]. |
| >>CSI-RS Resource Set Configuration | M |  | OCTET STRING | Contains the ltm-NZP-CSI-RS-ResourceSetToAddModList-r19 as defined in TS 38.331 [10]. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

9.2.3.xx3	Suggested LTM Candidate PSCell List

This IE provides suggested LTM candidate PSCell list.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Suggested LTM Candidate PSCell List |  | 1 |  |  | – |  |
| >Suggested Candidate PSCell Item |  | 1 .. < maxnoofLTMCells> |  |  | – |  |
| >>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>Early Sync Information Request | O |  | 9.2.1.xx3 |  | – |  |
| >>Request for CSI-RS Resource Configuration | O |  | ENUMERATED (true, …) |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

9.2.3.xx4	LTM Candidate PSCell List

This IE provides LTM candidate PSCell list.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| LTM Candidate PSCell List |  | 1 |  |  | – |  |
| >Candidate PSCell Item |  | 1 .. < maxnoofLTMCells> |  |  | – |  |
| >>PSCell ID | M |  | NR CGI 9.2.2.7 |  | – |  |
| >>TCI States Configurations List | O |  | OCTET STRING | Includes the LTM-TCI-Info IE, as defined in TS 38.331 [10]. | – |  |
| >>Early UL Sync Configuration | O |  | 9.2.1.xx6 | Early UL sync configurations for the UE. | – |  |
| >>Early UL Sync Configuration for SUL | O |  | 9.2.1.xx6 | Early UL sync configurations for the UE for SUL carrier. | – |  |
| >>Layer 1 Configuration | O |  | 9.2.3.xx2 |  | – |  |
| >>Complete Candidate Configuration Indicator | O |  | ENUMERATED (complete, ...) |  | – |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofLTMCells | Maximum no. of Cells configured for LTM allowed towards one UE, the maximum value is 8. |

<<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

(TP to TS 37.340 on Inter-CU LTM with DC) Inter-CU LTM on top of 37.340v18.6.0)

<<<<<<<<<<<<<<<<<<<< Start of Changes >>>>>>>>>>>>>>>>>>>>

10.4.2	MR-DC with 5GC

The SN Release procedure may be initiated either by the MN or by the SN and is used to initiate the release of the UE context and relevant resources at the SN. The recipient node of this request can reject it, e.g., if an SN change procedure is triggered by the SN.

In case of CPA, inter-SN CPC or subsequent CPAC, this procedure may be initiated either by the MN or the candidate SN, and it is used to cancel all the prepared PSCells at the candidate SN and initiate the release of related UE context at the candidate SN.

MN initiated SN Release

Figure 10.4.2-1: SN release procedure - MN initiated

Figure 10.4.2-1 shows an example signalling flow for the MN initiated SN Release procedure.

1.	The MN initiates the procedure by sending the SN Release Request message.

2.	The SN confirms SN Release by sending the SN Release Request Acknowledge message. If appropriate, the SN may reject SN Release, e.g., if the SN change procedure is triggered by the SN.

NOTE 00:	If inter-SN CPC is configured, upon reception of the SN Release Request Acknowledge message from the source SN, the MN cancels all CPC with the target candidate SN(s).

NOTE 00A:	If CPA or inter-SN CPC is configured, upon reception of the SN Release Request Acknowledge message from a candidate SN, the MN cancels all CPAC configurations in the candidate SN. In case of SN initiated inter-SN CPC, the MN triggers the Conditional PSCell Change Cancel procedure towards the source SN to inform it of the cancellation of CPAC in the candidate SN.

NOTE 00a:	If subsequent CPAC is configured, upon reception of the SN Release Request Acknowledge message from the source SN, the MN may retain the subsequent CPAC configuration or cancel the subsequent CPAC configuration. If the MN maintains the subsequent CPAC configuration, it should provide suitable execution conditions for the evaluation of the subsequent CPAC.

NOTE 00b:	If subsequent CPAC is configured, upon reception of the SN Release Request Acknowledge message from a candidate SN, the MN cancels all subsequent CPAC with the candidate SN. If applicable, the MN triggers the Conditional PSCell Change Cancel procedure towards the source SN, and triggers the MN initiated SN Modification procedure towards the other candidate SN(s), to inform them of the cancellation of the candidate SN.

NOTE 00x:	If LTM is configured, upon reception of the SN Release Request Acknowledge message from a candidate SN, the MN cancels all LTM configuration with the candidate SN. If applicable, the MN triggers the MN Initiated SN Modification procedure towards the source SN, and triggers the MN initiated SN Modification procedure towards the other candidate SN(s), to inform them of the cancellation of the candidate SN.

2a.	When applicable, the MN provides forwarding address information to the SN.

NOTE 0:	The MN may send the Xn-U Address Indication message to provide forwarding address information before step 2.

3/4.	If required, the MN indicates in the MN RRC reconfiguration message towards the UE that the UE shall release the entire SCG configuration. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

NOTE 1:	If data forwarding is applied, timely coordination between steps 1 and 2 may minimize gaps in service provision, this is however regarded to be an implementation matter.

5.	If PDCP termination point is changed to the MN for bearers using RLC AM, the SN sends the SN Status Transfer message.

6.	Data forwarding from the SN to the MN may start.

7.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 1a:	If data forwarding is applied, the order the SN sends the Secondary RAT Data Usage Report message and starts data forwarding with MN is not defined i.e., step 7 can take place before step 6. The SN does not need to wait for the end of data forwarding to send the Secondary RAT Data Usage Report message.

8.	If applicable, the PDU Session path update procedure is initiated.

9.	Upon reception of the UE Context Release message, the SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.

SN initiated SN Release

Figure 10.4.2-2: SN release procedure - SN initiated

Figure 10.4.2-2 shows an example signalling flow for the SN initiated SN Release procedure.

1.	The SN initiates the procedure by sending the SN Release Required message which may contain inter-node message to support delta configuration.

2.	If data forwarding is requested, the MN provides data forwarding addresses to the SN in the SN Release Confirm message. The SN may start data forwarding and stop providing user data to the UE as early as it receives the SN Release Confirm message.

NOTE 1b:	If inter-SN CPC is configured, upon reception of the SN Release Required message from the source SN, the MN cancels all CPC with the target candidate SN(s).

NOTE 1b1:	If CPA or inter-SN CPC is configured, upon reception of the SN Release Required message from a candidate SN, the MN cancels all CPAC configurations in the candidate SN. In case of SN initiated inter-SN CPC, the MN triggers the Conditional PSCell Change Cancel procedure towards the source SN to inform it of the cancellation of CPAC in the candidate SN.

NOTE 1c:	If subsequent CPAC is configured, upon reception of the SN Release Required message from the source SN, the MN may retain the subsequent CPAC configuration or cancel the subsequent CPAC configuration. If the MN maintains the subsequent CPAC configuration, it should provide suitable execution conditions for the evaluation of the subsequent CPAC.

NOTE 1d:	If subsequent CPAC is configured, upon reception of the SN Release Required message from a candidate SN, the MN cancels all subsequent CPAC with the candidate SN. If applicable, the MN triggers the Conditional PSCell Change Cancel procedure towards the source SN, and triggers the MN initiated SN Modification procedure towards the other candidate SN(s), to inform them of the cancellation of the candidate SN.

NOTE 1x:	If LTM is configured, upon reception of the SN Release Required message from a candidate SN, the MN cancels all LTM with the candidate SN. If applicable, the MN triggers the MN initiated SN Modification procedure towards the source SN, and triggers the MN initiated SN Modification procedure towards the other candidate SN(s), to inform them of the cancellation of the candidate SN.

3/4.	If required, the MN indicates in the MN RRC reconfiguration message towards the UE that the UE shall release the entire SCG configuration. In case the UE is unable to comply with (part of) the configuration included in the MN RRC reconfiguration message, it performs the reconfiguration failure procedure.

NOTE 2:	If data forwarding is applied, timely coordination between steps 2 and 3 may minimize gaps in service provision. This is however regarded to be an implementation matter.

5.	If PDCP termination point is changed to the MN for bearers using RLC AM, the SN sends the SN Status Transfer message.

6.	Data forwarding from the SN to the MN may start.

7.	The SN sends the Secondary RAT Data Usage Report message to the MN and includes the data volumes delivered to and received from the UE as described in clause 10.11.2.

NOTE 3:	If data forwarding is applied, the order the SN sends the Secondary RAT Data Usage Report message and starts data forwarding with MN is not defined i.e., step 7 can take place before step 6. The SN does not need to wait for the end of data forwarding to send the Secondary RAT Data Usage Report message.

8.	If applicable, the PDU Session path update procedure is initiated.

9.	Upon reception of the UE Context Release message, the SN releases radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue.