3GPP TSG-RAN WG3 Meeting #129	R3-255211

Bangalore, India, 25th – 29th May 2025

Agenda Item:		17.3

Source:		NEC

Title:	TP to BLCR TS38.423 on support on-demand SIB1

Document for:		Discussion and decision

- Introduction

In this contribution, we provide TP to BLCR TS38.423 on support on-demand SIB1 based on the proposals in R3-255210.

- TP to BLCR TS38.423

<Start of change>

### 8.x.x	OD-SIB1 Configuration Provision

#### 8.x.x.1	General

The purpose of the OD-SIB1 Configuration Provision procedure is to enable an NG-RAN node1 to provide OD-SIB1 configuration information to NG-RAN node2 and request NG-RAN node2 to transmit OD-SIB1 configuration information.

The procedure is also used to enable an NG-RAN node1 to request NG-RAN node2 to stop transmission of OD-SIB1 configuration.

The procedure uses non-UE -associated signaling.

#### 8.x.x.2	Successful Operation

Figure 8.x.x.2-1: OD-SIB1 Configuration Provision procedure, successful operation

If the start is included in the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, broadcast OD-SIB1 Configuration in SIBx (SIB naming FFS) and reply with the OD-SIB1 CONFIGURATION PROVISION RESPONSE message. The NG-RAN node2 stores the OD-SIB1 configuration information.

If the Assisted Cell is included in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall, if supported, broadcast the OD-SIB1 Configuration information in the cell indicated by the assisted cell.

If the “stop” is included in the Request Type IE in the OD-SIB1 CONFIGURATION PROVISION REQUEST message, the NG-RAN node2 shall stop broadcasting OD-SIB1 Configuration for the given NG-Cell ID in SIBx (SIB naming FFS). NG-RAN node2 removes the received OD-SIB1 Configuration.

#### 8.x.x.3	Unsuccessful Operation

Figure 8.x.x.3-1: OD-SIB1 Configuration Provision procedure, unsuccessful operation

If the NG-RAN node2 cannot broadcast OD-SIB1 Configuration in SIBx (Naming FFS), or a failure occurs during OD-SIB1 Configuration Provision, the NG-RAN node2 shall send OD-SIB1 CONFIGURATION PROVISION FAILURE message. The message shall contain the Cause IE with an appropriate value.

#### 8.x.x.4	Abnormal Conditions

Void.

### 8.x.y	OD-SIB1 Configuration Broadcast Stop Indication

#### 8.x.y.1	General

This procedure is initiated by an NG-RAN node2 to inform a neighbouring NG-RAN node1 about a previously admitted OD-SIB1 configuration provision being stopped. The procedure uses non-UE associated signalling.

#### 8.x.y.2	Successful Operation

Figure 8.x.y.2-1: OD-SIB1 Configuration Broadcast Stop Indication procedure, successful operation

The NG-RAN node2 shall indicate that a previously admitted OD-SIB1 configuration provision in the OD-SIB1 CONFIGURATION BROADCAST STOP INDICATION message is being stopped.

#### 8.x.y.3	Unsuccessful Operation

Not applicable.

#### 8.x.y.4	Abnormal Conditions

Void.

*************** Next change ***************

#### 9.x.x.x	OD-SIB1 CONFIGURATION PROVISION REQUEST

This message is sent by the NG-RAN node1 to the peer NG-RAN node2 to request or stop provisioning of an OD-SIB1 configuration in one assisted cell.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| CHOICE Request Type | M |  |  |  | YES | reject |
| >Start |  |  |  |  |  |  |
| >>OD-SIB1 configuration | M |  | OCTET STRING | Includes the OD-SIB1-CellConfig-r19 IE as specified in TS 38.331 [10] | – |  |
| >>NR CGI at NG-RAN node1 | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the OD-SIB1 configuration | – |  |
| >>NR CGI at NG-RAN node2 | O |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node2 requested to broadcast the OD-SIB1 configuration | – |  |
| >Stop |  |  |  |  |  |  |
| >> NR CGI at NG-RAN node1 | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the OD-SIB1 configuration | – |  |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.y	OD-SIB1 CONFIGURATION PROVISION RESPONSE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration will be provided by the NG-RAN node2.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| NR CGI at NG-RAN node1 | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the OD-SIB1 configuration | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.z	OD-SIB1 CONFIGURATION PROVISION FAILURE

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to indicate that the OD-SIB1 configuration cannot be provided in the requested cell of the NG-RAN node2.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| Cause | M |  | 9.2.3.2 |  | YES | ignore |
| NR CGI at NG-RAN node1 | M |  | NR CGI 9.2.2.7 | Indicates the NR cell of the NG-RAN node1 supports the OD-SIB1 configuration | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.3.3 |  | YES | ignore |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

#### 9.1.3.t	OD-SIB1 CONFIGURATION BROADCAST STOP INDICATION

This message is sent by an NG-RAN node2 to a peer NG-RAN node1 to report that an admitted OD-SIB1 configuration provision is being stopped.

Direction: NG-RAN node2  NG-RAN node1.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| NR CGI at NG-RAN node2 | O |  |  | RAN node1 supports the OD-SIB1 configuration |  |  |
| Provision Status | M |  | ENUMERATED (stopped, …) | Indicates the status of the OD-SIB1 configuration transmission | YES | reject |
| Interface Instance Indication | O |  | 9.2.2.39 |  | YES | reject |

<End of change>