3GPP TSG-RAN WG3 Meeting #129	R3-255235

Bengaluru, India, 25th - 29th Aug, 2025

Title: 	Emergency call back and paging

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# 1. Introduction

The last SA2-169 meeting sent an subsequent reply LS on the emergency call back and paging as follows [1].

1. Overall Description: SA WG2 thanks RAN3 for the LS on emergency call back and paging. SA2 could not reach consensus to agree to update our specifications. SA2 would request some clarifications from RAN3. For normal sessions, it is possible for SMF to keep the PDU session while the NG-RAN releases the RRC connection, e.g. AN release due to User Inactivity timer expired. However there is different understanding in SA2 regarding the case that emergency PDU session exists. Hence SA2 have the following questions: Q1： Is it possible to have different behaviour in NG-RAN node depending on the presence of the emergency PDU session to decide when to release RRC connection, compared to when there is no emergency PDU session? Q2: If yes, when the emergency PDU session exist, whether it is required to configure the NG-RAN to not release RRC connection until the emergency PDU session is released by the SMF? 2. Actions: To RAN3: ACTION: SA2 kindly requests RAN3 to answer the above questions.

RAN2-130 meeting also made the following agreement and provided the following LS [2].

1. Overall Description: RAN2 thanks SA2 for the reply LS on emergency service call back and paging. RAN2 further discussed this issue and confirm the feasibility of the solution mentioned in reply LS, that RAN can be aware of emergency PDU session based on the reserved ARP value. In addition to the endorsed solution from RAN3 (i.e. the gNB should not configure eDRX for RAN paging when releasing the UE to RRC_INACTIVE state), RAN2 also agreed that no new UE behavior will be specified for this case. It is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency callback. 2. Actions: To SA2, RAN3 group: ACTION: RAN2 kindly requests SA2 and RAN3 to take the above information into account.

This contribution provides our views to the two questions contained in the SA2 LS.

# 2. Discussion

## 2.1 Recap RAN3 and RAN2 discussion

The previous RAN3-127bis meeting already discussed the old reply LS from SA2 in [3], and made the following agreements.

- It can be left unspecified whether NG-RAN can release a UE with an emergency PDU session to RRC_INACTIVE state.

Also, regarding the open issues, RAN3 has endorsed the stage 2 CR containing the following note, and sent the LS to RAN2/SA2 in [4].

NOTE: If emergency PDU session resources are established, the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state. The gNB recognizes the emergency PDU session resources based on special ARP value of a QoS flow as specified in TS 23.501 [3].

- Observation 1:  RAN3 already agreed that “It can be left unspecified whether NG-RAN can release a UE with an emergency PDU session to RRC_INACTIVE state.”.

- Observation 2:  RAN3 already agreed that the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state if emergency PDU session resources are re-established.

Based on the LS in [3, 4], RAN2-130 meeting made conclusions and agreed the LS in [2]. It can be observed that from the RAN2 LS, it is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency call back, which is aligned with RAN3 conclusion.

- Observation 3:  RAN2 already concluded that it is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency callback, which is aligned with RAN3 conclusion.

## 2.2 Answers to the incoming SA2 LS

Regarding the 1st question from SA2, RAN3 actually already provided the response – it is left unspecified whether the NG-RAN can release or keep the UE in case of emergency PDU session. This was also confirmed by the RAN2 response. Hence the response to SA2 would be that it is left unspecified whether to have different NG-RAN behaviour to release/suspend the RRC connection when there is emergency PDU session. In short, it is up to NG-RAN implementation.

RAN3 to reply that for Q1, it is left unspecified whether to have different NG-RAN behaviour to release/suspend the RRC connection when there is emergency PDU session, i.e., it is up to NG-RAN implementation.

On the 2nd question, we understand this is related to the issue whether the idle UE can have the emergency PDU session or not, which may impact on the eDRX or the PEI monitoring. Given the response to Q1, there is no such requirement for the NG-RAN to delay the release of RRC connection, until the CN release the emergency PDU session.

RAN3 to reply that for Q2, there is no such requirements given the RAN3 reply to Q1.

# 3. Conclusion

Based on the discussion in this paper, we make the following observations and proposals.

- Recap RAN3 and RAN2 discussion

- Observation 1:  RAN3 already agreed that “It can be left unspecified whether NG-RAN can release a UE with an emergency PDU session to RRC_INACTIVE state.”.

- Observation 2:  RAN3 already agreed that the gNB should not configure the Extended DRX when sending the UE to RRC_INACTIVE state if emergency PDU session resources are re-established.

- Observation 3:  RAN2 already concluded that it is up to the gNB implementation whether to release the UE to RRC_IDLE or RRC_INACTIVE or keep the UE in RRC_CONNECTED state for fast emergency callback, which is aligned with RAN3 conclusion.

- Analysis of the LS

- RAN3 to reply that for Q1, it is left unspecified whether to have different NG-RAN behaviour to release/suspend the RRC connection when there is emergency PDU session, i.e., it is up to NG-RAN implementation.

RAN3 to reply that for Q2, there is no such requirements given the RAN3 reply to Q1.

The draft reply LS is provided in [5].

# 4. Reference

- S2-2505938 LS on energy saving indication from CN to RAN, SA2

- R2-2504847 Reply LS on emergency call back and paging, RAN2

- R3-251521 Reply LS on emergency call back and paging, SA2

- R3-252402 Reply LS on emergency call back and paging, RAN3

- R3-255236 [draft]Reply LS on emergency call back and paging, Huawei