3GPP TSG-RAN WG3 Meeting #129	R3-255237

Bengaluru, India, 25th - 29th Aug, 2025

Title: 	(TP to BLCR for TS 38.413, TS 38.423 and TS 38.300) Discussion on Low-power wake-up signal and receiver for NR

Source: 	Huawei

Agenda item:	18.2

Document Type:	Other

# 1. Introduction

In this contribution, we intend to finalize the RAN3 related issues on the procedure and configuration for RRC_IDLE/INACTIVE UEs.

# 2. Discussion

## 2.1 LP-WUS Subgrouping

- During recent RAN1 meetings it was confirmed that the maximum number of subgroups in one PO for LP-WUS will be 31.

Agreement at RAN1#120 Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31. Agreement at RAN1#120bis For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is - 7 for the case where 4 POs are mapped to one LO - 15 for the case where 2 POs are mapped to one LO

With the above agreements, RAN3-127bis meeting agreed that the maximum number of LP-WUS CN-based subgrouping is 31.

- The maximum number of subgroups is decided to 31.

Regarding the exact range of LP-WUS CN Subgroup ID (either [0, 30] or [1, 31]), from the formula of the UE subgrouping ID indicated in [2] below:

SubgroupID = (floor(UE_ID/(N*Ns*Np)) mod lp-SubgroupsNumForUEID) + (lp-SubgroupsNumPerPO – lp-SubgroupsNumForUEID), where: N: number of total paging frames in T, which is the DRX cycle of RRC_IDLE state as specified in clause 7.1 Ns: number of paging occasions for a PF Np is the number of subgroupsNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1 UE_ID: 5G-S-TMSI mod 1048576 lp-SubgroupsNumForUEID and lp-SubgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively Editor’s NOTE: The detailed parameters for LP-WUS subgrouping will be aligned with RRC specification.

It can be deduced that the UE subgroup ID shall always start from 0 if there is no CN subgroup ID assigned to the UE. Consequently, it is more appropriate to define the ​​LP-WUS CN Subgroup ID​​ range as ​​0 to 30​​ in the LP-WUSPS Assistance Information via NGAP/XnAP/F1AP. In addition, the procedure texts should be added for the LP-WUSPS Assistance Information included in the Core Network Assistance Information for RRC INACTIVE IE in NGAP.

RAN3 to set the range of LP-WUS CN Subgroup ID from 0 to 30 in LP-WUSPS Assistance Information in NGAP/XnAP/F1AP.

Add procedure texts for the LP-WUSPS Assistance Information included in the Core Network Assistance Information for RRC INACTIVE IE in NGAP.

## 2.2 The RRM relaxation/offloading configuration

In the latest RAN2 meeting, it was agreed that the R19 RRM measurement relaxation/offloading are carried in SIB2 [3].

Agreement at RAN2#130 - RRM relaxation / offloading configuration is provided in SIB2.

As observed from the RAN2 discussion, the LP-WUS related RRM measurement configuration includes the following:

- relaxedMeasurementForServingAndNeighboringCell : to allow relaxation of serving cell and neighboring cell RRM measurement requirements for cell reselection

- offloadMeasurementForServingCell : to allow offloading of serving cell RRM measurement on MR to serving cell RRM measurement on LP-WUR.

For the CU/DU split case, the gNB-CU shall be responsible to generate the LP-WUS related RRM relaxation/offloading configuration in SIB2. Hence there is no further RAN3 impact.

There is no RAN3 impact on the RRM relaxation/offloading configuration.

## 2.3 LP-WUS and eDRX

As indicated in [1], the LP-WUS could be supported together with eDRX. From RAN3 perspective, the existing procedures could apply. Specifically,

- for CN paging, the CN can provide both the LP-WUSPS Assistance Information IE and NR Paging eDRX Information IE in the NG Paging message;

- for RAN paging, the CN can provide both LP-WUSPS Assistance Information IE and NR Paging eDRX Information IE in the Core Network Assistance Information for RRC INACTIVE IE, then NG-RAN node can decide and propagate the RAN eDRX accordingly, and perform LP-WUS paging during the RAN eDRX cycle and PTW window (if the eDRX cycle is greater than 10.24seconds).

RAN3 can consider to have a stage 2 TP to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.

RAN3 can consider to have a stage 2 TP to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.  No further RAN3 signalling impact is foreseen.

## 2.4 Last used cell only

The agreed XnAP BLCR has introduced the LP-WUS subgrouping capability related information into the XnAP RAN paging message. Given that RAN2 did not touch or make any agreements on the last used cell only. Hence there is no further issue from RAN3 perspective (removing the editor’s note in XnAP BLCR).

No further issue on the last used cell from RAN3 perspective (removing the editor’s note in XnAP BLCR).

## 2.5 LP-WUS enabling control

RAN2 is considering whether to support the per-UE enabling/disabling of LP-WUS monitoring in IDLE/INACTVE mode, in order to further reduce false paging probability or to disable LP-WUS monitoring for UEs with delay-sensitive services as follows.

Agreement at RAN2#130 - RAN2 aim at supporting enabling/disabling LP-WUS monitoring in IDLE/INACTVE per UE, if the solution can be concluded in the August meeting.

This allows the network to decides whether a LP-WUS capable UE is allowed to use the LP-WUS functionality based on the conditions in the network (e.g., radio resources) and/or the characteristics of the UEs (e.g., paging probability, shorter DRX cycle, etc.).

According to the companies’ contributions in RAN2, there are two major views to enable/disable LP-WUS monitoring for one UE.

- via NAS signalling

- via RRC signalling

For NAS based solution, the CN can use NAS signalling to indicate whether a specific UE can use LP-WUS functionality, which is similar to Rel-17 CN-assigned subgrouping method for PEI. Figure 1 illustrates the procedure to disable or enable LP-WUS functionality using NAS signalling.

During either initial registration or the registration triggered by mobility, if the UE includes Requested LP-WUS and/or Requested CN Subgroup ID in the Registration Request message, the CN should be able to explicitly indicate to the UE to disable LP-WUS functionality via Registration Accept message. If CN disables LP-WUS functionality to the UE, it shall also indicate to RAN the disabling of LP-WUS functionality for the UE in the Initial Context Setup Request message.

If the conditions later become favourable for re-enabling LP-WUS for the UE, CN can re-enable it in the subsequent registration procedure via REGISTRATION ACCEPT message without any explicit indication. Naturally, it will also inform the RAN of the re-enablement.

Observation 1: For the LP-WUS enabling control via NAS signalling, the potential RAN3 impact is the introduction of a new indication e.g., in the Initial Context Setup Request message in the NGAP to enable or disable the LP-WUS monitoring of the UE.

Figure 1: Disabling/Enabling of LP-WUS with NAS signalling

Another possible direction is that the RAN can decide to enable or disable LP-WUS for the UE due to some conditions of the network side. This requires the gNodeB to inform CN that LP-WUS has been disabled or enabled for the UE. When there is paging for the UE, the CN shall further inform the involved gNodeBs about the LP-WUS disablement or enablement - either in the Paging message for UEs in the RRC_IDLE state, or in the Core Network Assistance Information for RRC_INACTIVE for UEs in the RRC_INACTIVE state.

So far, there was no consensus on the above solutions, and RAN3 shall monitor the RAN2 discussion at the parallel meeting.

RAN3 to monitor the LP-WUS enabling control discussion in RAN2 then act upon accordingly.

# 3. Conclusion

Based on the discussion in this paper, we make the following proposals.

- LP-WUS Subgrouping

- RAN3 to set the range of LP-WUS CN Subgroup ID from 0 to 30 in LP-WUSPS Assistance Information in NGAP/XnAP/F1AP.

- Add procedure texts for the LP-WUSPS Assistance Information included in the Core Network Assistance Information for RRC INACTIVE IE in NGAP.

- The RRM relaxation/offloading configuration

- There is no RAN3 impact on the RRM relaxation/offloading configuration.

- LP-WUS and eDRX

RAN3 can consider to have a stage 2 TP to capture that the eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.  No further RAN3 signalling impact is foreseen.

- Last used cell only

No further issue on the last used cell from RAN3 perspective (removing the editor’s note in XnAP BLCR).

- LP-WUS enabling control

Observation 1: For the LP-WUS enabling control via NAS signalling, the potential RAN3 impact is the introduction of a new indication e.g., in the Initial Context Setup Request message in the NGAP to enable or disable the LP-WUS monitoring of the UE.

RAN3 to monitor the LP-WUS enabling control discussion in RAN2 then act upon accordingly.

The TPs to NGAP, XnAP and stage2 are provided in the Annex to capture the above agreements. And the F1AP CR is provided in [4].

# 4. Reference

- R2-2503182 LS on LP-WUS subgrouping progress, RAN2

- RAN2 running TS 38.304 CR

- RAN2#130 chairman notes.

- R3-255238 (TP to BLCR for TS 38.473) Discussion on Low-power wake-up signal and receiver for NR, Huawei

# 5. TP to BLCR for TS 38.413 against R3-254002

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

## 8.3	UE Context Management Procedures

### 8.3.1	Initial Context Setup

#### 8.3.1.1	General

The purpose of the Initial Context Setup procedure is to establish the necessary overall initial UE context at the NG-RAN node, when required, including PDU session context, the Security Key, Mobility Restriction List, UE Radio Capability and UE Security Capabilities, etc. The AMF may initiate the Initial Context Setup procedure if a UE-associated logical NG-connection exists for the UE or if the AMF has received the RAN UE NGAP ID IE in an INITIAL UE MESSAGE message or if the NG-RAN node has already initiated a UE-associated logical NG-connection by sending an INITIAL UE MESSAGE message via another NG interface instance. The procedure uses UE-associated signalling.

For signalling only connections and if the UE Context Request IE is not received in the Initial UE Message, the AMF may be configured to trigger the procedure for all NAS procedures or on a per NAS procedure basis depending on operator’s configuration.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Initial context setup: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.3.4	UE Context Modification

#### 8.3.4.1	General

The purpose of the UE Context Modification procedure is to partly modify the established UE context. The procedure uses UE-associated signalling.

#### 8.3.4.2	Successful Operation

Figure 8.3.4.2-1: UE context modification: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the UE CONTEXT MODIFICATION REQUEST message, the NG-RAN node shall, if supported, replace the previously provided Core Network Assistance Information for RRC INACTIVE and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.4.2	Handover Resource Allocation

#### 8.4.2.1	General

The purpose of the Handover Resource Allocation procedure is to reserve resources at the target NG-RAN node for the handover of a UE. The procedure uses UE-associated signalling.

#### 8.4.2.2	Successful Operation

Figure 8.4.2.2-1: Handover resource allocation: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the HANDOVER REQUEST message, the target NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 8.4.4	Path Switch Request

#### 8.4.4.1	General

The purpose of the Path Switch Request procedure is to establish a UE associated signalling connection to the 5GC and, if applicable, to request the switch of the downlink termination point of the NG-U transport bearer towards a new termination point. The procedure uses UE-associated signalling.

#### 8.4.4.2	Successful Operation

Figure 8.4.4.2-1: Path switch request: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the Core Network Assistance Information for RRC INACTIVE IE is included in the PATH SWITCH REQUEST ACKNOWLEDGE message, the NG-RAN node shall, if supported, store this information in the UE context and use it for the RRC_INACTIVE state decision and RNA configuration for the UE and RAN paging if any for a UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the MICO All PLMN IE is included in the Core Network Assistance Information for RRC INACTIVE IE the NG-RAN node shall, if supported, consider that the registration area for the UE is the full PLMN and ignore the TAI List for RRC Inactive IE. If the Paging Cause Indication for Voice Service IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store and use it as specified in TS 38.300 [8]. If the PEIPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8]. If the CN MT Communication Handling IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and may subsequently request, based on implementation, the CN for MT communication handling as described in TS 23.502 [10]. If the LP-WUSPS Assistance Information IE is included in the Core Network Assistance Information for RRC INACTIVE IE, the NG-RAN node shall, if supported, store it and use it for LP-WUS paging subgrouping the UE in RRC_INACTIVE state, as specified in TS 38.300 [8].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.x	LP-WUSPS Assistance Information

This IE provides the information related to LP-WUS paging subgrouping for a particular UE, as specified in TS 38.304 [12].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUS CN Subgroup ID | M |  | INTEGER (0..30, …) |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

LoggedMDTTrigger ::= CHOICE{

periodical			NULL,

eventTrigger			EventTrigger,

choice-Extensions		ProtocolIE-SingleContainer { {LoggedMDTTrigger-ExtIEs} }

}

LoggedMDTTrigger-ExtIEs NGAP-PROTOCOL-IES ::= {

...

}

LPWUSPSAssistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions		ProtocolExtensionContainer { {LPWUSPSAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (0..30, ...)

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP to BLCR for TS 38.423 against R3-254004

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 8.2.5	RAN Paging

#### 8.2.5.1	General

The purpose of the RAN Paging procedure is to enable the NG-RAN node1 to request paging of a UE in the NG-RAN node2.

The procedure uses non UE-associated signalling.

#### 8.2.5.2	Successful operation

Figure 8.2.5.2-1: RAN Paging: successful operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If the MT-SDT Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the NR Paging Long eDRX Information for RRC INACTIVE IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

If the LP-WUSPS Assistance Information IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.300 [9].

If the Further Extended UE Identity Index Value IE is included in the RAN PAGING message, the NG-RAN node2 shall, if supported, use it according to TS 38.304 [33].

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.2.3.x	LP-WUSPS Assistance Information

This IE provides the information related to LP-WUS paging subgrouping for a particular UE, as specified in TS 38.304 [33].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| LP-WUS CN Subgroup ID | M |  | INTEGER (0..30, …) |  |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

XnAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) xnap (2) version1 (1) xnap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

LowerLayerPresenceStatusChange ::= ENUMERATED {

release-lower-layers,

re-establish-lower-layers,

...,

suspend-lower-layers,

resume-lower-layers

}

LPWUSPSassistanceInformation ::= SEQUENCE {

lPWUScNsubgroupID			LPWUSCNsubgroupID,

iE-Extensions				ProtocolExtensionContainer { { LPWUSPSassistanceInformation-ExtIEs} } OPTIONAL,

...

}

LPWUSPSassistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

LPWUSCNsubgroupID ::= INTEGER (0..30, ...)

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>

# 6. TP to BLCR for TS 38.300

<<<<<<<<<<<<<<<<<<<< Change Begins >>>>>>>>>>>>>>>>>>>>

### 9.2.10	Extended DRX for RRC_IDLE and RRC_INACTIVE

When extended DRX (eDRX) is used, the following applies:

-	For RRC_INACTIVE, eDRX configuration for RAN paging is decided and configured by NG-RAN. In RRC_INACTIVE the UE monitors both RAN and CN paging;

-	For RRC_IDLE, eDRX for CN paging is configured by upper layers. In RRC_IDLE the UE monitors only CN paging;

-	Information on whether eDRX for CN paging and RAN paging is allowed on the cell is provided separately in system information;

-	The maximum value of the eDRX cycle is 10485.76 seconds (2.91 hours) while the minimum value of the eDRX cycle is 2.56 seconds;

-	The hyper SFN (H-SFN) is broadcast by the cell and increments by one when the SFN wraps around;

-	Paging Hyperframe (PH) refers to the H-SFN in which the UE starts monitoring paging according to DRX during a Paging Time Window (PTW). The PH and PTW are determined based on a formula (see TS 38.304 [10]) that is known by the AMF, UE and NG-RAN;

-	H-SFN, PH and PTW are used if the eDRX cycle is greater than 10.24 seconds;

-	When the RRC_IDLE eDRX cycle is longer than the system information modification period, the UE verifies that stored system information remains valid before resuming/establishing an RRC connection;

-	The eDRX and LP-WUS may be used together by the gNB for RAN paging and CN paging.

<<<<<<<<<<<<<<<<<<<< Change Ends >>>>>>>>>>>>>>>>>>>>