3GPP TSG-RAN WG3 Meeting #129	R3-255241

Bengaluru, India, 25th - 29th Aug, 2025

Title: 	(TP to BLCR for TS 38.473) Finalizing common signal adaptation

Source: 	Huawei

Agenda item:	17.4

Document Type:	 Other

# 1. Introduction

In this contribution, we provide our further considerations on the common signal/channel adaptation, including SSB, PRACH and paging aspects.

# 2. Discussion

## 2.1 Paging adaptation

The previous RAN3-127bis meeting made the following agreement, based on the RAN2 liaison in [1].

- Introduce a new indication for NES paging adaptation in UE Paging Capability over F1.

- Meanwhile the following agreements were made in previous RAN2’s meetings.

Agreements @ RAN2#129bis meeting 1. For the case when both pei-Config-r17 and pagingAdaptationPEI-Config-r19 are configured, R19 UE supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 while other UE should monitor PEI according to pei-Config-r17. 2. For the case when pei-Config-r17 is configured and pagingAdaptationPEI-Config-r19 is absent, both R19 UE supporting paging adaption and other UE should monitor PEI according to pei-Config-r17. 3. Paging clustering/bundling/adaptation is not supported/applied in RRC_CONNECTED. Agreements @ RAN2#130 meeting Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band.

- For PEI, RAN2 agreed a dedicated new PEI capability in addition to the separate pagingAdaptationPEI configuration needed for R19 NES UEs. R19 UEs supporting paging adaption should monitor PEI according to pagingAdaptationPEI-Config-r19 when configured. In the existing F1AP spec, the PEI Subgrouping Support Indication IE is used to indicate the UE supports to receive the paging early indication on the associated cell from the CU to the DU. Hence, a new PEI capability indication should be introduced so that the DU can use the new pagingAdaptationPEI configuration before sending the paging adaptation based paging message towards the UE.

RAN3 to introduce the new pagingAdaptationPEI capability at the cell level in the F1AP Paging message.

## 2.2 SSB adaptation

For SSB adaptation, RAN1 and RAN2 made the following agreements:

Agreement@ RAN1#119 meeting Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode) Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for SCell Agreement@ RAN1#120 meeting For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement@ RAN1#120 meeting For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation Agreement@ RAN2#129 meeting 1. RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell. 2. RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. 3. For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC. Agreement@ RAN2#129bis meeting 1. Not support MAC CE based signalling to indicate SSB adaptation in addition to DCI agreed in RAN1. 2. For SSB adaptation, multiple additional SMTC configurations are configured to UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. 3. Upon reception of DCI format 2_9 for SSB burst periodicity switching, UE selects one SMTC configuration accordingly.

From the above agreements made in other WGs, it can be observed that for the SSB adaptation:

- it is only applicable for RRC connected UEs, but not for RRC idle/inactive UEs;

- Only the SSB burst periodicity adaptation for the NCD-SSB is supported.

- The SSB adaption is applicable only for SCells.

- The DCI format 2_9 is utilized to indicate the SSB adaptation.

From RAN3 perspective, since the NCD-SSB is always broadcasted with the periodicity adaptation, this cell can be considered as a normal cell. For the indicated SSB adaptation contents, it is the DU that is responsible to determine the SSB adaptation and send DCI format 2_9, hence there is no F1AP impact.

In addition, for the L3 measurement, RAN2 agreed that multiple additional SMTC configurations are configured to the UE via RRC, together with the mapping between SSB burst periodicity and SMTC configuration. In the existing F1AP specification,

- the cell level Measurement Timing Configuration IE referring to MeasurementTimingConfiguration is included in the Served Cell Information IE, then propagates to neighbour cells,

- the CU includes the UE level MeasurementTimingConfiguration IE of the CU to DU RRC Information IE as a container to the DU to generate the measurement gaps.

RAN2 agreed that the legacy SMTC is used for neighbour cell measurement, hence there is no RAN3 impact on the legacy MeasurementTimingConfiguration IE.

For SSB adaptation over SCell, RAN3 concluded there is no RAN3 impact.

## 2.3 PRACH adaptation

For PRACH adaptation, significant agreements were made in other WGs and provided as follows.  At previous RAN3-127bis meeting, the PRACH adaptation capability indication was agreed and captured in the F1AP BLCR.

Agreement@ RAN1#116bis meeting Support adaptation mechanisms of PRACH in time-domain for following: - UE in idle/inactive mode - UE in connected mode Agreement@ RAN1#120 meeting For DCI-based adaptation for additional PRACH resources, DCI 1_0 with P-RNTI indicates the availability information for additional PRACH resource from a reference point and for a validity time duration - FFS: Validity time duration for availability is configured by higher layer signaling or predefined - FFS: Location of the reference point defined in the specification - FFS: Value/granularity of the validity time duration. - FFS: Whether DCI can be used to explicitly deactivate the additional PRACH resources Agreement@ RAN1#120bis meeting For adaptation of PRACH in time-domain, at least for 4-step RACH, at least for DCI 1_0 with P-RNTI, - Support configuration of the additional PRACH resources within [the same] RACH-ConfigCommon in SIB1 used to configure the legacy PRACH resources - the legacy PRACH resources used for ‘additional RO validation before the SSB-RO mapping’ are configured in the RACH-ConfigCommon For DCI-based adaptation for additional PRACH resources, the following 1-bit field is used for adaptation indication in DCI format 1_0 with P-RNTI - Use one bit from the Bits 5-8 within the Short Message field (from upper layers) - Send LS to RAN2 to confirm the use of this bit. Above applies for cell that transmits the DCI for connected UEs and IDLE/INACTIVE mode UEs For DCI-based adaptation for additional PRACH resources, for the availability information of additional PRACH resources indicated by DCI 1_0 with P-RNTI - the validity duration is configured via higher layer signalling. For adaptation of PRACH in time-domain, for a connected mode UE, - One of the reserved bits of PDCCH order (DCI 1_0 with C-RNTI) is used for the new DCI field that indicates the availability of additional PRACH resources. Agreements @ RAN2#129 meeting 1. Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed. 2a. RAN2 starts CBRA for RACH adaptation. 2b. RAN2 starts 4-step RACH adaptation. 3. From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request. 4. From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources. 5a. From R2 perspective, RACH adaptation is not modelled as RA feature(s). 5b. From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain. 6. Will follow legacy mechanism regarding how to select RACH resource. Agreements @ RAN2#130 meeting 2. addl-RACH-Config-Adaptation is placed under RACH-ConfigCommon 3. Introduce addl-RACH-Config-Adaptation under BWP-UplinkCommon  RACH-ConfigCommon in SIB1. addl-RACH-Config-Adaptation can also be configured under BWP-UplinkCommon  AdditionalRACHConfig  RACH-ConfigCommon in SIB1.

From the above agreements, it can be observed that:

- PDCCH order (DCI 1_0 with C-RNTI)  and DCI 1_0 with P-RNTI (one bit in the short message field) can be used to indicate the availability information for additional PRACH resource from a reference point and for a validity time duration. The validity duration is configured via higher layer signalling.

- In the RAN2 running CR in [2], the detailed parameters which is the extension of the legacy common RACH configuration include the following:

In the current F1AP and XnAP specification, the NR PRACH Configuration is included in Served Cell information IE, containing the NR PRACH Configuration List for the UL carrier or the SUL carrier.  The motivation is to forward the PRACH configuration to other RAN nodes for RACH optimisation. And in the SON aspects, in the F1AP GNB-DU CONFIGURATION UPDATE ACKNOWLEDGE message and the GNB-CU CONFIGURATION UPDATE message, the Cells for SON IE is present to indicate the PRACH configuration of the served cells or neighbour cells. For the PRACH adaptation, RAN3 needs to discuss at least the following issues.

- If the additional PRACH configuration should be contained in the NR PRACH Configuration in the Served Cell information IE, and indicated as “additional PRACH resources”?

- Each time the DCI 1_0 with P-RNTI was sent to indicate the availability of the additional PRACH resources, or the validity duration is expired, whether the gNB-CU should be notified and neighbour cells be notified?

- Whether the Cells for SON IE should be extended to include the additional PRACH resource?

We consider due to the dynamic activation and implicit deactivation of the validity duration, for the NES WI, there is no need to further consider the exchange the additional PRACH configuration. If anything is needed, this can be further considered e.g., for future SON/MDT topic.

For PRACH adaptation, RAN3 concluded there has no additional work for the NES WI.

# 4. Conclusion

Based on the discussion in this paper, we make the following proposals.

Paging adaptation

- RAN3 to introduce the new pagingAdaptationPEI capability at the cell level in the F1AP Paging message.

SSB adaptation

For SSB adaptation over SCell, RAN3 concluded there is no RAN3 impact.

- PRACH adaptation

For PRACH adaptation, RAN3 concluded there has no additional work for the NES WI.

# 5. Reference

- R2-2501483 LS on paging enhancement in R19 NES, RAN2

- R2-250xxx Running RRC CR NES post130_v1.docx

# 6. TP to BLCR for TS 38.473 against R3-255081

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

## 8.7	Paging procedures

### 8.7.1	Paging

#### 8.7.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE.

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

At the reception of the PAGING message, the gNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the gNB-DU shall transfer it to the UE.

The RAN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The CN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Paging Cause IE may be included in the PAGING message. If present the gNB-DU shall, if supported, send it to UE according to TS 38.331 [8].

The PEIPS Assistance Information IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [6].

The RedCap Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging of the RedCap UE or the eRedCap UE.

The NES Paging Adaptation Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging adaptation to the UE.

The Last Used Cell Indication IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, consider the cell identified by the NR CGI IE as the last used cell of the paged UE, and use it as specified in TS 38.331 [8].

The Recommended SSBs List IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, use it to send the paging message over the indicated SSB beams.

The PEI Subgrouping Support Indication IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication as described in TS 38.300 [6] and TS 38.304 [24].

The PEI Subgrouping Support Indication – Paging Adaptation IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging adaptation related paging early indication as described in TS 38.300 [6] and TS 38.304 [24].

The UE Paging Capability IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, take it into account when paging the UE.

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.2.6	Paging messages

#### 9.2.6.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.39 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity |  |  |  |  |  |  |
| >>RAN UE Paging identity | M |  | 9.3.1.43 |  | - |  |
| >CN UE paging identity |  |  |  |  |  |  |
| >>CN UE paging identity | M |  | 9.3.1.44 |  | - |  |
| Paging DRX | O |  | 9.3.1.40 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Last Used Cell Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >>PEI Subgrouping Support Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >>PEI Subgrouping Support Indication – Paging Adaptation | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >>Recommended SSBs List |  | 0 .. 1 |  |  | YES | ignore |
| >>>Recommended SSBs List Item |  | 1 .. < maxnoofSSBAreas > |  |  | YES | ignore |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the recommended SSB beam for paging. | - |  |
| Paging Origin | O |  | 9.3.1.79 |  | YES | ignore |
| RAN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the RAN paging cycle as defined in TS 38.304 [24]. | YES | ignore |
| CN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the UE specific paging cycle as defined in TS 38.304 [24]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.258 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.3.1.259 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED(voice, …) | This IE indicates the paging cause is IMS voice, refer to TS 23.501[21]. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.269 |  | YES | ignore |
| UE Paging Capability | O |  | 9.3.1.270 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.1.285 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.3.1.286 |  | YES | ignore |
| MT-SDT Information | O |  | 9.3.1.289 |  | YES | ignore |
| NR Paging Long eDRX Information for RRC INACTIVE | O |  | 9.3.1.325 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.270	UE Paging Capability

This IE provides the UE Paging Capability information needed for paging.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| INACTIVE State PO-Determination | O |  | ENUMERATED(supported,…) | Corresponds to the inactiveStatePO-Determination contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | - |  |
| RedCap Indication | O |  | ENUMERATED(true,…) | Indicates that the paged UE is a Redcap UE or an eRedCap UE. | YES | ignore |
| NES Paging Adaptation Indication | O |  | ENUMERATED(supported,…) | Corresponds to the pagingAdaptation contained in the UERadioPagingInformation IE defined in TS 38.331 [8]. | YES | ignore |

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-SRSPosPeriodicConfigHyperSFNIndex,

id-candidatePSCellsToCancel,

id-ValidityAreaSpecificSRSInformationExtended,

id-OnDemandSIB1,

id-NESPagingAdaptationIndication,

id-PEISubgroupingSupportIndication-PagingAdaptation,

maxNRARFCN,

maxnoofErrors,

maxnoofBPLMNs,

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

PER-Scalar ::= INTEGER (0..9, ...)

PER-Exponent ::= INTEGER (0..9, ...)

PagingCell-Item ::= SEQUENCE {

nRCGI		NRCGI	,

iE-Extensions	ProtocolExtensionContainer { { PagingCell-ItemExtIEs } }	OPTIONAL

}

PagingCell-ItemExtIEs 	F1AP-PROTOCOL-EXTENSION ::= {

{	ID id-LastUsedCellIndication					CRITICALITY ignore 	EXTENSION LastUsedCellIndication						PRESENCE optional }|

{	ID id-PEISubgroupingSupportIndication			CRITICALITY ignore	EXTENSION PEISubgroupingSupportIndication			PRESENCE optional }|

{	ID id-Recommended-SSBs-List						CRITICALITY ignore	EXTENSION Recommended-SSBs-List						PRESENCE optional }|

{	ID id-PEISubgroupingSupportIndication-PagingAdaptation			CRITICALITY ignore	EXTENSION PEISubgroupingSupportIndication-PagingAdaptation			PRESENCE optional },

...

}

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

PDUSessionID ::= INTEGER (0..255)

PEISubgroupingSupportIndication ::= ENUMERATED {true, ...}

PEISubgroupingSupportIndication-PagingAdaptation ::= ENUMERATED {true, ...}

ReportingPeriodicityValue ::= INTEGER (0..512, ...)

Periodicity ::= INTEGER (0..640000, ...)

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

id-MobilityInitiation								ProtocolIE-ID ::= 859

id-ValidityAreaSpecificSRSInformationExtended		ProtocolIE-ID ::= 860

id-PLMNIndexNRAssistanceInfoForNetShar						ProtocolIE-ID ::= 861

id-OnDemandSIB1										ProtocolIE-ID ::= aaa

id-NESPagingAdaptationIndication					ProtocolIE-ID ::= bbb

id-OnDemand-SIB1-Cell								ProtocolIE-ID ::= ccc

id-PEISubgroupingSupportIndication-PagingAdaptation			ProtocolIE-ID ::= ddd

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>