3GPP TSG-RAN WG3 Meeting #129                                                                    R3-255243

Bengaluru, India, 25th – 29th, August 2025

Source:	CATT

Title:	(TP to BLCR for TS 38.401) On remaining issues of WAB

Agenda Item:	12.2

Document for:	Approval

1 Introduction

Some aspects with respect to WAB have not been concluded.

In this contribution, we will continue discussion about the open issues for WAB.

2 Discussion

2.1 Multi-hop WAB

In the last meeting [1], RAN3 have no consensus on supporting Solution 3 (spec-based solution) for multi-hop prevention.

The contribution (R3-253414) in last meeting elaborated many restrictions of Solution 1 (pure implementation-based solution). Based on so many restrictions, some companies have strong concern about how Solution 1 works and whether it can be accepted by operators.

Compared to Solution 1, Solution 3 can get rid of these restrictions. Although the changes to SIB and UE capabilities are necessary for Solution 3, the limited impact is foreseen which makes it worth doing.

Thus, we prefer to support Solution 3 as an important and necessary supplement to Solution 1.

Proposal 1-1: Support Solution 3 as an important supplement to Solution 1.

For Solution 3, WAB-gNB SIB broadcasts a specific indicator for WAB cell, and WAB-MT is able to avoid (re)selection to the WAB-gNB cells based on this indicator. RAN3 need to send an LS to RAN2 to trigger corresponding specification work for Solution 3. A draft LS is provided in another contribution [2].

Proposal 1-2: RAN3 send LS to RAN2 to trigger corresponding RAN2 specification for Solution 3.

2.2 Additional ULI

In the RAN3#127bis meeting [3], it was agreed that the additional ULI is introduced to the non-UE associated NG procedure:

RAN3#127bis: RAN3 to agree the WAB-gNB reports the additional ULI to the network within the NG Setup and RAN Configuration Update procedures. It is FFS whether the additional ULI shall be removed from NG UE associated procedures

The additional ULI has been introduced in non-UE associated messages (i.e., NG SETP REQUEST and RAN CONDIGURATION UPDATE) as the NGAP BLCR [4]. However, at present, stage-2 BLCR [5] and NGAP BLCR support WAB-gNB reporting the additional ULI for the served UEs to CN via UE associated messages. So, the open issue is whether the additional ULI in UE-associated NGAP messages is still needed.

We understand this issue depends on whether the content of additional ULI (i.e., pair of TAI and NR CGI) for different UEs under the same WAB-node can be different. If the PLMN/SNPN which the WAB-gNB broadcasts is the same as that serving the WAB-MT, and the WAB-MT accesses through TN, the additional ULI of UEs includes pair of TAI and NR CGI of the cell serving the WAB-MT. In that case, additional ULI for all the served UEs should be the same. However, in other cases, additional ULI of UE is determined based on WAB-node’s geo-location. It’s not clear whether a WAB-node’s geo-location can be mapped to different additional ULIs for different UEs which it serves. Namely, there are two understandings.

Understanding 1:

A WAB-node’s geo-location can be mapped to different additional ULIs for different UEs, i.e., the content of additional ULI for different UEs can be different. For example, the additional ULIs for UEs served by different serving cells of the same WAB-gNB are different.

In understanding 1, WAB-node should report all the additional ULIs to which the WAB-node’s geolocation can map via the non-UE associated messages over NGAP, so the non-UE associated messages should support carrying multiple additional ULIs. In addition, the additional ULI for each UE should be informed to AMF in the UE associated messages, which means the additional ULI in the UE associated NGAP messages in present BLCR can be kept.

Understanding 2:

A WAB-node’s geo-location should be mapped to one single additional ULI for all serving UEs.

In understanding 2, as the WAB-node’s geo-location should only map to one additional ULI, the WAB-node can report the single additional ULI to the AMF via the non-UE associated NGAP messages as defined in present BLCR. As all the served UEs share the same additional ULI which is reported via the non-UE associated message, the information does not need to be indicated again via the UE associated messages to CN. Thus, the additional ULI in the UE associated NGAP messages in present BLCR can be removed.

In our view, as the coverage of WAB-gNB is normally small no matter one or multiple cells the WAB-gNB serves, the location of all the UEs served by the same WAB-node could be considered the same. There is no need to have the WAB-gNB’s geo-location mapping to different additional ULIs for different UEs, i.e., all the UEs served by the same WAB-node should have the same additional ULI. So, understanding 2 is preferred.

Proposal 2-1: A WAB-node’s geo-location should be mapped to one single additional ULI for all serving UEs. The content of additional ULI (i.e., pair of TAI and NR CGI) for different UEs under the same WAB-node should be the same.

Proposal 2-2: Remove the additional ULI IE in the UE associated NGAP messages in present BL CR for TS 38.413.

The associated section (X.5) in stage-2 BLCR should be updated for the report of the additional ULI to CN via the non-UE associated NGAP procedures.

Proposal 2-3: Update the BLCR to TS 38.401 for the report of additional ULI to CN via the non-UE associated NGAP procedures.

A TP is provided to BLCR of TS 38.401 in the Annex.

2.3 WAB integration

During the WAB-node integration procedure, the WAB-MT establishes the BH PDU session(s), and the WAB-gNB is initialized and sets up the NG/Xn connection(s). These steps will only be performed if the WAB-MT and WAB-gNB are “authorized” respectively. If the WAB-MT is not authorized, it should not establish the BH PDU session(s). (It has been captured in BLCR of TS 38.401 [5] that WAB-MT will not establish the BH PDU session(s) until its authorization status changes to “authorized”.)

Similarly, the WAB-gNB should not be initialized if it is not authorized by OAM, in other words, the WAB-gNB should be authorized by OAM before the WAB-gNB is configured by the OAM. However, present BLCR of TS 38.401 states that the WAB-gNB is authorized after the WAB-gNB is configured by OAM.

Section X.1 in BLCR to TS 38.401: Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling. Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases: Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is configured by the OAM (e.g., with the information needed to establish NG connections towards one or more AMF(s) and the WAB-gNB is service-authorized by the SeGW or by the OAM.

Proposal 3-1: In phase 2-1 of WAB-node integration procedure, WAB-gNB should be authorized by OAM or by the SeGW before the WAB-gNB is configured by OAM.

In phase 2-1, the WAB-gNB is authorized by SeGW, however, SeGW is out of RAN/3GPP scope, a note should be added to clarify that.

Proposal 3-2: A note should be added for WAB-node integration procedure that the SeGW is out of RAN’s scope.

A TP is provided to BLCR for TS 38.401 in the Annex.

2.4 Non-terrestrial BH

Three scenarios of WAB-gNB using non-terrestrial network as backhaul are illustrated as below. In scenario A and B, NTN radio access is used by WAB-MT, i.e., the WAB-MT accesses the BH NG-RAN via NTN RAT which could be LEO, MEO or GEO. In scenario C, NTN backhaul is used between BH NG-RAN and the BH 5GC which could be LEO, MEO, GEO, DYNAMIC LEO, DYNAMIC MEO, DYNAMIC GEO.

During movement, the WAB-MT may change the accessing network, for example, from terrestrial network to non-terrestrial network (NTN radio access or NTN backhaul), or from one type of non-terrestrial network to another type (e.g. from LEO to GEO). So, above scenarios conclude all the scenarios of satellite backhaul category changing in case of WAB.

In Rel-18, SA2 has studied and standardized how to support the change of satellite backhaul category. Based on the SA2 specification, the change of satellite backhaul type impacts QoS and policy control in PCF. For the scenarios above, it is SA2’s responsibility to study whether the existing mechanism can work or not.

In the RAN3#128 meeting [1], some discussion aiming at NGAP enhancement of WAB-gNB informing UE 5GC that the backhauling of UE is using NTN:

If non-terrestrial link is used between WAB MT and BH gNB and/or between BH gNB and BH CN, the WAB-gNB informs UE’s CN that the BH includes a non-terrestrial link. FFS how a WAB node know the BH-gNB is using a non-terrestrial link. Possible options include BH-gNB informs WAB-gNB via Xn.

However, there is no concrete reason to add the indication of “BH includes a non-terrestrial link” on NG interface.

Based on the discussion papers last meeting (R3-253344, R3-253390) regarding this issue, it seems the only usage of such indication is to support QoS enhancement based on different backhaul categories of UE in WAB scenario, e.g., the PCF makes policy decision based on backhaul category of WAB BH for UE. However, the policy strategy (including QoS matter) is normally studied in SA2’s and it’s out of RAN3 scope. RAN3 don’t have enough knowledge to find out the technical gap, given SA2 already standardized the mechanism to support the change of satellite backhaul category. So, even if there is a need to introduce the NTN link indication over NGAP for QoS enhancement, it should be determined by SA2.

QoS enhancement is not within Rel-19 WAB scope (both RAN3 and SA2). Note that SA#106 meeting approved a Rel-20 SA1 WI [6] on VRM on requirement of QoS management when VMR use backhaul links with different categories of TN and NTN. A correction [7] to TS 22.261 was approved to capture these requirements for Rel-20:

S1-244766: Subject to regulatory requirements and operator policy, the 5G network shall support a mechanism to determine suitable QoS parameters for traffic relayed via a mobile base station when the connection quality of the serving mobile base station relay changes e.g. during mobility between terrestrial access network and satellite access network.

Thus, SA2 should take charge of the discussion on how to fulfill this requirement of QoS enhancement from Rel-20. If SA2 see a necessity to enhance NGAP, they can send LS to RAN3 for alignment.

Observation 1: QoS enhancement for WAB is neither in RAN3 scope nor in R19 WAB scope. And there is SA1 Rel-20 WI on the requirements of QoS management for VMR using NTN backhaul. It makes no sense that RAN3 discusses this issue from Rel-19.

Proposal 4: RAN3 not to discuss the enhancement of NGAP for QoS management of NTN WAB BH in Rel-19.

Regarding how a WAB-node know the BH-gNB is using a NTN link to send the NTN link indication to UE 5GC, RAN3 thought about one possible solution that BH-gNB informs the WAB-gNB over Xn. However, this solution requires upgrade of BH-gNB and enhancements to XnAP. The BH-gNB should be upgraded to find the co-location of WAB-MT and WAB-gNB, and send the BH link category to WAB-gNB over XnAP. Due to the usage of Xn connection, this solution cannot be used when the PLMN(s) served by the BH-gNB and the PLMN(s) broadcasted by the WAB-gNB are different or the Xn interface does not exist between the WAB-gNB and BH-gNB.

Observation 2: The BH-gNB based solution requires upgrade of BH-gNB and it cannot be used in some scenarios.

3 Conclusion

In the previous sections we made the following proposals:

Multi-hop WAB

Proposal 1-1: Support Solution 3 as an important supplement to Solution 1.

Proposal 1-2: RAN3 send LS to RAN2 to trigger corresponding RAN2 specification for Solution 3.

Additional ULI

Proposal 2-1: A WAB-node’s geo-location should be mapped to one single additional ULI for all serving UEs. The content of additional ULI (i.e., pair of TAI and NR CGI) for different UEs under the same WAB-node should be the same.

Proposal 2-2: Remove the additional ULI IE in the UE associated NGAP messages in present BL CR for TS 38.413.

Proposal 2-3: Update the BLCR to TS 38.401 for the report of additional ULI to CN via the non-UE associated NGAP procedures.

WAB integration

Proposal 3-1: In phase 2-1 of WAB-node integration procedure, WAB-gNB should be authorized by OAM or by the SeGW before the WAB-gNB is configured by OAM.

Proposal 3-2: A note should be added for WAB-node integration procedure that the SeGW is out of RAN’s scope.

Non-terrestrial BH

Observation 1: QoS enhancement for WAB is neither in RAN3 scope nor in R19 WAB scope. And there is SA1 Rel-20 WI on the requirements of QoS management for VMR using NTN backhaul. It makes no sense that RAN3 discusses this issue from Rel-19.

Proposal 4: RAN3 not to discuss the enhancement of NGAP for QoS management of NTN WAB BH in Rel-19.

Observation 2: The BH-gNB based solution requires upgrade of BH-gNB and it cannot be used in some scenarios.

4 Reference

- RAN3#128 minutes, RAN3 Chair

- R3-255673, [Draft] LS on Multi-hop Topology Avoidance for WAB, CATT, China Telecomm, Huawei, NTT Docomo, Lenovo, Samsung, NEC, RAN3#129

- RAN3#127bis minutes, RAN3 Chair

- R3-253081, (BL CR to 38.413 for WAB) Support for Wireless Access Backhaul, Huawei, Ericsson, Nokia, Nokia Shanghai Bell, China Telecom, ZTE, Qualcomm, Samsung, CATT, Jio Platforms (JPL), Lenovo, RAN3#128

- R3-253078, (BL CR to 38.401 for WAB) Support for Wireless Access Backhaul, Ericsson, ZTE, Nokia, Nokia Shanghai Bell, Huawei, Samsung, Lenovo, Qualcomm, Jio Platforms, RAN3#128

- SP-241821, New WID on Vehicle-Mounted Relays Phase3 (VMR_Ph3), SA#106

- S1-244766, Additional requirements on QoS management for VMR backhaul change, CATT, Qualcomm, SA1#108

5 Annex (TP to BL CR for TS 38.401)

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>Start of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<

# X	Wireless Access Backhaul

## X.1	WAB-node integration procedure

Figure X.1-1 WAB-node integration procedure

Phase 1: WAB-MT setup. The WAB-MT of a WAB-node connects to the network in the same way as a UE by performing RRC connection setup procedure with the BH-RAN-node. The WAB-MT then performs, authorization and authentication with the BH-5GC. After the WAB-MT is authorized, the WAB-MT can establish one or more PDU sessions for backhauling.

Phase 2: WAB-gNB setup. This phase includes the following 3 sub-phases:

Sub-phase 2-1: WAB-gNB initialization. In this phase, the WAB-gNB is service-authorized by the SeGW or by the OAM. After the WAB-gNB is authorized, the WAB-gNB is configured by the OAM (e.g., with the information needed to establish NG connections towards one or more AMF(s)).

NOTE: The SeGW is out of RAN scope.

Sub-phase 2-2: NG connection setup. The WAB-gNB establishes NG connection(s) toward the AMF(s). This step follows legacy procedures. After the NG is set up, the WAB-gNB can start serving UE(s).

Sub-phase 2-3: Xn connection setup. If needed, the WAB-gNB can establish Xn connection(s) towards the BH-RAN-node and/or other NG-RAN node(s).

---------------------------------------Next Change -------------------------------------------

## X.5 	User Location Information for UEs served by a WAB-gNB

In addition to the User Location Information (ULI) for UEs served by a WAB-gNB, the WAB-gNB also provides the core network with Additional ULI via the NG Setup or RAN Configuration Update procedures, which includes a TAI and a NR CGI pertinent to the WAB-gNB’s broadcasted PLMN/SNPN.

If the PLMN/SNPN broadcasted by a WAB-gNB is the same as the PLMN/SNPN serving the WAB-MT, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI includes the TAI and the NR CGI of the cell serving the WAB-MT.

If the PLMN/SNPN serving the WAB-MT is different from the WAB-gNB’s broadcasted PLMN/SNPN, and the WAB-MT connects to the BH-gNB by means of a terrestrial link, the Additional ULI is determined by the WAB-gNB, based on the WAB-node’s geo-location.

If the WAB-MT connects to the BH-gNB by means of a non-terrestrial link, the Additional ULI is determined by the WAB-gNB, based on WAB-node’s geo-location. This applies regardless of whether the PLMN/SNPN serving the WAB-MT is the same as, or different than, the WAB-gNB’s broadcasted PLMN/SNPN.

In case Additional ULI changes, e.g., due to WAB-node movement, the WAB-gNB derives the new Additional ULI and reports it via the RAN Configuration Update procedures, if required by the core network.

>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>End of Changes<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<