3GPP TSG-RAN WG3 Meeting #129                                                                  R3-255249

Bengaluru, India, August 25th – 29th, 2025

Source:	CATT, ZTE, China Telecomm, Samsung

Title:	(TP to BLCR for TS 38.473) Adaptation of paging

Agenda Item:	17.4

Document for:	Discussion and Approval

1 Introduction

During RAN3#127bis meetings[1], following agreement was achieved for paging adaptation:

RAN3#127bis Introduce a new indication for NES paging adaptation in UE Paging Capability over F1.

In the last RAN2 meeting[2], there are several agreements regarding to paging adaptation as well as PEI enhancement (the agreements in yellow are made for paging adaptation and those in green are for PEI):

Agreements on stage-3 details

- Proposal (include ARFCN inside the list with the PCI to have a NES-CellId list associated to a OD-SIB1 config) is not proceeded.

- Keep RRC CR as it is for the short message and UE behaviour.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum PO per PF, i.e., maxPO-perPF-r19 is 8.

- The maximum offset value is extended to 32 radio frames.

- The value range for the new optional firstPDCCH-MonitoringOccasionOfPO-r19 field parameter is extended to accommodate SCS=480kHz and N=T/32.

- A new pei-ConfigBWP IE is not introduced for Rel-19 UEs that support adaptive paging.

- Introduce a new optional firstPDCCH-MonitoringOccasionOfPEI-O-r19 field parameter for Rel-19 UEs that support adaptive paging.

- The maximum number of PEI occasion per paging frame is extended to 8.

- The maximum offset value is extended to 32 radio frames.

- Rel-19 excluded cell list can be configured as an empty list. No new UE behavior is introduced (i.e. the UE supporting OD-SIB1 ignores legacy excluded cell list only if NES excluded cell list is present).

- Same as the other 2 agreed cases of OD-SIB1 acquisition failure, capture the failure case of OD-SIB1 window expiry in TS 38.304. RAN2 confirm no other cases need to be considered and specified in TS 38.304.

- The capability for paging adaption to be included in UE-RadioPagingInfo is a separate capability from other NES features and no need to define a common capability for all NES features.

- Use a “container” using OCTET STRING with content generated by UE for paging adaption capability.

- The paging adaption capability is per UE.

- Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band.

- Rely on PHY layer specs to capture availability conditions related to usage of additional PRACH resources. It can be revisited if needed after checking PHY running specifications.

- A note in TS 38.321 is added to clarify that UE selects only amongst preambles configured for the next valid RO(s) for the selected SSB, when CB-PreamblesPerSSB is configured for additional ROs. Wording of the note can be refined during the running CR review discussion.

- The following two proposals were discussed and not pursued:

- (P9 R2-2503712) If UE has on-going RACH procedure upon reception of DCI with RACH adaptation indication, the UE stops the on-going RACH procedure, and re-initiates the RACH procedure with updated RACH resources.

- (P8 R2-2503945) RAN2 to discuss whether the RA procedure can be terminated earlier if the selected RA resource is deactivated.

In this contribution, we provide our further considerations on paging adaptation and PEI enhancement in Rel-19.

2 Discussion

2.1 Paging adaptation indication

In RAN3#127bis meeting, a new indicator of paging adaptation capability which is added in UE Paging Capability IE in F1AP PAGING message has been introduced to F1AP BLCR. The FFS is kept in semantics description for stating the corresponding IE defined in RRC spec for this new indicator.

#### 9.3.1.270 UE Paging Capability This IE provides the UE Paging Capability information needed for paging. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. INACTIVE State PO-Determination; O; ENUMERATED(supported,…); Corresponds to the inactiveStatePO-Determination contained in the UERadioPagingInformation IE defined in TS 38.331 [8].; -. RedCap Indication; O; ENUMERATED(true,…); Indicates that the paged UE is a Redcap UE or an eRedCap UE.; YES; ignore. NES Paging Adaptation Indication [FFS]; O; ENUMERATED(supported,…); Corresponds to the [FFS] contained in the UERadioPagingInformation IE defined in TS 38.331 [8].; YES; ignore. Editor’s Note: FFS on the name and semantics description, pending to RAN2’s update.

It can be observed from above agreements that the paging adaptation capability is determined as a per-UE capability by RAN2, thus it’s correct for RAN3 to add a per-UE indicator for paging adaptation capability in F1AP PAGING message.

According to the agreements, RAN2 decided to use the RRC container generated by UE for all Rel-19 and further paging capabilities to contain the paging adaptation capability. However, RAN2 has not started the detailed design for the RRC container, thus the corresponding RRC IE to the new F1AP indicator for paging adaptation capability is not clear now. RAN3 can continue wait for RAN2 and keep the FFS.

Proposal 1: In F1AP BLCR, keep FFS for the corresponding RRC IE to the new indicator for paging adaptation capability in UE Paging Capability IE.

2.2 Paging adaptation PEI

In Rel-19, PEI is enhanced to adapt to the paging adaptation. A new UE capability for PEI has been introduced to Rel-19 NES according to RAN2 agreements:

Introduce new capability for R19 PEI in NES and follow the existing PEI capability definition to have this new capability per band.

For a UE supporting paging adaptation PEI, if the configuration of PEI for paging adaptation is signaled in system information, the UE can monitor the PEI occasion according to this configuration and ignore the legacy PEI configuration. And RAN2 decided to follow the existing PEI capability definition to have this new capability per band.

For Rel-17 PEI, the PEI capability is reported to gNB by UE per-band, and the PEI support indication is included per cell in the F1AP PAGING message. At reception of the PEI support indication, the gNB-DU considers that the corresponding cell is supported by UE to receive the PEI. Therefore, if the new PEI capability for paging adaptation is also per band reported by UE, support of this capability should also be indicated per-cell in paging message over F1.

Proposal 2: Introduce a new per-cell support indication for paging adaptation PEI in PAGING message over F1.

A TP to F1AP BLCR is provided in Annex to introduce the new support indication for paging adaptation PEI.

3 Conclusion

In the previous sections we made the following observations and proposals:

Proposal 1: In F1AP BLCR, keep FFS for the corresponding RRC IE to the new indicator for paging adaptation capability in UE Paging Capability IE.

Proposal 2: Introduce a new per-cell support indication for paging adaptation PEI in PAGING message over F1.

4 Reference

- Draft Report of 3GPP TSG RAN3 meeting #127bis, MCC

- Draft Report of 3GPP TSG RAN2 meeting #130, MCC

5 Annex (TP to BLCR of 38.473)

-------------------------------------Start of TP---------------------------------------------

### 8.7.1	Paging

#### 8.7.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE.

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

At the reception of the PAGING message, the gNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the gNB-DU shall transfer it to the UE.

The RAN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The CN UE Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 38.304 [24].

The NR Paging eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Paging Cause IE may be included in the PAGING message. If present the gNB-DU shall, if supported, send it to UE according to TS 38.331 [8].

The PEIPS Assistance Information IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging subgrouping of the UE, as specified in TS 38.300 [6].

The RedCap Indication IE may be included in the UE Paging Capability IE in the PAGING message, and if present the gNB-DU shall, if supported, use it for paging of the RedCap UE or the eRedCap UE.

The Last Used Cell Indication IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, consider the cell identified by the NR CGI IE as the last used cell of the paged UE, and use it as specified in TS 38.331 [8].

The Recommended SSBs List IE may be included in the Paging Cell Item IEs IE of the PAGING message, and if present the gNB-DU shall, if supported, use it to send the paging message over the indicated SSB beams.

The PEI Subgrouping Support Indication IE may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication as described in TS 38.300 [6] and TS 38.304 [24].

The Paging Adaptation PEI Subgrouping Support Indication IE [naming FFS] may be included in the Paging Cell Item IEs IE in the PAGING message, and if present the gNB-DU shall, if supported, consider that the cell identified by the NR CGI IE is supported by the UE to receive the paging early indication for paging adaptation as described in TS 38.300 [6] and TS 38.304 [24].

The UE Paging Capability IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, take it into account when paging the UE.

The Extended UE Identity Index Value IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The Hashed UE Identity Index Value IE may be included in the PAGING message, and if present the gNB-DU shall, if supported, use it according to TS 38.304 [24].

The MT-SDT Information IE may be included in the PAGING message. If present the gNB-DU shall, if supported, use it for MT-SDT paging as specified in TS 38.331 [8].

The NR Paging Long eDRX Information for RRC INACTIVE IE may be included in the PAGING message, and if present, the gNB-DU shall, if supported, use it according to TS 38.304 [24].

#### 8.7.1.3	Abnormal Conditions

Not applicable.

-----------------------------------------------------Next Change-----------------------------------------------------

#### 9.2.6.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| UE Identity Index value | M |  | 9.3.1.39 |  | YES | reject |
| CHOICE Paging Identity | M |  |  |  | YES | reject |
| >RAN UE Paging identity |  |  |  |  |  |  |
| >>RAN UE Paging identity | M |  | 9.3.1.43 |  | - |  |
| >CN UE paging identity |  |  |  |  |  |  |
| >>CN UE paging identity | M |  | 9.3.1.44 |  | - |  |
| Paging DRX | O |  | 9.3.1.40 | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX | YES | ignore |
| Paging Priority | O |  | 9.3.1.41 |  | YES | ignore |
| Paging Cell List |  | 1 |  |  | YES | ignore |
| >Paging Cell Item IEs |  | 1 .. <maxnoofPagingCells> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Last Used Cell Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >>PEI Subgrouping Support Indication | O |  | ENUMERATED(true, …) |  | YES | ignore |
| >>Recommended SSBs List |  | 0 .. 1 |  |  | YES | ignore |
| >>>Recommended SSBs List Item |  | 1 .. < maxnoofSSBAreas > |  |  | YES | ignore |
| >>>>SSB Index | M |  | INTEGER (0..63) | Identifier of the recommended SSB beam for paging. | - |  |
| >>Paging adaptation PEI Subgrouping Support Indication [naming FFS] | O |  | ENUMERATED(true, …) |  | YES | ignore |
| Paging Origin | O |  | 9.3.1.79 |  | YES | ignore |
| RAN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the RAN paging cycle as defined in TS 38.304 [24]. | YES | ignore |
| CN UE Paging DRX | O |  | Paging DRX 9.3.1.40 | This IE indicates the UE specific paging cycle as defined in TS 38.304 [24]. | YES | ignore |
| NR Paging eDRX Information | O |  | 9.3.1.258 |  | YES | ignore |
| NR Paging eDRX Information for RRC INACTIVE | O |  | 9.3.1.259 |  | YES | ignore |
| Paging Cause | O |  | ENUMERATED(voice, …) | This IE indicates the paging cause is IMS voice, refer to TS 23.501[21]. | YES | ignore |
| PEIPS Assistance Information | O |  | 9.3.1.269 |  | YES | ignore |
| UE Paging Capability | O |  | 9.3.1.270 |  | YES | ignore |
| Extended UE Identity Index Value | O |  | 9.3.1.285 |  | YES | ignore |
| Hashed UE Identity Index Value | O |  | 9.3.1.286 |  | YES | ignore |
| MT-SDT Information | O |  | 9.3.1.289 |  | YES | ignore |
| NR Paging Long eDRX Information for RRC INACTIVE | O |  | 9.3.1.325 |  | YES | ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

-------------------------------------End of TP---------------------------------------------