3GPP TSG-RAN WG3 Meeting #129                                                                               R3-255250

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	(TP to BLCR of TS 38.300) On remaining issues of LP-WUS

Agenda Item:	18.2

Document for:	Discussion and Decision

1 Introduction

In the last meeting[1], RAN3 has some issues leftover for LP-WUS, as captured by chair’s notes:

(TP To BL CR for TS 38.300) support of homogeneous deployment for LP-WUS in R3-253831 noted, wait for reply LS from SA2 To be continued... FFS on the range of LP-WUS CN Subgroup ID IE included in LP-WUSPS Assistance Information pending on RAN1 decision on which subgroup ID(s) would be reserved for common codepoint.

In this contribution, we discuss those issues.

2 Discussion

2.1 UE paging capability loss

Regarding the issue of UE paging capability loss in the LS from SA2[3], RAN2 discussed it in RAN2#129bis[2], and sent a LS(R2-2503172/R3-253012)[4] to SA2/RAN3 as below:

R2-2503172: For UE Radio Paging Capabilities introduced up to Rel-18, RAN2 agreed not to introduce any standard solution, i.e., rely on proper network implementation, as follows. - We will not change legacy UE capabilities and rely on network implementation To address the issue for UE Radio Paging Capabilities introduced in Rel-19 onwards, RAN2 made the following agreement: - Use a “container” using OCTET STRING with content generated by UE when new UE Radio Paging Capabilities are introduced. This is applicable only to new Rel-19 and future capabilities added to paging capabilities. This will only work for gNB that support this Rel-19 “container” In particular for Rel-19 LP-WUS feature, if it is deployed, all the gNBs belonging to a certain area need to support the Rel-19 “container”.

According to the LS, RAN2 indicated that they will not enhance RRC spec for UE Radio Paging Capability to Rel-18 or earlier Releases, and they will introduce a RRC container containing UE’s paging capabilities for Rel-19 or Rel-19 onwards. It means that UE paging capability of LP-WUS (or paging features beyond Rel-19) will not be made lost by gNBs of Rel-19 or on-wards, but this problem still exists for gNBs of Rel-18 or before Rel-18.

Therefore, RAN2 suggested a way in the LS to handle this issue by implementation, i.e., all the gNBs of the area deploying the LP-WUS (or paging features beyond Rel-19) should be at least upgraded to Rel-19.

However, according to SA2’s reply[5] to RAN3, SA2 assumes that the way recommended by RAN2 cannot address the issue effectively unless “the area” mentioned by RAN2 is the whole NR coverage, because the UE may come from anywhere of NR coverage. Meanwhile, SA2 has studied another way of implementation which is inspired by the solution captured in TS 23.501 clause 5.4.4.3 with respect to retrieving the UE paging capability of desired NR-RAN RAT when UE moves between LTE and NR in 5GS.

R3-255026: The 5GS design means that the UE Radio Capability for Paging is loaded into the AMF only in the cell where the UE attaches (does “initial registration”) or first moves from LTE to NR. After that the UE can move throughout the whole PLMN and normally there is no event that triggers any update of the UE Radio Capability for Paging information stored in the AMF. Hence the “certain area” mentioned by RAN 2 is the whole NR coverage of that PLMN. It is essential that operators are able to use RAN equipment from different vendors and that competition between those RAN vendors is not inhibited by RAN 2 requiring all RAN vendors to upgrade their equipment before new UE functionality can be used with the first RAN vendor to develop a solution. Hence this RAN2 solution is highly undesirable compared to what was proposed in RAN2’s earlier LS in R2-2501388=S2-2504576. SA2 are investigating whether a solution that builds on the current process in TS 23.501 clause 5.4.4.3 for handling UEs that support both “LTE connected to 5GC” and NR can be used to solve the problem. For example, when (as already specified) the UE Radio Capability for Paging is sent in the NGAP Initial UE Context Setup Request to the gNB, the gNB could use the UE Radio Access Capabilities (received in the same message) to check that the UE Radio Capability for Paging contains the full information on paging related features. If the gNB detects an omission, then the gNB should ensure that an update of the UE Radio Capability for Paging Information is sent to the AMF. Clause 5.4.4.3 in TS 23.501: a) UE radio capability for paging information: - The UE Radio Capability for Paging Information contains information derived by the NG-RAN node (e.g. band support information) from the UE Radio Capability information. The AMF stores this information without needing to understand its contents. As the AMF only infrequently (e.g. at Initial Registration) prompts the NG-RAN to retrieve and upload the UE radio capabilities i.e. UE Radio Capability information to the AMF and the AMF may be connected to more than one NG-RAN RAT, it is the responsibility of the NG-RAN to ensure that UE Radio Capability for Paging Information (which is derived by the NG-RAN node) contains information on all NG-RAN RATs that the UE supports in that PLMN. To assist the NG-RAN in this task, as specified in TS 38.413 [34], the AMF provides its stored UE Radio Capability for Paging Information in every NG-AP INITIAL CONTEXT SETUP REQUEST message sent to the NG-RAN. - The UE Radio Capability for Paging Information is maintained in the core network, even during AMF reselection and is stored in the UCMF alongside the UE Radio Capability information associated to a UE Radio Capability ID.

SA2 asks RAN3 and RAN2 to provide views on the potential solution. According to SA2’s solution, AMF can always provide the stored UE radio capability as well as the UE radio capability for paging to gNB in NGAP INITIAL CONTEXT SETUP REQUEST message. At reception of this message, Rel-19 gNB could check whether the UE Radio Capability for Paging IE contains the full content of paging related information included in the UE Radio Capability IE. If not, the Rel-19 gNB would determine there is a paging capability loss and initiate a UE capability retrieving procedure to UE to update the UE radio capability for paging. In that way, the updated UE radio capability for paging is sent to AMF when the UE moves from an area where RAN equipment is not upgraded and accesses the upgraded RAN equipment. Therefore, SA2 solution can work only if all the gNBs within UE’s tracking area(s) deploying the LP-WUS are upgraded to Rel-19 and support the implementation proposed by SA2.

SA2’s solution requires no enhancement to AMF but special handling at Rel-19 gNB. We understand this is more complex than updating UE radio capability for paging in case the UE moves from LTE to NR in 5GS, where the gNB only needs to check whether the UE Radio Capability for Paging IE contains NR related IE. SA2’s solution requires gNB always to compare the content of UE Radio Capability for Paging IE and the UE Radio Capability for Paging IE at reception of INITIAL CONTEXT SETUP REQUEST message. Nevertheless, SA2 solution can work from RAN point of view.

Observation 1: SA2’s solution can work only if all the gNBs within UE’s tracking area(s) deploying the LP-WUS are upgraded to Rel-19 and support the implementation as SA2’s solution.

RAN3 can capture SA2’s solution into BLCR of 38.300.

Proposal 1: To address the problem of paging capability loss, a gNB supporting LP-WUS can check whether the UE Radio Capability for Paging IE contains the full content of paging related information included in the UE Radio Capability IE at reception of each NGAP INITIAL CONTEXT SETUP REQUEST message. If not, the gNB would determine a missing of paging capability and initiate a UE capability retrieving procedure to UE to update the UE radio capability for paging.

RAN’s implementation to handle the issue of paging capability loss can be captured in BLCR of TS 38.300. A TP to 38.300 BLCR is provided in Annex.

Proposal 2: Capture RAN’s implementation for handling the issue of paging capability loss into BLCR for TS 38.300.

RAN3 can send reply LS to SA2 to confirm SA2’s solution and attach the TP to 38.300 BLCR to the reply LS.

Proposal 3: Send reply LS to SA2 to confirm SA2’s solution and attach the TP to 38.300 BLCR to the reply LS.

2.2 Range of CN Subgroup ID

During the RAN1#120, RAN1#120bis and RAN1#121 meetings, the agreements achieved regarding to the numbers of subgroups per PO and the PO-to-LO association/mapping include:

RAN1#120: Agreement Confirm the following working assumption with the modification: Working Assumption The maximum number of subgroups per PO supported in Rel-19 is 31 32. RAN1#120bis: Agreement The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4. Agreement For Option 2, at least one codepoint corresponding to each of the subgroups in each PO is supported. - For codepoint corresponding to more than one subgroups: - Alt 2: One codepoint for each PO corresponding to all the subgroups in the PO Agreement For Option 2, a common codepoint per PO is always used and the maximum number of subgroups supported per PO is - 7 for the case where 4 POs are mapped to one LO - 15 for the case where 2 POs are mapped to one LO Agreement Regarding whether there is any restriction on mapping multiple POs to one LO, no additional constraint for mapping multiple POs to one LO RAN1#121: Agreement For PO-to-LO association and codepoint determination, assume NPOLO is the number of POs associated with a LO, NSGPO is the number of subgroups per PO for LP-WUS, and iSG is the subgroup ID of a UE (0≤iSG<NSGPO). - The PO index within the LO is defined as iPO=UE_ID mod N∙Ns+is mod NPOLO, - The reference PF for the LO of a PO is provided by (SFN for PF) – floor(iPO/NS) * T/N, which is the first PF of the PF(s) associated with the LO. - For the codepoints, - If NSGPO>1, - The number of information bits in LP-WUS is log2NPOLO*(NSGPO+1). - Alt 2: (the codepoints for each PO are consecutive) - The codepoint for subgroup iSG in PO iPO is iPO*(NSGPO+1)+iSG. - The codepoint for all the subgroups in PO iPO is (iPO+1)*NSGPO+1-1. - If NSGPO=1, the number of information bits in LP-WUS is log2NPOLO, and the codepoint for PO iPO is iPO. - Here UE_ID (for LP-WUS), N, NS, is and T are defined in TS 38.304

According to RAN1’s agreements, a common codepoint (i.e., codepoint for all subgroups) per PO associated with the LO is used.

The maximum number of subgroups supported per PO is:

-	7 for the case where 4 POs are mapped to one LO

-	15 for the case where 2 POs are mapped to one LO

-	31 for the case where 1 PO is mapped to one LO

The subgroup IDs per PO and the common codepoint in all three cases are calculated as:

| Number of POs associated with a LO | Maximum number of subgroups per PO | Subgroup IDs per PO | Codepoint for all subgroups per PO |
| --- | --- | --- | --- |
| NPOLO=4 | NSGPO=7 | {0, …, 6} | For iPO=0, codepoint is 7; For iPO=1, codepoint is 15; For iPO=2, codepoint is 23; For iPO=3, codepoint is 31. |
| NPOLO=2 | NSGPO=15 | {0, …, 14} | For iPO=0, codepoint is 15; For iPO=1, codepoint is 31; |
| NPOLO=1 | NSGPO=31 | {0, …, 30} | iPO=0, codepoint is 31. |

It can be observed that the maximum subgroup ID for a UE is 6, 14 and 30 respectively in case 4, 2 and 1 PO is mapped to one LO, and the minimum subgroup ID for a UE is 0 in all three cases. So, the range of CN Subgroup ID IE included in LP-WUSPS Assistance Information can be determined as {0, …, 30}.

Proposal 4: The range of CN subgroup ID in LP-WUSPS Assistance Information is {0, …, 30}.

3 Conclusion

According to the analysis in section 2, we have following observations and proposals:

Proposal 1: To address the problem of paging capability loss, a gNB supporting LP-WUS can check whether the UE Radio Capability for Paging IE contains the full content of paging related information included in the UE Radio Capability IE at reception of each NGAP INITIAL CONTEXT SETUP REQUEST message. If not, the gNB would determine a missing of paging capability and initiate a UE capability retrieving procedure to UE to update the UE radio capability for paging.

Proposal 2: Capture RAN’s implementation for handling the issue of paging capability loss into BLCR for TS 38.300.

Proposal 3: Send reply LS to SA2 to confirm SA2’s solution and attach the TP to 38.300 BLCR to the reply LS.

Proposal 4: The range of CN subgroup ID in LP-WUSPS Assistance Information is {0, …, 30}.

4 Reference

- RAN3#127bis Minutes, RAN3 chair

- draft_MeetingReport_RAN2#129bis, MCC

- R3-250021(S2-2412876), LS Reply on LP-WUS subgrouping, SA2, RAN3#127

- R3-253012 (R2-2503172), Reply LS on paging capability loss issue, RAN2, RAN3#128

- R3-255026, Response to Reply LS on paging capability loss issue, SA2

Annex

TP to BLCR for TS 38.300

----------------------------- Start of the TP -------------------------------

### 9.2.5	Paging

>>>>>>>>>>>>>>>>>>> Unchanged part is skipped <<<<<<<<<<<<<<<<<<<<<<<

UE ID based subgrouping: For UE ID based subgrouping, the gNB and UE can determine the subgroup ID based on the UE ID and the total number of subgroups for UE ID based subgrouping in the cell. The total number of subgroups for UE ID based subgrouping is decided by the gNB for each cell and can be different in different cells. The following figure describes the procedure for UE ID based subgrouping:

Figure 9.2.5-2: Procedure for UE ID based subgrouping

1.	The gNB determines the total number of subgroups for UE ID based subgrouping in a cell.

2.	The gNB broadcasts the total number of subgroups for UE ID based subgrouping in a cell.

3.	UE determines its subgroup in a cell.

4.	When paging message for the PEI capable UE is received from the CN at the gNB or is generated by the gNB, the gNB determines the PO and the associated PEI occasion for the UE.

5.	Before the UE is paged in the PO, the gNB transmits the associated PEI and indicates the corresponding subgroup derived based on UE ID of the UE that is paged in the PEI.

NOTE:	Paging to the UE may be lost in case the UE moves from a gNB which does not support forwarding the Rel-17 PEI or Rel-19 LP-WUS radio paging capability to the AMF. It is up to the network implementation to avoid the issue. For example, a gNB supporting PEI or LP-WUS can check whether the UE Radio Capability for Paging IE contains the full content of paging related information included in the UE Radio Capability IE at reception of each NGAP INITIAL CONTEXT SETUP REQUEST message. If not, the gNB may determine a missing of radio paging capability and initiate a radio capability retrieving procedure to UE to update the UE radio capability for paging.

----------------------------- End of the TP ---------------------------