3GPP TSG-RAN WG3 Meeting #129	R3-255257

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	22.2

Source:	Ericsson

Title:	(TP to BL CR for TS 38.401) Stage-2 Updates for SL Multi-hop

Document for:	Discussion, Agreement

# Introduction

This contribution provides a discussion on NR Sidelink Multi-hop Relay based on the progress in RAN3 #128 and offline agreements.

Reuse the Peer UE ID IE as counterpart information with the update for the semantics description.

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

FFS whether the same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

FFS whether upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1.

FFS whether to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel is needed can be further discussed in next meeting.

The target is to close the WI with the simplest approach for Rel-19.

# Discussion

## Allocation of PC5 RLC Channel ID

During the discussion of PC5 RLC channel ID allocation, RAN3 considered whether the same channel IDs should be assigned toward between intermediate Relay UE and its Parent UE, and between intermediate Relay UE and its Child UE.

It was noted that maintaining different PC5 RLC channel IDs may help avoid any ambiguity in, for example, QoS flow mapping, and channel release handling. Considering that it has been agreed to reuse Peer UE ID as counterpart information last meeting, it seems reasonable to reuse the Peer UE ID IE in the PC5 RLC Channel corresponding lists within the F1AP UE CONTEXT MODIFICATION REQUEST message, and to include the same IE in the UE CONTEXT MODIFICATION RESPONSE, UE CONTEXT MODIFICATION REQUIRED, and UE CONTEXT MODIFICATION CONFIRM messages within the PC5 RLC channel lists.

- RAN3 agrees to differentiate the PC5 RLC channels between intermediate Relay UE and Parent UE, and intermediate Relay UE and Child UE,

- Reuse the Peer UE ID IE in the PC5 RLC Channel to be Setup List in the F1AP UE CONTEXT MODIFICATION REQUEST message.

- Introduce the Peer UE ID IE to the F1AP UE CONTEXT MODIFICATION RESPONSE/REQUIRED/CONFIRM messages within the PC5 RLC Channel lists.

## First Relay UE Awareness in gNB-DU

Regarding the identification of the First Relay UE at the gNB-DU, the question was raised whether explicit indicator is required upon reception of an RRCSetupRequest message from a Remote UE. RAN2 clarified that the gNB-DU can derive the complete hop sequence by interpreting the received SidelinkUEInformation messages originating from the First Relay UE, through the intermediate relay UE and the Last Relay UE. The path information can be recoganized by the network. Since relay UEs can also operate as Remote UEs, and the existing procedure already provides sufficient path awareness, no additional signaling enhancement is required for the gNB-DU to know the First Relay UE.

- The gNB-DU is able to know the path information via the received SUI messages from the Relay UEs.

- No further signaling enhancement is needed for the gNB-DU to differentiate the First Relay UE.

## F1 Enhancements for PC5 RLC Multiplexing

In relation to the multiplexing of PC5 Relay RLC channels over F1 signaling, it requires to check whether enhancements are necessary to explicitly indicate multiplexing. It was concluded that the current framework is sufficient, as multiplexing can be implicitly known by the network based on the existing identifiers, including SRB IDs, DRB IDs. SRAP headers can carry the RB IDs and Layer 2 ID. These parameters allow the network to use the same PC5 RLC channel when appropriate, without adding new signaling to the F1AP procedures.

- RAN3 agrees no signaling impact is foreseen to support the multiplexing of PC5 Relay RLC channels.

- Agree the TP to BL CR for TS 38.401 in the Annex and and the TP to BL CR for 38.473 in [1].

# Conclusions and Proposals

In this contribution we made observations and proposals for Sidelink Multi-hop, as listed below.

Observation 1	The gNB-DU is able to know the path information via the received SUI messages from the Relay UEs.

Proposal 1	RAN3 agrees to differentiate the PC5 RLC channels between intermediate Relay UE and Parent UE, and intermediate Relay UE and Child UE,

Proposal 2	Reuse the Peer UE ID IE in the PC5 RLC Channel to be Setup List in the F1AP UE CONTEXT MODIFICATION REQUEST message.

Proposal 3	Introduce the Peer UE ID IE to the F1AP UE CONTEXT MODIFICATION RESPONSE/REQUIRED/CONFIRM messages within the PC5 RLC Channel lists.

Proposal 4	No further signaling enhancement is needed for the gNB-DU to differentiate the First Relay UE.

Proposal 5	RAN3 agrees no signaling impact is foreseen to support the multiplexing of PC5 Relay RLC channels.

Proposal 6	Agree the TP to BL CR for TS 38.401 in the Annex and and the TP to BL CR for 38.473 in [1].

# Reference

[1] R3-255258, (TP to BL CR for TS 38.473) Support of SL Multi-hop in F1AP, Ericsson

# Annex

TP to BL CR for TS 38.401:

### 8.19.xx	Remote UE initial access for Multi-hop Layer-2 UE-to-Network Relay

The signalling flow for Remote UE Initial access is shown in Figure 8.19.xx-1.

Figure 8.19.xx-1: Overall procedure for Remote UE’s initial access via multi-hop relay

1.	The U2N Remote UE, the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE perform discovery procedure, and establish PC5 connection using NR ProSe procedure.

2.	The U2N Remote UE sends an RRCSetupRequest message to the First U2N Relay UE via PC5 Relay RLC channel.

3.	The First U2N Relay UE withholds the received RRC message. If the First U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should send its own RRCSetupRequest message to the Intermediate U2N Relay UE via PC5 Relay RLC channel in order to trigger the RRC establishment/resume procedure to enter RRC_CONNECTED state upon reception of the RRC message from the U2N Remote UE. If the Intermediate U2N Relay UE is in RRC_IDLE/RRC_INACTIVE state, it should trigger the RRC establishment/resume procedure in clause 8.19.1 to enter RRC_CONNECTED state upon reception of the RRC message from the First U2N Relay UE. If all Relay UEs are in RRC_CONNECTED state, this step could be skipped.

Note x: If in RRC_IDLE or RRC_INACTIVE state, the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last Relay UE should trigger the RRC establishment/resume procedure in sequence.

4.	The First U2N Relay UE in RRC_CONNECTED state sends the SidelinkUEInformationNR message to the gNB-DU via the Intermediate U2N Relay UE and Last U2N Relay UE.

5.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the First U2N Relay UE by encapsulating the SidelinkUEInformationNR message to gNB-CU, and gNB-CU allocates the local ID of U2N Remote UE.

6.	The gNB-CU sends the UE CONTEXT MODIFICATION REQUEST message of the Last U2N Relay UE to gNB-DU. Such message may request the establishment of Uu Relay RLC channel(s) and PC5 Relay RLC channel(s) for the transmission of U2N Remote UE’s SRB0.

7.	The gNB-DU sends the UE CONTEXT MODIFICATION RESPONSE message of the Last U2N Relay UE to gNB-CU.

8.	The gNB-CU sends the DL RRC MESSAGE TRANSFER message of the Last U2N Relay UE to gNB-DU by encapsulating the RRCReconfiguration message, which contains the local ID allocated to the U2N Remote UE. The RRCReconfiguration message shall also contain the Uu Relay RLC channel(s) configuration and PC5 Relay RLC channel(s) configuration if not configured and bearer mapping for relaying of U2N Remote UE’s SRB0.

9.	The gNB-DU sends the RRCReconfiguration message to the Last U2N Relay UE to configure the local ID of the U2N Remote UE, the Uu Relay RLC channel(s) configuration, PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0.

10.	The Last U2N Relay UE sends the RRCReconfigurationComplete message to gNB-DU.

11.	The gNB-DU sends the UL RRC MESSAGE TRANSFER message of the Last U2N Relay UE by encapsulating the RRCReconfigurationComplete message to gNB-CU.

12.	The gNB-CU configures the Intermediate U2N Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the Intermediate U2N Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

13.	The gNB-CU configures the First U2N Relay UE with the local ID allocated to the U2N Remote UE, PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB0. According to the configuration from gNB-CU, the First U2N Relay UE may establish a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB0 over PC5. This step follows the same signaling flow as described in steps 6-11.

14.	After receiving the local ID of the U2N Remote UE and the PC5 Relay RLC channel(s) configuration and bearer mapping for relaying of U2N Remote UE’s SRB0, the First U2N Relay UE sends the RRCSetupRequest message of the U2N Remote UE to gNB-DU via the Intermediate U2N Relay UE and the Last U2N Relay UE. The local ID of the U2N Remote UE and RB ID for SRB0 are conveyed in the SRAP header.

15.	The gNB-DU allocates a C-RNTI and a gNB-DU UE F1AP ID for the U2N Remote UE and sends the INITIAL UL RRC MESSAGE TRANSFER message to gNB-CU by encapsulating the RRCSetupRequest message of the U2N Remote UE. In addition, the local ID of the U2N Remote UE, the gNB-DU UE F1AP ID of the Last U2N Relay UE and the sidelink configuration container for the PC5 Relay RLC channel configuration for relaying of U2N Remote UE’s SRB1 are included in the INITIAL UL RRC MESSAGE TRANSFER message.

16.	The gNB-CU allocates a gNB-CU UE F1AP ID for the U2N Remote UE and generates a RRCSetup message towards the U2N Remote UE. The RRC message is encapsulated in the DL RRC MESSAGE TRANSFER message, and includes the configurations of PC5 Relay RLC channel and bearer mapping at least for the transmission of U2N Remote UE’s SRB1.

17.	The gNB-DU sends the RRCSetup message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

18.	The gNB-CU configures the Last U2N Relay UE with PC5 Relay RLC channel, Uu Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the Last U2N Relay UE establishes a PC5 Relay RLC channel for relaying of U2N Remote UE’s SRB1 over PC5 and establishes a Uu Relay RLC channel for relaying of U2N Remote UE’s SRB1 towards gNB-DU if not configured yet.

The gNB-CU configures the First U2N Relay UE and the Intermediate U2N Relay UE with PC5 Relay RLC channel and bearer mapping for relaying of U2N Remote UE’s SRB1. According to the configuration from gNB-CU, the First U2N Relay UE and the Intermediate U2N Relay UE establish the PC5 Relay RLC channels for relaying of U2N Remote UE’s SRB1 over PC5 if not configured yet.

NOTE 1: Step 18 can be performed earlier, e.g., via Steps 6-13.

19.	The U2N Remote UE sends the RRCSetupComplete message to the gNB-DU via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

20.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

21.	Upon receiving the RRCSetupComplete message of U2N Remote UE, the gNB-CU sends the INITIAL UE MESSAGE message to the AMF.

22.	The AMF sends the INITIAL CONTEXT SETUP REQUEST message to the gNB-CU.

23.	The gNB-CU sends the UE CONTEXT SETUP REQUEST message to establish the U2N Remote UE context in the gNB-DU. Such message may request the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs, and may also encapsulate the SecurityModeCommand message.

24.	The gNB-DU sends the SecurityModeCommand message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

25.	The gNB-DU sends the UE CONTEXT SETUP RESPONSE message of the U2N Remote UE to the gNB-CU, which contains the configuration of PC5 Relay RLC channels for the transmission of U2N Remote UE’s SRB2 and DRBs.

26.	The U2N Remote UE responds with the SecurityModeComplete message.

27.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and sends it to the gNB-CU.

28.	The gNB-CU generates the RRCReconfiguration message for U2N Remote UE and encapsulates it in the DL RRC MESSAGE TRANSFER message. The RRCReconfiguration message contains the configuration of PC5 Relay RLC channels and bearer mapping for the transmission of U2N Remote UE’s SRB2 and DRBs.

29.	The gNB-DU sends RRCReconfiguration message to the U2N Remote UE via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

30.	The U2N Remote UE sends RRCReconfigurationComplete message to the gNB-DU via the First U2N Relay UE, the Intermediate U2N Relay UE and Last U2N Relay UE.

31.	The gNB-DU encapsulates the RRC message in the UL RRC MESSAGE TRANSFER message and send it to the gNB-CU.

32.	The gNB-CU sends the INITIAL CONTEXT SETUP RESPONSE message to the AMF.

33.	The gNB-CU configures additional Uu Relay RLC channels between the gNB-DU and the Last U2N Relay UE, and additional PC5 Relay RLC channels for the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE for relaying of U2N Remote UE’s DRBs and SRBs. Also, such step may configure the bearer mapping between U2N Remote UE’s DRB/SRB and PC5/Uu Relay RLC channel at the First U2N Relay UE, the Intermediate U2N Relay UE, and the Last U2N Relay UE.

NOTE 2: This step can be performed earlier.