3GPP TSG-RAN WG3 Meeting #129	R3-255274

Bengaluru, India, 25 – 29 August 2025

Agenda Item:		11.4

Source:		Ofinno

Title:	Mobility Optimization for NR-DC

Document for:		Discussion and decision

# Introduction

During multiple previous meetings many companies have proposed to include predicted PSCell into DC procedures (e.g., SN addition). This issue is still open.

This contribution discusses introducing PSCell ID in DC procedures. Accompanying TP to BL CR for 38.423 proposes a possible implementation.

# Discussion

The MN may use its AI/ML model to predict the UE trajectory. The MN may use such prediction to select a target SN for SN addition or change. UE trajectory prediction is cell-level granularity, which assumes that the MN has very good idea about potential PSCell and/or SCG configuration for the UE. Once such inference results are available, it is not clear why they shall be discarded instead of sending them to the SN to assist in selecting PSCell for the UE / configuring SCG for the UE.

Providing this and maybe additional assistance information from the MN to the SN may be a future compatible solution taking into account that more inference results may be available at the MN in future as RAN2 progresses with AI/ML for mobility work (e.g., SHO, RLF, ToS, measurement predictions).

In previous RAN3 discussions, it was mentioned that in legacy behaviour, PSCell selection / SCG configuration is done by SN, not MN. As a possible compromise, the MN may send to the SN several PSCell / SCell predicted candidates for the SN to take into account during PSCell/SCell selection. This may be further enhanced by adding priority/preference information to each predicted candidate PSCell/SCell to assist SN in selecting PSCell / configuring SCG for the UE.

Proposal 1: MN sends to SN predicted PSCell / SCell candidates during DC procedures (e.g., SN addition) for a UE. SN may take this assistance information into account while selecting PSCell / configuring SCG for the UE.

Proposal 2: For predicted PSCell / SCell candidates, MN may send to SN priority/preference of selecting a predicted candidate PSCell / SCell. SN may take this assistance information into account while selecting PSCell / configuring SCG for the UE.

# Proposals

Proposal 1: MN sends to SN predicted PSCell / SCell candidates during DC procedures (e.g., SN addition) for a UE. SN may take this assistance information into account while selecting PSCell / configuring SCG for the UE.

Proposal 2: For predicted PSCell / SCell candidates, MN may send to SN priority/preference of selecting a predicted candidate PSCell / SCell. SN may take this assistance information into account while selecting PSCell / configuring SCG for the UE.

# Reference

[1] R3-255001, RAN3#129 Meeting Agenda

[2] R3-255002, RAN3#128 Meeting Report

[3] R3-255044, (BL CR to TS 38.423) Support of enhancements on AI/ML for NG-RAN