3GPP TSG-RAN WG3 Meeting #129	R3-255276

Bengaluru, India,  25th - 29th August 25

Agenda Item:	19.2

Source:	Charter Communications, Qualcomm Inc., Fraunhofer HHI, Fraunhofer IIS

Title:	[TP for BL CR TS 38.300] Introduction of SRS resource configurations for Evolution of NR duplex operation Sub-Band Full Duplex

Document for:	Discussions & Approval

# 1	Introduction

This TP follows discussions in R3-255267.

# 2	Text Proposal

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

17.2 Cross-Link Interference Management

When different TDD DL/UL patterns are used between neighbouring cells, UL transmission in one cell may interfere with DL reception in another cell: this is referred to as Cross Link Interference (CLI).

To mitigate CLI, gNBs can exchange and coordinate their intended TDD DL-UL configurations for gNB-to-gNB CLI and SRS Resource configurations for UE-to-UE CLI, over Xn and F1 interfaces; and the victim UEs can be configured to perform CLI measurements. There are two types of CLI measurements:

- SRS-RSRP measurement in which the UE measures SRS-RSRP over SRS resources of aggressor UE(s);

- CLI-RSSI measurement in which the UE measures the total received power observed over RSSI resources.

Layer 3 filtering applies to CLI measurement results and both event triggered and periodic reporting are supported.

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>