3GPP TSG-RAN WG3 Meeting #129	R3-255279

Bengaluru, India, 25 – 29 August 2025

Agenda Item:		21.3

Source:		Ofinno

Title:	Remaining issues on XR uplink rate control

Document for:		Discussion and Decision

# Introduction

In last RAN3 meeting, the following agreements were achieved on support of XR uplink Rate Control:

CU sends the uplink rate control indication per QoS flow over F1 to DU.

Other additional assistance information from CU to DU?

- Recommended UL bit rate infor per QoS flow

- Measured bit rate per QoS flow

- No additional information

This contribution discusses some potential issues on it.

# Discussion

In last RAN3 meeting, we agreed to introduce a per-QoS Flow indication over F1 interface to enable the gNB-CU to inform the gNB-DU that the QoS Flow allows rate adaptation in the uplink direction.

Similar principle should be applied to NR-DC operations. That is, during SN Addition Preparation procedure and MN initiated SN modification procedure, MN may send the Indication of Bitrate Adaptation as part of the QoS profile to SN to enable uplink rate control.

Proposal 1: During SN Addition Preparation procedure and MN initiated SN modification procedure, MN sends the Indication of Bitrate Adaptation as part of the QoS profile to SN to enable uplink rate control.

In the current Xn BLCR [3], the Indication of Bitrate Adaptation IE has been added to the QoS Flow Level QoS Parameters for handover scenario, as shown below. It can be used for SN Addition Preparation procedure and MN initiated SN modification procedure.

///////////////////////////////////////////////////////////////////////skip unrelated///////////////////////////////////////////////////////////////////////

#### 9.2.3.5	QoS Flow Level QoS Parameters

This IE defines the QoS Parameters to be applied to a QoS flow.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| CHOICE QoS Characteristics | M |  |  |  | – |  |
| >Non Dynamic 5QI |  |  |  |  |  |  |
| >>Non dynamic 5QI Descriptor | M |  | 9.2.3.8 |  | – |  |
| >Dynamic 5QI |  |  |  |  |  |  |
| >>Dynamic 5QI Descriptor | M |  | 9.2.3.9 |  | – |  |
| … | … |  |  |  |  |  |
| PDU Set QoS Parameters |  | 0..1 |  | Indicates the PDU Set QoS Parameters. | YES | ignore |
| DL PDU Set Information Marking Support Indication | O |  | ENUMERATED (true, …) |  | YES | ignore |
| Indication of Bitrate Adaptation | O |  | ENUMERATED (uplink, …) | Indicates that the QoS Flow allows rate adaptation in the indicated direction. | YES | ignore |

However, stage 2 and 3 text descriptions are missing for SN Addition Preparation procedure and MN initiated SN modification procedure. Here we propose to add the stage 2 and stage 3 text descriptions for TS 38.423 and TS 37.340.

Proposal 2: To agree the text proposals in [5] for BLCRs TS 38.423 and TS 37.340.

FIG. 1

In addition, based on the current assumption, it is possible that an XR service may be mapped to one DRB in RAN side. The DRB may be split and served by a master node and a secondary node as shown in FIG. 1. Thus, each node has its own MAC entity. Master node and secondary node may send its own recommended bit rate to UE. UE may be confused on how to report it to its upper layer for the same DRB to apply the rate control.

On the other hand, even though only master node or secondary node determines to send a recommended bit rate to UE, the other node may not know the situation and waste the resources allocated for that or lack of information to increase the resources for a potential increased bit rate.

Therefore, at least it is necessary for MN/SN to notify SN/MN the recommended bit rate that its MAC CE sends to UE. Based on that, the SN/MN may take corresponding actions based on implementations.

Proposal 3: At least it is necessary for MN/SN to notify SN/MN the recommended bit rate that its MAC CE sends to UE for split bearer case.

# Proposals

Proposal 1: During SN Addition Preparation procedure and MN initiated SN modification procedure, MN sends the Indication of Bitrate Adaptation as part of the QoS profile to SN to enable uplink rate control.

Proposal 2: To agree the text proposals in [5] for BLCRs TS 38.423 and TS 37.340.

Proposal 3: At least it is necessary for MN/SN to notify SN/MN the recommended bit rate that its MAC CE sends to UE for split bearer case.

# Reference

[1] R3-255001, RAN3#129 Meeting Agenda

[2] RP-240107, Revised WID on XR for NR Phase 3

[3] R3-254024, (BL CR to 38.423) Support of XR enhancements

[4] R3-254019, (BL CR to 37.340) Support of XR enhancements

[5] R3-255280, (TP for XR BL CR for TS 38.423, TS 37.340) Support of UL rate control