3GPP TSG-RAN WG3 Meeting #129	R3-255302

Bengaluru, India, Aug 25th - Aug 29th, 2025

Agenda item:	13.3

Source:	Qualcomm Incorporated

Title:	Signalling enhancements for Intra-CU Conditional LTM

WID/SID:	NR_Mob_Ph4-Core – Release 19

Document for:	Discussion and decision

# Introduction

In the last RAN3 meeting (RAN3 #128 [1]), significant progress was made towards completing the specification work on this topic. Below is the list of agreements made in the meeting.

RAN3 #128 agreements:

RAN3 confirms that the network can trigger a LTM Cell Switch Command MAC CE towards a candidate cell with C-LTM candidate configuration.

The candidate (initial source) DU need to know the L1 C-LTM and the L3 C-LTM.

The DU needs to know to which other candidate cell(s) to generate the L1 execution condition for L1 C-LTM.

WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

WA: To introduce a new IE with a list of candidate cells for L1 execution condition.

It is acknowledged that the C-LTM indicator needs to be sent to the initial source DU for the very first C-LTM.

The candidate DU generates the TAT for its own candidate cells.

In this contribution, we discuss the remaining signalling aspects of intra-CU conditional LTM (CLTM).

# 2 Remaining signalling aspects of Intra-CU conditional LTM (CLTM)

In the previous RAN3 meetings, the following were agreed:

- During Intra-CU CLTM preparation, the gNB-CU determines an initial list of candidate cells for Intra-CU CLTM, and it also determines, for each candidate cell, whether to configure an L1 execution condition or an L3 execution condition.

- When the gNB-CU determines that an L1 execution condition should be configured for a candidate cell, the gNB-CU transmits an L1 execution condition request indication, to the source gNB-DU or the candidate gNB-DU. Upon receiving the message, the source gNB-DU or the candidate gNB-DU generates the L1 execution condition configuration for the candidate cell and transmits it to the gNB-CU in the response message.

- Furthermore, the DU entity has a characteristic behaviour for CLTM, e.g., it needs to generate L1 execution conditions for candidate cells upon receiving a corresponding request from the gNB-CU.

Consequently, the WAs from the previous meeting can be confirmed.

Proposal 1. Confirm the following WAs from the previous meeting ([1]):

WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

WA: To introduce a new IE with a list of candidate cells for L1 execution condition.

- The gNB-CU includes in a new IE a list of LTM candidate cells for which L1 execution conditions should be configured in the UE Context Setup Request and UE Context Modification Request messages to a candidate gNB-DU and the source gNB-DU, respectively.

  - This IE is not present if gNB-CU decides that for all candidate cells L3 execution conditions should be configured.

- The candidate gNB-DU and the source gNB-DU include the generated L1 execution conditions from the prepared candidate cell to other candidate cells in the UE Context Setup Response and UE Context Modification Response messages, respectively.

The overall signalling flow for the Intra-CU CLTM procedure is then as shown in Figure 1.

When the parallel preparation of LTM candidate cells is completed, i.e., upon completion of Steps 5a and 5b in Figure 1, the gNB-CU knows about all the prepared candidate cells. The gNB-CU can then generate the L3 execution conditions based on this information and the UE RRC measurement reports.

Proposal 2. Upon receiving information regarding the prepared LTM candidate cells from its underlying DUs, the gNB-CU may generate the L3 execution conditions for Intra-CU CLTM:

- The gNB-CU can generate the L3 execution conditions from the source cell to other candidate cells, as well as the execution conditions from a candidate cell to other candidate cells, i.e., for subsequent LTM.

The gNB-CU transmits the L3 execution conditions and the L1 execution conditions for the prepared cells to the UE in the RRC reconfiguration message containing the LTM configuration for Intra-CU CLTM.

Figure 1: Signalling flow for Intra-CU CLTM with L1 and L3 execution condition configuration

5. RACH-less Conditional intra-CU LTM is supported.

7. UE based TA measurement mechanism is supported for conditional intra-CU LTM.

8. PDCCH ordered early TA acquisition is supported for conditional LTM.

8. The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

9. The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

10. Candidate cell TA is maintained by a new timer.

RAN2 made the following agreements regarding Early TA acquisition for Intra-CU CLTM.

Based on the agreements above, we think that for the Early TA RACH configuration procedure, the related Rel-18 intra-CU LTM procedures can be simply reused.

Proposal 3. The Rel-18 Intra-CU LTM Early TA RACH configuration and TA Information Transfer procedures can be reused for Early TA acquisition for Intra-CU CLTM.

For Intra-CU CLTM, it was agreed in RAN2 that a TA timer value associated with the TA value of a candidate cell needs to be signalled to the UE, since the UE is responsible for checking the validity of the TA. A TA timer value needs to be provided to the UE for each candidate cell, and the gNB-CU provides the TA timer value to the UE through RRC signalling.

The candidate DU generates the TAT for its own candidate cells.

It was agreed in the last RAN3 meeting [1], that the candidate gNB-DU can determine the TA timer value associated with a TA value for a candidate cell that is managed by the candidate gNB-DU.

We think that the candidate gNB-DU should signal the TA timer value of a candidate cell to the gNB-CU. The gNB-CU can then provide the TA timer value for the candidate cell to the UE through RRC signalling.

Also, the gNB-CU should provide the TA timer value of a candidate cell to the source gNB-DU. This enables the source gNB-DU to track the validity of the TA value provided to the UE. If the validity expires, then the source gNB-DU can initiate a PDCCH-ordered RACH procedure to the UE to acquire the TA for the candidate cell.

Proposal 4. A candidate gNB-DU signals the TA timer value of its candidate cell to the gNB-CU.

- The UE Context Modification Response message can be used, or

- The TA Information Transfer procedure can be used to provide the TA timer value to the source gNB-DU, which then uses the UE Context Modification Response message to signal the TA timer value to the gNB-CU.

Observation 1. The gNB-CU should provide the TA timer value of a candidate cell received from a candidate gNB-DU to the source gNB-DU, so that the source gNB-DU can track the validity of the TA value provided to the UE.

Proposal 5. The gNB-CU provides the TA timer value of a candidate cell received from a candidate gNB-DU to the source gNB-DU using the UE Context Modification Request message.

The Rel-18/Rel-19 LTM CSI configuration procedure can be reused for Intra-CU LTM for providing the UE with configuration based on SSBs and/or CSI-RSs for performing L1 measurements and reporting.

# 3 Conclusion

Based on the above discussion, we recommend that RAN3 discuss the following observations and proposals.

Proposal 1. Confirm the following WAs from the previous meeting ([1]):

WA: To introduce one codepoint in the legacy LTM indicator IE, namely “C-LTM”.

WA: To introduce a new IE with a list of candidate cells for L1 execution condition.

- The gNB-CU includes in a new IE a list of LTM candidate cells for which L1 execution conditions should be configured in the UE Context Setup Request and UE Context Modification Request messages to a candidate gNB-DU and the source gNB-DU, respectively.

  - This IE is not present if gNB-CU decides that for all candidate cells L3 execution conditions should be configured.

- The candidate gNB-DU and the source gNB-DU include the generated L1 execution conditions from the prepared candidate cell to other candidate cells in the UE Context Setup Response and UE Context Modification Response messages, respectively.

Proposal 2. Upon receiving information regarding the prepared LTM candidate cells from its underlying DUs, the gNB-CU may generate the L3 execution conditions for Intra-CU CLTM:

- The gNB-CU can generate the L3 execution conditions from the source cell to other candidate cells, as well as the execution conditions from a candidate cell to other candidate cells, i.e., for subsequent LTM.

Proposal 3. The Rel-18 Intra-CU LTM Early TA RACH configuration and TA Information Transfer procedures can be reused for Early TA acquisition for Intra-CU CLTM.

Proposal 4. A candidate gNB-DU signals the TA timer value of its candidate cell to the gNB-CU.

- The UE Context Modification Response message can be used, or

- The TA Information Transfer procedure can be used to provide the TA timer value to the source gNB-DU, which then uses the UE Context Modification Response message to signal the TA timer value to the gNB-CU.

Observation 1. The gNB-CU should provide the TA timer value of a candidate cell received from a candidate gNB-DU to the source gNB-DU, so that the source gNB-DU can track the validity of the TA value provided to the UE.

Proposal 5. The gNB-CU provides the TA timer value of a candidate cell received from a candidate gNB-DU to the source gNB-DU using the UE Context Modification Request message.

# References

[1] Chair notes, RAN3 #128 meeting.