3GPP TSG-RAN WG3 Meeting #129	R3-255306

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	17.3

Source:	ZTE Corporation

Title:		(TPs to BL CR for 38.420, 38.470 and 38.473) Support of on-demand SIB1

Document for:	Discussions & Approval

1	Introduction

- This paper provides further discussion on on-demand SIB1.

2	Discussion

At the RAN3-128 meeting, it was agreed that the CU shall inform the DU when the UL WUS Configuration broadcast request for a NES cell has been accepted/rejected by Cell A. Based on this information, the DU determines whether allow the cell to enter the On-Demand SIB1 state. The following agreement was reached:

The NES gNB-CU sends the indication to DU. The NES gNB-DU MAY go to OD-SIB 1 up to gNB-DU decision.

In the current F1AP BL CR, the following IE is introduced in the GNB-CU CONFIGURATION UPDATE message to capture the above agreement.

| On-Demand SIB1 Cell |  | 0..1 |  |  | YES | ignore |
| --- | --- | --- | --- | --- | --- | --- |
| >NR CGI | M |  | 9.3.1.12 |  | - |  |
| >On-Demand SIB1 Indicator | M |  | ENUMERATED (start, stop, ...) |  | - |  |

However, the current definition of the On-Demand SIB1 Indicator IE is somewhat confusing. The use of "start" or "stop" appears to imply that the CU is making the decision to initiate or terminate On-Demand SIB1 for the NES cell. But according to our understanding of the agreement, it is the DU that determines whether to allow the cell to enter the On-Demand SIB1 state, based on the "accept" or "reject" status of the cell's associated UL WUS Provision. Therefore, we propose revising the agreement to clarify that this indicator should be, i.e., "accept" or "not accept".

Proposal 1: Revise the agreement from "The NES gNB-CU sends the indication to DU. The NES gNB-DU MAY go to OD-SIB 1 up to gNB-DU decision" to "The NES gNB-CU sends the indication of the "accept" or "reject" status of the associated UL WUS Provision request of a NES cell to DU. The NES gNB-DU MAY go to OD-SIB 1 up to gNB-DU decision".

Furthermore, we assume that the NR CGI in the On-Demand SIB1 Cell IE refers to a NES cell in the DU. However, the Cell ID of Cell A, which either "accepts" or "rejects" the UL WUS Provision request associated with this NES cell,is currently missing. Based on the following agreement, only one Cell A ID is required to be included:

One “Provision Request message includes one “OD-SIB1 config R19” referring to the TS 38.331 definition, it is a RRC Container in octet string (presence M)  +  one NES Cell ID (presence M ) +  one Cell-A ID (presence O )

Proposal 2: Introduce a NR CGI IE for a Cell A (which either "accepts" or "rejects" the UL WUS Provision request) in the On-Demand SIB1 Cell IE in GNB-CU CONFIGURATION UPDATE message.

Consider above enhancements for OD-SIB1 , we propose to capture the following to the TS38.470:

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and may activate or deactivate cells. The gNB-CU Configuration Update function may activate SSB beams of cells. The gNB-CU Configuration Update function may indicate the cells where the gNB-DU is allowed to deactivate the SSB beams. With the gNB-CU Configuration Update function, energy saving with cell activation and cell deactivation can be supported as defined in TS 38.300 [8]. The gNB-CU Configuration Update function may indicate the acceptance  status of a cell's associated UL WUS Provision which the gNB-DU considers for On-demand SIB1 for network energy saving. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. The gNB-DU Configuration Update function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

Proposal 3: Add the description of the functionality of "the CU indicating the acceptance status of a cell's associated UL WUS Provision to the DU via gNB-CU Configuration Update" into TS 38.470.

Proposal 4: RAN3 agrees the corresponding TPs to BL CR for TS38.470 and TS38.473 in annex.

Regarding the XnAP, since the details have been discussed, and the basic mechanism is stable, the corresponding IE names should be also unchanged. Therefore, the Editor’s Notes in BL CR to 38.420 could be removed.

Proposal 5: Remove the Editor’s Notes in BL CR to 38.420.

3 Conclusion

Proposal 1: Revise the agreement from "The NES gNB-CU sends the indication to DU. The NES gNB-DU MAY go to OD-SIB 1 up to gNB-DU decision" to "The NES gNB-CU sends the indication of the "accept" or "reject" status of the associated UL WUS Provision request of a NES cell to DU. The NES gNB-DU MAY go to OD-SIB 1 up to gNB-DU decision".

Proposal 2: Introduce a NR CGI IE for a Cell A (which either "accepts" or "rejects" the UL WUS Provision request) in the On-Demand SIB1 Cell IE in GNB-CU CONFIGURATION UPDATE message.

Proposal 3: Add the description of the functionality of "the CU indicating the acceptance status of a cell's associated UL WUS Provision to the DU via gNB-CU Configuration Update" into TS 38.470.

Proposal 4: RAN3 agrees the corresponding TPs to BL CR for TS38.470 and TS38.473 in annex.

Proposal 5: Remove the Editor’s Notes in BL CR to 38.420.

Annex: TP to BL CR for TS38.420

Change Begins

5.2.4	Energy saving function

This function enables decreasing energy consumption by indication of cell activation/deactivation, SSB beam activation/deactivation over the Xn interface or enabling the neighbour NG-RAN node(s) to broadcast the UL WUS configuration.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

6.2.5	Energy saving procedures

-	Cell Activation procedure: enables an NG-RAN node to request the activation of a previously deactivated cell or SSB beam hosted in another NG-RAN node.

-    UL WUS Configuration Provision procedure: enables an NG-RAN node to provide UL WUS configuration to another NG-RAN node and request the receiving NG-RAN node to start or stop broadcasting the UL WUS configuration.

-    UL WUS Configuration Transmission Status Update procedure: enables an NG-RAN node to inform the requesting NG-RAN node that the UL WUS configuration broadcasting has stopped.

Change Ends

Annex: TP to BL CR for TS38.470

Change Begins

5.2	F1-C functions

5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU. The F1 setup function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and may activate or deactivate cells. The gNB-CU Configuration Update function may activate SSB beams of cells. The gNB-CU Configuration Update function may indicate the cells where the gNB-DU is allowed to deactivate the SSB beams. With the gNB-CU Configuration Update function, energy saving with cell activation and cell deactivation can be supported as defined in TS 38.300 [8]. The gNB-CU Configuration Update function may indicate the acceptance  status of a cell's associated UL WUS Provision which the gNB-DU considers for On-Demand SIB1 operation for network energy saving. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. The gNB-DU Configuration Update function also allows to send the information of a mobile IAB-MT from the co-located mobile IAB-DU to the gNB-CU.

The F1 setup and gNB-DU Configuration Update functions allow to inform the S-NSSAI(s), NSAG(s), CAG ID(s) and NID(s) supported by the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide information on either RedCap access configuration, or eRedCap access configuration, or both at the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide barring exemption for emergency call information at the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide barring configuration information for 2Rx XR UEs at the gNB-DU.

The F1 setup and gNB-DU Configuration Update functions allow to provide the UL WUS configuration information for on-demand SIB1 at the gNB-DU.

The F1 setup and gNB-CU Configuration Update functions allow to inform the NID(s) available at the gNB-CU.

The F1 resource coordination function is used to transfer information about frequency resource sharing between gNB-CU and gNB-DU. In case of split gNB architecture, the gNB-CU may consolidate the outgoing messages from multiple gNB-DUs and distribute the incoming messages to the involved gNB-DUs, to perform resource coordination.

The gNB-DU status indication function allows the gNB-DU to indicate overload status to gNB-CU. In case of IAB, the IAB-donor-DU or IAB-DU can indicate the downlink congestion status to the IAB-donor-CU.

The network access rate reduction function is used to indicate to the gNB-DU that the rate at which UEs are accessing the network need to be reduced.

The F1 removal function is used to remove the interface instance and all related resources between the gNB-DU and the gNB-CU in a controlled manner.

The F1 TA information transfer function is used to transfer the TA information from the gNB-DU to the gNB-CU and the gNB-CU to the gNB-DU.

Change Ends

Annex: TP to BL CR for TS38.473

<<<<<<<<<<<<<<<<<<<<Start Change >>>>>>>>>>>>>>>>>>>>

8.2.5	gNB-CU Configuration Update

8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the Cells Allowed to be Deactivated List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider that it is allowed to deactivate the SSB beams within the indicated cells for network energy saving purpose.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If the On-Demand SIB1 Cell IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it for On-Demand SIB1 operation.

<<<<<<<<<<<<<<<<<<<<Next Change >>>>>>>>>>>>>>>>>>>>

9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >>gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| >>MBS Broadcast Neighbour Cell List | O |  | 9.3.1.226 |  | YES | ignore |
| >>SSBs within the cell to be Activated List | O |  | 9.3.1.326 | List of SSB beams within the cell requested to be activated. | YES | reject |
| <<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>> |  |  |  |  |  |  |
| Extended gNB-CU Name | O |  | 9.3.1.206 |  | YES | ignore |
| Cells Allowed to be Deactivated List |  | 0..1 |  |  | YES | ignore |
| >Cells Allowed to be Deactivated List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| On-Demand SIB1 Cell |  | 0..1 |  |  | YES | ignore |
| >NR CGI | M |  | 9.3.1.12 | This IE indicates a NES cell in the gNB-DU. | - |  |
| >NR CGI of Assisting Cell | M |  | 9.3.1.12 |  |  |  |
| >On-Demand SIB1 Indicator | M |  | ENUMERATED (accept, reject, ...) | This IE indicates the acceptance status by the indicated assisting cell regarding the UL WUS Provision request associated with the indicated NES cell. | - |  |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |
| maxnoofSSBAreas | Maximum no. SSB Areas that can be served by a cell. Value is 64. |

<<<<<<<<<<<<<<<<<<<<Next Change >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<<<<<<<<<<<<<<<<<<<< Unmodified Text Omitted >>>>>>>>>>>>>>>>>>>>

On-Demand-SIB1-Cell ::= CHOICE {

nRCGI							NRCGI,

nRCGIOfAssistingCell			NRCGI,

on-demandSIBIndindicator		ENUMERATED{accept, reject, ...},

choice-extension				ProtocolIE-SingleContainer { { On-Demand-SIB1-Cell-ExtIEs} }

}

On-Demand-SIB1-Cell-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End Change >>>>>>>>>>>>>>>>>>>>