3GPP TSG-RAN WG3 Meeting #129	R3-255307

Bengaluru, India, 25 - 29 August 2025

Agenda Item:	17.4

Source:	ZTE Corporation

Title:		(TP to BL CR for 38.470) Support of Paging Adaptation

Document for:	Discussions & Approval

1	Introduction

Paging adaptation has already been supported over F1AP, as agreed in previous RAN3 meetings. A new indicator for paging adaptation capability, added to the UE Paging Capability IE in the F1AP PAGING message, has also been introduced in F1AP BLCR.

Furthermore, in Rel-19, PEI has also been enhanced to support paging adaptation. RAN2 has agreed to introduce a new UE capability for PEI adaptation per band. We also believe it is necessary for the CU to indicate such PEI adaptation capability for each paging cell in the F1AP PAGING message.

Considering the above F1AP enhancements for paging adaptation in support of network energy saving, we propose to update TS 38.470 to reflect this new functionality.

Proposal：Update TS 38.470 to reflect the new functionality of paging adaptation in support of network energy saving.

2	TP for 38.470

<<<<<<<<<<<<<<<<<<<< START OF CHANGE >>>>>>>>>>>>>>>>>>>>

5.2.5	Paging function

The gNB-CU is responsible for filtering target cells for paging based on the UE Radio Capability for Paging. The gNB-CU may further send a list of recommended SSB beams to the gNB-DU for paging of UEs in RRC_INACTIVE.

The gNB-DU is responsible for transmitting the paging information according to the scheduling parameters provided. The gNB-DU also takes the UE paging capability into account for paging when provided by the gNB-CU.

The gNB-CU provides paging information to enable the gNB-DU to calculate the exact PH, if the eDRX is configured, PO and PF. The gNB-CU determines the PA. The gNB-DU consolidates all the paging records for a particular PH, PO, PF and PA, and encodes the final RRC message and broadcasts the paging message on the respective PH, PO, PF in the PA.

The paging function also supports CN controlled subgrouping paging for UE Power Saving. The paging function also supports paging adaptation for Network Energy Saving.

<<<<<<<<<<<<<<<<<<<<END OF CHANGE >>>>>>>>>>>>>>>>>>>>