3GPP TSG-RAN3 WG3 Meeting #129	R3-255313

Bengaluru, India, 25 – 29 August 2025

Agenda Item:	16.2

Source: 	Qualcomm Incorporated

Title: 	Interface management procedures and Reader selection

Document for:	Discussion/Decision

# Background

In this paper, we provide our views on remaining issues related to interface management and functionalities to support Topology 1 of AIoT Topology.

# Discussion

In RAN3#127, RAN3 agreed to support the scenario where a gNB only supports communication with A-IoT devices over A-IoT radio and this is also captured in the BL CR to 38.300.

Besides the legacy deployment scenario (i.e., gNB supporting only Uu), RAN3 needs to discuss how to support different deployment scenarios as shown in the figure below:

- gNB supporting only A-IoT radio with direct connectivity to AIOTF (case 1)

- gNB supporting only A-IoT radio with indirect connectivity to AIOTF (case 2)

- gNB supporting both Uu and AIoT with direct connectivity to AIOTF (case 3)

- gNB supporting both Uu and AIoT with indirect connectivity to AIOTF (case 4)

There are several deployment scenarios possible for A-IoT based on whether a gNB supports A-IoT and/or NR Uu and depending on whether a direct or indirect connectivity to AIOTF is supported by that gNB.

For the above deployment scenarios, RAN3 further needs to discuss how to handle the A-IoT non applicable mandatory IEs.

## NG Setup Request

The following agreements and working assumptions were achieved in the previous meetings:

RAN3 agreed to introduce an AIoT indicator in the NGAP Setup Request message. The detail of this indicator will be further discussed.

WA: Define the A-IoT Support IE in the NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu, …)

Currently, NG Setup Request contains some mandatory IEs (e.g., TA List, TAI Slice Support List, Default Paging DRX) which are not applicable to A-IoT.

NG Setup Request contains some mandatory IEs (e.g., Supported TA List, TAI Slice Support List, Default Paging DRX) which are not applicable to A-IoT

We think we can handle the different deployment scenarios (cases 1-4 mentioned in the background section) and the mandatory IEs by including an “A-IoT support” IE in NG Setup Request with appropriate code-points – {A-IoT only, A-IoT and NR Uu}. Therefore, the following is proposed.

Alternatively, there were some proposals to extend the RAT Information IE in NG Setup Request to include a codepoint for A-IoT. Even if we define such a new codepoint for A-IoT, that will not be enough to distinguish a gNB supporting only A-IoT vs. gNB support both A-IoT and Uu. Also, this RAT Information IE is associated with the TAC of the indicated PLMNs, whereas we agreed to define a new A-IoT area (to not reuse TAC) for A-IoT. Hence, it is not appropriate to reuse the existing IE as well.

Including a new codepoint for A-IoT in RAT Information IE in NG Setup Request can’t distinguish a gNB supporting only A-IoT vs. a gNB supporting both A-IoT and NR Uu. Also the RAT Information IE is associated with the TAC of the indicated PLMN(s) and is hence not suitable to reuse this IE (as we agreed on a new A-IoT area and not reuse TAC).

Including an “A-IoT support” IE in NG Setup Request with appropriate code-points e.g., {A-IoT only, A-IoT and NR Uu} can distinguish a gNB supporting only A-IoT vs. a gNB supporting both A-IoT and NR Uu and handle the mandatory IEs in the receiver appropriately.

| >>RAT Information | O |  | 9.3.1.125 | RAT information associated with the TAC of the indicated PLMN(s). | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

Turn the previous WA:Define the A-IoT Support IE in NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu,..) into agreement

If this “A-IoT Support” IE is included in NG Setup Request and is set to “A-IoT only”, the AMF can ignore those IEs that are not applicable to A-IoT (e.g., TAI Slice Support List, Default Paging DRX) and consider only the A-IoT applicable IEs upon receiving the NG Setup Request. Further, gNB can include random/junk values in those A-IoT non-applicable mandatory IEs.

If this “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT and NR Uu”, the AMF will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

If the “A-IoT support” IE is included in NG Setup Request and is set to “A-IoT only”, the AMF (in case of indirect connectivity) can ignore those IEs that are not applicable to A-IoT (e.g., TAI Slice Support List, Default Paging DRX) and consider only the A-IoT applicable IEs upon receiving the NG Setup Request.

If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT and NR Uu”, the AMF (in case of indirect connectivity) will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

## NG Setup Response

NG Setup Response contains some mandatory IEs (e.g., AMF Name, GUAMI, Relative AMF Capacity), all of which are not relevant to A-IoT e.g., there is no GUAMI at A-IoT device or there is no requirement to support AMF load balancing for which Relative AMF Capacity was defined.

To handle the A-IoT non-applicable mandatory IEs, the following open issue was also captured in the previous meeting:

Whether to have A-IoT Support Indicator in the NG SETUP RESPONSE message?

NG Setup Response includes some mandatory IEs (e.g., AMF Name, GUAMI, Relative AMF Capacity), all of which are not applicable to A-IoT

There were several options discussed in the previous RAN3 meeting, but there was no consensus:

- Option 1: A-IoT Support Indication

- Option 2: known via OAM, gNB can ignore the not applicable IEs in case of direct connectivity.

- Option 3: reuse existing IE (e.g., AMF name) to indicate that the CN node is AIOTF

We think none of the options are necessary and this can be handled at the gNB based on what it sent in NG Setup Request.

If the gNB includes an “A-IoT support” IE with codepoint “A-IoT only” in NG Setup Request, gNB can ignore the A-IoT non applicable mandatory values (e.g., AMF Name, GUAMI, Relative AMF capacity) upon receiving this in NGAP Setup Response. Otherwise, it can consider those IEs. In the former case, AMF/AIOTF can include random/junk values in those A-IoT non applicable mandatory IEs

If the gNB includes an “A-IoT support” IE with codepoint “A-IoT only” in NG Setup Request, gNB can ignore the A-IoT non applicable mandatory values (e.g., AMF Name, GUAMI, Relative AMF capacity) upon receiving them in NG Setup Response. Otherwise, the gNB considers those IEs.

## Reader selection

The following was agreed in RAN3#127:

AIoT RAN node receives the requested service area information (encoded as area and/or reader ID list) from AIoT CN.

Upon receiving only the area in Inventory Request, the gNB selects readers within the indicated area

Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers

WA: Upon receiving only the reader list in Inventory Request, the gNB selects the readers indicated by the reader list

FFS: Upon receiving both the area and the reader list in Inventory Request, the gNB selects the readers within the indicated area and the readers within the reader list

Reader Selection refers to SA2 TS 23.369 clause 5.3.3. Further check is needed.

In case the AIOTF indicates a list of readers in which it wants to perform inventory, there was discussion last meeting whether the gNB can perform the “final” selection of readers (e.g., can reject the readers indicated by the AIOTF or can select new readers outside the indicated list).

Some of the reasons provided by the proponents were that the gNB might be doing some coverage adaptation (e.g., due to CCO) and should therefore decide whether it can use the reader. In our view, this does not require the gNB to have the final say. The gNB can still try using that reader via repeated A-IoT paging attempts and the inventory might fail in the worst case. But there is no reason for gNB to ignore the readers indicated by the AIOTF.

Further in our view, the gNB should not override the selection of AIOTF as the gNB might not know why the AIOTF requested the gNB to use a certain reader e.g., this could be a reader in the coverage overlap of multiple gNBs which can be also used for the A-IoT devices not in the gNB’s coverage or this could be a “special” reader which the AF/AIOTF wants to use. In case the AIOTF only indicates an area, gNB can “select” a subset of readers within the indicated area as already agreed in RAN3.

An AIOTF may want to use specific readers for performing Inventory (e.g., those readers in the coverage overlap of multiple gNBs or in a certain location) and the gNB might not have such knowledge.

A gNB performing some link adapdation of Readers (e.g., Reader’s coverage adjustment or Reader’s Tx/Rx power adaptation) can still attempt to perform inventory using the list of readers indicated by the AIOTF and perform multiple rounds of inventory if it fails. There is no need for gNB to downselect or upselect from the list of readers indicated by the AIOTF.

We therefore propose to turn the WA and FFS into agreement as well to cover all possible scenarios.

Upon receiving only the reader list in Inventory Request, the gNB uses all the readers indicated by the reader list to perform inventory.

Upon receiving both the area and the reader list in Inventory Request, the gNB selects all the readers within the reader list and may select additional readers within the indicated area.

# Summary

Based on the discussion above, we have following observations and proposals:

Observation 1:	There are several deployment scenarios possible for A-IoT based on whether a gNB supports A-IoT and/or NR Uu and depending on whether a direct or indirect connectivity to AIOTF is supported by that gNB.

2.1 NG Setup Request

Observation 2:	NG Setup Request contains some mandatory IEs (e.g., Supported TA List, TAI Slice Support List, Default Paging DRX) which are not applicable to A-IoT

Observation 3:	Including a new codepoint for A-IoT in RAT Information IE in NG Setup Request can’t distinguish a gNB supporting only A-IoT vs. a gNB supporting both A-IoT and NR Uu. Also the RAT Information IE is associated with the TAC of the indicated PLMN(s) and is hence not suitable to reuse this IE (as we agreed on a new A-IoT area and not reuse TAC).

Observation 4:	Including an “A-IoT support” IE in NG Setup Request with appropriate code-points e.g., {A-IoT only, A-IoT and NR Uu} can distinguish a gNB supporting only A-IoT vs. a gNB supporting both A-IoT and NR Uu and handle the mandatory IEs in the receiver appropriately.

Proposal 1:	Turn the previous WA:Define the A-IoT Support IE in NG Setup Request as ENUMERATED (A-IoT only, A-IoT and NR Uu,..) into agreement

Proposal 2:	If the “A-IoT support” IE is included in NG Setup Request and is set to “A-IoT only”, the AMF (in case of indirect connectivity) can ignore those IEs that are not applicable to A-IoT (e.g., TAI Slice Support List, Default Paging DRX) and consider only the A-IoT applicable IEs upon receiving the NG Setup Request.

Proposal 3:	If the “A-IoT support” flag is included in NG Setup Request and is set to “A-IoT and NR Uu”, the AMF (in case of indirect connectivity) will consider the A-IoT applicable IEs for A-IoT communication and the A-IoT non-applicable IEs for Uu communication.

2.2 NG Setup Response

Observation 5:	NG Setup Response includes some mandatory IEs (e.g., AMF Name, GUAMI, Relative AMF Capacity), all of which are not applicable to A-IoT

Proposal 4:	If the gNB includes an “A-IoT support” IE with codepoint “A-IoT only” in NG Setup Request, gNB can ignore the A-IoT non applicable mandatory values (e.g., AMF Name, GUAMI, Relative AMF capacity) upon receiving them in NG Setup Response. Otherwise, the gNB considers those IEs.

2.3 Reader selection

Observation 6:	An AIOTF may want to use specific readers for performing Inventory (e.g., those readers in the coverage overlap of multiple gNBs or in a certain location) and the gNB might not have such knowledge.

Observation 7:	A gNB performing some link adapdation of Readers (e.g., Reader’s coverage adjustment or Reader’s Tx/Rx power adaptation) can still attempt to perform inventory using the list of readers indicated by the AIOTF and perform multiple rounds of inventory if it fails. There is no need for gNB to downselect or upselect from the list of readers indicated by the AIOTF.

Proposal 5:	Upon receiving only the reader list in Inventory Request, the gNB uses all the readers indicated by the reader list to perform inventory.

Proposal 6:	Upon receiving both the area and the reader list in Inventory Request, the gNB selects all the readers within the reader list and may select additional readers within the indicated area.