3GPP TSG-RAN3 Meeting #129	R3-255317

Bengaluru, India, Aug 25 - 29, 2025

Agenda Item:	11.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on remaining issues in AI/ML enabled CCO

Document for:	Discussion/Decision

# Background

In this paper we provide our views on the final remaining open issues in AI/ML enabled CCO.

RAN3#128 Adopt Opt2 (gNB-CU directly forwards the received further CCO state of neighbor cells to gNB-DU) Timing information for predicted CCO issue over Xn is not needed? Current mechanism enables the update of predicted CCO issue and/or future CCO state. A new detected CCO issue/a new predicted CCO issue for the same affected cells and beams after a predicted CCO issue will cancel the prediction. Check the scenario whether has the possibility that there will be isolated multiple CCO issues detected for different cells or beams? Specific Cancel towards each CCO issue is needed? Specify mechanisms (F1, Xn) to update/cancel a prediction, no new IE is needed to identify the previous prediction (affected cells and beams will identify the previously signalled issue). Addition of new code-point in predicted coverage modification cause (Xn) / predicted CCO issue (F1), with details FFS.

# Discussion

## Exchange Time for Predicted CCO Issue over Xn

It has been proposed since few meetings by some companies to send Time for Predicted CCO issue to neighbouring CU over Xn and neighbouring CU can pass the same information to neighbouring DU. Currently the Predicted CCO issue is passed as Predicted Coverage Modification cause. Some companies in the previous meeting mentioned that if Predicted CCO issue time is provided to the neighbouring node, the neighbouring node can decide on a CCO state change time to align with the time associated with predicted CCO issue. Additional information on “time for predicted CCO issue” at the source may assist the neighbouring nodes in better decision on future CCO state. It is ok to exchange this information between neighbours.

Exchange Time for Predicted CCO issue (“Time for Predicted CCO issue” IE) to the neighbouring CU over Xn along with future CCO state and associated time as an additional information to assist the neighbouring node to derive its own future CCO state at an appropriate future time instance.

## Update/Cancel Predicted CCO Issue

In the last RAN3 meeting it was agreed to specify mechanism to update or cancel a previously predicted CCO issue. It was also agreed not to introduce new IE for the same. There was also a proposal to say that missing Predicted CCO issue or cause will indicate cancellation. There was some support to introduce a new code point in the Predicted coverage modification cause value to indicate cancellation of the prediction.

We prefer to explicitly indicate cancel using a new code point “cancel” in the Predicted coverage modification cause value. This would mean the both Predicated CCO issue and its corresponding future CCO state is cancelled.

Introduce a new codepoint “cancel” in Predicted Coverage Modification Cause value over Xn to indicate cancellation of both Predicted CCO issue and its corresponding Future CCO state.

Introduce a new codepoint “cancel” in Predicted CCO Issue value over F1 to indicate cancellation of Predicted CCO issue.

## Future CCO State Feedback

It was agreed in the last meeting to use legacy UE performance for feedback of predicted CCO issue and future CCO state. The coverage or capacity change is for a cell and not for a UE. With legacy UE performance it is unclear which cell the UE belongs to (whether it is the cell where the future CCO state was applied or a normal cell). It is useful to enhance the legacy UE performance by including the Cell ID where the UE belong to along with the performance metrics as feedback, so that the source can know if the UE is an impacted UE due to Coverage or capacity change in the cell where the UE currently is.

Along with legacy UE performance, legacy SON reports like RLF report, RACH failure reports can also be used to understand if coverage change has a positive impact on UEs or not.

Agree to include current NR-CGI of the UE in the legacy UE Performance as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

Legacy SON reports like RLF, RACH failure reports can also be used as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn. No spec impact is expected with this.

# Summary

Based on these observations and discussion above, we have the following proposals:

Proposal 1:	Exchange Time for Predicted CCO issue (“Time for Predicted CCO issue” IE) to the neighbouring CU over Xn along with future CCO state and associated time as an additional information to assist the neighbouring node to derive its own future CCO state at an appropriate future time instance.

Proposal 2:	Introduce a new codepoint “cancel” in Predicted Coverage Modification Cause value over Xn to indicate cancellation of both Predicted CCO issue and its corresponding Future CCO state.

Proposal 3:	Introduce a new codepoint “cancel” in Predicted CCO Issue value over F1 to indicate cancellation of Predicted CCO issue.

Proposal 4:	Agree to include current NR-CGI of the UE in the legacy UE Performance as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn.

Proposal 5:	Legacy SON reports like RLF, RACH failure reports can also be used as feedback for the predicted CCO issue and future CCO state exchanged between neighbours over Xn. No spec impact is expected with this.