3GPP TSG-RAN WG3 Meeting #129	R3-255328

Bengaluru, India, 25th ~29th Aug, 2025

Title:	[TPs to TS 38.300, 38.413 BL CRs] A-IoT Session Release and Inventory Complete Indication

Agenda Item:	16.2

Source:	Huawei, China Telecom, China Unicom, Lenovo

Document for:	other

# Introduction

In last RAN3 meeting, several agreements and leftover issues were listed about A-IoT Session Release and Inventory Complete Indication.

In this contribution, we further discuss related leftover issues.

# 2	Discussion

## 2.1	The interaction between the two Session Release procedures

Introduce new CN triggered Class 1 Session Release procedure, which includes A-IoT SESSION RELEASE REQUEST message and A-IoT SESSION RELEASE RESPONSE message. Introduce a class2 procedure to allow NG-RAN node to trigger Session Release procedure towards AIoT CN. - The name of the procedure, the name of the messages and the purpose of the procedure are FFS. - The procedure text regarding the interaction with the A-IoT Session Release procedure is FFS.

In last meeting, there was a question about whether there has to be a CN triggered Class 1 Session Release procedure after the gNB triggerred class2 release procedure.

In NGAP, for UE Context Release, PDU Session Release, Broadcast Sesssion Release, handshake between gNB and CN are always required. it is not reliable to release a session based on a class 2 procedure.

There was also a question about inventory only case, can the gNB release the session after providing the inventory complete indication in the Inventory Report Transfer IE to the AIOTF. Besides the reliable issue mention above, in this solution, as the Inventory Report Transfer IE is transparent to the AMF, in indirect connectivity, the AMF is not able to release the session context.

Therefore, the CN needs to trigger a class 1 release procedure to release stored context, NG resources and A-IoT radio resources at the gNB, with or without the gNB triggered class 2 Session Release procedure. In other words, the Class 1 CN triggered session release procedure should be performed at the end for all scenarios.

There was a comment about whether the gNB has to wait for the CN initiated session release before start another session over the radio, to address this, maybe we can clarify that after requests a session release procedure, the gNB can start another session over the air interface of the same reader(s), without waiting for the completion of the CN release procedure for the first session.

A-IoT Session release should not only rely on the class 2 A-IoT Session Release Request procedure, the Class 1 CN triggered A-IoT Session Release procedure should be performed at the end for all scenarios.

May clarify that after triggers the A-IoT Session Release Request procedure, the gNB can start another A-IoT session over the air interface of the same reader(s), without waiting for the completion of the CN triggered A-IoT Session Release procedure of the first session.

## 2.2	When and how to provide Inventory Complete indication

In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the complete of the triggered Inventory session. - FFS on the detail about Inventory Complete indication via new procedure or introducing indication IE in current Inventory Report message. - In the case of Command after Inventory scenario, whether need to introduce the complete indication needs to be further checked.

TS 23.369: ### 7.2.3 Naiotf_AIoT_Command service operation Service operation name: Naiotf_AIoT_Command. Description: The NF consumer requests a command operation for one or multiple AIoT Device(s). Inputs, Required: 1) AF ID. 2) At least one of the following parameters are included: - Target Area information for the command operation. - Information about the target AIoT Device(s): - either the AIoT Device ID(s) or the filtering information(see clause 5.8) for multiple AIoT Devices. 3) Notification Endpoint. 4) Command Type: Read, Write or Permanent Disable. Inputs, Optional: Information to be used for resource allocation: - Approximate number of AIoT Devices. - Approximate message size from the AIoT Device for Read Operation. - If the Command Type is Read, the offset to read application data from and the length of application data to read shall be included. - If the Command Type is Write, the offset where to write the application data, the application data to write and its length shall be included. Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure. Outputs, Optional: None.

TS 23.369: ### 6.2.3 Command Procedure 8. For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size], RAN AIoT Device NGAP ID for each AIoT Device) to the NG-RAN directly or as a NGAP AIoT information via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data. The Correlation ID is as the same as the Correlation ID generated in step 4. The RAN AIoT Device NGAP ID for each AIoT Device is used by the NG-RAN to determine the AIoT device context in NG-RAN as specified in TS 38.413 [10]. The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2. NOTE: Command Request(s) can be sent to NG-RAN when inventory procedure is ongoing.

About whether need to introduce the complete indication in the case of Command after Inventory scenario, from our view, the AIOTF also needs to get the Inventory Complete Indication from the gNB to know the completion of Inventory, otherwise, the AIOTF has to wait and triggers the Session Release after the timer expires, radio resource waste and latency to complete a session are foreseen.

In Command after Inventory scenario, the gNB also needs to send Inventory Complete Indication to the A-IoT CN node.

About how to provide the Inventory Complete Indication to the AIoT CN, there are several options:

- Option 1: Introduce an Inventory Completed Indication IE in the Inventory Report Transfer IE, and change the Reader Report List IE to optional.

- Option 2: Change the Class 2 release request procedure name to A-IoT Session Notification, with two branches “Inventory Complete Indication” and “release request indication” in the message.

- Option 3: introduce a new Class 2 procedure, named as e.g., “Inventory Complete procedure”.

All the above 3 options are workable, considering of specification impacts, the isolation of Inventory procedures and the Session Release Request procedure, we would like to support option 1 to address this issue.

Introduce Inventory Completion Indication IE in the Inventory Report Transfer IE, and change the Reader Report List IE to optional.

# 3	Conclusion

In this contribution, we discussed A-IoT Session Release and Inventory Complete, got the following proposals:

- A-IoT Session release should not only rely on the class 2 A-IoT Session Release Request procedure, the Class 1 CN triggered A-IoT Session Release procedure should be performed at the end for all scenarios.

May clarify that after triggers the A-IoT Session Release Request procedure, the gNB can start another A-IoT session over the air interface of the same reader(s), without waiting for the completion of the CN triggered A-IoT Session Release procedure of the first session.

In Command after Inventory scenario, the gNB also needs to send Inventory Complete Indication to the A-IoT CN node.

Introduce Inventory Completion Indication IE in the Inventory Report Transfer IE, and change the Reader Report List IE to optional.

The corresponding TPs are provided in section 4 and 5.

# 4	TP to TS 38.300 TP

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 16.xx.x3	Inventory procedure

Figure 16.xx.x3-1 depicts the basic communication between the gNB and the A-IoT CN node for the Inventory procedure.

Figure 16.xx.x3-1: Inventory procedure

1.	The A-IoT CN node initiates the Inventory procedure over NG-C by sending the Inventory Request message to the gNB. The Inventory Request message includes a Correlation ID and A-IOTF Identifier to identify the inventory session. The Inventory Request message also includes an A-IoT Device Identification Requested corresponding to the A-IoT device(s) which are targeted for the inventory. The A-IoT Device Identification Requested may indicate inventory for a single device, a group of devices or for all devices.

The Inventory Request message also includes Requested Service Area Information (list of A-IoT Area IDs and/or reader list), the Expected D2R message size and may include Inventory Assistance Information e.g., Approximate number of Target A-IoT devices.

Editor’s Note 1:	Whether the Expected D2R message size is mandatory is FFS.

The Inventory Request message may also include a follow-on command indicator to indicate that at least for one of the targeted A-IoT device(s) a Command procedure will follow.

Editor’s Note 2:	Specification of further parameters of the Inventory Request message (e.g., other assistance information from 5GC) needs further work.

2.	The gNB allocates and co-ordinates the usage of A-IoT radio resources.

3.	The gNB confirms the request from the A-IoT CN node by replying with the Inventory Response message. The Inventory Response message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message.

NOTE 2:	If the gNB is not able to perform the Inventory procedure, it rejects the request and sends an Inventory Failure message to the A-IoT CN node.

4.	The gNB performs the Inventory procedure towards the A-IoT device(s) over the A-IoT radio interface.

5./6.	Upon receiving the inventory result from the A-IoT device(s), the gNB sends the Inventory Report message to the A-IoT CN node. If the Inventory procedure concerns multiple A-IoT devices, multiple Inventory Report messages may be sent to the A-IoT CN node.

The Inventory Report message includes the AIOTF Identifier and the Correlation ID received in the Inventory Request message and further includes the A-IoT NAS PDU(s) carrying the inventoried A-IoT device ID(s).

If a follow-on Command indicator was received at step 1, the Inventory Report message also includes a RAN A-IoT device NGAP ID for each A-IoT device which was inventoried.

NOTE 3:	If the Inventory procedure concerns multiple A-IoT devices, an Inventory Report may include reports from multiple A-IoT devices.

Upon completion of the inventory procedure, the gNB sends an Inventory Complete Indication in the last Inventory Report message.

NOTE x:	If there is no follow-on Command, after sending the Inventory Complete Indication, the gNB may start another A-IoT Session over the A-IoT radio immediately.

7.	If there is no follow-on Command, the A-IOT CN triggers the A-IoT CN node initiated A-IoT Session Release procedure as defined in clause 16.xx.x5.

### 16.xx.x4	Command procedure

Figure 16.xx.x4-1 depicts the basic communication between the gNB and the A-IoT CN node for the Command procedure.

Figure 16.xx.x4-1: Command procedure

The Command procedure addresses only one A-IoT device.

1.	Prior to the Command procedure, the Inventory procedure is performed involving the concerned device and potentially other A-IoT devices.

NOTE 4: the step 2/3/4 may happen in parallel of the Inventory procedure involving other A-IoT devices or in parallel to other command procedures targeting other A-IoT devices within the same session.

2.	The A-IoT CN node initiates the Command procedure over NG-C by sending the Command Request message to the gNB in order to send a command to a single A-IoT device. The Command Request message includes the same Correlation ID and AIOTF Identifier as the ones used in its corresponding inventory procedure.

The Command Request also includes the RAN A-IoT device NGAP ID and the A-IoT NAS PDU corresponding to the targeted A-IoT device and may include some command assistance information. This command assistance information may contain the estimate of expected D2R message size, command type (FFS).

3.	The gNB allocates and co-ordinates the usage of A-IoT radio resources and performs the command procedure over the A-IoT radio interface.

4.	The gNB replies to the Command Request with the Command Response messageincluding the AIOTF Identifier the Correlation ID and the RAN A-IoT device NGAP ID received in the Command Request, and the A-IoT NAS PDU received from the device.

NOTE 5:	If the gNB is not able to perform the Command procedure, it rejects the request and sends a Command Failure message to the A-IoT CN node.

5.	Upon completion of all the transmission for all related devices within the session, the A-IOT CN triggers the A-IoT CN node initiated A-IoT Session Release procedure as defined in clause 16.xx.x5.

### 16.xx.x5	A-IoT CN node initiated A-IoT Session Release procedure

Figure 16.xx.x5-1 depicts the A-IoT CN node initiated A-IoT Session Release procedure, which is enable the A-IoT CN node to order the release of the A-IoT session context and resources related to a previously established A-IoT Session.

Figure 16.xx.x5-1: A-IoT CN node initiated A-IoT Session Release procedure

1.	The A-IoT CN node initiates the A-IoT Session Release procedure over NG-C by sending the A-IoT Session Release Command message to the gNB. The A-IoT Session Release Command message includes the Correlation ID and the AIOTF Identifier to identify the session to be released.

2.	The gNB releases the session context, the associated NG resources and the A-IoT radio resources.

3.	The gNB confirms the release to the A-IoT CN node by replying with the A-IoT Session Release Complete message. The A-IoT Session Release Complete message includes the AIOTF Identifier and the Correlation ID received in the A-IoT Session Release Command message.

### 16.xx.x6	gNB initiated A-IoT Session Release procedure

Figure 16.xx.x6-1 depicts the gNB initiated A-IoT Session Release Request procedure, which is to enable the gNB to request the A-IoT CN node to release the A-IoT session context and resources related to a previously established A-IoT Session.

Figure 16.xx.x6-1: gNB initiated A-IoT Session Release procedure

1.	The gNB initiates the A-IoT Session Release Request procedure over NG-C by sending the A-IoT Session Release Request message to the A-IoT CN node in order to request the release of an A-IoT session. The A-IoT Session Release Request message includes the Correlation ID and the AIOTF Identifier to identify the session requested to be released.

NOTE y:	After step 1, the gNB may start another A-IoT Session over the A-IoT radio immediately.

2.	The A-IoT CN node accepts the request and triggers the CN initiated A-IoT Session Release procedure.

3.	The CN initiated A-IoT Session Release procedure is performed as described in section 16.xx.x5.

<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>

# 4	TP to TS 38.413 TP

<<<<<<<<<<<<<<<<<<<< First Change >>>>>>>>>>>>>>>>>>>>

### 8.xx.2	Inventory Report

#### 8.xx.2.1	General

The purpose of the Inventory Report procedure is to enable the NG-RAN node to provide Inventory Report to the A-IoT CN node following a successful Inventory Request procedure. This procedure applies only if the NG-RAN node is a gNB.

#### 8.xx.2.2	Successful Operation

Figure 8.xx.2.2-1: Inventory Report

The NG-RAN node initiates the procedure by sending an INVENTORY REPORT message for the Inventory service.

Interactions with the A-IoT Session Release procedure:

In case the Inventory Complete Indication IE is included in the INVENTORY REPORT message and set to “true”, if there is no follow-on command to be transmitted, or all the follow-on command transmissions have been completed, the A-IoT CN node should initiate the A-IoT Session Release procedure for the A-IoT session denoted by the AIOTF Identifier IE and the A-IoT Correlation Identifier IE contained in the INVENTORY REPORT message.

#### 8.xx.2.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 8.xx.4	A-IoT Session Release

#### 8.xx.4.1	General

The purpose of the A-IoT Session Release procedure is to enable the A-IoT CN node to terminate all activities related to the indicated A-IoT session. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

#### 8.xx.4.2	Successful Operation

Figure 8.xx.4.2-1: A-IoT Session Release, successful operation.

The A-IoT CN node triggers the procedure by sending a AIOT SESSION RELEASE COMMAND message to the NG-RAN node. Upon reception of AIOT SESSION RELEASE COMMAND the NG-RAN node shall terminate all activities and release the context and resources related to the A-IoT session denoted by the AIOTF Identifier IE and the Correlation ID IE and respond with an AIOT SESSION RELEASE COMPLETE message.

#### 8.xx.4.3	Unsuccessful Operation

Not applicable.

#### 8.xx.4.4	Abnormal Conditions

Void.

### 8.xx.5	A-IoT Session Release Request

#### 8.xx.5.1	General

The purpose of the A-IoT Session Release Request procedure is to enable the NG-RAN node to request the A-IoT CN node to release the A-IoT session. This procedure applies only if the NG-RAN node is a gNB. The procedure uses non-UE associated signalling.

#### 8.xx.5.2	Successful Operation

Figure 8.xx.5.2-1: A-IoT Session Release Request

The NG-RAN node initiates the procedure by sending an AIOT SESSION RELEASE REQUEST message to the A-IoT CN node.

Interactions with the A-IoT Session Release procedure:

Upon reception of the AIOT SESSION RELEASE REQEUST message, the A-IoT CN node should initiate the A-IoT Session Release procedure for the A-IoT session denoted by the AIOTF Identifier IE and the A-IoT Correlation Identifier IE contained in the AIOT SESSION RELEASE REQUEST message.

#### 8.xx.5.3	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

#### 9.3.x.4	Inventory Report Transfer

This IE provides the inventory report related information from the NG-RAN node to the AIOTF.

In indirect communication, this IE is transparent to the AMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| A-IoT Correlation Identifier | M |  | 9.3.3.xx1 |  |
| Global gNB ID | M |  | 9.3.1.6 |  |
| Reader Report List |  | 0..1 |  |  |
| >Reader Report Item |  | 1..< maxnoofReaders> |  |  |
| >>Reader Index | M |  | 9.3.1.aa1 |  |
| >>Device Report List |  | 1 |  |  |
| >>>Device Report Item |  | 1..<maxnoofDevices> |  |  |
| >>>>A-IoT NAS PDU | M |  | OCTET STRING |  |
| >>>>RAN A-IoT Device NGAP ID | O |  | 9.3.3.yy1 |  |
| Inventory Complete Indication | O |  | ENUMERATED (true, …) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofReaders | Maximum no. of Readers served by a gNB. Value is 65535. |
| maxnoofDevices | Maximum no. of Devices within a reader. Value is 65535. |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

NGAP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-Access (22) modules (3) ngap (1) version1 (1) ngap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

//skip unchanged part

InventoryReportTransfer ::= SEQUENCE {

correlationIdentifier			AIoT-CorrelationIdentifier,

globalgNB-ID					GlobalGNB-ID,

readerReportList				AIoT-ReaderReportList												OPTIONAL,

inventoryCompleteIndication		ENUMERATED {true, ...}												OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { InventoryReportTransfer-ExtIEs} }		OPTIONAL,

...

}

InventoryReportTransfer-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>