3GPP TSG-RAN WG3 Meeting #129	R3-255348

Bangaluru, IND, 25th – 29th August, 2025

Agenda item:	22.2

Source: 	Samsung

Title: 	Remaining issues on multi-hop for SL relay

Document for:	Discussion and decision

Introduction

Last meeting discussed the multi-hop SL relay, and the following agreements and open issues are captured,

Reuse the Peer UE ID IE as counterpart information with the update for the semantics description.

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

FFS whether the same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

FFS whether upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1.

FFS whether to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel is needed can be further discussed in next meeting.

In this contribution, we further discuss this open issue.

Discussion

Last meeting continued to discuss details on multi-hop relay.

The first open issue is the following WA,

WA: RAN3 supports that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

According to the discussion last meeting, most of the companies believed that the above WA is aligned with the design of single-hop relay, and will bring minimum RAN3 spec impact. The only thing RAN3 needed to confirm is that RAN2 has the similar understanding.

After further check with RAN2, our understanding is that RAN3 can confirm this understanding, and turn the WA into agreement with some rewording.

After confirming the above WA, the only spec impact is the IE description of Remote UE Local ID in TS 38.473 as follows,

9.3.1.267	Remote UE Local ID

This IE uniquely identifies a L2 U2N Remote UE within the connected Relay UE, or within the Last Relay UE in case of Multi-hop relay communication.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Remote UE Local ID | M |  | INTEGER (0..255, ...) | Corresponds to the sl-LocalIdentity contained in the SL-SRAP-Config IE defined in TS 38.331 [8]. |

Proposal 1: RAN3’s common understanding is that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

The next open issue is the Editor’s note captured in 38.473 BL CR,

Editor note’s: FFS on whether peer UE ID is needed during UE context setup procedure.

In our understanding, this open issue can be discussed case by case,

- For the UE context setup for the Last relay UE, since the SUI can only be sent after UE context setup procedure. In this case, the peer UE ID is not needed.

- For the UE context setup for the Intermediate relay UE, the intermediate relay UE needs to establish PC5 RLC Channel during connection setup/resume, so the F1AP also needs to exchange PC5 RLC channel to be setup list related information. In such case, the Peer UE ID indicating the intermediate relay UE’s parent relay UE is needed to be included.

As a summary, for the UE context setup for the intermediate relay UE, the Peer UE ID indicating its parent relay UE is needed. So the semantic description in current 38.473 BLCR is needed to be updated as follows,

| >>Peer UE ID | O |  | BIT STRING (SIZE(24)) | Corresponds to information provided in the sl-DestinationIdentityL2-U2U contained in the SL-TxResourceReqL2-U2U IE or corresponds to information provided in sl-SourceIdentityRelayUE contained in the SL-TxResourceReqDisc IE, defined in TS 38.331 [8]. This IE is included if the gNB-CU UE F1AP ID and/or gNB-DU UE F1AP ID are associated with a L2 U2U Remote UE or L2 U2U Relay UE or L2 U2N Relay UE in case of Multi-hop relay communication.. | YES | reject |
| --- | --- | --- | --- | --- | --- | --- |

Note that the above update is only subject to the Peer UE ID contained in UE CONTEXT SETUP REQUEST message. We can keep the current semantic description of the Peer UE ID contained in UE CONTEXT MODIFICATION REQUEST message as it is.

Proposal 2: For F1AP UE context setup for the intermediate relay UE, the Peer UE ID indicating its parent relay UE is needed.

The next open issue is,

FFS whether the same PC5 RLC channel ID can be allocated toward the parent UE and child UE.

In order to have a reasonable conclusion, we can firstly check whether the same PC5 RLC channel ID can be allocated toward two Remote UEs for the Relay UE in U2U relay. In our understanding, the F1AP allows the operation that the same PC5 RLC channel ID can be allocated toward two Remote UEs for the Relay UE in U2U relay, and the PC5 RLC channels are distinguished by the Remote UE Local ID together with the PC5 RLC Channel ID.

Observation 1: PC5 RLC channels are distinguished by the Remote UE Local ID together with the PC5 RLC Channel ID in U2U relay scenario, which indicates that the same PC5 RLC channel ID can be allocated toward two Remote UEs for the Relay UE in U2U relay scenario.

As a result, we do not see any issue that the multi-hop relay reuse the similar design logic.

Proposal 3: The same PC5 RLC channel ID can be allocated toward the parent UE and the child UE, i.e. the PC5 RLC channels are distinguished by the Peer UE ID together with the PC5 RLC Channel ID in multi-hop relay scenario.

Another thing to notice is that for U2U relay case, the Peer UE ID IE is only included in PC5 RLC Channel to be Setup List, but not included in PC5 RLC Channel to be Modified/Released List.

Firstly, recall that the motivation to include the Peer UE ID IE only in PC5 RLC Channel to be Setup List in U2U relay scenario is to ensure the correct RRC configuration generated by DU, for which the motivation is different from the purpose of including the Peer UE ID IE for multi-hop relay scenario.

In addition, the reason to not including the Peer UE ID in Modified/Released list is that for U2U Relay case the PC5 RLC channel can be uniquely identified by PC5 RLC Channel ID and Remote UE Local ID contained in the Modified/Released List.

However, for multi-hop relay case, especially for the Second Relay UE, it is possible that the established PC5 RLC channels are not established for only one Remote UE. So it is not appropriate to include Remote UE local ID in Modified/Released list in all cases.

Observation 2: For e.g. the Second Relay UE, it is possible that the established PC5 RLC channels are established for multiple Remote UEs. So it is not appropriate to include Remote UE local ID in PC5 to be Modified/Released List in F1AP UE CONTEXT MODIFICATION REQUEST message for the Second Relay UE context.

If we do not use Remote UE Local ID IE in Modified/Released List, in order to uniquely identify a PC5 RLC Channel by gNB-DU, the Peer UE ID IE is also needed to be included in Modified/Released List.

By following similar reason, the Peer UE ID IE is also needed to be included in RC5 RLC Channel related lists in F1AP UE CONTEXT MODIFICATION RESPONSE/REQUIRED/CONFIRM message.

According to the discussion last meeting, some company raised the point that if we follow the design logic in Proposal 3, the current IE definition for PC5 RLC Channel ID should also be updated as follows,

9.3.1.265	PC5 RLC Channel ID

This IE uniquely identifies a PC5 Relay RLC channel for a L2 U2N Remote UE, or a L2 U2N Relay UE, or a L2 U2U Remote UE, or L2 U2U Relay UE, or a L2 Intermediate U2N Relay UE on each hop.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PC5 RLC Channel ID | M |  | INTEGER (1.. 512, ...) |  |

Proposal 4: Add Peer UE ID IE in PC5 RLC Channel to be Modified/Released List in F1AP UE CONTEXT MODIFICATION REQUEST message.

Proposal 5: Add Peer UE ID IE in PC5 RLC channel related lists in F1AP UE CONTEXT MODIFICATION RESPONSE/REQUIRED/CONFIRM message.

The next open issue is,

FFS whether upon the reception of the RRCSetupRequest message, the gNB-DU needs to know that which Relay UE is the First Relay UE of the U2N Remote UE to configure lower layer configuration of Remote UE’s SRB1.

On one hand, it is true that the gNB-DU does not know the First relay UE at the time of configuring Sidelink Relay Configuration contained in F1AP INITIAL UL RRC MESSAGE TRANSFER message. On the other hand, although the gNB-DU does not know the First relay UE at that time, the gNB-DU is still able to provide a reasonable lower layer configuration of Remote UE’s SRB1 by implementation. In our understanding, the information on first relay UE might be known by gNB-DU unless some information contained in the lower layer configuration of Remote UE’s SRB1 is highly dependent on the First relay UE and has to be configured to be totally different for different First relay UEs; otherwise, we do not see strong motivation to make enhancement toward this open issue.

Proposal 6: No need for gNB-DU to know which Relay UE is the First Relay UE of the Remote UE to configure lower layer configuration of Remote UE’s SRB1.

The last open issue is,

FFS whether to enhance the F1 signaling to support the multiplexing of PC5 Relay RLC channel is needed can be further discussed in next meeting.

It should be noted that in legacy single hop scenario, the Uu RLC channel is also used for multiplexing the data transferred over multiple PC5 RLC Channels for different Remote UEs, but the gNB-DU is never additionally informed by the gNB-CU that one specific Uu RLC channel is setup for which Remote UEs. We think the similar logic can also be applied to the multi-hop scenario, i.e. the gNB-DU does not need to know that a specific parent hop PC5 RLC channel is setup for which Remote UEs.

Proposal 7: No need to introduce information related to the multiplexing of PC5 RLC channel over F1AP.

Conclusion

In this paper, we provide discussion on multi-hop relay. The following proposals are provided,

Proposal 1: RAN3’s common understanding is that the remote UE local ID uniquely identifies a multi-hop remote UE within the last relay UE.

Proposal 2: For F1AP UE context setup for the intermediate relay UE, the Peer UE ID indicating its parent relay UE is needed.

Observation 1: PC5 RLC channels are distinguished by the Remote UE Local ID together with the PC5 RLC Channel ID in U2U relay scenario, which indicates that the same PC5 RLC channel ID can be allocated toward two Remote UEs for the Relay UE in U2U relay scenario.

Proposal 3: The same PC5 RLC channel ID can be allocated toward the parent UE and the child UE, i.e. the PC5 RLC channels are distinguished by the Peer UE ID together with the PC5 RLC Channel ID in multi-hop relay scenario.

Observation 2: For e.g. the Second Relay UE, it is possible that the established PC5 RLC channels are established for multiple Remote UEs. So it is not appropriate to include Remote UE local ID in PC5 to be Modified/Released List in F1AP UE CONTEXT MODIFICATION REQUEST message for the Second Relay UE context.

Proposal 4: Add Peer UE ID IE in PC5 RLC Channel to be Modified/Released List in F1AP UE CONTEXT MODIFICATION REQUEST message.

Proposal 5: Add Peer UE ID IE in PC5 RLC channel related lists in F1AP UE CONTEXT MODIFICATION RESPONSE/REQUIRED/CONFIRM message.

Proposal 6: No need for gNB-DU to know which Relay UE is the First Relay UE of the Remote UE to configure lower layer configuration of Remote UE’s SRB1.

Proposal 7: No need to introduce information related to the multiplexing of PC5 RLC channel over F1AP.

Reference